��������������������������������������������������������������������������

Function:  FloppyTst

Author:    Dave Peckham

Purpose:   FloppyTst determines the status of the specified floppy

disk drive.

FloppyTst has been tested with the Summer '87 Clipper.

History:   Dec  9, 1986   FloppyTst 1.0 written in Clipper

by Charlie Butler with calls to the

ROMBIOS, Allocate and DeAlloc routines

from Tom Rettig's Library.

Feb 12, 1988   Converted FloppyTst to external assembler

function written in MASM 3.0.  Function now

accepts integer instead of character disk

specifiers.

Converted Int 13h BIOS calls to

Int 25h (MS-DOS Absolute Disk Read) and

Int 26h (MS-DOS Absolute Disk Write) calls

for greater MS-DOS compatibility

Feb 17, 1988   Added support for "Sector not found"

and "Unknown Media" errors

Feb 18, 1988   Function now accepts character parameters.

For example, call with "A", "a", "B" or "b".

Date:      February 17, 1988

Version:   2.0 for Clipper '87

Language:  Microsoft Macro Assembler (MASM) version 3.0

Syntax:    FloppyTst (<expC>)

where <expC> is "A", "a", "B" or "b"

Returns:   A <expN> that will be one of the following:

0 - Drive loaded and ready to read or write

1 - Drive door open or diskette inserted upside-down

2 - Diskette is unformatted

3 - Diskette has a write-protect tab installed

4 - Diskette is of incorrect media type for this floppy drive

8 - Diskette has been damaged (Sector not found) or

is not formatted

15 - Invalid drive specifier passed in <expN>

n - Any return code other than those listed here indicates

that an error did occur, though the source of the error

can not be exactly determined

Notes:     The <expC> passed as a parameter must be one of the following

character values: "A", "a", "B" or "b".  Thus, calls of the

form FloppyTst ("A") and FloppyTst ("B") are legal.  If <expC>

is not one of these values, the value 15 will be returned as

the function value.  The function may return a misleading

value if drive B: is tested on a machine with only an A: drive.

The result will usually correspond to the status of the A: drive.

The DOS Critical Interrupt handler may, under certain

circumstances, report a confusing or ambiguous error.

For example, attempting to read or write to a non-DOS

diskette or a 1.2Mb diskette in a 360k drive cause

FloppyTst to return eight instead of four.  If you

understand this, you can provide better error messages

for users.

For those who have replaced the MS-DOS Critical Error Handler

with their own, you must be sure that your replacement handler

returns the standard MS-DOS error codes, or that you

modify the error routines in this assembler module to

conform to your return codes.

Algorithm:

1. Verify that valid drive specifier has been passed.

2. Read sector zero into buffer using MS-DOS interrupt 25h,

Absolute Read.

3. Write buffer to sector zero using MS-DOS interrupt 26h,

Absolute Write.

4. The return value is based upon error codes returned from

the MS-DOS Critical Error handler.

Assembling and Linking:

masm floptst,floptst;

plink86  fi your_prog,floptst  li extend,clipper

where "your_prog" is your Clipper source file

��������������������������������������������������������������������������