I B M C O M

IBM RS232 interrupted driven communications routines. BBS version

has small model lib file for ibmcom.

Since the BBS version supplies only object,the following assumptions

have been made:

o Buffer input size is defined to be 2000 characters per

port.

o Buffer output size for putsi_com() routine is defined to be

80 characters.

Purchasers of the source can change the above buffer size's.  If you

whish to obtain source and/or to see these routines continue to be

supported with ehancements,bug fixes, protocals, terminal

emulators,etc.

Send $15 to:

Intrinsic Computers

RR 4

Oak Ridge,NJ   07438

Send $25 and we'll give you the latest

version on a diskette plus code for a

reentrant xmodem recieve protocal. We

also are available for consulting.

Description of Routines:

int init_handl( int port_num)

Initializes port_num interrupt routines.

Must be called after init_port(),but before

using any other routines. Returns FALSE if

port_num is not 1 or 2,otherwise TRUE.

void init_buffer_out(int port_num)

Sets up the circular buffers for interrupt

output (specifically putsi_com()).  Should

be called only once.

int init_port(int port_num,int baudrate,char parity,

int data_bits,int stopbit)

Must be called before init_handl(). It sets

the port_num's baud rate,parity,etc..

Variables used:

port_num   1 or 2

baudrate   3 or 300   for 300 baud

6 or 600   for 600 baud

12 or 1200  for 1200 baud

24 or 2400  for 2400 baud

48 or 4800  for 4800 baud

96 or 9600  for 9600 baud

19 or 19200 for 19200 baud

38 or 38400 for 38400 baud

56          for 56000 baud

112          for 112000 baud

parity    'E' for even

'N' for none

'O' for odd

data_bits  5,6,7, or 8

stopbit    1 or 2

Returns

TRUE if params ok.

FALSE if illegal params.

int check_port_hw(int port_num)

Used to check if a port error has occured or

if no error it returns the number of char

in the ports interrupt buffer. If error has

occurred it returns the NEGATIVE of the port's

line control register.  In programmers terms

if the return value is >= 0 you got the # chars

in port_num's buffer, otherwise if its < 0, you

got an error on the port.  The error is cleared

by calling this routine.

int check_port_buf(int port_num)

Returns port_num's buffer overflow status.

1 for yes its overflowed

0 for no overflow

unsiged com_status(int port_num)

Returns port_num's port status.

void disable_txmit_int(int port_num)

Turns off interrupt driven output. You DON'T

call this unless you want to stop midstream

the output of a string. Normally the

interrupt handler inself will turn off Tx

interrupts when it finishes the string.

int disable_port_int(int port_num)

Turns off interrutps for port_num. Returns FALSE if

illegal port_num, otherwise TRUE.

int reset_port(int port_num)

Resets port_num so that it will not interrupt

the IBM anymore. Used to halt communications

Should be called before exiting your programs

or interrupts will still be active and

hang the machine (or worse). Returns FALSE if illegal

port_num, otherwise TRUE.

void putc_com(int port_num,char *string)

Outputs the string to port_num

Does not buffer the character. It uses a

polling technique.

int puts_com(int port_num,char *string,int num_char)

Outputs the indicated num_char from string

to port_num using a polling technique. Returns

TRUE.

int putsi_com(int port_num,char *string,int num_char)

Same as puts_com() except it outputs the

string using an interrupt driven technique.

You must call init_buffer_out() once before

ever using putsi_com. Returns TRUE if able to

handle the string passed to it, otherwise FALSE.

See the demo program for further explanation.

unsigned char getc_com(int port_num)

Gets a char from port_num's buffer.If the

buffer is empty it returns 0xFF which could

be a valid character. This routine should be

used in conjunction with check_port_buf() to

see if any characters are in the buffer

before calling this routine. Returns char gotten.

int gets_com(char *buffer,int num_bytes,int port_num)

Gets num_bytes from port_num and puts it into buffer.

Returns actual number of bytes transferred into buffer.

void report_serial_status(unsigned error_code)

Will print out a nice report of the port status if you pass

it the port status (obtained from com_status(). Note, not

all status bits are errors, some are normal.

Linking Instructions:

Must be linked with a main().  Supplied object in

BBS version is small model only.