Table of Contents

--------------------------------------------------------------------

1        Hardware Description

1.0   General Description of the 8259 hardware

1.1   8259 registers

1.1.1 The IRR

1.1.2 The IMR

1.1.3 The ISR

1.2   Cascading of 8259's

1.3   Special Fully Nested Mode

1.4   Triggering of 8259 interrupt request.

1.5   Default IR7.

1.6   Fully Nested Mode.

1.8   Setting priorities

1.9   Buffered mode

2     Programming the 8259

2.1   Initialization of the 8259.

2.2   fully nested mode

2.3   special mask mode.

2.4   Interrupt vector.

2.5   Special Fully Nested Mode.

2.6   Automatic rotation.

2.7   Poll Mode.

--------------------------------------------------------------------

1        Hardware Description

1.0  General Description of the 8259 hardware

The purpose of the 8259 interrupt controller is to:

provide an external asynchronous input which informs the CPU to:

- complete whatever instruction it is currently executing

- save the CS, IP, and flags registers

- clear the TRAP and interrupt enable flags (TF, IF)

- begin executing at the address indicated by the interrupt

vector supplied by the 8259

Clearing IF in the CPU prevents any other interrupts from occurring

until the software executes an IRET or specifically enables

interrupts to the CPU by executing STI.

The 8259 accepts requests from 8 external lines, queues the requests in

priority order, and issues an interrupt to the CPU if the CPU's IF is

set and if the priority of the highest priority interrupt is higher

than that of the interrupt being serviced.  In Special Masked Mode

the priorities within the 8259 are disregarded and any unmasked

interrupt can interrupt an interrupt being serviced.

1.1   8259 registers

Three registers within the 8259 handle the interrupt requests:

- Interrupt Request Register (IRR)

- Interrupt Mask Register (IMR)

- In-Service Register (ISR).

1.1.1 The IRR

The IRR receives and stores the requests until they are accepted by

the CPU.  Upon acceptance by the CPU, the bit in the IRR is cleared

and the bit in the ISR is set, indicating that the interrupt is

being serviced.

The value of the IRR can be examined by invoking

Interrupts_Requesting_Service.

This register is cleared when Initialize_Master_8259 or

Initialize_Slave_8259 is invoked.

1.1.2 The IMR

The IMR prevents further interrupts from being accepted by the IRR.

The value of the IMR can be examined by invoking the VALUE_OF_IMR.

Each of the 8 interrupts can be enabled or disabled.

1.1.3 The ISR

The ISR indicates which interrupts are currently being processed by

the CPU.  As long as an ISR bit is set, the 8259 will not attempt to

request an interrupt for that same level.

The value of the ISR can be examined by invoking the VALUE_OF_ISR.

The ISR bits can be cleared (allowing that specific interrupt to be

serviced again) in different ways:

- Automatic EOI: (AEOI) the ISR bit which caused the interrupt is

cleared by the 8259 itself once the CPU begins to process that

interrupt.  If this is not used, code in the interrupt handler

will have to issue an EOI.

- Non specific EOI: command is issued by the CPU to clear the bit

of the highest priority interrupt being serviced.  This is

useful when several handlers pass control to a common routine

to complete the processing.  The common routine can issue

an EOI without knowing which level it should be directed at.

(However it must know which chip to issue it to).

@SEE */<2.2

- Specific EOI: command is issued by the CPU to clear the bit

representing a specific interrupt.

1.2  Cascading of 8259's

Up to 8 8259's can be cascaded to one "master" 8259 for a maximum of

64 interrupt levels.

(Each 8259 can generate 8 different INTERRUPT_LEVELS)

When a slave is cascaded to a master, one of the master's interrupt

levels is used for handling the slave.  When the slave's interrupt

request is acknowledged by the master, the master signals the

slave to pass the corresponding interrupt vector to the CPU.  This is

done by the hardware and does not require programming.

1.3   Special Fully Nested Mode

Only applies to cascaded systems and only to the master.

This mode allows higher priority interrupts from a slave to

interrupt an in-process interrupt even though it is from the

same slave.

(All the interrupts from one slave use the same IR on the Master

and if this mode (Special Fully Nested Mode) were not in effect

the higher priority interrupt would be locked out of the master).

@SEE */<2.5

1.4   Triggering of 8259 interrupt request.

The interrupt request to the 8259 can be edge (low to high) or

level triggered.

Level triggered mode is complicated by the requirement to remove

the interrupt request to the 8259 before the ISR bit is cleared

to avoid another interrupt for the same signal.

1.5   Default IR7.

If the interrupt request line to the 8259 goes low (interrupt

request disappeared) before the CPU has responded (INTA),

the 8259 will cause an IR7 to occur.

(This situation could occur as the result of noise).

The 8259 would not set an ISR bit in this case because it was not

a true interrupt.  The interrupt handler for level 7 must thus

examine the ISR to verify that its interrupt request is valid.

1.6   Fully Nested Mode.

Default mode after intialization.  Only higher priority interrupts

will be serviced by the CPU during servicing of an interrupt.

(Only if the CPU Interrupt Flag has been set - by STI or IRET).

Setting a mask bit in the IMR inhibits a request for the

corresponding level.

1.8   Setting priorities

Interrupt priorities can be set by software.  The priorities

determine the order in which the interrupts are handled within

the 8259 as well as whether an interrupt to the CPU will occur

when in nested mode (i.e., Fully Nested or Special Fully Nested).

Setting priorities consists of specifying which level is to be

the lowest;  this can be done specifically or automatically

(an interrupt level becomes the lowest priority after being

serviced).  When an interrupt level is assigned the lowest

priority, the level which numerically follows it becomes the highest

level.  (Ex., if 4 is assigned the lowest, then the list of levels

in descending order of priority would be 5,6,7,0,1,2,3,4).

1.9   Buffered mode -  Hardware data bus buffers may be installed for the 8259.

This does not affect programming other than requiring that the chip be

notified that it is buffered when initializing it by software.

2     Programming the 8259

2.1   Initialization of the 8259.

Each 8259 has to be initialized before other programming of chips can be

done.  The initialization procedure specifies:

- One chip or cascaded with others          @SEE */<1.2

- Use of automatic EOI, or handled by CPU.  @SEE */<1.1.3

- Buffered mode                             @SEE */<1.9

- Slave or Master                           @SEE */<1.2

- if slave then its slave ID (0..7) (which corresponds to the

interrupt level in the master chip which handles the slave).

- If master then the list of interrupt levels which are slaves

- Interrupt number (0..31)    @SEE */<2.4

Initializing results in:

-    clearing the IMR

-    setting priority of IR0 to 0 (highest priority)

-    setting 8259 to Fully Nested Mode.     @SEE */<1.6

The chip is initialized when the PIC_8259 class is instantiated

and when INITIALIZE_8259 is invoked

2.2  fully nested mode. default after initialization.  When an interrupt

is being serviced (ISR bit is set), interrupts of the same level

and lower are disabled; those of higher are not.

Note.

If an interrupt is being serviced (ISR bit is set) and a higher

priority interrupt handler is also being serviced and masks the

lower priority interrupt, then when the lower priority interrupt

handler attempts to issue a non-specific EOI, it will not clear the

ISR bit corresponding to its level because of the unusual situation

where its IMR has been masked and yet it is being serviced.

After the lower level interrupt is no longer being serviced,

reading the ISR bit will indicate that it is still being serviced.

Be careful when masking interrupts from within an interrupt handler

if non specific EOIs are used.

If it must be done, make sure the interrupt to be masked is not

being serviced.  Note that in fully nested mode there is no need

to mask lower priority interrupts.

2.3   special mask mode.

Priorities have no significance in this mode.  Masks are effective

in disabling interrupts (as they are in nested mode also).  Non

specific EOI should not be used in this mode.

2.4   Interrupt vector.

These numbers map to the interrupt vector space, 0 .. 1K.

(256 vectors * 4 bytes/vector).

Each 8259 is supplied with an interrupt vector (0..248) which

indicates a set of 8 interrupt vectors.  Note that only numbers which

are multiples of 8 may be specified, so rather than allow for an

erroneous value, the initialization function of PIC_8259 only accepts

the values 0..31 which represent the 32 possible blocks of 8 vectors

each.

When the 8259 passes an interrupt number to the CPU, the resulting

vector will be this number plus the interrupt level (0..7).

(INTEL specifies that 0..31 are reserved for future use and should not

be used for "user" vectors).  (MS DOS does not respect this).

2.5  Special Fully Nested Mode.

If Special Fully Nested Mode is used, then a non specific EOI should

be issued to the slave, its ISR should then read to determine whether

there are other interrupts for that chip which are being serviced.

If not, then a non specific EOI may be issued to the master to clear

the ISR for the level corresponding to the slave.  This checking and

issuing of EOI to the master is already done automatically by

Slave_8259::Issue_Specific_EOI_Command or

Slave_8259::Issue_Non_Specific_EOI_Command.

(If Special Fully Nested Mode is specified).

There is no need to add code to clear the master if the PIC_8259 class

is used.

2.6  Automatic rotation.

Using automatic rotation results in the interrupt level which was just

serviced to be assigned the lowest priority.

This prevents one interrupt level from locking out the lower level

ones if interrupts are occurring as fast as they are being handled.

When issuing an EOI, automatic rotation can also be specified.  If

AEOI is in effect (EOIs are not issued by CPU) then setting the priority

can be done specifically or can be specified to be handled automatically

by the 8259 every time it clears an ISR bit.

2.7 Poll Mode.

If the CPU is not permitted to respond to the 8259 INT request

(by physically not connecting the INTA line, or disabling interrupts

in the 8259), the Poll command can be used to determine if there is

an interrupt request pending.  The 8259 will inform the CPU if there is

an interrupt pending (IRR /= 0) and identify the level of the one having

the highest priority.  For the 8259 this is equivalent to the CPU

acknowledging an interrupt (The IRR bit is cleared and the ISR bit is set).

If AEOI is not in effect an EOI must be issued.