RS232.C v1.6 User Documentation

copyright C. Karcher 1992,93,94,95

>   About RS232.C:

RS232.C was written to provide all of the basic functionality needed

to employ serial I/O in any application written with Borland 'C'

language compilers.  Some features are:

1. Ease of use.  No assembly language or library files are used and a

simple "#include" statement is all that is required to access all of

the functions provided.

2. Both input and output are buffered and interrupt driven for

efficiency.

3. Serial ports 1 - 4 are supported on PC, AT and PS/2 compatibles.

Chained interrupts used on port 3 and 4 are allowed for so as not to

interfere with devices such as a mouse or printer.  Transmission

speeds of 110 to 115200 baud are available.

4. Detection and utilization of hardware buffered UARTs (NS16550AF

etc.) found in some machines is automatic.

5. Interrupt driven hardware and XON/XOFF flow control is provided for.

6. All source code is included.  RS232.C can be used with all memory

models.

This is user supported software and may be distributed freely in a

whole and unmodified state.  It may be used in part or wholly, free of

royalty, to develop any commercial applications provided that the

developer has registered RS232.C.  Registration entitles the developer

to any future enhancements or upgrades which may be released.

Registration also includes technical support as far as can be provided

via mail, telephone or electronic messaging.  To register RS232.C,

send a check or money order for $20.00 to (please indicate the

version level or date of the version you have - if an updated

version is available, it will be mailed to you):

Chris A. Karcher

9537 Evanston Ave. N.

Seattle WA 98103-3131

Compuserve members can register RS232.C through Compuserve by

typing GO SWREG and selecting Register Shareware from the menu.

The registration ID for RS232.C is 4460.

The author may be contacted by mail at the above address, by telephone

at (206)789-7945 or via Compuserve EMAIL to user ID 76406,536, or

by mail on Internet at 76406.536@compuserve.com.

The author of RS232.C makes no expressed or implied warranty of any

kind with regard to the software or accompanying documentation.  In no

event shall the author be held liable for incidental or consequential

damages in connection or arising out of performance or use of any part

of this software.

The distribution should include the following files:

RS232.C   - serial communication routines source code

RS232.DOC - documentation (this file)

RS_DEMO.C - sample program demonstrating RS232.C

>   Why use RS232.C?

Borland's C compilers (and probably all DOS based C compilers) do

provide some basic support for serial communications but that support

makes use of calls to BIOS routines only.  The BIOS serial routines

are adequate only for the slowest of transmission speeds and do not

exploit all of the capabilities of the hardware found in PC's.  If

efficient serial communication is required, the programmer must

provide all of the low level software.  RS232.C does just that by

taking care of all the low level details and providing the programmer

with all the high level functions needed for efficient serial I/O.

>   How to use RS232.C:

All that is required to use RS232.C is to include it in your source

code.  The simplest way to do that is with a compiler '#include'

directive, treating RS232.C just as you would a header file.  Having

done that, three basic steps are all that are required to perform

serial I/0:

1. Allocate memory for input and output buffers.  (It can be

pre-allocated at compile time or allocated via some memory

allocation function like malloc at runtime.)

2. "Open" the port with the function rs_initport, using the

desired communication parameters.

perform I/O with functions provided

.     .    .      .        .

.     .    .      .        .

.     .    .      .        .

3. "Close" the port when finished performing I/O with the

function rs_close before ending the program.

The file RS_DEMO.C, included with the distribution, is a sample

terminal program demonstrating most of the functions available with

RS232.C.

The following code demonstrates an extremely simple but functional

program which turns a PC into a dumb terminal capable of communicating

with a modem or another computer:

/*

TERM: a light weight dumb terminal program to demonstrate rs232.c

C. Karcher

*/

#include<conio.h>

#include"src\rs232.c"       /* include rs232 variables and functions */

main()

{

int key = 0;

char input_buffer[1024],output_buffer[1024]; /* allocate buffers */

/* open port 1 with 2400 baud, no parity, 8 data bits, 1 stop bit,

a 1024 byte input buffer and a 1024 byte output buffer */

if(rs_initport(RS_PORT1,RS_B2400,RS_NOPAR,RS_DBIT8,RS_SBIT1,

1024U,input_buffer,1024U,output_buffer) > 0)

cprintf("Terminal mode active - press escape to exit\r\n");

else{

cprintf("Unable to open COM port\r\n");

return 0;

}

/* turn on Data Terminal Ready */

rs_modctrl(RS_WRTMCR,RS_MCRDTR,RS_LINON);

/* display any characters received and send any keystrokes

typed until escape is pressed */

do{

if(rs_inrcvd())               /* if a character was received */

putch(rs_getbyt());         /*    display it */

if(rs_keyhit())               /* if a key has been pressed */

rs_sndbyt((key = getch())); /*    send it */

}while(key != 27);            /* if the key pressed was ESC,

end */

/* turn off Data Terminal Ready */

rs_modctrl(RS_WRTMCR,RS_MCRDTR,RS_LINOFF);

/* close the port and exit */

rs_close();

return 0;

} /* end TERM */

>   RS232.C Functions:

The following is a list of the functions contained in RS232.C and a

brief explanation of the purpose of each one.  A detailed description

of each function and it's use follows:

rs_initport - Prepares a COM port for use.

rs_sndbyt   - Sends a single byte.

rs_sndstr   - Sends a string of bytes.

rs_getbyt   - Gets a single byte.

rs_getstr   - Gets a string of bytes.

rs_inrcvd   - Returns number of bytes which been received.

rs_peek     - Non-destructive read of next character in input.

rs_outfre   - Returns amount of free space remaining in output buffer.

rs_scanin   - Scans buffer for a string and returns offset if found.

rs_error    - Returns code for last error detected.

rs_modctrl  - Controls or returns status of modem control lines.

rs_break    - Sends break to remote equipment.

rs_clrout   - Clears output buffer.

rs_clrin    - Clears input buffer.

rs_keyhit   - Determines if a key has been pressed.

rs_timer    - Times events or operations.

rs_setflow  - Establishes or returns status of flow control.

rs_close    - "Closes" port prepared by rs_initport.

>   Function Reference: (Note: The pre defined constants shown in

parentheses with some of the function argument

descriptions or return value descriptions are

defined in RS232.C and may be used for

convenience.)

>   rs_initport:

>   Purpose:

Prepares a COM port for use.

>   Prototype:

int rs_initport(char port, long baud, char parity, char data_bits,

char stop_bits, unsigned in_buf_size, char *in_buf,

unsigned out_buf_size, char *out_buf);

>   Arguments:

port: Character indicating which port to use.  May be one of

the following:

'1' (RS_PORT1) - COM port 1

'2' (RS_PORT2)

'3' (RS_PORT3)

'4' (RS_PORT4) - COM port 4

'0' (RS_USERPORT) - user defined port -- see notes

baud: The long value indicating the baud rate.  Can be any of

the following:

110L (RS_B110) - 110 baud

300L (RS_B300)

600L (RS_B600)

1200L (RS_B1200)

2400L (RS_B2400)

4800L (RS_B4800)

9600L (RS_B9600)

19200L (RS_B19K)

38400L (RS_B38K)

57600L (RS_B57K)

115200L (RS_B115K) - 115200 baud

parity: A character indicating the type of parity checking to use

as follows:

'N' (RS_NOPAR) - no parity

'E' (RS_EVPAR) - even parity

'O' (RS_ODPAR) - odd parity

'S' (RS_SPPAR) - parity bit always space

'M' (RS_MKPAR) - parity bit always mark

data_bits: A character indicating how many data bits to use in each

byte:

'8' (RS_DBIT8) - 8 data bits

'7' (RS_DBIT7) - 7 data bits

stop_bits: A character indicating the number of stop bits to use:

'1' (RS_SBIT1) - 1 stop bit

'2' (RS_SBIT2) - 2 stop bits

in_buf_siz: An unsigned integer between 2 and 32768 indicating the

size in bytes of the input buffer.  The value must be a

power of 2 (i.e. 2,4,8,16,256,512,1024 etc.).

in_buf: Character pointer to the input buffer allocated by the

application.

out_buf_siz: The unsigned value between 2 and 32768 indicating the

size in bytes of the output buffer.  The value must be a

power of 2 (i.e. 2,4,8,16,256,512,1024 etc.).

out_buf: Character pointer to the output buffer.

Note: If RS_POLLED_XMIT is defined, the output buffer is not used

for serial output.  In this case the output buffer size may be 0

and the output buffer pointer may be NULL.

>    Return value:

rs_initport can return the following integer values indicating

the success or failure of the operation:

4 (RS_UART4) Success, UART detected was a 16550 capable of

hardware buffering.  This capability is automatically

utilized.

3 (RS_UART3) Success, UART detected was a 16550 incapable of

hardware buffering.

2 (RS_UART2) Success, UART detected was an 8250A or 16450.

1 (RS_UART1) Success, UART detected was an 8250B

0 (RS_NOUART) Failure, no UART detected.

-1 (RS_BADIBUF) Failure, input buffer size invalid or input

buffer pointer is a NULL pointer.

-2 (RS_BADOBUF) Failure, output buffer size invalid or output

buffer pointer is a NULL pointer.

-3 (RS_BADPORT) Failure, invalid port argument.

-4 (RS_BADPAR) Failure, invalid parity argument.

-5 (RS_BADDBIT) Failure, invalid data_bits argument.

-6 (RS_BADSBIT) Failure, invalid stop_bits argument.

-7 (RS_BADBAUD) Failure, invalid baud argument.

>    Examples:

if(rs_initport(RS_PORT1,RS_B2400,RS_NOPAR,RS_DBIT8,RS_SBIT1,

1024U,input_buffer,1024U,output_buffer) > 0)

cprintf("Terminal mode active - press escape to exit\r\n");

port_ok = rs_initport(port,RS_B2400,RS_NOPAR,RS_DBIT8,RS_SBIT1,

BUF_SIZ,in_buf,BUF_SIZ,out_buf);

>    Notes:

If a port has already been "opened" with rs_initport, it will

automatically be closed before another port (or the same port) is

opened.  Only one port may be opened at any one time although the

interrupt service routine installed by rs_initport can coexist

with others sharing the same IRQ (interrupt request).  An example

of this would be serial mouse installed on COM port 1 and an

application using rs_initport to prepare access to COM port 3.

In this case, both the mouse driver and the application would be

using IRQ4.  Some serial applications "take over" the interrupt

completely which would disable use of the mouse.  The interrupt

service routine installed by rs_initport "chains" the interrupt

so that if it is invoked by other than the targeted device,

control is passed to the appropriate routine.

To determine the appropriate I/O address for a given COM port's

UART, rs_initport first uses the BIOS data area and, failing

this, uses the "standard" I/O addresses.  Standard IRQ's are also

assumed.  If a serial adapter has been installed with other than

the standard I/O addresses and IRQ's, rs_initport will not find

the UART and will fail.  The standard addresses and IRQ's assumed

are:

port          I/O address            IRQ

COM1          3F8                    4

COM2          2F8                    3

COM3          3E8 (PS/2 = 3220)      4 (PS/2 = 3)

COM4          2E8 (PS/2 = 3228)      3 (PS/2 = 4)

If the port argument is '0' (the character, not the integer),

rs_initport makes no attempt to determine the base I/O

address of the UART nor the IRQ.  Instead, the values defined

by the struct rs_user are used.  If rs_user was never

initialized, rs_initport returns -3.  The structure has the

following form, and it's members may be accessed globally by

the application.  For more information, see the section at

the end of this document file regarding use of more than one

port simultaneously.

struct rs_userport{

int base;

char irq;

}rs_user;

If a UART capable of FIFO buffering is detected, this feature is

automatically enabled.  This can greatly reduce the amount of

overhead due to interrupts and can prevent "lost characters" due

to receive overruns at higher transmission speeds.

rs_initport does not alter the state of any of the modem control

output lines (DTR or RTS).  They must be explicitly controlled

using rs_modctrl.

>   rs_sndbyt

>   Purpose:

Sends a single byte.

>   Prototype:

int rs_sndbyt(int rs_snd);

>   Arguments:

The byte to send (int converted to char).

>   Return Value:

Returns 0 if the byte was successfully written to the output

buffer.  Returns -1 no port open or byte could not be written.

>   Examples:

if(rs_sndbyt('A'))

printf("Unable to send message\n");

rs_sndbyt(getche());

>   Notes:

rs_sndbyt writes the character into the output buffer and

returns immediately without waiting for the character to

actually be transmitted.  If there is no room in the output

buffer, rs_sndbyt will wait until there is room unless output is

currently disabled via flow control.  In this case, rs_sndbyt

returns -1 to indicate failure.

If transmit interrupts are disabled by defining RS_POLLED_XMIT,

the buffer is not used and the character is sent when the UART

is free to send it.  If output is disabled via flow control,

the character is not sent and rs_sndbyt returns -1.

>   rs_sndstr

>   Purpose:

Sends a string of bytes.

>   Prototype:

int rs_sndstr(int rs_sndcnt, char *rs_sndstr);

>   Arguments:

rs_sndcnt indicates the number of bytes to be sent - if a number

other than 0 is specified, the string need not be nul

terminated.  If rs_sndcnt is 0, rs_sndstr is treated as a nul

terminated string and characters will be sent up to but not

including the terminating nul.  rs_sndstr is a character pointer

to the string to be sent.  rs_sndstr can be larger than the size

of the output buffer and up to 32767 bytes.

>   Return Value:

Function returns 0 on success.  If no port is open, it returns

-1.  If the entire string could not be written to the output

buffer, the number of characters written is returned.

>   Examples:

rs_sndstr(0, "Hello, world.");

if(rs_sndstr(strlen(msg),msg)

printf("Message could not be sent\n");

>   Notes:

Like rs_sndbyt, rs_sndstr writes the characters to the output

buffer and returns immediately - it does not wait for the

characters to actually be transmitted.  If there is insufficient

room in the output buffer, it waits until there is unless output

is currently disabled via flow control.  In this case, rs_sndbyt

returns the number of characters that were written to the

buffer.

If transmit interrupts are disabled by defining RS_POLLED_XMIT,

the output buffer is not used and the function does not return

until the entire string has been sent or output becomes disabled

via flow control.

>   rs_getbyt

>   Purpose:

Gets a single byte.

>   Prototype:

int rs_getbyt(void);

>   Arguments:

none

>   Return Value:

Returns the next received byte (converted to int) available.  If

no characters are currently waiting to be read from the input

buffer, returns -1.

>   Examples:

while(rs_inrcvd()

putch(rs_getbyt());

if((ch = rs_getbyt()) < 0)

printf("No received character\n");

>   Notes:

rs_getbyt does not wait for incoming characters - it returns -1

immediately if no received characters are available.

>   rs_getstr

>   Purpose:

Gets a string of bytes.

>   Prototype:

int rs_getstr(int rs_getcnt, char *rs_getbuf);

>   Arguments:

rs_getcnt is the number of characters to be read from the input

buffer.  If rs_getcnt is 0, characters are read from the input

buffer until a nul character is encountered, which is included

in rs_getbuf.  rs_getbuf is a character pointer to the string

which the received characters will be written in to.  rs_getbuf

must be large enough to hold the entire string plus a

terminating nul character.  rs_getstr can be used to get a

string up to the input buffer size.

>   Return Value:

Returns -1 if no port is open or the number of characters read

from the input buffer and written to the target string.

>   Examples:

if(rs_getstr(8,msg) < 8)

printf("Message incomplete\n");

>   Notes:

If the requested number of characters are not available,

rs_getstr does not wait for them to become available but instead

reads as many as are available and returns the number copied.

>   rs_inrcvd

>   Purpose:

Returns number of bytes which been received.

>   Prototype:

unsigned rs_inrcvd(void);

>   Arguments:

none

>   Return Value:

Returns the number of characters which have been received and

are ready to be read from the input buffer by rs_getbyt or

rs_getstr.

>   Examples:

in_cnt = rs_inrcvd();

rs_getstr(rs_inrcvd(),msg);

>   Notes:

rs_inrcvd will never return a value greater than the input

buffer size specified when rs_initport was called, even if an

input buffer overflow has occurred.  Use rs_error to determine

if there has been an overflow.

>   rs_peek

>   Purpose:

Returns next available character from input buffer.  Leaves

character in buffer.

>  Prototype:

int rs_peek(void);

>  Arguments:

none

>  Return Value:

Next available character in input buffer.  If there is not

currently a port open, or if there are no characters waiting

to be read from the buffer, returns -1.

>  Examples:

next_char = rs_peek();

if(rs_peek() >= 0){

if(rs_peek() == JUNK)

rs_getbyt();

else

message = rs_getbyt();

}

>   rs_outfre

>   Purpose:

Returns amount of free space remaining in output buffer.  If

RS_POLLED_XMIT is defined, this function will not return a

meaningful value.

>   Prototype:

unsigned rs_outfre(void);

>   Arguments:

none

>   Return Value:

Unsigned integer indicating current amount of free space in

bytes that are available in the output buffer.

>   Examples:

if(rs_outfre() == 0)

printf("Serial port busy - try again later\n");

printf("%u bytes are currently buffered for output\n",

output_buffer_size - rs_outfre());

>   rs_scanin

>   Purpose:

Scans characters not yet read from input buffer for the

specified string.

>   Prototype:

int rs_scanin(char *);

>   Arguments:

Character pointer of string to search for.

>   Return Value:

Int containing either the offset into the buffer of the

first occurence of the search string (if found) or -1 if the

string was not found.

>   Examples:

if(rs_scanin("CONNECT") < 0)

printf("No connect message from modem\n");

offset = rs_scanin("BYE");

if(offset)

rs_getstr(offset + 1,buf);

>   rs_error

>   Purpose:

Returns code for last error detected.

>   Prototype:

int rs_error(void);

>   Arguments:

none

>   Return Value:

Returns integer whose bit pattern indicates the last error which

occurred as follows:

bit 0 (RS_RBER): 1 = receive buffer overflow

bit 1 (RS_ROER): 1 = receive data overrun (UART received data

faster than it could be read by the interrupt

service routine)

bit 2 (RS_PERR): 1 = parity error

bit 3 (RS_FERR): 1 = framing error

bit 4 (RS_BKDT): 1 = break detected

Returns 0 if no error has been detected since last call to

rs_error or since port was opened.

>   Examples:

if(rs_error())

printf("Serial port error\n");

if(rs_error() & RS_PERR)

printf("Parity error!\n");

>   Notes:

A call to rs_error returns a code for any errors which have

occurred and clears the error flags.  Subsequent calls will not

indicate the same error unless it has reoccurred.

Most UARTs will indicate a framing error when a break is

detected.

>   rs_modctrl

>   Purpose:

Controls or returns status of modem control lines.

>   Prototype:

int rs_modctrl(int rs_cmd,...);

>   Arguments:

rs_cmd = 2 (RS_GETMCR): Return status of modem control output

lines.

rs_cmd = 1 (RS_WRTMCR): Change status of modem control output

lines.  2 additional arguments are required with this

command. The first is an integer with a bit pattern

indicating which line(s) to control:

1 (RS_MCRDTR): Data Terminal Ready line.

2 (RS_MCRRTS): Ready To Send line.

The second is an integer determining whether the

line(s) should be turned on or off:

0 (RS_LINOFF): Turn line off.

1 (RS_LINON): Turn line on.

rs_cmd = 0 (RS_GETMSR): Return status of modem control input

lines.

>   Return Value:

If rs_cmd = 0 (RS_GETMSR), an integer with a bit pattern

indicating the requested status is returned:

0x01 (RS_CTSCHG) CTS line changed states

0x02 (RS_DSRCHG) DSR line changed states

0x04 (RS_RICHG)  RI line changed states

0x08 (RS_DCDCHG) DCD line changed states

0x10 (RS_CTSSTE) state of CTS line

0x20 (RS_DSRSTE) state of DSR line

0x40 (RS_RISTE)  state of RI line

0x80 (RS_DCDSTE) state of DCD line

If rs_cmd = 1 (RS_WRTMCR), zero is returned.

If rs_cmd = 2 (RS_GETMCR), an integer with a bit pattern

indicating the current state of the modem control output lines

is returned:

0x01 (RS_MCRDTR): 1 = DTR on

0x02 (RS_MCRRTS): 1 = RTS on

If an operation fails (no port open), -1 is returned.

>   Examples:

if(! (rs_modctrl(RS_GETMSR) & RS_MSRDSR))

printf("Remote equipment is not ready\n");

/* set Data Terminal Ready output line on */

rs_modctrl(RS_WRTMCR,RS_MCRDTR,RS_LINON);

if(rs_modctrl(RS_GETMCR) & RS_MCRRTS)

printf("Ready To Send output line is currently off\n");

>   Notes:

When using rs_modctrl to return the status of the modem status

input lines (rs_cmd = RS_GETMSR), the lower 4 bits of the value

returned indicate that the corresponding modem status lines have

changed states since the last time the status was read.

The next time the status is read, these bits will be cleared

unless a line has again changed states.

When using rs_modctrl to set modem control output lines, more

than 1 line can be turned on or off with the same call by or'ing

the bit pattern for the desired lines (i.e.

rs_modctrl(WRTMCR,RS_MCRDTR | RS_MCRRTS,RS_LINON) ).

>   rs_break

>   Purpose:

Sends break to remote equipment.

>   Prototype:

int rs_break(void);

>   Return Value:

Returns 0 on success, -1 if no port open.

>   Examples:

if(rs_error())

rs_break();

>   Notes:

An RS232 line is normally in the "marking" state when idle.

rs_break places the line in a spacing state for approx. 1

character period.  It is used by some communications equipment

to signal some event.

If a buffer transmit is currently in progress, rs_break will

wait until the buffer is empty before sending the break unless

output is currently disabled via flow control.  In this case,

rs_break will send the break immediately.

>   rs_clrout

>   Purpose:

Clears output buffer (and the UART output FIFO if used).  If

RS_POLLED_XMIT is defined, clears the output FIFO only.

>   Prototype:

void rs_clrout(void);

>   Return Value:

none

>   Examples:

if(rs_error() & RS_BKDT)  /* break detected */

rs_clrout();            /*  stop transmission, clear output */

>   Notes:

rs_clrout will immediately stop transmission and clear the

output buffer.  It has no effect if a port is not open.

>   rs_clrin

>   Purpose:

Clears input buffer.

>   Prototype:

void rs_clrin(void);

>   Return Value:

none

>   Examples:

if(rs_error()){ /* if an error occurred */

rs_clrin();   /*    clear input buffer */

rs_sndstr(0,"resend last message");

}

>   Notes:

rs_clrin will immediately clear the input buffer of any

characters not yet read.  If a UART FIFO is in use, it will also

be cleared.

>   rs_keyhit

>   Purpose:

Determines if a key has been pressed.

>   Prototype:

int rs_keyhit(void);

>   Return Value:

Returns 0 if no keys have been pressed, 1 if one or more keys

have been pressed but not yet read.

>   Examples:

if(rs_keyhit())

rs_sndbyt(getch());

>   Notes:

When constant keyboard monitoring is required, rs_keyhit should

be used in place of library functions like kbhit().  rs_keyhit

reads the BIOS keyboard data area directly to determine if any

keys have been pressed.  Library functions like kbhit() call a

BIOS interrupt routine to accomplish the same thing.  This can

cause loss of incoming serial data when using high transmission

speeds.  A port need not be open to use this function.

>   rs_timer

>   Purpose:

Times events or operations.

>   Prototype:

unsigned rs_timer(int rs_cmd);

>   Arguments:

One integer argument determines the action which will be taken:

0 (RS_CLRTIM): The current timer value is returned and the

timer is set to 0.

1 (RS_GETTIM): The current timer value is returned and timing

continues.

>   Return Value:

rs_timer returns an unsigned integer indicating the number of

"ticks" (18.2 per second) which have elapsed.

>   Examples:

rs_timer(RS_CLRTIM);

rs_sndstr(msg_len,msg);

while(! rs_inrcvd()){

if(rs_timer(RS_GETTIM) > 18){ /* wait approx. 1 second */

printf("Message response timed out\n");

break;

}

}

>   Notes:

rs_timer reads the BIOS time data area directly and avoids use

of BIOS interrupt calls to do this.  It should be used instead

of library routines such as biostime() to avoid loss of incoming

serial data when performing high speed communication.  The value

returned may be in error by as much as 1 "tick" so should be

considered a fairly low resolution timer.  A port need not be

open to use this function.

>   rs_setflow

>   Purpose:

Establishes or returns status of flow control.

>   Prototype:

int rs_setflow(int rs_cmd,...);

>   Arguments:

rs_cmd can be one of the following:

0 (RS_FLWOFF) -  Disables flow control.

1 (RS_FLWHDW) -  Enables hardware flow control.  Requires an

additional integer argument to define hardware

line(s) to use for flow control:

1 (RS_FLWCTS) - Use Clear To Send

2 (RS_FLWDSR) - Use Data Set Ready

4 (RS_FLWRI) - Use Ring Indicator

8 (RS_FLWDCD) - Use Data Carrier Detect

2 (RS_FLWXON) -  Enables XON/XOFF flow control.  Requires two

additional parameters, the first of which

defines the character to use for XON, the

second defining the character to use as XOFF.

The following two pre-defined constants are

the standard XON and XOFF characters:

0x11 (RS_XON)

0x13 (RS_XOFF)

3 (RS_FLWSTAT) - Returns the status of flow control.  1

indicates that output is currently disabled

by flow control.  0 indicates output is

currently enabled.

4 (RS_FLWINS)  - Inserts a control character in the output

stream.  The control character to be sent is

defined by one additional argument.

>   Return Value:

Returns -1 if no port open.  Except when rs_cmd is 3

(RS_FLWSTAT), returns 0 on success.  When rs_cmd is RS_FLWSTAT,

value returned is status of flow control.

>   Examples:

/* set flow control to use Data Set Ready hardware line */

rs_setflow(RS_FLWHDW,RS_FLWDSR);

/* set flow control to XON/XOFF using standard XON and XOFF

characters */

rs_setflow(RS_FLWXON,RS_XON,RS_XOFF);

if(rs_setflow(RS_FLWSTAT))

printf("Output currently disabled via flow control\n");

/* send XOFF character to remote */

rs_setflow(RS_FLWINS,RS_XOFF);

/* disable flow control */

rs_setflow(RS_FLWOFF);

>   Notes:

Enabling flow control, whether with hardware line monitoring or

with XON/XOFF characters, will cause output to stop immediately

when the defined condition is met (the selected hardware line

goes to logic 0 or the character used as XOFF is received).  Any

characters buffered for output will remain buffered and

transmission will resume when the hardware line being used for

flow control goes to logic 1 or the character used for XON is

received.  Flow control effects output only.  Characters can

continue to be received or written to the output buffer up to

it's capacity when output is disabled via flow control.  Only

one type of flow control may be used at once.

If the remote equipment is using XON/XOFF type flow control,

rs_setflow can be used to control remote output with the

RS_FLWINS command.  The control character will be sent

immediately, even if characters are already being transmitted

via the output buffer.  The buffer will continue to be sent

after the control character has been transmitted.

>   rs_close

>   Purpose:

Closes" port prepared by rs_initport.

>   Prototype:

void rs_close(void);

>   Return Value:

none

>   Notes:

This function should always be used when an application has

"opened" a port with rs_initport.  rs_close disables interrupts

from the port and restores the previous interrupt vector.

Failure to call rs_close before exiting from the application can

very likely cause undefined behavior.  The function can be

registered with the library function atexit() to insure that it

gets called.  It is probably also a good idea to install a

ctrl-break handler which calls rs_close to prevent exiting the

application without closing the port:

int c_break(void)

{

rs_close();

return(0);

}

/* somewhere in application */

ctrlbrk(c_break); /* library function ctrlbrk registers

cleanup function */

If a buffer transmit is currently in progress, rs_close waits

for it to finish before closing the port unless output is

currently disabled via flow control.

>   General Notes:

>   Baud Rates:

Although RS232.C supports speeds up to 115.2K baud, the higher

speeds may not work on some machines due to a variety of

factors.  Foremost is the speed of the CPU itself - slower

machines may not be capable of sustained input at high speeds.

Other factors may effect the maximum usable baud rate also.  For

instance, even though a machine's CPU is fast enough to support

high speeds, other software running on the machine may prevent

it.  (One example is a 386 CPU running an extended memory

manager which places the CPU in protected mode.  This can cause

interrupt service routines to be slower.)  The BIOS in some

machines may have routines which are too slow, such as the timer

interrupt service routine - this can effect time critical

routines like the serial interrupt service routine in RS232.C.

Only experimentation can determine what the highest usable speed

is on any given machine and configuration.

>   Interrupts:

Because machines equipped with COM ports 3 and 4 use hardware

interrupt lines shared with ports 1 and 2, a problem can arise

with some hardware.  For example, an internal modem is installed

as COM1 and you are attempting to use COM3 for some other

purpose.  Because both ports use interrupt request line IRQ4, if

the modem board is "holding" IRQ4 inactive, COM3 will never be

able to signal an interrupt and thus will appear dead.  Most

serial adapter boards do not behave this way and will "float"

the interrupt request line when not signaling an interrupt.

Also, the problem can only occur if software accessing the

problem board leaves interrupts enabled at the board.

(Specifically, the signal OUT2 is left active with modem control

register bit 3.)

If your application installs any of it's own interrupt routines,

they must re-enable interrupts as soon as possible after entry

to prevent loss of incoming serial characters.

RS232.C re-prioritizes hardware interrupts to give serial

interrupts the highest priority.  While this will have no

noticeable effect with most applications, it should be taken

into account in those applications which make use of hardware

interrupts from other sources.

>   Polled vs. interrupt driven output:

The default method of character transmission by RS232.C uses

interrupts.  Each time a character is written a character is

written to the UART's transmit holding register, an interrupt is

generated by the UART when the transmit holding register becomes

available for the next character to be sent.  It may be

desirable under certain circumstances to disable this feature

and use "polled transmission" instead.  This requires that

software constantly poll the UART and write the next character

when the transmit holding register becomes free.  The method

used is determined at compile time.  If the macro RS_POLLED_XMIT

is defined, the polling method will be used.  The following line

placed at the beginning of the application's source code will

accomplish this:

#DEFINE RS_POLLED_XMIT

The following describes some advantages and disadvantages to

both methods:

Interrupt driven output:

Advantages:  This is the more efficient method of output.  No

CPU time is wasted polling the UART.  Characters may be queued

for output and the application may go on processing without

waiting for the characters to actually be sent.

Disadvantages:  Some additional overhead is incurred in the

interrupt service routine.  At high transmission speeds this

may cause problems on slower machines or under certain

circumstances.  Interrupt driven output may not work properly

with some non-standard hardware.

Polled output:

Advantages:  This method makes the interrupt service routine

slightly "leaner" and simplifies some of the code elsewhere

making the overall code size slightly smaller.  This method

may prove to be more reliable on some hardware.

Disadvantages:  While the application is sending characters,

it can do nothing else and no output buffer is used.  This

method is less efficient for throughput.

>   Flow control and handshaking:

Flow control is provided for only in respect to stopping and

starting output as requested by remote equipment.  To utilize

flow control on remote equipment, the application must monitor

the conditions requiring flow control (i.e. input buffer free

space) and take the appropriate action when needed (like sending

XOFF or dropping a hardware line).

Handshaking is something that is completely arbitrary and the

only requirement is that the local and remote equipment must

agree on the protocol.  RS232.C provides a function for

determining the status of and controlling hardware handshake

lines but it is up to the application to determine how it should

be used.

>   Compiling:

If RS232.C can be simply included (whether by "pasting" it

directly into an applications source code or with a preprocessor

"#include" directive) no special compiler considerations are

needed.  If desired, RS232.C can be split into separate header

and code files to follow more conventional 'C' conventions.

This is accomplished by splitting the file where indicated.

Programs using RS232.C can be compiled and linked from the IDE

if desired.

Borland's newer compilers have many optimization options and, so

far, none have caused problems with RS232.C.  One necessity is a

must when using Turbo C 2.0 or earlier compilers: Stack checking

must be disabled.  With stack checking enabled in the older

compilers, stack checking code is generated even in interrupt

functions.

Any of the memory models can be used with RS232.C - the only

drawback is the extra function call overhead and/or memory

access overhead incurred with the large models.  This may

prevent an application from being able to use the higher baud

rates on some machines.

>   Other Compilers:

RS232.C will compile and run with other DOS based compilers with

minor changes.  The following lists types/functions/macros which

may need attention under compilers other than those produced by

Borland:

function type interrupt - must take care of saving and

restoring registers and provide IRET instruction

unsigned char inportb(int portid) /* reads a byte from

a hardware port */

void outportb(int portid,unsigned char value) /* writes a

/* byte to a port */

void far *MK_FP(unsigned seg, unsigned ofs) /* takes integer

segment and offset

arguments and returns

far pointer */

The code is C++ compatible via "#ifdef __cplusplus"

preprocessor directives.  In order to use RS232.C in

a C++ program, the header file must be separated from

the main body of the code (name it RS232.H) and RS232.C

must be compiled separately (as in a project file).

>   16550 FIFO mode:

The 8250A/B and 16450 UARTs found in most machines contain only

a one byte buffer for input and output.  This means that for

every byte which is received, an interrupt is generated which

invokes a routine to retrieve the incoming byte and store it.

This must take place before the next incoming byte has been

fully received to avoid loss of data.  The same is true of

outgoing data - after each byte being transmitted has been

sent, an interrupt is generated to invoke a routine to write the

next byte to the transmit register.  In some situations, this is

simply too much overhead or the interrupt service routines can

not be allowed to react quickly enough to keep up with the data.

The more sophisticated 16550 series of UARTs overcome this with

the inclusion of 16 byte FIFOs (first-in-first-out-registers)

for both input and output.  Software must be able to recognize

and enable this feature when it's available, and software

determines how it is used.  Code in RS232.C checks for the

presence of FIFO capable UARTs and automatically employs them.

Outgoing data is written to the UART at 16 bytes per interrupt

and incoming data signals generates a receive interrupt after 4

bytes have accumulated in the FIFO - though up to 16 bytes can

be received before the interrupt service routine is able to

respond, without losing any data.  These defaults can be changed

by altering the corresponding constants in the function

rs_initport (see comments in RS232.C).

>   Using More Than 1 COM Port Simultaneously:

While RS232.C provides no direct support for simultaneous serial

port access, it is easy enough to accomplish.  The solution is

to include 2 copies of RS232.C in your source code.  Because all

variable and function names in RS232.C begin with 'rs_', this

makes it easy.  To allow an application to access 2 ports at

once you can do the following:

Create 2 copies of RS232.C renaming them to RS232A.C and

RS232B.C.

Using Borland's built in editor (or any editor capable of

search and replace), edit RS232A.C performing a global

search and replace of the string "rs_" with "rs1_".  Do

the same for "RS_" replacing it with "RS1_".

Do the same thing with RS232B.C, replacing "rs_" with

"rs2_" and "RS_" with "RS2_".

Include both modified copies in the source code for your

application.  This provides 2 complete sets of functions

(the function names now start with "rs1_" or "rs2_",

depending on which port is being accessed) allowing access

to 2 different ports and simultaneous serial port I/O.

While not the most elegant of solutions, it is workable

and RS232.C is small enough so as not to make inclusion of

2 copies a problem. (RS232.C adds about 3400 bytes to an

executable file.)

Only certain ports can be used simultaneously.  Because COM1

uses the same hardware interrupt as COM3 and COM2 uses the same

interrupt as COM4, you will most likely not be able to use COM1

with COM3 or COM2 with COM4.  The problem arises due to the

design of the hardware - if a UART has its OUT2 hardware line at

logic 1 (necessary to enable interrupts), another UART sharing

the same interrupt will not be able to signal an interrupt.

This is true of any serial communications software which uses

interrupts.  A serial mouse driver using COM1 preventing serial

I/O on COM3 is one example of how this problem may arise even

when using only one port in an application.  One way to get

around this problem (other than opening and closing the ports as

needed using rs_close) is to turn off OUT2 on the UART which is

NOT currently being utilized.  OUT2 is controlled by bit 3 of

the modem control register and can be controlled using the

rs_modctrl function of RS232.C.  The following is an example

which would allow interrupts to occur on a port opened with

rs1_initport and reenable interrupts on a port opened with

rs2_initport when finished:

rs2_modctrl(RS2_WRTMCR,rs2_modctrl(RS2_GETMCR) & '\xF7');

rs1_modctrl(RS1_WRTMCR,rs1_modctrl(RS1_GETMCR) | '\x08');

perform I/O with rs1...

.     .   .    .

.     .   .    .

.     .   .    .

rs1_modctrl(RS1_WRTMCR,rs1_modctrl(RS1_GETMCR) & '\xF7');

rs2_modctrl(RS2_WRTMCR,rs2_modctrl(RS2_GETMCR) | '\x08');

resume I/O with rs2...

One very real drawback to this method is that no interrupts

can occur on the on the disabled port and data could be

lost.  Some serial adaptor allow for the conflict

caused by shared interrupts by or'ing the interrupt request

output of the conflicting UARTs.  Only testing can ascertain

if this is so.  Another workaround (and only if the hardware

supports it) is to use an alternate IRQ which does not

conflict with another device.  See the function description

for rs_initport for instructions on how to use an alternate

IRQ and/or base address.