Rapid Contour Plots By Bilinear Patch

In this technique, the points of the original data array are viewed as

being samples of a function that is continuous, with piecewise

continuous partial derivatives.  This function is presumed to be

bilinear within the squares delineated by the data points.  In order

to prepare the contour plot of the entire region, we merely prepare a

contour plot of each square "patch" delineated by four adjacent data

points.  There is one potential problem with this method, and that is

if a contour value is exactly equal to one of the data values (one of

the values exactly on the corner of a patch), then the plot becomes

ill-conditioned.  In the software example provided, this is readily

prevented.  The data values take on only integer values, while the

contour levels are floating point.  We simply prevent any contour

value from taking on an exact integer value by adding a small number

(constant called epsilon) if we determine that the contour value is an

exact integer.

Once this conditioning problem is resolved, it may readily be seen

that within a single patch, for a specified contour level, exactly

three possibilities exist:  no contour line crosses the patch,

determined if the contour level is either less than the minimum of the

four corner values or greater than the maximum of the corner values;

one contour line crosses the patch; or two contour lines cross the

patch.  In the case where one contour line crosses the patch, the

bilinear equation is solved for the endpoints of the contour line and

the line is plotted.  In the case where two contour lines cross the

patch, we determine the four endpoints, and then must decide which

pairs of endpoints to match to draw the appropriate contour lines.

Define the bilinear coefficients for the patch in a coordinate system

local to the patch, so that the x value ranges from 0 to 1 and the y

value ranges from 0 to 1, and let the bilinear equation be:

value(x,y) = ax + by + cxy +d.

If we now attempt to parametrize the contour line in x in terms of y

we find:

x = (value - contour level - by - d)/(a + cy).

We then see, that because of the nature of the local coordinate

system, one of the contour lines must have the y value of both of its

endpoints greater than -a/c, and the other contour line must have both

of its endpoints less than -a/c.  (It should be noted that because of

the non-integral nature of the contour level, we cannot have two

contour lines in a patch when c=0).  This means that if we simply sort

the four endpoints in increasing value of their y-values, that the two

endpoints with the lowest y-values form a pair for one contour line

and the two endpoints with the highest y-values form a pair for the

other contour line.

In the software example provided, contour lines are approximated by

drawing straight lines from one edge of the bilinear patch to the

other.  In cases where the number of data points are large relative to

the screen pixel density (say 20 x 20 data points for an EGA display),

this is adequate for reasonable contour plots.  If the data are

sparse, it may be desirable to plot the contour line more accurately

within the patch, using the parametrized equation above.

The software example provided consists of three modules and one main

program.  The first module is c_defs.pas and contains the definitions

used for the contour plotting software, the pointers to the data array

and the contour level array, and a procedure for allocating memory to

these arrays.  The data array is called data_array_pointer, which

points to longint data points, via:  data=data_array_pointer^[x]^[y],

with 0<=x<max_x_size and 0<=y<max_y_size.  The contour level array

points to type float (currently set to be single for speed with

coprocessors), and is accessed via:  contour level = contours^[i],

with 0<=i<max_contours.  These data arrays are allocated via a call to

init_array.

The second module is video.pas and contains the routines necessary for

manipulating the display.  These procedures are:  init_graphics, which

loads the BGI driver for the graphics display (this procedure requires

that the Borland BGI driver file for the display be in the default

directory); close_graphics, which returns the display to text mode;

and make_line, which plots a single contour line through a single

patch.  It should be noted, that as written make_line takes as an

argument the contour number.  The color of the contour line is

specified by this contour number, so that on EGA and VGA displays, the

contour plot is color-coded to make it easier to interpret.

The third module is contour.pas and contains the routines necessary

for contour plotting.  The only procedure available for external

calling is contour_plot, which performs the entire plot function.

The test program, test.pas, is a skeleton of a general user program

employing the contour plot modules.  It performs the operations

minimally necessary for operation.  First, it calls init_array to

allocate storage.  Then, it fills the data array.  In this example,

the data array consists of a sum of two two-dimensional Gaussians plus

noise.  The contour level array is then filled with values (which need

not be in any particular order).  Init_graphics is called to set the

display to graphics mode, contour_plot is called to perform the plot,

and finally close_graphics is called to return the display to text

mode.

The test package is compiled using the Borland command-line compiler,

version 5.0, by:  tpc /B test.pas.  All modules internally use

compiler options $N+ to compile with IEEE floating point and $E+ to

include the emulator, so that the test program will exhibit identical

behavior whether a coprocessor is present or not (except for speed).