High Intensity Background Colors

by Everett Ogden

In some of my dBASE programs I have used high-intensity backgrounds to

highlight the field areas on my input screens.  I used a "bright black"

background (actually dark gray), which I preferred to the usual alternatives,

such as blue.  Bright background is a documented alternative to flashing

characters (a certain bit in a certain register in the display adapter selects

one or the other), but it is used so little that some hardware and most

programs don't support it.  If you want to try it out, the following DEBUG

script file will create two files in your current directory: BLINKOFF.COM and

BLINKON.COM.  Create it with any editor that produces plain ASCII text.  Be

sure to include the blank line before the RCX.  Save it as BLANK.SCR and

execute it with the command DEBUG<BLANK.SCR (this assumes that DEBUG is

available in the current directory or in one designated in your PATH command).

N BLINKON.COM

A 100

MOV AL,29

MOV DX,03B8

OUT DX,AL

MOV DX,03D8

OUT DX,AL

MOV AX,1003

MOV BL,01

INT 10

INT 20

RCX

13

W

N BLINKOFF.COM

E 0101 09

E 010E 00

W

Q

To try it, display some blinking characters on the screen and run BLINKOFF from

the DOS prompt.  The characters should stop blinking, and the background should

get brighter.  Try a CLS to see if clearing the screen resets blinking.  If

not, run BLINKON to return to normal.

This works to varying degrees on IBM MDA and CGA displays, and Zenith CGA, EGA

and (sometimes) VGA displays.  The IBMs reset to blinking mode whenever the

screen is cleared.  Most Zeniths don't.  Recent Zenith VGA cards may give a

large block cursor when you run this program (but not all the time).  It had no

effect on at least one CGA system I tried (I don't remember the brand).  All

considered, it may not be worth the bother.

I used to use this in dBASE II, in a modified version that I POKEd into memory

and then CALLed each time I ERASEd the screen.  With either FoxBASE+ or dBASE

III Plus you can RUN the programs above.  Or you can LOAD and CALL them if you

make a few minor changes.  The INT 20 instruction must be changed to RETF and

the file extensions should be changed to .BIN instead of .COM.  Finally, you

could change the length (the number on the line after the RCX command) to 12,

since RETF is one byte while INT 20 is two.  But it won't hurt to leave it

alone.