STANDARD MICROSYSTEMS CORPORATION

PC Agent/SNMP

For Ethernet and TokenCard EliteSeries LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992, 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide:

o  Provides an introduction to PC Agent/SNMP.

o  Lists the system requirements for running PC Agent/SNMP.

o  Explains how to install PC Agent/SNMP.

o  Describes how to use PC Agent/SNMP.

o  Describes the configuration file contents.

o  Lists installation error messages.

o  Lists installation warning messages.

o  Describes how to unload PC Agent/SNMP and the unloading error

messages.

INTRODUCTION

PC Agent/SNMP is a TSR (Terminate and Stay Resident) program. As a

true SNMP Agent, it allows your PCs and LAN adapters to be directly

monitored by any SNMP Manager, such as HP OpenView, Sun SunNet

Manager and SMC's own EliteView. In addition, PC Agent is network

operating system-independent, so it offers you end-to-end management

of your entire star or bus network, even if the server is inoperable.

SYSTEM REQUIREMENTS

PC Agent/SNMP can run on XT, AT, PCMCIA, EISA, and Micro Channel

Architecture computers. An MS-DOS or PC-DOS operating system

(version 3.1 or above) is required, and an agent-compatible network

driver must be resident in memory. The SMC network drivers that are

compatible with PC Agent/SNMP are :

o  NDIS Token-Ring driver - SMC8100.DOS (ver. 1.00 and above)

o  NDIS Ethernet driver - SMC8000.DOS (ver. 2.00 and above)

o  ODI Token-Ring driver - SMC8100.COM (ver. 1.03 and above)

o  ODI Ethernet driver - SMC8000.COM (ver. 3.03 and above)

o  IPX driver - IPX.COM (ver. 1.00 and above for Token-Ring

or 5.00 for Ethernet)

This TSR occupies approximately 22K bytes and can be loaded into

either high or low memory.

PC Agent/SNMP can be used with IPX and TCP/IP network. Although PC

Agent/SNMP does not require the loading of a network operating system

or protocol stack, an IP address must be available if it is to

communicate with SNMP Network Management Stations using TCP/IP

protocol.

INSTALLATION PROCEDURE

To load PC Agent/SNMP:

1. Type: PCAgent  [Configuration File]

The [Configuration File] parameter is optional. If Configuration

File is not specified, PCAGENT.CFG will be used as the default

configuration file.

2. Press the Enter key.

The Configuration File is used to pass user-defined information to PC

Agent. This information may include the name of the person using the

PC and the PC's location, make and model. For details on the format

and contents of this file, see the topic "CONFIGURATION FILE

CONTENTS."

PC Agent/SNMP consists of a single executable file: PCAGENT.EXE. It

can be loaded only after the appropriate driver (IPX, ODI, or NDIS)

has been loaded into memory.

For example, adding the following lines to the AUTOEXEC.BAT file

loads the SMC Ethernet ODI Driver (SMC8000.COM) followed by PC

Agent/SNMP, with PCDATA.CFG used as the Configuration File:

LSL

SMC8000

PCAGENT PCDATA.CFG

Note that PC Agent/SNMP may be executed immediately after the driver

is loaded, as in the examples above, to best ensure that load-time

requirements are met. Since PC Agent/SNMP is a TSR, it cannot be

loaded after WINDOWS is running.

However, it may be more convenient to place Configuration Files for

the various PCs on a server where they can be controlled by a Network

Administrator. In this case, the Login sequence would precede the

loading of PC Agent. For example, if the Network Administrator has

allocated a user area on network drive F for the various users'

Configuration Files, the following lines in user Kathy's AUTOEXEC.BAT

file loads PC Agent in a NetWare environment:

LSL

SMC8000

IPXODI

NETX

LOGIN

PCAGENT F:\USERS\AGTSTUFF\KATHY.CFG

OPERATION OF PC AGENT/SNMP

PC Agent/SNMP maintains a database of statistical and configuration

information in its RAM space. The statistics are gathered during

network operation, and include such information as physical level

counters and the total number of transmit and receive packets. Most

of the configuration information is obtained directly from the PC and

the adapter. The remainder is obtained from the Configuration File.

When PC Agent/SNMP is loaded, it first examines its PC and adapter

and initializes its database accordingly. Then, if a Configuration

File has been specified in the command line, PC Agent uses the file

to initialize, and in some cases overwrite, the information in its

database. If a Configuration File has not been specified in the

command line, PC Agent will use PCAGENT.CFG as the default

Configuration File and assume that it is located in the same directory

as PC Agent.

For example, the agent may initialize the display type value in its

database to "EGA/VGA" (a generic description). However, if the

display type is also present in the Configuration File, its value

would replace the information obtained by PC Agent. Often the

description entered into the Configuration File is more specific than

that, which can be obtained by PC Agent (for example, "ATI VGA Wonder

w/512KB").

CONFIGURATION FILE CONTENTS

Each line in the Configuration File contains both the name of an

Object (variable) and its value. In most cases, this configuration

information is optional. However, one Object, the IP address of the

agent (smcDosWsIpAdr), must be provided if the agent is to be

addressed by Network Management Stations using TCP/IP protocol.

The syntax of lines in the configuration file is very simple, as

shown below:

ObjectName = ObjectValue

where ObjectName is the name assigned to the Object and ObjectValue

is the value defined by the user.

Object names are case-sensitive; those which can be assigned a value

in the Configuration File are listed below. Note that the first

three listed, smcDosWsIpAdr, ipDfGwy and ipSubNetMask are required if

the agent is to communicate with SNMP network management stations

using TCP/IP protocol. Otherwise, they are optional.

smcDosWsIpAdr

This contains the IP address for PC Agent/SNMP. If the PC also has

a TCP/IP stack, the address assigned to the agent may be identical

to the one assigned to the TCP/IP stack. However, it is often

convenient to assign a different IP address to the agent to avoid

confusion. Note that the agent must be assigned an IP address in

the Configuration File regardless of whether a stack is loaded on

the PC with the same IP address. Since TCP/IP stacks vary widely

in the way their IP addresses are specified, the agent has no way

of "discovering" the IP address of such a stack.

SYNTAX :   IP address in the form of xxx.xxx.xxx.xxx, where xxx is

an integer from 0 to 255.

EXAMPLE :  smcDosWsIpAdr = 129.254.2.3

ipDfGwy

This is the default gateway for the IP network. It is used when

traps are configured for IP addresses on another subnet.

SYNTAX :   IP address in the form of xxx.xxx.xxx.xxx, where xxx is

an integer from 0 to 255.

EXAMPLE :  ipDfGwy = 129.254.0.99

ipSubNetMask

This is the IP subnet mask. It indicates which bits in the IP

address field refer to the network address.

SYNTAX :   IP address in the form of xxx.xxx.xxx.xxx, where xxx is

an integer from 0 to 255.

EXAMPLE :  ipSubNetMask = 255.255.0.0

ComStr

This is the password that the SNMP Manager uses for read/write

operations. If this object is not defined in the configuration

file, this object will have a default value of 'public'.

SYNTAX :   a string of maximum 12 characters.

EXAMPLE :  ComStr = mypassword

sysContact

This is usually the person to call for help. It is initialized by

the agent with SMC's technical support telephone number in the USA.

SYNTAX :   a string of maximum 45 characters.

EXAMPLE :  sysContact = Technical Support 1-800-123-2345

sysName

This is usually the owner/user of the PC. It is not initialized by

the agent.

SYNTAX :   a string of maximum 45 characters.

EXAMPLE :  sysName = Sally Jones

sysLocation

This is usually the physical location of the PC. It is not

initialized by the agent.

SYNTAX :   a string of maximum 45 characters.

EXAMPLE :  sysLocation = somewhere on earth

smcDosWsPcTyp

This is the make and model of the PC. It is not initialized by the

agent.

SYNTAX :   a string of maximum 40 characters.

EXAMPLE :  smcDosWsPcTyp = IBM AT

smcDosWsPcProc

This indicates the processor type and whether or not there is a

co-processor. It is initialized by the agent based on discovery

during the examination of the PC.

SYNTAX :   a string of maximum 30 characters.

EXAMPLE :  smcDosWsPcProc = Processor 80386

smcDosWsPcBios

This is the BIOS manufacturer and version number. The PC tries to

discover this information by scanning for copyright notices in the

BIOS area.

SYNTAX :   a string of maximum 50 characters.

EXAMPLE :  smcDosWsPcBios = ABC BIOS V1.10

smcDosWsPcVideo

This is the type of video adapter on the PC.

SYNTAX :   a string of maximum 25 characters.

EXAMPLE :  smcDosWsPcVideo = Super VGA

smcDosWsTrapDstAdr.1

smcDosWsTrapDstAdr.2

smcDosWsTrapDstAdr.3

In a TCP/IP network environment, these are the IP addresses of

network management stations to which traps (alerts) should be sent

in case an alarm situation occurs. In an IPX network environment,

these are the IPX network addresses (in decimal) of the network

management stations where traps should be sent. Traps can be sent

to a maximum of 3 management stations(Adr.1, Adr.2 and Adr.3).

SYNTAX :   In a TCP/IP network environment, the syntax is an IP

address in the form of xxx.xxx.xxx.xxx, where xxx is

an integer from 0 to 255.

In an IPX network environment, the syntax is

xxx.xxx.xxx.xxx, where xxx corresponds to each byte of

the 4 bytes IPX network address.

EXAMPLE :  smcDosWsTrapDstAdr.1 = 129.254.2.1

smcDosWsTrapDstPro.1

smcDosWsTrapDstPro.2

smcDosWsTrapDstPro.3

These are numbers between 2 and 6 indicating the protocol to be

used when sending traps to the network management stations having

an IP address or IPX network address of smcDosWsTrapDstAdr.1,

smcDosWsTrapDstAdr.2 and smcDosWsTrapDstAdr.3, respectively.

In an IPX network, if the network management station is on another

network, the protocol specified here should be acceptable by the

router on the local network.

1 = not used

2 = TCP/IP (Ethernet & Token-Ring)

3 = IPX with Ethernet II frame type (Ethernet ONLY)

4 = IPX with 802.2 (LLC) frame type (Ethernet & Token-Ring)

5 = IPX with SNAP frame type (Ethernet & Token-Ring)

6 = IPX with 802.3 frame type (Ethernet ONLY)

SYNTAX :   number between 1 and 6

EXAMPLE :  smcDosWsTrapDstPro.1 = 2

INSTALLATION ERROR MESSAGES

"PC AGENT/SNMP was not installed.

Too many parameters in command line."

"PC AGENT/SNMP was not installed.

Invalid parameter in command line."

"PC AGENT/SNMP was already installed."

"PC AGENT/SNMP was not installed.

DOS version too old (below 3.10); use DOS 3.10 or above."

"PC AGENT/SNMP was not installed.

Internal error (1) : Call SMC Technical Support."

"PC AGENT/SNMP was not installed.

Internal error (2) : Call SMC Technical Support."

"PC AGENT/SNMP was not installed.

PC AGENT/SNMP compatible network driver was not loaded."

"PC AGENT/SNMP was not installed.

There is no network adapter for the driver."

"PC AGENT/SNMP was not installed.

Unable to open configuration file <file name>."

INSTALLATION WARNING MESSAGES

"PC AGENT/SNMP Configuration file error :

invalid MIB variable <name>.

Line : < erroneous line in configuration file >"

"PC AGENT/SNMP Configuration file error :

invalid IP address <the IP address>.

Line : < erroneous line in configuration file >"

"PC AGENT/SNMP Configuration file error :

invalid numeric value for MIB variable.

Line : < erroneous line in configuration file >"

"PC AGENT/SNMP Configuration file error :

MIB variable <name> cannot be set from configuration file.

Line : < erroneous line in configuration file >"

*****************************************************************

Warning: PC AGENT/SNMP was not able to find a free interrupt vector

for application trap API. External application programs

will not be able to issue traps.

*****************************************************************

UNLOADING PROCEDURE

To unload PC AGENT/SNMP:

1. Type: PCAGENT /u (or /U)

2. Press Enter.

UNLOADING ERROR MESSAGES

"Not able to unload PC AGENT/SNMP.

PC AGENT/SNMP was not installed."

"Not able to unload PC AGENT/SNMP."

EtherCard PLUS, TokenCard, EliteSeries, and SuperDisk are trademarks,

and SMC and Standard Microsystems are registered trademarks, of

Standard Microsystems Corporation. Other trademarks mentioned herein

belong to their respective companies.