STANDARD MICROSYSTEMS CORPORATION

Novell Netware IPX Shell

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Novell NetWare IPX Workstation

driver files from the SuperDisk to your PC.

o  How to use SuperDisk with Novell's "WSGEN" process to create a

workstation driver.

o  How to build the NetWare IPX shell.

o  Preparing your operating system so you can attach to the network.

o  Error messages.

Note: This driver supports all 8216 (Ultra), 80xx (Elite), and

80xx (PLUS) Series ISA and Micro Channel Ethernet adapters.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default Destination Directory is a:\.  Please make a

backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create and installation diskette.

For your convenience, EZStart will copy the files IPX.COM and

NETX.EXE. The IPX.COM file is a fully functional IPX driver for

all SMC EtherCard Elite Ultra and EtherCard Elite Adapters, both ISA

and Micro Channel. You can load this file directly to your system

and bring up the SMC adapter immediately.  If you need to build a

special IPX driver for your environment, see the instructions under

"BUILDING THE NETWARE IPX SHELL".

CREATING THE IPX WORKSTATION DRIVER USING "WSGEN" FROM THE

HARD DISK

1. Copy the NetWare 2.2 floppy diskettes to your computer's

hard drive.

2. Create a directory called LAN_DRV_.200. This directory must

be at the same level as the LAN_DRV_.001 directory. Copy

all .OBJ files that are in the root directory on the

SuperDisk into the LAN_DRV.200 directory.

3. Copy the SMC8000W.LAN file from the root directory of the

diskette you created above to the \WSGEN directory on

your computer's hard drive. If you are generating a router,

also copy this file to the \ROUTEGEN directory.

4. Run WSGEN to create a workstation driver. For more

information on using this software, refer to the NetWare 2.2

Installation Manual.

CREATING THE IPX WORKSTATION DRIVER USING "WSGEN" FROM A

FLOPPY DISKETTE

1. Insert the Novell diskette labeled WSGEN into drive A and

execute WSGEN.

2. When prompted for a LAN_DRV_??? diskette, insert the

diskette you created above into drive A. For more

information on using this software, refer to the NetWare 2.2

Installation Manual.

BUILDING THE NETWARE IPX SHELL

1. Determine the LAN adapter configuration. For information on RAM,

Base I/O, and IRQ, refer to the supplied file SYSMAP.DOC.

2.  Run the Novell workstation generation program "WSGEN".

3.  Select the SMC Ethernet Adapter DOS IPX Driver.

4.  Select the Software Configuration.

For all SMC ISA Adapters: Choose the default configuration 'First

Software Configured Adapter'. The shell driver will read the

specific configuration information directly from the adapter.

Western Digital Adapters, which are not software configurable:

When configuring a workstation that contains a jumper-configured

adapter, configuration entries must specify a base I/O, IRQ, and

RAM size that match those selected on the adapter. If none of

the choices in the table matches the hardware configuration

desired, select the "Driver Configurable by Jumpers Utility"

entry and run Novell's JUMPERS.EXE version 2.00 configuration

program. This utility lets you configure the LAN software to

use any supported hardware configuration parameters. For more

information on JUMPERS.EXE, refer to the Novell documentation.

The JUMPERS utility is available from Novell or CompuServ.

The RAM location for jumper-configured adapters is set by

the driver to the location specified in the configuration

selected.

There are configuration selections designating RAM sizes

of 8, 16, and 32 Kbytes of RAM. Adapters with 8 Kbytes

of RAM can be used with configuration selections that

specify 8, 16 or 32 Kbytes of RAM. Adapters with 32 Kbytes

of RAM must be used with 32 Kbyte RAM configuration

selections.

The Ethernet Adapter AT bus 16-bit adapters can function

in two modes, 16-bit mode and 8-bit mode. In 16-bit mode, the

adapter performs 16-bit data transfers and uses the full 16

Kbytes of RAM. In 8-bit mode, the adapter performs 8-bit data

transfers and uses only 8 Kbytes of RAM.

To operate the 16-bit adapter in 16-bit mode, place the adapter

in a 16-bit bus slot and configure the driver to a

configuration selection which specifies 16 or 32 Kbytes of RAM.

To operate the 16-bit adapter in 8-bit mode, configure the

driver to a configuration selection that specifies 8 Kbytes of

RAM, or place the adapter in an 8-bit bus slot and specify any

driver configuration.

Micro Channel Adapters: When one Micro Channel LAN adapter card

is installed in a workstation, choose the default configuration

'First Software Configured Adapter'. The shell driver will read

the specific configuration information directly from the

adapter.

If the computer contains more than one Ethernet Micro Channel

LAN adapter, choose 'First, Second, Third or Forth Software

Configured Adapter' to select the Micro Channel adapter that

IPX.COM is to use. First, Second, etc., are determined by the

slot number of the adapter; for example if two adapters are in

slots 2 and 4 of the workstation, the adapter in slot 2 is the

'First'.

PREPARING YOUR OPERATING SYSTEM

1. Use any text editor to add the following lines to your

AUTOEXEC.BAT file:

IPX.COM

NETX.COM

2. Reboot your computer.

ERROR MESSAGES

"Could not find an adapter at I/O base address XXX"

The driver was unable to find an SMC Ethernet LAN adapter

at the specified base I/O address. Check for adapter

conflicts and retry the installation. Check that the adapter

is seated properly in adapter slot.

"Could not find an adapter at slot X (Micro Channel only)"

The driver was unable to find an SMC Ethernet LAN adapter

at the specified slot number. Check for adapter conflicts

and retry the installation. Check that the adapter is seated

properly in adapter slot.

"Adapter hardware failure during initialization"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test

process due to a hardware problem. Check the adapter

for configuration conflicts.

"Cannot access adapter RAM properly"

The adapter could not read from or write to the adapter RAM.

Verify that another adapter does not share the same memory

space as the LAN adapter.

"Other system RAM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

other RAM in the system. Reconfigure the adapter to use

a different RAM address and retry the installation.

"Other system ROM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

ROM in the system. Reconfigure the adapter to use a

different RAM address and retry the installation.

"IRQ higher than 7 not valid in 8-bit slot"

The IRQ number specified cannot be used if the adapter

is inserted into an 8-bit slot. Select a different IRQ

number.

"Adapter unable to process a multicast address"

A node address was given on the command line or the board is

configured for a node address that is reserved for multicast

address. Broadcast and multicast addresses are not allowed

as valid node addresses. Use the node address override

with caution as this can cause problems on your network.

"Configuration corrupted; run software setup"

The driver failed to initialize the adapter due to invalid

data in configuration EEROM. Run the software setup utility

and reconfigure the adapter.

"I/O base address chosen for driver not valid"

Reconfigure the adapter with another I/O base address,

then reinstall.

"I/O base address for driver out of range"

The selected I/O base address is not within the valid

range of I/O base addresses for this adapter. Select

another I/O base address and reinstall.

"IRQ higher than 7 not valid in 8-bit slot"

When using an 8-bit slot, the IRQ value selected must be

3 - 7. Change the IRQ value or move the adapter to a 16-bit

slot.

"IRQ chosen for driver not valid (out of range)"

The IRQ value read from the software-configurable adapter is

not one of the IRQs supported by the adapter.

"Adapter RAM base address on illegal boundary"

The RAM base value is invalid for this adapter. Reconfigure

the RAM base address and reinstall.

"RAM base address for driver out range"

The memory base address cannot be used for the configuration

or the installed adapter.

"RAM size chosen for driver out of range"

The memory size cannot be used for this configuration or

adapter.

"Invalid error message"

The error return code was not valid. Reinstall or run the

adapter diagnostics.

"IRQ chosen for driver does not match adapter configuration"

The driver configuration does not match the jumper

configuration on the adapter. Change the jumpers on the

adapter to match the RAM size configuration.

"RAM size chosen for driver does not match adapter

configuration"

The driver configuration does not match the RAM size

selected by jumpers on the adapter. Change the jumpers

on the adapter to match the RAM size configuration.

"Adapter hardware failure detected (self test)"

The adapter did not pass the initialization self-test

procedure due to a hardware problem. Check the adapter

for configuration conflicts.

"Cannot access adapter RAM properly"

The adapter could not read from or write to the adapter

RAM. Verify that another adapter does not share the same

memory space as the LAN adapter.

"Adapter RAM conflicts with other system RAM"

Adapter RAM must be reconfigured into another area of

the System Memory Map. Then reinstall.

"Adapter ROM conflicts with other system ROM"

Adapter ROM must be reconfigured into another area of

the System Memory Map. Then reinstall.

"Adapter hardware failure during initialization"

The adapter has not passed the initialization procedure.

Run the adapter diagnostics and check for proper

configuration.

"Configured RAM size invalid"

Configured RAM size does not match the physical RAM size.

SMC and Standard Microsystems are registered trademarks; and EtherCard

Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite 16C Ultra,

Elite Series, EliteView, EZStart, SuperDisk, and UltraChip are

trademarks of Standard Microsystems Corporation.  Other products and

company names are registered trademarks or trademarks of their

respective holders.