Connect�FACSys

==============

��������

�����   ����  ������

��   �����        ���

��� ��� ����    ����

���      ������

���           ���� ���          ���  ���

��� ��� ���  ����� ��� ��� ���  ��� �����  ����

��� ���� ���  ���  ��� ���� ��� ���  ���  ������

��� ���  ���  ���  ��� ���  ��� ���  ���� ������

T   E   C   H   N   O   L   O   G   I   E   S

11433 Cronridge Drive, Suite H

Owings Mills, MD 21117

800-678-1097 (within U.S.A.)

+1-410-363-1097

FAX: +1-410-363-0846

MHS: Sales @ Infinite

CompuServe: >MHS:Sales @ Infinite (or 73270,405)

Internet: Sales@Infinite.MHS.CompuServe.com

Technical Support provided on the CompuServe Information Service in

NVENA Section 4 (GO INFINITE)

Introduction to Connect�FACSys

==============================

Product Overview

----------------

Facsimile (Fax) is an increasingly popular method of transmitting

images and text to all points of the world via standard telephone

lines. It has become an indispensable business tool in almost

every office.  The Connect�FACSys product offers a

comprehensive feature set, integration with facsimile hardware,

Microsoft Windows support, full featured Hewlett Packard

emulation, support for the DCA/Intel CAS standard, and MHS

and NetWare compatibility.  Together these features provide you

with a powerful fax solution.

Connect�FACSys creates a fax gateway using a dedicated PC/AT

or greater. All users on a Novell LAN can have access to the fax

system and simultaneously send and receive transmissions

through one gateway.

Connect�FACSys provides excellent output quality, an easy to

use DOS or Windows interface, and compatibility with Novell

print services. Inbound fax transmissions can automatically print

to any Hewlett Packard (HP) LaserJet or compatible printer using

the Autoprint function.  Alternatively, inbound faxes can be

directed to any Windows graphic compatible printer from a

Menu.

Connect�FACSys was designed around the DCA/Intel

Communicating Application Specification (CAS) developed for

Intel facsimile products. CAS is a transfer agent which provides a

standard for facsimile communications between standard

facsimile equipment and personal computer systems.

Connect�FACSys offers many features that allow you to optimize

your inbound and outbound fax environment.  These include:

* automatic or manual routing of incoming facsimile transmissions

* ability to affix your company logo/letterhead to your outgoing

facsimiles

* attach your actual signature to an outgoing facsimile

* e-mail users can send a message with attachments through any

application that supports NetWare MHS or Connect�.

* personal phonebook

* group broadcasting

* delayed/scheduled transmission

* user/administrative security

* system logging

* management reporting

Connect�FACSys in a DOS Environment

===================================

Connect�FACSys provides you with several ways to work with

your outgoing and incoming faxes.  You can send your files

directly from applications like Lotus and Word Perfect, or, you

can send files directly from DOS.  Simply use one of the

Terminate and Stay Resident (TSR) programs, or use the DOS

command line utilities. You may also send files from a User

Interface similar to that of Windows.

Connect�FACSys in a Windows Environment

=======================================

You may operate Connect�FACSys from a Windows 3.1

environment.  When using Connect�FACSys from within

Windows you can take advantage of all the features that the

environment offers. For instance, several applications under

Windows can be run at one time, allowing you to switch easily

between them for an integrated work environment. With

Windows, you can transfer data between Connect�FACSys and

other standard DOS applications as well as other Windows

applications.

Pre-Installation

================

Included with this package are the diskettes containing all of the

required program and data files for the operation of

Connect�FACSys. The CAS software is also included.

System Requirements:

-------------------

* Connect�FACSys requires an IBM PC/AT or compatible

computer with a 286 processor or greater.

* 640 KB of RAM

* Recommended expanded memory is 1MB EMS for the

Connect�FACSys server

* 1 floppy disk drive (High density)

* DOS 3.1 or higher

* An Intel, Hayes, or Ferrari facsimile board, or other

supported fax board

* A Network Interface Card

* Novell NetWare Operating System 2.1x or higher

* IPX V3.10 or higher

* NETX V3.26 or higher

* RJ11 Telephone Jack/Wall Outlet

Optional Requirements:

----------------------

* An MHS-compatible messaging engine such as: NetWare

MHS, NetWare Global MHS, or Infinite Technologies

Connect� if you want to use Connect�FACSys with an e-

mail program.

* Microsoft Windows Version 3.1, if you will be using the

Windows interface.

* Eicon PostScript board for routing inbound faxes to

postscript printers.

Preparing the FAX Server

------------------------

CONFIG.SYS

----------

Set both Files and Buffers to 40.  Standard memory statements

are acceptable, but delete any references to Windows software,

such as stacks.  Delete any settings that were previously entered

to support your fax cards, unless you have an Intel CCP board.

AUTOEXEC.BAT

------------

Load your network drivers, and login.  Path and prompt

statements are acceptable. Delete any references to Windows

software such as "smartdrv.exe" and any settings that were

previously entered to support fax cards.

Network Drivers

---------------

Check your network drivers.  Connect�FACSys requires the

following minimum version drivers:

IPX/SPX         3.10

NETX            3.26a

LSL             2.0

IPXODI          2.0

If you are unsure of your version numbers, run the NVER

command at the fax server.

Note: For NetWare 3.12 or 4.x users only.  The Connect�FACSys Fax

Server station will not run with NetWare's VLM.  You can use any

of the drivers listed above.  However, the VLM can be used at any

workstation that is sending or receiving a fax.

SHELL.CFG

---------

Create a shell.cfg file in the same directory where your IPX

driver is located; the file should contain "showdots=on".

README.TXT

----------

Please check the readme.txt file which comes on your

Connect�FACSys diskettes for any last minute information

regarding the product that is not included in the documentation.

FAX Board Setup

===============

The following information is designed to provide you with

additional information on configuring Connect�FACSys with

particular fax boards.  If your fax board is not listed in this

section, proceed to the Installation.

Brooktrout

----------

With multiple card installations, the DMA channel selection can

be the same or different channels, but the interrupt selections

must be different for each card.

Intel

-----

Note for users of the Intel SatisFAXtion board - If you are installing

an Intel SatisFAXtion board, during the installation we recommend

typing install -X NIC I/O address (where the I/O address is that of the

NIC if the NIC uses an I/O address).

1. Perform the I/O address setup with the Intel software; avoid

I/O address 310.

Note:  if setting up multiple fax cards you must perform the address

selection with the Intel software.  Only one card can be installed at

a time in the fax server during this process.  The address selection

for Intel NIC cards uses the same Intel software.  If you use an Intel

NIC, remove it from the fax server during the address setup.

2. If you have an Intel CCP, our install program will copy a

configuration file to your boot disk and update your

config.sys at the Connect�FACSys Fax Server.

Ferrari Fax

-----------

You are allowed a maximum of 4 Ferrari Fax boards on a fax

server.When setting up your I/O address, switches 3 through 6

on all boards must have the same settings.  Use this table for

switch settings 1 and 2:

Board   Switch  1       Switch  2

1       on              on

2       on              off

3       off             on

4       off             off

Install all 4 boards in the Connect�FACSys Fax Server.

Gamma Board

-----------

Follow these steps to install the Gamma Board.

1. Install the gamma board software on the network using the

software package provided with the gamma board.

2. The installation creates an environment variable called

GFAX=path. This is the path pointing to the directory

where GammaFax is installed.

3. In that directory you will find a file named GFAX.$DC

(basic ASCII file).

4. Append/Edit to that file the following line:

GFXFORM # 3

Where # = the board number.  For multiple boards, you

should follow the syntax:

GFXFORM 1 3 on the first line

GFXFORM 2 3 on the second line.

GFXFORM 3 3...

GFXFORM 8 3 (maximum)

GFXFORM # 3 on the last line with # being the total # of boards

(maximum is 8).

5. Run GFQM.BAT from the GammaFax directory.

6. Complete your Connect�FACSys installation.  Then, change

to the \FAXLINK directory (the directory where you

installed Connect�FACSys) and run casdrv.exe and then

faxlink.exe

Installation

============

Please review the list of System Requirements and the pre-

installation notes in the previous section before beginning your

installation.  This installation procedure requires you to be

familiar with NetWare and the MS-DOS operating system.

Overview

--------

For a successful installation, you must login to the network as the

system SUPERVISOR since changes will be made to the

NetWare system files.  Also, you must install the software from

the workstation that will operate as your Connect�FACSys

server.

First, run the install program included with your

Connect�FACSys product.  After you've run this program,

complete any additional installation procedures necessary for

your system, and your installation will be complete.

Users of Ferrari, GammaFax, Brooktrout and Intel products

should consult the previous section of the manual for information

on installing these devices before proceeding with the installation

process.

Installing the Connect�FACSys Server

------------------------------------

You must install Connect�FACSys from the workstation that will

operate as your Connect�FACSys server.  To start the

installation, place the Connect�FACSys diskette labeled

"Installation and System Disk 1 of 2" in Drive A:.  Change to

the A: drive, and type INSTALL.

Note for users of the Intel SatisFAXtion board: If you are installing an

Intel SatisFAXtion board, during the installation we recommend typing

install -X NIC I/O address (where the I/O address is that of the NIC if

the NIC uses an I/O address).

When the Welcome screen appears, press ENTER; and a menu

screen will be displayed.

Highlight the desired selection using the up and down arrow keys

and press Enter.  After completing an option you will return to

this menu.  For a first time installation you must go through the

menu in order from top to bottom.  The initial installation

process will lead you through each option sequentially.  Later,

you may change any aspect of your installation by selecting an

individual option.

1. Device Selection

When you select Device Selection, a will be presented from which

to select the board and the number of devices.

You will be able to select only one board type, (i.e.: Intel

SatisFAXtion or Class II). You select the board by highlighting it

using the UP/DOWN ARROW keys and pressing ENTER.

2. Install Connect�FACSys

A screen will be displayed, asking where to copy the Connect�FACSys

Server and DOS Application files.

Enter the drive and path where you wish to install the files.  By

default, the Server's files will be placed in the FAXLINK

directory and the DOS application files in the FACSYS directory.

You may specify different directories during the installation.  The

install will create the directories if they do not exist.  Avoid

using NetWare Map Roots.  (Note: Windows Application files are

installed after the DOS application is installed.  See "Install

Windows Application Files" at the end of this section)

Printer Emulation Options

-------------------------

This screen also displays the options for the type of printer

emulation you will use.

Connect�FACSys supports PCL5 and/or Postscript emulation only if

you have an Eicon Script card installed in your Connect�FACSys

Server.  Otherwise, PCL4 emulation (HP Laserjet II) is

provided.  If you have an Eicon card, you must select the

Options button.  In the menu select either PCL or Postscript.

You can choose one type of printer emulation per fax server.  In

an environment with multiple fax servers, each fax server can

have its own printer emulation type. (Refer to the section called

"Preferred Fax Server" for choosing how to route faxes to a

particular fax server.)

If you are interested in additional information regarding the

Eicon Script board contact Eicon Technologies at 514-631-2592.

For additional information on the PCL4 emulation included, refer

to the Technical Reference.

Installation Activities

-----------------------

The installation will then automatically do the following; (DO

NOT DO THIS MANUALLY!)

1. Copy all Connect�FACSys Server and DOS Application files

to the network.

2. Create a NetWare print queue named PRINTQ_FAX. Note:

Do not change the name of the print queue.

3. Grant basic rights to the group EVERYONE for the basic

operation of Connect�FACSys.

4. Create two groups named FAX_ADMIN and FAX_ROUTER. Any

users added to the group FAX_ADMIN will have supervisory

access to Connect�FACSys.  The installation will make the

supervisor and the user who is installing Connect�FACSys a

member of this group. Members of the group FAX_ROUTER can

view the 1st page of all unrouted faxes.

3. Configure Devices

Select Number of Devices

------------------------

For each board, you will need to select the number of devices by

pressing the Add button.  With some boards, the number of

devices is self-detected.

Set up each device by highlighting that particular device and

pressing the Setup button.  The installation routine will provide

you with a setup screen for each device.

Setup

-----

CSID            The CSID for the board can be a fax number where you

plan to receive faxes, or your company name.  This

information is displayed in the header of each outgoing

fax.  The size of the entry is limited to 20 characters.

Board on-line   Select if the board is on-line or not.

On-Line         Enable both send and receive.  The configuration can be

changed later.

Settings        Set the I/O address, IRQ or COM port, etc.  Note: You

cannot change the I/O address on an Intel board from this

installation.  You must use the Intel software to do so.

Save            When you've finished setting up all the selected boards,

save the configuration by selecting the Close button

(twice).  Answer yes to save the configuration.

Special Note for the Intel SatisFAXtion 400: There are two types of

Intel Satisfaxtion 400 boards.  If you have the new fax card, you

must copy the download.402 and loader.402 to the FAXLINK

directory with the filenames of download.ifp and loader.ifp

respectively.  Retrieve these two files from you Intel software.  If

you are unsure which card you have check the entry panel.  The old

card has a round scanner port on the I/O entry panel, the new one

does not.

If you selected the Intel CCP, you will have to download the

drivers to your boot disk and update the config.sys file.

Connection CoProcessor Board Users

----------------------------------

Check to make sure you have created a config.sys file on the fax

server's boot disk.

Class II Devices

----------------

For Class II devices, the following information is provided.

Option          Description

------------    -------------------------------------------------------

Quad 'A'        This option was created for Australian modems that have

Prefix          a synchronization problem.

Left to Right   You may need to enable this option if you're using a

Bit             Rockwell modem.

Scan Line 32    Needed for Australian modems that fail to supply the

necessary filler bytes for slow fax machines.

Enable          This option was implemented for Class I devices in slow

Underrun        network environments.  Do not use with Class II devices.

Rec. begin      Normal setting is DC2.  Some original Class II devices

char            may operate with DC1.

4. Driver Environment

Select Driver Environment; and update the screen with the following

information.  The installation routine modifies the configuration

file where all the driver configurations are stored.

Company Name    Name displayed in the FROM: field on all faxes if a

user does not have a User Full Name established in

the NetWare bindery.  You can manually change this

when sending a fax from within Windows.

Queue Size      Maximum number of entries in ALL queues

combined.  Also determines how much disk space

will be used storing faxes.  The maximum number of

entries is 32000, while the default setting is 200.

Answer on Ring  Number of rings the board will accept before

answering.

Dial Retries    Number of attempts a board will make to send a fax if

the fax fails on the first try.

Retry Interval  Amount of time, in seconds, between the retries.

Dial Timeout    The amount of time, in seconds, used to hold the line

active with a ring.  The timer starts upon detection of

the dial tone.

Dial Prefix     Prefix you will optionally use when dialing out, such

as with a PBX.  Include a comma(s) when pause time

is required.  Refer to the Driver Environment section

for delay time selections.

Dial Mode       Mode of dialing

Options are: Tone or Pulse

Speaker Mode    Options are: Always On, On Until Carrier, or

Always Off

Speaker Action  Options are: No Tone, Tone on Error, Tone After

Every Event.

The Layout option may be selected, and additional items can be

defined on the page layout screen.

Default logo    Enter the name of your logo file for use when the

cover page option is enabled.  If you don't have a

logo file, enter LOGO.PCX.  Refer to the instructions

on creating a logo later in this manual.  Note:  An

error message will occur when sending a fax with a

cover page if you do not have a logo file.

Lines per page  Enter the number of lines per page

Fill Page...    Instructs the fax board to send a full page length for

each page, regardless of whether the page is filled

with text.

Pause Line for  Select the number of seconds (2 or 6) to wait when a

Comma           comma is encountered during dialing.

Time format     Choose the U.S. standard or military time as you

want it to appear on all outgoing faxes.

Date format     Choose the date format for all outgoing faxes.

Header format   Select None, Partial, or Full

5. Server Environment

After you choose Server Environment, you may update the following

information:

Auto Logout     When enabled, automatically logs the Fax

Server off the network when the down

command is issued.  This is a recommended

security feature.

Disable Device  When you down the fax server, you prevent the

on Exit         fax server from storing received faxes on the

file server while the fax card or modem is still

enabled to receive.  Enabling this option with a

yes disables the fax card or modem.

User Notify     When enabled, informs the fax originator of the

success or failure of their fax transmission.  If

the user is running CASTOFF or Windows and

not loading NWPOPUP in the WIN.INI file,

the user will not receive a notification.

Use Expanded    Defines whether or not EMS will be used at the

Memory          fax server for the PCL4 conversion.  This

feature increases fax server performance.

Default PRINTQ  Enter the name of the NetWare print queue to

to process      process.

Print Option

------------

You may also select the Print button to configure the printing of

incoming faxes.  The following information should be entered:

Enable Auto     You will select the printer type, form type, and

Print...        identify the file server and print queue that

drives your selected printer.

Print Path      Enter "File Server/Print Queue".  This is the

print queue that drives your selected printer.

Form Type       Select Letter, Legal, or A4.

Printer         HP-PCL (Hewlett Packard Laser printer or

Emulation       compatible), Postscript, or Epson or Epson

compatible.

NOTE: An Epson or compatible printer must

be a 24 pin printer.

Print           Select the print resolution; Draft or Quality.

Resolution

Receive         Send a NetWare broadcast message to the

Notification    selected user's login_name.

Enable Time     Prints the time of receipt on the lower right

Stamp           corner of each page.

E-mail Option

-------------

You may choose to integrate Connect�FACSys with MHS

electronic mail.  To do so, select the Email option; and update the

following information:

Enable E-Mail   Indicates to enable integration of MHS or

Services        Connect�-based electronic mail.

MHS Host Name   Enter the name of the MHS host with which the

Connect�FACSys server will communicate.

Fax Gateway     Enter the name that has been assigned to the

Name            Connect�FACSys gateway on the MHS host.

Receive         Enter the name of the MHS e-mail user to

Notification    receive notification of faxes received.

Short cover     Indicates to use a short cover page containing

page            fax header information, instead of the full cover

page which includes message text.

Local Delivery  Indicates whether to deliver incoming faxes

directly to the E-Mail application or to MHS or

Connect� for delivery.

E-Mail          Enter the name of the Preferred Application of

Application     the E-Mail product you are using.

OCR Option

----------

You may choose to use Optical Character Recognition as a means

of routing incoming faxes.  To do so, select the OCR option; and

update the following information:

Enable OCR for  Enables OCR.

Received Faxes

OCR Timeout     Indicate the amount of time that the OCR

engine should search for the routing code and

delimiters.

The FAXLINK.INI file will be modified through your choices.

6. Product Licensing

Your Connect�FACSys product must be registered.  If you do not

have a completed  registration form you can register the product

temporarily for 60 days (demo products for 30 days).  Your

product will cease to operate after 60 days if you do not get a

registration code from Infinite Technologies and register

Connect�FACSys.

1. Select the Product Registration, and select the file server

that you want to be able to fax from by highlighting the

appropriate server and press Enter.

2. You will be prompted with the question, "Do you have a

completed registration form?".  If you have the completed

form, answer Yes and enter the registration number.  If you

answer No, the product will have a temporary registration for

60 days (30 days if a demo copy).

The product can be registered any time during the 60 day period.

The product will no longer function after the 60 day period if you

fail to permanently register the product!  To register your

Connect�FACSys product, contact  Infinite Technologies at 410-

363-1097.

Note:  The product registration number is generated from your font

license number and your NetWare serial number.  If you plan to

upgrade NetWare you will receive a new NetWare serial number

from Novell.   In order for Connect�FACSys to work with the new

release of NetWare, you will have to provide Infinite Technologies

with a copy of your Novell upgrade form that states the new serial

number so we can generate a new registration number.  To avoid

interruption of your faxing capabilities, it is recommended that you

retrieve your new number before updating your NetWare software.

Windows Applications File

-------------------------

Insert the diskette labeled "Windows Client Applications

Diskette" in Drive A and load Windows.  Then, run A:SETUP.

Additional Installation Procedures

----------------------------------

This is a list of some procedures which must be configured

manually by the installer before Connect�FACSys can be run

successfully.  Be sure you have run the Install program before

completing this list.

Installation - Other Procedures

-------------------------------

Use your NetWare PrintCon Utility to set a print job for BYTE

STREAM.  If  you do not do this, users will incur errors when

attempting to print faxes through the PCL print services.  You

must then copy this print job to all users.

Check that you have successfully completed the Install program

included with Connect�FACSys and that Base I/O and DMA

parameters have been set correctly.

Initialize the Fax Server

-------------------------

You're ready to initialize and test your fax server.  You must be

residing in the directory where you have installed the

Connect�FACSys server program files.

Type:

CASDRV.EXE <enter>

FAXLINK.EXE <enter>

MONITOR <enter>

The screen will display two status windows.  When the colon

prompt appears in the left window, the fax server is operational.

Additional Setup

----------------

From SYSCON:

1. Enter SYSCON and create a Login_name for the

Connect�FACSys Fax Server such as FAX.  Include this

Login_name in the group called FAX_ADMIN.  In the fax

server's login script include a CASTOFF statement.

2. During the Connect�FACSys installation, two groups called

FAX_ADMIN and FAX_ROUTER were created.  Members

of the FAX_ADMIN group can perform all the supervisory

functions of the fax server.  The members of the

FAX_ROUTER group can view the first page of all unrouted

faxes and route them to the appropriate network user.  You

may want to add users to these groups to perform these

administrative functions.

Phonebooks

----------

We recommend that all phonebooks be placed in the

FAXLINK\USER directory.  The product installation gave all

users read, write, create and erase rights to this subdirectory.

The recommended naming convention is Login_name.pbf.  This

naming convention is useful if you plan to create batch files to

load the Connect�FACSys TSR.

Example:

FACSTSR /P=F:\FAXLINK\USER\%USER%.PBF

(%USER% is a defined variable in the system login script)

Search Drive

------------

It's recommeneded to map a search drive to the FACSYS

directory in the system login script if the users will be running

Connect�FACSys executable files.

Autoexec.bat

------------

If you have a local hard drive in your Connect�FACSys server,

we recommend that you include the following statement in your

autoexec.bat file:  SET CASTMP=C:\.  This temporary storage

is used when network traffic is very heavy.

Logo File

---------

Be sure to create a logo file using the FAXLOGO logo creation

utility described later in this manual.  If you send a fax with a

cover page, the fax server looks for a logo file in the FAXLINK

directory.  If you don't have a logo in the directory, an error

message will result when sending a cover page with your fax.

Note: Each user can create their own personal logo file.  The file

name must be login_name.pcx and it must be stored in the

FAXLINK\USER directory.

Basic Testing from a Workstation

================================

1. At a workstation, setup a search drive for the FACSYS

directory:

Example: MAP S3: = fileserver_name/volume:FACSys

2. Send a simple fax, such as your autoexec.bat file, using the

faxsend utility from the DOS prompt.

Type: FAXSEND -F=C:\autoexec.bat -T = [fax machine phone #]

Test File Transmit and Receive

------------------------------

1. Change directory to FAXLINK\USER

2. Type FAX at the DOS prompt, and press <Enter> to clear

the welcoming message.

Create a Phonebook

------------------

1. Press <ALT>-P, New

2. The name of the phonebook should be login_name.pbf

3. Press <ALT>-P, Person

4. Press <ALT>-A (Add), and fill out the name and fax

number for a fax machine that is in your office.  Enter the

number in the fax information field.

5. Enter OK.  Then press the <Escape> key to exit the

phonebook.

Note: It is recommended that all phonebooks be created and stored in the

FAXLINK\USER directory since the installation sets up the

appropriate accesss rights for this directory.  Also, there is a limit of

65,000 entries per phonebook.

Establish Start-up Options

--------------------------

1. Select Start-up options by entering ALT-M, O.

2. Check Unread Faxes at Startup: If you  enable this option, it

will search the queue directory for faxes that are routed to the

individual.  You will save time in accessing the front end if

you do not enable this function.

3. Show Only Personal Faxes in Log Queue:  members of

FAX_ADMIN or fax_router should not check the box

4. Enter F:\FAXLINK\USER\LOGIN_NAME.PBF in the

default phonebook section.

Fax your Test File to Your Fax Machine

--------------------------------------

1. Enter File Transmit by Pressing ALT-F, T.

2. Press <Tab> and the Space Bar to highlight the

appropriate text file in the Files List Box.

3. Use the down arrow key to select [c:] and Press <Enter> to

select the C drive as your current path.

4. Press the Space Bar again, use the arrow key to select the

autoexec.bat file and press enter.  The file name should

appear in the File List Box.

5. Select the phonebook entry by Pressing ALT-P

6. Press the Space Bar and highlight your name and Press

<Enter>.

7. Press <Enter> again to send immediately.

Monitor Events

--------------

Press ALT-M, Supervisory Options, Monitor events.

This is a real time display of your fax lines.  Upon entry you will

see dial, send and/or idle for current line status.

Send a Fax to Connect�FACSys

----------------------------

1. Fax a blank copy of your corporate letter head to the fax

server.  This will be used later to generate a logo file for your

Connect�FACSys server.

2. Enter FAX at the DOS prompt and Press ALT-F, R.  The

screen will display information regarding the received fax.

3. Highlight the job by Pressing the Space Bar, then Press

ALT-V to view the fax.

4. Verify that your fax was received successfully.  Then Press

<Esc> to return to the File Receive menu.

5. Press <Esc> to exit File Receive.

Test The PCL Conversion Of The Fax Server

-----------------------------------------

This is done by loading the FACSTSR. Enable capturing to

Printq_fax and perform a print job.

1. Type FACSTSR /P=F:\FAXLINK\USER\LOGIN_NAME.PBF at a

DOS prompt and press <Enter>.

2. Press the Hot Keys (ALT-ESC) to bring up the DOS

Redirector.

3. Find "capture: prn" on the bottom right part of the screen.

Press <F3> to change the capture to fax; then press

<Esc>.

4. At the DOS prompt, type Capture Sh and press <Enter>.

Your capture statement should show that LPT1 is captured to

Printq_fax.  The timeout value should be at least 5 seconds.

5. Press the Print Screen key; the DOS Redirector will pop up.

6. Highlight your fax machine from the Phonebook list and

press <F1> to Add, and <F10> to Send.

7. Your fax server will display a message indicating that it has

received a print job from Printq_fax.

8. After the fax server converts the job, it will be sent to your

fax machine.

Send A Fax From Wordperfect 5.1

-------------------------------

If you use DOS applications, the procedures to send a fax are

basically the same as the procedure above for testing the PCL

Conversion.  The DOS Redirector will appear when you perform

the keystrokes for printing the document.

1. While the TSR is loaded, enter WordPerfect.  While in

WordPerfect, you can press the hot keys to verify that your

capture is still set to fax.

2. Create a document for test purposes, or open an existing

document.

3. Select WordPerfect Print (Shift-F7).

4. Press "S" to select an HP II laser print driver. Using other

print drivers will generate PCL conversion errors at the fax

server.  If you want to fax using the HP III or Postscript

driver you need the Eicon Script card in your

Connect�FACSys Fax Server.

5. Press "3" to Edit; Select LPT1 or Printq_fax as your port

selection.

6. Press <Enter> until you return to the initial screen and

press 1 to print the full document.

7. The DOS Redirector will pop up.

8. Highlight your destination from the phonebook and Press

<F1> to Select, and Press <F10> to Send.

This completes the basic testing of Connect�FACSys.

If you have are having problems with your installation, read

through the installation instructions again, attempt all corrections

and review the Error Log in the Technical Reference section.  If

you continue to have problems, you may contact our Technical

Support Department.

Phone:          410-363-1097

FAX:            410-363-0846

MHS:            Support @ Infinite (via NHUB or CompuServe)

Internet:       Support @ Infinite.ihub.com

IMPORTANT! Please fill out the product license form included

with the application software and mail to Infinite Technologies

for permanent product registration.

Customize your Fax Server

=========================

Connect�FACSys Server Initialization File

Configuration information for the Connect�FACSys server is

stored in the FAXLINK.INI file. Within this file you can install

soft fonts and cartridge fonts, select your memory configurations,

and control network functions. You can use any standard text

editor to edit this file.

The following table shows each section of the FAXLINK.INI

file, and the options available in each section.

FAXLINK.INI Table

-----------------

Keyword         Description

[Font]          Font Section

Use for loading HP downloadable and cartridge

fonts.  Before doing this, you should consult your

HP LaserJet manual and be familiar with this

process.  You should know the difference

between cartridge fonts and downloadable soft-

fonts.  You cannot access cartridge fonts and

load them into your PC.  You can, however,

program soft-fonts into the server's memory and

they then act as cartridge fonts.

Connect�FACSys comes pre-configured with

several internal fonts representing Courier,

Times Roman, Helvetica, and Line Printer

typefaces.  To pre-load your fonts (using your

fonts as an HP font cartridge), you must insert

the font files into FAXLINK.INI.  Each line

within this section must contain the valid font file

name, the alternate pre-processed file name, and

an assigned font ID for loading.

Example:

cr120rpn.pcp=cr120rpn.pzp, 10000

In the above example, the Courier 12 point font

(cr120rpn.pcp) is processed into the

cr120rpn.pzp file and assigned the font ID value

of 10000.  Each font loaded must have a unique

ID number assigned.

[Panel]         Panel Section

Symbol Set      The default symbol set may be controlled by this

parameter. Valid arguments are Roman8,

ASCII, and PC8.

PrimaryFont     When loading fonts within the [fonts] section,

this field overrides the default font upon

downloading. This is similar to the HP's control

panel feature. If no font id is given, (i.e.,

10000), the system will assume that the first font

downloaded within the [fonts] list is the primary

font.

AutoReset       This controls the autoreset feature of the PCL

processor after every print job. You can select

either YES or NO as a valid argument.

Form            Optional forms/paper sizes may be selected when

using the PCL conversion processor. Valid

arguments are Legal, Letter, Auto, and A4.

PageLength      This controls the lines printed on a page. This

normally applies to text only.

Copies          To control the number of output copies, you can

select a value of 1 through 99.

Orientation     To control the default orientation, you can select

either Portrait or Landscape. When using

Landscape, make sure that Landscape fonts are

downloaded.

[Engine]        Engine Control

Section

Expanded Memory During processing, the PCL conversion

processor uses the EMS memory for image

construction if YES is set. The processor's

conversion speed will be significantly increased

using EMS.

InitMemory      Some EMS drivers require memory initialization

before use. If this field is set to YES, the

processor will zero all EMS memory prior to

startup. This feature normally executes within

one or two seconds and is recommended for use.

DotsPerInch     This field requires a horizontal and vertical DPI

resolution for which the PCL processor will

produce an output. Normal facsimile resolutions

are 200 X 200.

SendMode        This field is used to control the sending

resolution for any document created by the

LaserJet emulator for transmission. Valid

arguments are either Fine or Standard.

Fill Page       This feature will force the output of the PCL

Processor to 11 inches regardless of the actual

output size. This option may be set to YES or

NO.

Display         This option gives you the ability to report

Warnings        warning messages to the system console during

PCL processing. You may set this to YES or

NO.

DisplayErrors   This option is similar to the warning messages,

but will only report severe error messages that

usually produce corrupted faxes. You may set

this to YES or NO. We recommend that this

feature remain enabled since fatal errors should

be reported.

[FACSYS]        Connect�FACSys Server Control

Device          The fax board being used.

Note: When selecting the line routing [routing]

options, the Auto Print, Print Quality, Print

Queue, and Print Time [Stamp] options are

controlled by the options available in the

[routing] section of the FAXLINK.INI. file

(included in this chapter).

AutoPrint       The Connect�FACSys server is capable of

automatically printing received facsimiles to a

network printer. Set YES to enable or NO to

disable this feature. If you select YES, make

sure that your target print queue is properly

configured.

PrintQuality    This option controls the print quality for in-

coming facsimile data. Enter HIGH or LOW as

a valid argument. When selecting HIGH, the

print resolution is set at 300 dpi. When selecting

LOW the print resolution is 150 dpi which gives

you more speed because of reduced amounts of

data sent to your network print queue.

PrintQueue      If you select this feature, the facsimile image will

be automatically sent to the appropriate print

queue. You can select the server name and

queue.

Print Time      This option permits you to print an optional time

stamp on each fax page received in the

lower/right hand corner. Set to YES to enable

this feature.

UserNotify      Setting UserNotify to YES allows a document

sender to receive a notification that the

transmission succeeded or failed. Detailed errors

can be observed within the Connect�FACSys Log

Queue.

AnswerOnRing    The facsimile hardware is capable of waiting for

a specific number of rings before answering the

phone line. You can set 1 through 255 rings

before pickup. Setting 0 disables in-coming line

traffic.

ForceLogout     This is an important security feature. It is

recommended that this be set to YES. The

Connect�FACSys server will automatically

logout from all servers when the DOWN

command is issued from the console.

OCR             This option enables the Optical Character

Recognition (OCR) for receiving facsimiles. The

OCR scanning decodes preset routing codes. This

process utilizes the OCR routing table.

OCRTimeout      This value tells the scanning process to abort the

search for the routing code after the specified

number of seconds. Use 0 to disable this

feature.

DOS IDLE        Gives TSRs and drivers more time to run during

idle time. Can be 0-1000, 0 is disable, 1-1000 is

frequency, 1 being the highest.  This should be

set to 1 if you are using a Class I or II device,

and set to 50 for all other fax devices.

E-Mail          Set this option to YES, to enable the MHS

processing module. Message Handing Service

(MHS) out-bound facsimiles are supported by

Connect�FACSys from your e-mail application.

When using this feature, you must configure

your MHS Connectivity Manager correctly to

make certain that Connect�FACSys has been set

as a valid gate for out-going facsimiles. Refer to

your MHS manual for further details.

[e-mail]        Configure Connect�FACSys with e-mail

Short Cover     Defines length of cover page.

Page

Print Notify    Provides Fax print job status via MHS e-mail

using NOTIFY files. (see MHS Notification in

index)

MHS Host        The MHS Host name used with your

Connect�FACSys server.

MHSGateway      The Gateway name defined under Novell's

MHS. Default is Connect�FACSys.

AppName         E-mail application to receive send and receive

notifications.

Local Delivery  This option, when set to YES, enables local

delivery of Faxes directly to the user's mailbox.

[settings]      Settings Section

Default         The default setting for the device(s) as defined

during the installation of Connect�FACSys. We

recommend that you choose the options of SEND

RECEIVE during installation (which can be

modified at any time by re-running install with

the fax server down), and modify these

parameters through the FAXLINK.INI file.

Line            Sets the function of Send and Receive, Send

Only, Receive Only, or Disable for each line.

Example: default = SR

The default for all devices is Send and Receive.

All available devices will be capable of sending

and receiving faxes unless a particular device is

specified otherwise as with the following

examples:

Example: line0 = SO

The first fax line is changed from Send and

Receive to Send Only.

Example: line1 = RO

The second fax line is changed from Send and

Receive to Receive Only.

[routing]       Routing Section

Default         The default setting used to define the inbound

printing and notification functions and

specifications. You should note that setting

individual routing options such as DID, DTMF,

or OCR overrides the notification and printing

definition of these settings, except that the printer

type defined (i.e. PCL) remains constant.

Example:

default=1,HPPCL,INFINITE/PRINTQ_1, AUTO,1,1,EVERYONE

Example:

line0=0,HPPCL,INFINITE/PRINTQ_1,AUTO, 1,1,EVERYONE

Example:

line1=1,EPSON,INFINITE/PRINTQ_0,LETTER, 0,1,SALLY,JOHN,

BILL @ INFINITE

Use the following options, in order, as in the

above examples, line#=option,option, ...

Enable - (0=off, 1=on)

Print Driver Name - (HPPCL, EPSON, PSCRIPT)

Printer Path - (NetWare printer path for auto

print of received faxes)

Form - (LETTER, LEGAL, A4, AUTO) (Note: This is the

paper tray type)

Quality - (0=draft, 1=Quality)

Time Stamp - (0=off, 1=on)

Receive User Notify - (0=off, 1=on) (Note: The

above options are used with the HPPCL option

only)

[queues]        Queues Sections

N/A             The print queue(s) which will be attaching to the

fax server is/are defined here. No other print

servers should be servicing a queue attached to

the Connect�FACSys fax server. Also, only one

fax server per print queue is permitted, though

you may have multiple print queues servicing a

single fax server. Add the print queues to this

section below the one defined during the

installation of Connect�FACSys.

Configuring Inbound Routing

===========================

Connect�FACSys supports a number of inbound routing

mechanisms, all based on the routing code entered for the user.

The following types of routing are supported:

Manual Routing

--------------

Manual Routing is accomplished through the Connect�FACSys

front end:

1. Enter the Connect�FACSys front end and press <Enter> to

clear the welcoming message.

2. Press ALT-F R  (file / receive)

3. Press the Space Bar and use the arrow keys to highlight an

unrouted fax.

4. Press ALT-V to view the fax.

5. After you have determined who the fax should be routed to

press <Escape> to return to the Receive Facsimile screen.

6. With the fax highlighted, press ALT-R to route the fax.

7. A list of network users will appear from the bindery.  Select

the appropriate user and press <Enter>.

8. If you desire, you can send a network notification.

Direct Inline Dial  (DID)

-------------------------

This is the most accurate routing system for incoming faxes.  You

must purchase a bank of phone numbers and assign a number to

each person who will be receiving faxes.  Your PBX system will

also require a special setup to support this function.  The user's

routing ID must be the last four digits of the phone number.  The

only boards that we support for DID routing are Brooktrout and

Gammalink.

Dial Tone Modulated Frequency (DTMF)

------------------------------------

After the routing table has been setup, users can inform the people

who send them faxes and use DTMF of their routing ID.  DTMF

may only be employed from a fax machine.  The user sending the

user faxes will need to reference their fax machine's documentation

to learn how to use this feature.

Optical Character Recognition (OCR)

-----------------------------------

OCR will scan the header of inbound faxes for routing codes

enclosed in "(( ))".  This option can be enabled in the [FACSys]

section of the FAXLINK.INI by setting OCR=yes, and the

OCRTIMEOUT value between 90 and 160.  Refer to the

FAXLINK.INI section earlier in this section for additional

information on enabling OCR.

CSID Routing

------------

A CSID code or descriptor is used by most fax machines and fax

servers to identify the fax originator.  The CSID descriptor

typically appears on the first line of the first page of a fax.

CSID Routing allows for the routing of incoming faxes based on

the CSID received on the fax.  However, you must first create a

ROUTER.INI file in the FAXLINK directory that will contain the

CSID entries and the associated routing IDs which have been

entered for your users.  With CSID Routing, when a fax is

received, Connect�FACSys retrieves the CSID descriptor, then

scans the ROUTER.INI file and attempts to locate a matching

description.  Letters in a company's name are not case sensitive.

Once a match is found, the associated routing code is used for

delivery.

Example of the ROUTER.INI file:

908-271-9752=310

12139400223=308

Optus=311

Please note that CSID Routing is limited in that all faxes received

from a single CSID location are routed to a single routing ID.

Note: The fax number in the ROUTER.INI file must be exactly as

printed on the top of the sender's fax.  The number to the right of

the equals sign must be valid routing ID's.

To enable CSID Routing, in the [FACSys] section of the

FAXLINK.INI add "CSIroute=yes.

Hewlett Packard PCL Processor

=============================

The Hewlett Packard PCL processor, part of your

Connect�FACSys software package, is capable of converting any

HPPCL (Level 4) commands, text, fonts, and graphics into a fax

image of normal print output.

The extended set of commands to use the PCL processor are

listed below.

There are six special printer escape sequences supported by the

PCL processor. Each command is preceded by the ESC

character, followed by the LEFT SQUARE BRACKET ([), and,

finally, the command code (D, T, N, C, S, or P). The circumflex

(^) character may be used in place of the ESC character.

Escape          Description/Action

ESC[D           Sets the dialing number for the fax job. Any

numeric characters are permitted including the

dialing control prefixes. Up to 31 characters are

supported.

P - Pulse Dialing

T - Tone Dialing

,  -  Pause

ESC[T           The fax transmission may be scheduled using this

escape. The data should be formatted in the

following manner:

MM/DD/YY HH:MM

If this is omitted, the fax job will be scheduled

immediately in the sending queue.

ESC[N           This field sets the target name which will appear on

the top of the fax transmission header. If omitted,

the Connect�FACSys server will place the dialing

number selected. Up to 31 characters are supported.

ESC[C           This is the cover page control command which will

create a cover page for the fax transmission. You

may place up to 512 characters (including new-line

characters) within this escape. If omitted, no cover

page will be selected.

ESC] or Null    This is the command termination sequence.

ESC[S           This identifies the name of the person sending a Fax

print job.

ESC[P           This command is used to set the Fax priority from 0

through 255 (255 being the highest).

ESC[B           This field allows for the inclusion of the billing or

matter code.

When inserting these facsimile control commands within your

document, you typically represent the ESC with <27>.  If you

are entering multiple commands, you must be certain that a

continuation escape sequence (another <27>) separates the

commands.  The end of your facsimile control command or

commands must have a NULL character (<0>) to indicate the

end-of-command. The following example illustrates how to send

a single in-line facsimile control command.

Example: WordPerfect for DOS

<27>[D1-800-123-4567<0>

The following example uses two printer escape sequences to set

the dialing target and schedule a transmission time using the

continuation method.

Example: WordPerfect for DOS

<27>[D1-800-123-4567<27>[T06/01/91 10:10<0>

Note: Some applications do not permit the use of the<R>NULL

character. You may optionally terminate your command sequence

by using the RIGHT BRACKET ] as follows:

Example: WordPerfect for DOS

<27>[D1-800-123-4567<27>[T06/01/91 10:10<27>]

Installing with E-Mail

======================

Connect�FACSys supports NetWare MHS or compatible message

routers for popular e-mail packages such as BeyondMail, Da

Vinci eMAIL, ExpressIT!, Notework, and other SMF compatible

e-mail systems.  You may send and receive faxes directly with

your e-mail application through the Connect�FACSys gateway via

your  message router.

Note: For each application you must consult your e-mail installation and

user guide for the correct syntax to successfully transmit an e-mail

message through the MHS Gateway.

Gateway Installation

====================

To install Connect�FACSys as a Gateway, follow the instructions

in your NetWare MHS or compatible router manual for gateway

installation.

Define the gateway with a gateway name of FACSYS and a

description of FAX SERVER.

Note: Two of the Novell NetWare MHS v1.5 enhancements are multiple

recipients and multiple attachments.  Define the gateway for

Connect�FACSys under the Novell Netware MHS Directory as

SMF70 to take advantage of these features.  However,

Connect�FACSys v3.30 remains compatible with Novell Netware

MHS v1.1.

The fax server running Connect�FACSys will poll the FACSYS

directory. When a job is submitted, the Connect�FACSys server

places it in the SEND QUEUE for transmission.

Editing the FAXLINK.INI File

----------------------------

FAXLINK.EXE reads the faxlink.ini file when it starts. The

following parameters make the file specific to MHS:

[FACSys]

e-Mail=yes

[E-MAIL]

MHSGateway=             Enter the Connect�FACSys gateway name

defined under Novell MHS

DefaultRecipient=       Enter a local MHS user

AppName=                The e-mail application name where Send

and Receive Notifications are sent. Valid

entries are:

NW for Notework

MHS for Right Hand Man

DVSEMAIL for Da Vinci

POSTMAST for ExpressIT!

LocalDelivery=          Yes/No, depending on whether you want

Local Delivery of the received fax to the

App or to the MHS Connectivity Manager.

You must now set the MV environment variable so the

Connect�FACSys server knows where MHS is installed. You can

add the MV variable to the login script of the Connect�FACSys

server by using Syscon and adding DOS SET MV="server

name\volume:" in the login script.  Server name being the server

where MHS is installed, and volume being the volume where

MHS is installed.

After all the changes are made, you must reboot your fax server

to activate the changes.

Using E-Mail Message Routing

============================

Connect�FACSys provides a gateway for Novell NetWare MHS

(v1.1 and up) compatible e-mail software packages to send

facsimiles. E-mail applications that strictly adhere to the Standard

Message Format (SMF) can communicate with the gateway

defined for Connect�FACSys under the MHS or compatible

message router Directory Manager.

Sending Facsimiles via E-Mail

-----------------------------

An e-mail application sends a fax to a remote fax machine via

Connect�FACSys by sending a message with an extended address

to the Connect�FACSys gateway.  For example, Pat at Infinite

wants to send a facsimile to Mr. Joe Swift at 908-271-0698.  He

types:

Joe @ FACSys {FAX: 1-908-271-0698; TO: Joe Swift}

After sending the fax, Connect�FACSys delivers a Send

Notification to the e-mail message author.  This notifies the

message author of a successful or unsuccessful transmission.

This extended address is a standard MHS e-mail format; some

applications may have formats that differ from this example.

Refer to your e-mail manual for the format information you need.

A variety of additional extended address options are available.

These include:

Extended Address Options

------------------------

FAX             The number to be dialed.  Composed of digits and the

following characters:

-       separator

,       pause

!       hook flash (for special PBX functions)

P       pulse dialing

T       tone dialing

M       manual dialing

TO              The recipient's full name which may include a router

ID.

RESOLUTION      The desired resolution.  Options:

high    200 x 200 dpi

low     100 x 200 dpi

LOGO            The pcx logo file name.  The file must reside in the

FAXLINK directory and the full path and drive letter

must be specified.

TIME            The time to send the facsimile which follows SMF

syntax.  For example:

dd-mmm-yy hh:mm:ss

dd      day

mmm   month (Jan, Feb, Mar, etc.)

yy      year

hh      hour (24 hour time, e.g. 23:00 for

11:00 p.m.)

mm      minute

ss      second

TEXT            Specifies standard or compressed mode. Options:

80      standard

132     compressed

ROUTER ID       A unique nine digit number that identifies a particular

person.  The number must be enclosed in double left

and double right parentheses.  When the router id is

detected, a fax can be routed directly to a user's

mailbox.  Example: ((1234))

Based on the prior information, a Connect�FACSys user could

specify the following in an extended address:

Pat @ FACSys {FAX: 1-908-271-9572; TO: Robert Johnson ((123));

RESOLUTION: Low; LOGO: it.pcx; TIME: 1-Sep-94 23:00:00}

This address specifies the dial number, the recipient's full name,

resolution, a logofile name, and send time.

Receiving a Fax via E-Mail

--------------------------

Some e-mail applications have the capability to view a fax.  The

Connect�FACSys administrator may let Connect�FACSys route

incoming faxes to a specific application by enabling e-mail and

assigning an AppName in faxlink.ini.

Connect�FACSys can also route received faxes manually and

automatically.  It sends a Receive Notification to the e-mail user

associated with the identified router ID as set up in the

Connect�FACSys Supervisor Routing Table. For fast response,

local delivery is performed if the recipient is local.

FAXLINK.TXT

-----------

Faxlink.txt is a text file that is located in the FAXLINK

directory.  The contents of this file will appear on the bottom of

all EMAIL/MHS faxes.  This file may be changed to contain any

information the user wishes.  It is limited to 24 lines and must be

a TXT file.

cc:Mail Compatibility

---------------------

cc:Mail users who wish to run Connect�FACSys have to use the

cc:Mail Router.  Two cc:Mail Router Link types are used. We

describe cc:Mail separately as it is unique from standard MHS e-

mail packages.

1. MHSLINK: - cc:Mail MHSLINK is used if the cc:Mail

Router runs with the MHS Connectivity Manager to

communicate with MHS compatible applications.

a. Define a cc:Mail Router in NetWare MHS with the

name of the cc:Mail Post Office.

b. Define a Post Office in cc:Mail with the name of the

Connect�FACSys Gateway, with the address being

the path of the cc:Mail Router defined previously,

mhslink file_server\volume:mhs\mail\gates\ccmailpo.

cc:Mail is responsible for carrying messages between the cc:Mail

Post Office and the cc:Mail Router. The MHS or compatible

message router does the routing between the cc:Mail Router and

the Connect�FACSys Gateway.

2. SMFLINK: - cc:Mail SMFLINK is used if the cc:Mail

Router will only talk to the Connect�FACSys Gateway.

a. MHSHost can be left blank in faxlink.ini

b. Specify "Yes" to LocalDelivery in faxlink.ini

c. Define AppName as "ccMail" in faxlink.ini

d. The Connect�FACSys Gateway will still use the

MHS directory structure even if MHS is not

installed.  For example, the path below needs to be

created if MV=FS1\sys: and MHSGateway=FACSys:

FS1\sys:mhs\mail\gates\FACSys

Also, there will be four subdirectories  - "in", "out",

"iparcel", and "oparcel" under FACSys

e. Define a Post Office in cc:Mail with the name of the

Connect�FACSys Gateway, and the address being the

path of the Connect�FACSys Gateway, smflink

file_server\volume:mhs\mail\gates\FACSys

The cc:Mail Router communicates with the cc:Mail Post Office

and the Connect�FACSys Gateway directly without the MHS

Connectivity Manager.

Sending a Facsimile via cc:Mail

-------------------------------

To send a facsimile from cc:Mail:

1. Select "Prepare new message" from MAIN MENU.

2. Select "Address to person".

3. Select Connect�FACSys Gateway.

4. Type "user {extended address}" after the prompt "To:".

5. Repeat steps 1-4 for multiple recipients.

Note: The cc:Mail Router uses SMF64 and does not support multiple

attachments.

Receiving a Facsimile via cc:Mail

---------------------------------

A Connect�FACSys user will get a Receive Notification in

cc:Mail if the MHS Host specified in his Connect�FACSys

Router Table entry is a cc:Mail Post Office.

To read the Receive Notification from cc:Mail:

1. Select "Read inbox message" from MAIN MENU.

2. Select a Connect�FACSys Fax Gateway message.

3. Select "Display Items" from ACTION MENU to view body text.

4. Select "Choose another item" in ACTION MENU for attachment.

5. Select "Copy file item to DOS" to save attachment.

Incoming facsimiles in DCX format are received as attachments

by cc:Mail users.

Connect�FACSys provides a DOS DCX viewer for viewing and

printing DCX images.

You may disable the Connect�FACSys Receive Notification by

leaving DefaultRecipient blank in your faxlink.ini file.

DaVinci Multiple Attachments

----------------------------

To send multiple attachments from DaVinci e-mail, you must

first modify your DaVinci EMAILSYS.INI file.  Go to the

[Dispatcher] segment of that file.  Add the line Add_Wrap =

512 at the end of the [Dispatcher] segment, as recommended by

DaVinci.

For Further information on this, please contact DaVinci directly.

Connect�FACSys MHS Notification Extensions

------------------------------------------

Connect�FACSys offers three message scripts to report on any

fax event which has originated from either printing or e-mail

services.

The Connect�FACSys administrator can create and edit files with

any standard text editor (i.e. EDIT, NOTEPAD) and create more

elaborate system messages for fax events. These files are known

as Message Scripts.

Notification message scripts (3 in all).

notify.ok       Message script for faxes that are successfully

transmitted.

notify.err      Message script for failed faxes.

notify.rcv      Message script for received faxes.

Within each message script file, the administrator may utilize any

of the Message Tokens listed below.  There is no limit to the

number of lines any given message script may contain. However,

each line within the message script should not exceed 128

characters.  Each token can be identified within the message

script by embedding the token between the ~ (tilde) character as

follows:

Example:

If a line within the script reads:

Fax was sent by user ~User~ on ~Date~ @ ~Time~.

The following output is generated:

Fax was sent by user JEFF on 23-Jun-94 @ 2:34p.

Message Script Tokens

---------------------

The following tokens are valid within any of the message script

files. Note that some of the tokens may be abbreviated.

Token and

Abbreviation    Description                     Format

-------------   ---------------------           ------------------------

Connect/Con     Connection time/duration        mm:ss

of event

Date            Date of event                   dd-mmm-yy

Destination     Destination name                string

/Dest

Device/Dev      Device used for sending         integer

Error/Err       Error number (hexadecimal)      hex integer

Fax             Fax number dialed               string

Pages/Page      Pages actually transmitted      integer

Remote/Rem      Remote fax CSID                 string

Retries/Retr    Number of dial retries          integer

Sender          Sender name (descriptive)       string

Status/Stat     Description of fax status       string

Time            Time of event (12 hour)         hh:mm a/p

Time24          Time of event (24 hour)         hh:mm

User            Logged user name on network     string

Fax Server Selection

====================

Connect�FACSys provides you with the ability to select any fax

server on your network system for sending, receiving, and

manipulating fax documents.

You can select a fax server in one of the following three ways

whether you are working from DOS, Windows or the TSR.

Default Fax Server

------------------

Each user on the network system may select a default fax server

by setting the DOS environment variable FAXSERVER. From

the DOS prompt (or a batch file), you may set the default fax

server as follows:

SET FAXSERVER=FAX

In the above example, the FAXSERVER environment variable is

set to the fax server FAX. If there are multiple fax servers within

the internetwork, the server named "FAX" will be selected as the

default server. This applies to the DOS/Windows front-end

applications, TSRs, and command line utilities.

Explicit Fax Server

-------------------

An explicit path is when the user invoking an application passes

additional parameters to the program to "override" the default

fax server selection. All applications support the /S=Server

option except for the DOS TSR; it utilizes /F=Server.

Implied Fax Server

------------------

If the default and explicit fax servers have not been specified, the

applications will attempt to locate the fax server located nearest

to them.   This may not be the preferred FAX server.  From the

Connect�FACSys User Interface (both the DOS and Windows

versions) users can select a different fax server.

DOS/Windows Fax Server Selection Dialog

From either DOS or Windows, choose the "Select Fax Server"

option from the Manager menu to manually select the fax server

of your choice. This dialog displays all known fax servers on the

internetwork to the user. Included in the display are rudimentary

job statistics as follows:

Tasks           Total fax jobs pending transmission.

Received        Total facsimiles received.

Logs            Total recorded activity for sending and

receiving facsimile transmissions.

By observing this information, you can select the fax server with

the fewest number of jobs pending so that your fax job will be

processed as quickly as possible.

To select a fax server, highlight the fax server you want from the

listbox provided.  Then, click the OK button and all

transmissions, receives, and log queue activity will be performed

on the fax server you have chosen.

The "Jobs" button control lists all queued print jobs remaining

within the PRINTQ_FAX network print queue where the selected

fax server resides.

While within the print queue job dialog function, you may

optionally delete a print queue job similar to that of the NetWare

utility PCONSOLE and rush your job to the beginning of the list.

To successfully rush a print job, you must have the

Q_OPERATORS security equivalence within the NetWare

system bindery

The User Interface

==================

The Connect�FACSys User Interface lets you control basic fax

functions, manage your fax server environment, queues and

phone directories, and perform supervisor functions

Select options from the Connect�FACSys User Interface by using

the mouse or a keystroke equivalent.  To select a menu option

using your keyboard, hold down the <ALT> key and press the

letter underlined in the menu option.

Windows Interface

-----------------

To run the Windows version of the User Interface, load

C2FACSYS.EXE under Windows.  Once the application is

installed you may double click on the Connect�FACSys icon to

load.  You may also execute the Windows version of the User

Interface by typing WIN C2FACSYS at the DOS prompt.

DOS Interface

-------------

To run the DOS version of the User Interface, type FAX to

execute FAX.EXE

Basic FAX Functions

-------------------

With Connect�FACSys, six basic fax functions are initiated from

the File pull down menu from the User Interface screen.  To get

to this screen, DOS users type FAX and Windows users double

click on the Connect�FACSys icon installed under Windows.

All network users can invoke the Connect�FACSys User

Interface to transmit faxes.  To receive, route, view and save

faxes, and to perform management activities, you must be added

via SYSCON as a member of the group FAX_ADMIN.

From the File Menu you can:

* View a fax

* Route a fax

* Save a fax

* Send a fax

* Print a fax

* Exit the program

View a Fax

----------

You may view a fax using the Open function from the File

Menu.  This lets you look at a file that has been received and

saved to disk (an incoming facsimile). All incoming fax files are

in DCX format.

Open

----

To Open a file, highlight the file in the list box or type the

desired file into the edit box.  Use the UP/DOWN ARROW Keys

to scroll through a directory or sub-directory. The file you select

will be displayed in the edit box.

Connect�FACSys permits you to view and print the image under

Windows.  DOS users have an additional print "button" to print

images directly.  Select <Cancel> or File/Exit to return to the

Connect�FACSys User Interface.

Close

-----

This allows you to exit from viewing a file. All files opened are

automatically closed when this is selected.  To exit from viewing

a file in the DOS version, just press <Esc> and you will be

returned to the previous screen.

Information

-----------

The information option lets you see the total number of pages in

the file, the number of pixels each page contains, and the current

page number.  To see information, make sure you have selected

Open and chosen a file.  Then choose the Information option.  To

exit this option, select <Enter> to return to the parent

Window.

This option is not available under DOS.

Send a Fax

----------

Text, PCX and DCX files can be sent to a fax machine using this

Transmit function.  To transmit a document intact, (with full

fonts and graphics) use the TSR Hot-Key (for DOS users), from

within your application (i. e., WordPerfect, SuperCalc) or the

Windows Printer Driver (for Windows users). For further

information, please reference the DOS TSRs section.

You may send a fax from the Connect�FACSys user interface by

selecting the Transmit option from the File pull down menu.

This option is available in Windows and DOS

From the Connect�FACSys User Interface, select the Transmit

function from the File pull down menu.

Files Box

---------

Use the UP/DOWN ARROW keys to move to

the file you want to transmit. Highlight that file

by pressing the SPACE BAR.  Press

<ALT>+A (for Add).  Or, you can press the

ENTER key. The file now appears in the File

List box. You can select additional files to

transmit in the same manner.

To Delete a file, tab to the File List box,

highlight the file.  Press <ALT>+D (for

Delete)

Destination     The Phonebook Directory list box displays your

current telephone book entries. Select a

destination from this list and press <Enter>.

To send a fax to a destination that is not listed in

your phonebook, select OK.  A dialog box

appears and allows you to enter the fax/phone

number of your destination.  Note: This new

number will not be saved in your phonebook, it's

used for this transmission only.  Select the OK

button to return to the previous menu.

If the destination you select doesn't have a phone

number listed, an error message will be displayed

in the Queue Menu.  To correct the error, you

must add the number to the phonebook (or enter

it in the dialog box) and transmit the file again.

Send Options    Select options for your fax file transmission.  The

following options are available:

Small Text

When sending text files you have the ability to

select from two text sizes before you transmit.

The default is 80 columns by 66 rows.

Compressed mode is 132 columns by 88 rows.

Select Small Text (132 Column) only if you

choose this over the default.

Resolution Mode

Resolution mode offers two options. The default

is 100 x 200 resolution mode which has a faster

transmission speed, but reduces the resolution of

your text file. Selecting the 200 x 200 Fine

Resolution will enhance the quality of your

transmission, but limits the transmission speed.

Enter a check mark next to Fine Resolution if

you choose this over the default.

Binary

Available only when both the sending and

receiving ends have facsimile hardware boards

that provide this capability.  When you choose

Binary transmission mode, you must verify that

the destination site has an Intel or Ferrari

compatible facsimile board, and you must check

the "Compatible Fax Board" box in the

phonebook entry.

Cover Page

A cover page can be attached to the fax in the

Menu.  You must prepare your cover page before

transmitting a file.

To edit cover page information, choose the Cover

option.  When you have completed the "To:",

"From:", and "Message:" fields with the

appropriate information, select the Include Cover

Page option at the bottom of the screen by

tabbing to the option and pressing the space bar

to fill the square brackets with an X.  If you are

using a mouse, you may click on the field at the

bottom of the dialog box to indicate that a cover

page should be sent.

Priority Fax

Places your fax at the top of the send queue.

OK              To transmit your file(s) to all destinations you

have selected.

CANCEL          To return to the previous menu screen.

Receive File

------------

The incoming facsimiles are stored as files in the receive queue

area of the file server.  From the Receive File option on the File

Menu you can View, Route and Save incoming facsimiles.  You

must be a member of the FAX_ADMIN group to Receive Files.

View

----

To view a file in the receive queue, do the following:

1. Select the facsimile you want to view by highlighting it

using the spacebar.

2. Select View.

3. The document appears on your screen. To scroll through the

document, use the UP/DOWN ARROW Keys.

DOS Users

---------

The upper left hand corner of the image screen displays the status

of the current view.  The status shows either "Loading" or

"View".  The page count is displayed in the upper right hand

corner.

The following buttons are displayed at the bottom of the view

screen:  Exit, Previous, Next, Rotate, Invert, Up Arrow, Down

Arrow.  Select these buttons to move through the fax and alter it

for increased legibility.

Windows Users

-------------

Windows users automatically launch the FaxView program to

view facsimiles.

The pull down menu and button bars offer choices for saving the

image to disk, printing and adjusting the size of an image.  You

can also choose to rotate or invert a fax for increased legibility

and ease of editing.  For additional information on options

available in the Windows FaxView application, refer to the

FaxView section later in this manual.

Print

-----

Connect�FACSys provides two options for the resolution

(quality) of your printed document.  Quality Mode, 200 x 200

pixels resolution, enhances the quality of the print out, but

requires more time to print the document.  The Draft Mode, 100

x 200 pixels resolution,  prints more quickly.  Certain files, such

as graphic image files, should utilize the Quality Mode.

DOS Users

---------

1. From the Receive screen, select the document you want to

print as described above under Viewing.

2. Select Print.

3. Select your Print Options to indicate the print quality

desired and page ranges, if any.  Press the OK button to

print.

Windows Users

-------------

Before printing, make sure you configure your Printer Setup

properly.  Select the printer you want to use and make the

appropriate screen option choices for your selection.

Note: You must select a portrait printout orientation under Printer Setup

for received faxes.

The Print option lets you determine which pages to print.  To

print, do the following:

1. View the file through the Receive File option, or open a

previously saved fax using the Open File option.

2. Return to the File Menu and select Print.

3. To print all pages, select OK.

4. To print selected pages, type the number of the beginning

page on the From: line, then, type the ending page number

on the To: line. Your selected page range will print.

The document can also be printed by selecting the Print Option in

the File Menu.

Route - Windows and DOS

-----------------------

This option lets you send a network broadcast message to another

user when a fax for that person has arrived in the received files

list.  This action also assigns the person's name in the USER

column of the list.

To route a received message, select a file from the Received File

list and do the following:

1. Select the Route control button.

2. The Connect�FACSys router dialog box appears and

displays the names of users on your network.  Select a name

from the Network Users box.

3. To notify the addressed person immediately about the file

transfer, check the box for Message Router.  If the box is

not checked, the file is routed to their faxbox without a

network broadcast message.  The user won't be aware of the

fax until he or she runs the Connect�FACSys User Interface.

Save

----

To save files, clean the receive queue, or to archive your files,

follow these steps:

1. Select the Receive File option from the File Menu.

2. In the current queue entries box, select the fax to be saved

and then click on the Save button.

3. A destination box is shown. Enter the destination path and

file name. Select OK.

The Manager Menu

================

The Manager Menu provides the Connect�FACSys Supervisor(s)

with the ability to generate reports and a way to check the status

of program and transmission information.

Version Information

-------------------

Version Information describes the product name, copyright

information, the version of software and CAS. It will also

indicate the board that is installed.

Network Connection

------------------

This option provides reference information required by

Connect�FACSys Supervisor(s). The connection number of your

workstation, the Internetwork address and the login time are

displayed.

Select Fax Server

-----------------

A list of your available fax servers displays.  Use the UP/DOWN

ARROW keys to move to the preferred fax server. In DOS,

highlight the preferred fax server by pressing the SPACE BAR.

Press ENTER.  In Windows, double click on the fax server

you're selecting.

Startup Options

---------------

Each user has their own startup option file. The file sets personal

default settings when starting Connect�FACSys. The startup

screen for both the DOS and Windows interface offer identical

options.  The screen from the Windows version is shown.

Text Size       Two text sizes are available within Connect�FACSys.

Normal          sets faxes at 80 x 66 format.

Condensed       sets faxes at 132 x 88 format.

Phonebook

---------

Select either the system phonebook or a user's personal phone

book.  Specifies a path and filename for the user's default

phonebook.

Example: F:\FAXLINK\USER\TIM.PBF

This is the default phonebook used when executing functions

from the user interface such as File Transmit, or phonebook

editing functions. (See the Phone Menu Section later in this

manual for more information on selecting alternate phonebooks.)

OK              Saves the entries selected in this dialog.

Cancel          Exits this dialog without saving the modified

entries in this dialog.

Phone           Provides a list of available phonebooks when

entering the Default Phone Path and Name.

Color           Provides color options for further

customization.

Resolution

----------

Choose between Standard and Fine resolution with Connect�FACSys.

Standard resolution transmits more quickly; however, fine resolution

is recommended for dense graphic images.

Fine            sets resolution to 200 x 200 dpi

Standard        sets resolution to 100 x 200 dpi

Check Unread Faxes at Startup

-----------------------------

If this box is checked, the system searches for faxes which have

been routed to you and are Unread when you execute the user

interface. A dialog box displays how many are unread.  Selecting

this option causes a slight delay in accessing the user interface

because the queues are searched each time you execute the user

interface.

Show Only Personal Faxes in Log Queue

-------------------------------------

When you execute a log queue search, this option displays only

those faxes that are relevant to that user.

Supervisor Functions

--------------------

User Routing Table

------------------

Connect�FACSys offers a special process enabling you to

automatically route your newly received fax transmissions to a

user workstation with a network message/notification.

This feature implements an Optical Character Recognition (OCR)

scan on the first page of the facsimile received.  When the unique

router ID is identified, the incoming fax is automatically routed

to the identified user.  Several rules must be followed and

maintained in order for this automatic routing to take place.

The OCR option must be turned on within the FAXLINK.INI

file.  To do this, see the FAXLINK.INI File description in the

Installation section.

What the Sending Site Needs to Know

-----------------------------------

To activate automatic routing, the sending site must place a

numerical identifier (specific to the individual recipient of the fax)

on the cover page of the sending document. The identifier may

contain any valid digit string of 1-8 digits in length.  The

numerical string must be embedded in a set of identifiers. The

identifiers are a double left and a double right parenthesis. For

example, if user ROB selects the numbers 3412 as a router ID,

the sending site must place the string ((3412)) on the fax that is

being sent to ROB.  If the router id is invalid or missing, the

incoming fax is placed into the supervisory bin. The

Connect�FACSys supervisor may then manually route the

facsimile to the appropriate user on the network, and notify the

user of this event.

To optimize the performance of the character recognition

software, you should follow these rules:

1. Always use the numerical ((1-8 digit)) sequence embedded

between the double left and double right parentheses.

2. Always use a courier typeface (or common font style) of

approximately 12 points in size or greater when sending a

fax to the Connect�FACSys server.

3. Do not use a proportional font because the characters may

appear to run together.

4. Always make certain that the facsimile document is not

smudged or distorted before transmitting.

5. Place the router ID at the highest location possible on the

cover page of the document to reduce the time necessary to

successfully decode the received fax since the OCR software

scans the page from top to bottom.

6. Avoid slashed zeros. The OCR process may interpret this as

the digit eight '8'.

User Routing Setup

------------------

Only Connect�FACSys Supervisor(s) can access the Automatic

In-Coming Router Table found in the Manager Menu.  The

routing table lets you control the internal security of received

facsimile data.

Whenever a received facsimile containing a router id is detected,

the Connect�FACSys auto-router notifies the network user at the

workstation(s) he or she may be logged into. If auto-route is not

set, the user is notified of a newly received fax upon execution of

the Connect�FACSys User Interface.

To configure your routing table, do the following:

Select the Supervisor Function, User Routing Table option.

Select the user for whom you want to define routing from the

User Routing dialog box.  Then, click on the "Route..." button.

From the displayed screen, you include the following information:

Router ID       The user's numerical route id is displayed.  To

create or change this number, simply enter a

valid number (8 characters maximum) and select

OK.

Use MHS         Check this option if you are utilizing OCR and

MHS inbound features to route faxes via e-mail

packages.

MHS Host Name   Insert the MHS Host Name.

MHS User Name   Insert the MHS User Name.

Network Receive Check this box to send a broadcast notification to

Notification    the user whenever a facsimile is received.

Auto Print      Select this option to automatically print incoming

fax documents to the network printer, after they

are OCR routed to the user.  Note: If AutoPrint

is disabled in FAXLINK.INI, this feature is

disabled.

Print Queue

Type the name of the server and queue to which

you want OCR routed documents printed.

OK              Accept this configuration and return to the User

Routing Dialog to select the next network user.

Cancel          Return to the User Routing Dialog without

accepting changes.

Delete          Delete this user from the User Routing Table

Once you have configured the routing table, you are ready to

begin testing your Connect�FACSys routing feature. The sending

site should be aware of your network router id to engage the

auto-route feature upon receipt.

Monitor Events

--------------

This enables the Connect�FACSys Supervisor(s) to monitor the

events currently being processed. The event descriptions are self

explanatory. If you select ABORT, the facsimile currently in

progress will be aborted.

Run Diagnostics

---------------

The facsimile front-end processor can take advantage of the

internal diagnostic functions of the facsimile hardware device.

Upon selecting "Run Diagnostics", the facsimile communication

server will begin the diagnostic tests for the facsimile hardware

device and display the results of the diagnostics process. If an

error code is generated, the code will appear in the dialog box. If

no error return code is generated, the dialog function will display

that the Server has passed the diagnostic test.

Autoreceive State

-----------------

The Autoreceive State, accessible by Connect�FACSys

Supervisor(s), allows you to set the number of rings prior to

pick-up by the fax board. The range is 1 to 255 rings.

The current autoreceive state or value appears in the Answer

Line On Ring edit box. If you desire to change the value, do the

following:

1. Select one of the valid numbers mentioned above and type it

in the Answer Line On Ring edit box .

2. Select OK to save the change. You will be returned to the

Manager Menu.

Disabling the Facsimile Device

------------------------------

Entering a value of zero disables the facsimile device from

answering any incoming calls. Since fax transmissions occupy

125 kilobytes per page, allowing an unattended system to answer

all calls could result in rapid and severe disk space loss.

Therefore, if the system is to be unattended for a lengthy period,

you should consider setting the ring option to 0 (zero).

Note: To make permanent changes consult your FAXLINK.INI file.

Shutdown Server

---------------

This option facilitates a remote shutdown in the event of system

failure. Access to this option is limited to Connect�FACSys

Supervisor(s).  To activate the shutdown, click on the shutdown

Server Control option, select OK and the facsimile

communication server will be signaled to immediately halt

operation. To abort this process, select CANCEL.

When the facsimile communication server is shutdown, the front-

end processor remains active and is able to fully utilize the

graphics and printing functions. Any communication service or

queue function, however, will not be processed and the user will

be notified that the facsimile communication server has not been

found on the network.

The Queue Menu

==============

Overview

The Queue Menu provides three basic functions:  Through the

queue menu you can:

* View three different send/receive queues

* Kill a queue

* Generate management reports

All transmissions (sending and receiving) are divided into three

(3) queue categories.

* Send Queue - Contains all of the current files waiting to be

transmitted. All transmissions are sent on a (First in, First

out) FIFO basis.

* Receive Queue  - Stores all incoming facsimile

transmissions.

* Log Queue - Displays the final status for all events,

whether they are successful or not. Each event is displayed

with the date and time the transaction occurred and

reception status.

To view a queue, select the Queue Control option.  Select the

desired queue and it will be displayed for you.

Each queue offers three identical buttons:

Cancel          Returns you to the User Interface

Info            Gives you detailed information on the job in the

queue.

The Status line will display one of the following conditions:

* A Successful Completion

* Aborted By User

* Line is Busy or Not Answering

* Event is being processed

Note:  For binary file transfers, all of the above

information is identical, except, that the facsimile

resolution and number of pages will be replaced with

an indication that a binary file has been transferred,

along with the size of the file in bytes. Keep in mind,

binary files may only be sent and received by other CAS

compatible PC facsimile devices.

Delete          Removes the selected job from the Queue

In addition, the following queue options are available.

Send Queue - Assign Priority

----------------------------

Allows you to rush a fax job by moving it to the top of the Send

Queue.  Select the job you want to move.  DOS Users press

<ALT> -P  and Windows Users click on the "Priority" button.

Receive Queue - View

--------------------

Allows you to view the selected job in the queue.  For additional

information on viewing a fax from the Windows version, refer to

the FaxView section later in this manual.

Log Queue - Resend Faxes

------------------------

You can resend faxes which failed to transmit. To re-send the

fax, highlight the fax which has an Error status. Tab to the

Resend button and press enter or press <ALT>+R to resend

that fax.

Kill Queue

----------

Access to this option is limited to Connect�FACSys

Supervisor(s).  Initiating this command results in the entire

contents of the queue being erased.

To kill a queue, choose Kill Queue, select the queue you wish to

kill, and click on OK.

You may specify the ending date of a date range to Kill a

particular queue.  When you access the Kill Queue screen, you

will see a Date Box. The date which is displayed (default) is the

current system date. If you wish to kill the queue(s) for a period

starting with the oldest fax and continuing through a date earlier

than the current date, use the UP/DOWN ARROW keys to

change the end date. Proceed as instructed.

Management Reports

------------------

Management reports are available through a query application

included with the Connect�FACSys Windows Application.  Any

Connect�FACSys supervisor can run these reports.

To run a management report, access the Query application by

selecting the Management Reports option from the Queue pull-

down menu.  The Query module appears.

Select the Query Menu, which is the only option displayed, then

select the Query Options to set your report defaults.  Once these

defaults are set, the report will be run the same way until you

change your Query Options.  The following screen displays the

Query Options that are available:

Options         Description

--------------  ----------------------------------------------------

Date and Time   Sorts all events in chronological order.  The order

of events will display from most current to less

recent.

User Name       Sorts events by network user ID and subtotal each

user. A grand total will be shown at the bottom of

the report.

Connection Time Sorts by the actual connection time down to the

seconds level. This is used where telephone time

is crucial for billing purposes rather than the

actual number of pages or files.

Billing ID      Track for accounting purposes.

Export File     The Export File Window allows you to select a

format if you wish to save a report file to a

specific format.  For example, you may wish to

merge data into a time and billing or accounting

application.  Press on the DOWN ARROW in the

Export File window and make your file type

selection. Here you choose the file type that you

wish to export.

Output to       Directs the output of the report to your default

Printer         printer.

Note: If you desire to select a printer other than

your default, you must return to the

Connect�FACSys Windows Application by

touching the menu with your mouse. Press the

ALT+F Key simultaneously.  The printer dialog

box appears.  Choose the printer and select OK.

Now, return to the Query Manager Menu and

select Execute to print your report.

Show Errors in  Includes any errors that appear in the queue to the

Report          report.

Show            Gives you totals of the number of transmissions,

Summary/Totals  number of failures/errors, total connection time,

and total number of pages transmitted.

Once your report defaults have been set, select OK.  If you select

SAVE, these defaults will be saved to your USER.CFG file.

To generate the report, select EXECUTE at the Query Menu.

To return to the User Interface, select the exit option from the

Query Menu

Windows FaxViewer(TM) Menu Overview

===================================

The FaxViewer* is a Windows based viewer designed to be

launched from Connect�FACSys and e-mail products used in

conjunction with Connect�FACSys.  This menu allows users to:

* View faxes on screen

* Save individual pages or sections of a fax as Windows

bitmaps (BMP files), or PCX files.

* Copy pages or sections of pages to the Windows clipboard.

The FaxViewer functions are available from the pull down

menus: File, Edit and Page, or from the button bar that runs

across the top of the screen.

Launch the FaxViewer

--------------------

You can launch the FaxViewer from within Connect�FACSys by

selecting View from the File pull down menu on the User

Interface or by selecting a file in the Receive Queue and clicking

on the View button.

The FaxViewer opens and displays the file you've selected.  You

can then select pages or partial pages and copy them to the

clipboard or save them as Windows bitmap or PCX files.

From the FaxViewer, you can make choices using either the pull

down menus (File, Edit, Page) or using the button bar that

stretches from left to right across the top of the screen.  Some

choices are available from both the menus and the button bar.

For example, to Print a message you can either select the Print

function from the File menu or click on the Printer icon in the

button bar.

Brief descriptions of each button bar activity are displayed at the

bottom of your screen when you hold the mouse over the top of

the icon in the button bar.

Selecting a Partial Page

------------------------

Full page is the default mode.  You may work with partial pages

by choosing the Rubberband option from the Page menu.  When

this option is selected, you select partial pages by holding down

the left mouse button and dragging the mouse.  When you've

selected your desired area, release the mouse button.  Then you

may print, zoom, and perform other options on just the selected

portion of the page.

FaxViewer Options

-----------------

The following section offers a brief description of the FaxViewer

options.

Open

----

Open the fax file that you want to view.  Select Open from the

File pull down menu or click on the file folder icon.  You will be

presented with a dialog box allowing you to select the file you

want to view from your FACSYS directory.  You can choose to

display a list of files of type .DCX, .PCX and .FAX.

Print

-----

Select your preferred printer using the Print Setup option from

the File menu.

You may choose your default printer or select a printer from the

list.  The Print Setup dialog also lets you choose portrait or

landscape orientation and lets you select the paper size and

source.  The Options button offers additional choices.  For

additional information on printing options, including advanced

work with font selection, consult the on-line help by pressing F1.

Once your printer is configured, you may print the displayed file

by choosing Print from the File menu or clicking on the Printer

icon.

Copy to Clipboard

-----------------

Copy the displayed fax to the clipboard.  From the clipboard you

may paste the fax into other Windows applications.

Copy to File

------------

Selecting this option allows you to save the fax to a different file.

From the displayed dialog box, enter the filename for the file.

Center Fax

----------

This option, available from the Page menu or by selecting the

Center icon centers the fax that you are viewing.

Next and Previous Page

----------------------

Use the Next and Previous page options to move through your

fax, page by page.  The page number you are viewing is

displayed in the lower left corner of the FaxView screen.

Rotate Fax

----------

Selecting the Rotate fax function rotates your fax image 180

degrees on screen.

Invert Fax

----------

Inverting a fax changes the contrast - what was black becomes

white and vice versa.  Inverting can make certain faxes easier to

read.

Zoom

----

The zoom option allows you to magnify a page, or a portion of a

page.  You may choose between actual size, 1/2, 1/3, or 1/4

magnification.

Page Number

-----------

The page number option allows you to jump to a specific page in

a multiple page fax.  Enter your destination page number in the

dialog box and FaxView will display that page.

Phone Menu

==========

You may create, maintain and select different phonebooks for

different uses.  This section describes how to create phonebooks

and select a new phonebook.  It also covers modifying your

phonebook through a variety of options.

The phonebook defined in the default phonebook field (Manager,

Startup Options) will be the one used when opening dialog boxes

such as File Transmit, or when selecting the phonebook editing

options, described below.

If the default phonebook field is blank, you must Select a

phonebook each time you choose a function pertaining to

phonebooks such as transmitting a file, editing phonebook

entries, etc.

Creating a New Phonebook

------------------------

To create a new phonebook, click on the New option. This opens

the dialog allowing you to create a new phonebook. Enter the

name of the new phonebook. Note the displayed directory. The

directory path may be entered when entering the new phonebook

name. Press Enter or click on the OK button.

Opening an Existing Phonebook

-----------------------------

To open a phonebook, click on the Select option. This displays

the Open dialog box.

Name

----

The structure of the directory where you reside when executing

the Connect�FACSys front-end manager is displayed when this

option is selected. A list of available phonebooks is displayed.

Select a phonebook from the list.  You may also click on

Directory to select a phonebook stored in an alternate directory.

Information

-----------

Selecting this option displays the name of the currently selected

phonebook as well as the number of persons and groups held in

the phonebook.

Persons

--------

Selecting Persons opens the Person Maintenance dialog.  From

this list you update individual records.

Person List

-----------

All of the records saved to this phonebook will be displayed.

They will be displayed in Last Name, First Name order.

Note: If you select the "Use company name in list" option, the company

name will appear in the alphabetized list.

Cancel

------

Click on the Cancel button to return to the Connect�FACSys user

interface.

Delete

------

To Delete an entry from the Person List, select/highlight that

entry and click on the Delete button.

Adding or Editing Records

-------------------------

Select Add to add a new record to this phonebook or select Edit

to modify an existing record.  Selecting either Add or Edit

presents you with the Person Record dialog box.

Person Record Dialog Box

------------------------

Enter information about this person in the dialog box.  A brief

description of the fields follow.

Last Name

---------

Enter or modify the last name of the person to whom you will

transmit a fax in this field.

First

-----

Enter or modify the first name of the person to whom you will

transmit a fax in this field.

Company

-------

Enter or modify the company name to which you will transmit a

fax.

Use Company Name

----------------

Select this box if you want the company name, rather than an

individual's name, to appear in the alphabetized listing of

phonebook entries.

Billing

-------

You may optionally enter a billing code for this entry. This may

be used for importing data into an accounting or chargeback

system.

Notes

-----

You may optionally enter notes for this entry.

Fax Number Field

----------------

Tab from field to field and enter or modify the Country Code,

Area Code and fax number to which the fax will be sent.

Voice Number Field

------------------

You may optionally enter or modify the voice telephone number

for this entry.

OK

--

To save this entry to the phonebook, select OK.

Cancel

------

To leave the Person Entry screen without saving your changes,

choose the Cancel button.

Groups

------

This opens the Group Maintenance dialog.

Group List

----------

The list box displays the currently available groups. To select a

group, click on the desired group.

Delete

------

To Delete a group, click on the Delete button.

Cancel

------

To leave the Group Entry screen without saving your changes,

choose the Cancel button.

Add or Edit

-----------

Select Add or Edit, as appropriate to create a new group or to

modify an existing group.

Group Entry

-----------

Group Name

----------

The name of the group which was selected while in Group

Maintenance appears in this box.

Group Members

-------------

This list box displays current members of the selected group.

Available Members

-----------------

The list box displays all of the users in your currently selected

phonebook who have not yet been added to the current group.

OK

--

To save the name of a new group, click on the OK button.

Add

---

To add Available Members to the group, select the users desired,

and click on the Add button.

Remove

------

To remove users from a group, select the users from the Group

Members list box, and click on the Remove button.

Close

-----

To exit the Group Maintenance dialog, click on the Close button.

Import

------

To import data from a comma separated value ASCII file into a

phonebook, use the following syntax:  Last name, first name,

company name, billing code, notes, fax number, voice number.

The fields are limited to the following lengths:

Last Name       18

Notes           31

First Name      18

Fax Number      13

Company Name    31

Voice Number    13

Billing Code    21

Note: To convert your phonebook from FACSys version 3.30b or older

to Connect�FACSys version 3.40, please see "Upgrading to

Connect�FACSys Version 3.4" later in this section.

Export

------

This will export from a phonebook to an ASCII file.

Print

-----

This will print a hard copy of the selected phonebook. You may

optionally print by person name or company name fields.

Rebuild

-------

This will re-index the selected phonebook.

Converting your Phonebook

-------------------------

To convert your phonebook to Connect�FACSys version 3.40,

use the PDIR utility.

Set DOS Variable LOCALAREA

You will need to set the DOS variable LOCALAREA = [Your

Local Area Code] to fully utilize the Connect�FACSys

phonebook structure. This can be done by simply adding the

following to your Login Scripts:

DOS set LOCALAREA = "410"

Upgrading to Connect�FACSys Version 3.40

========================================

To upgrade to your new version 3.40 of Connect�FACSys, please

follow the following steps:

Note: When you upgrade your phonebook you will lose the use

Company Name status and will have to modify such entries to

re-initialize use Company Name.

1. Make a backup copy of the files that exist in your

FAXLINK directory (or alternately selected directory where

you installed Connect�FACSys previously).

2. Delete all of the files from this directory.

3. Install Connect�FACSys version 3.40 from the new diskettes

you received.

Note: When you install Connect�FACSys version 3.40, you should have

a valid .pcx logo file defined. For GammaLink board users, the file

must be a valid TIFF file. The correct configuration for .pcx file is

2 colors, 1 plane, and 1700 x 800 PELs.

4. You must upgrade your phonebooks as there are more

available data and fields. To do so, run PDIR (see index).

5. Make sure that you run the product registration portion of

the install. If the product has already been installed, you

will be advised of such. If you have not registered

Connect�FACSys, please make sure to get the completed

registration card to us (you may mail or fax to 410-363-0846

or 410-363-3779).

Fax Server Console Commands

===========================

This interface is for the use of the LAN Supervisor(s) only.

Connect�FACSys employs a "virtual" console design which helps

segregate status information within your server's display console.

In previous versions of FACSys, all keyboard entry, event status

messages, and system warnings were displayed to the same

display which constantly "scrolled" important information off the

display prior to recording the possible error condition.

With the virtual console implementation, a separate user console

and monitoring console are provided.  Under this scenario, you

may enter console commands, view system configuration

information, and record important system messages while rapid

network communications can be monitored in another.

The following commands are entered and displayed from the

main console window. Reference to the "system password"

indicates that the function requires the password of the logged

object (i.e. user) at the fax server.

Note: To use the following commands, type the command which appears

in bold followed by ENTER.

config

------

System configuration information will inform the user of the

current server configuration, service information, routing

buffers, and memory utilization.

down

----

This command is utilized to shutdown the fax server and perform

the following functions.

* Close all network communication channels.

* Detach as a print server from the network print queue.

* Cancel all service advertising functions.

* Optionally perform network logout and device shutdown.

This function requires the system password.

Help

------

On-line help

hp

--

When utilizing the PCL engine within the fax server, this

command will display rudimentary information pertaining to the

status of the PCL processor.  This function will indicate the

actual fonts that are pre-loaded, current memory mode, and the

network print job number.

kill

----

Deletion of queues can be accomplished from the front-end

software programs (Windows/DOS) as well as from the system

console.  The queue parameter is the queue intended for deletion

as follows:

kill task

kill receive

kill log

The parameter may be the task (send), receive, or log queue.

This function does require the system password.

line

----

Line control is offered from the system console to control the

"device mode" for any given fax hardware device. Each and

every fax card installed within the fax server is referred to as a

line or channel.

With V3.30, you may optionally set the device mode of each

device as follows:

line [device number 0...n] mode

line 0 SO

---------

The above example sets the device mode for line 0 to SO (Send

Only). Valid modes are as follows:

S or SO         Send Only

R or RO         Receive Only

SR              Send and Receive

OFF or DISABLE  Disable Send and Receive

line show

---------

Displays the current line/channel status

mhs

---

The fax server also serves as an MHS gateway which is capable

of integrating full send and receive MHS functions with the MHS

connectivity manager.  This command simply displays the MHS

status and/or is used to enable/disable MHS services.

Command         Function

----------      -----------------------------

mhs             Show MHS server status

mhs on          Enable MHS services

mhs off         Disable MHS services

When enabling the MHS services from the system console, make

certain that the MHS host and gateway are properly configured

within the server initialization file or errors will occur.

monitor [partial]

-----------------

The virtual monitor display will appear as a long vertical window

next to the main console area. All network communications

packets serviced will be displayed within this scrolling window.

The user console is not affected by this operation.

off

---

This function disables the monitoring features of the system

console and clears the virtual console/display. Only one virtual

console is visible when executed.

ocr

---

The OCR (Optical Character Recognition) module is responsible

for the automated routing of in-bound facsimile documents. This

function will  enable/disable the OCR routing feature.

Command         Function

----------      -----------------------------

ocr             Display OCR status:

ocr on          Enable OCR

ocr off         Disable OCR

ocr timeout 60  Set OCR timeout value

In the last example, the OCR timeout value is set to 60 seconds

which instructs the OCR to abort scanning any facsimile

document if the routing code is not located within this time

interval. If this value is set to zero, no timeout threshold will be

employed and OCR scanning will continue until the entire

page/document has been processed.

This feature may also be controlled from the fax server

initialization file.

print

-----

The fax server is capable of automatically printing incoming

facsimile documents to any HP LaserJet compatible printer on

your network. This function will enable/disable and control the

target printer during operation.

Command                         Function

--------------------------      -----------------------------

print # [#=channel 0..N]        Display auto-print status

print # off                     Disable auto-printing

print # on                      Enable auto-printing

print # on SERVER/PRINTQ_0      Enable auto-print target

print path (server name is

optional)

print # form letter or          Change auto-print form

print form legal or

print form A4

print # time on or              Enable date & time stamp on

print time off                  auto-print jobs

queue

-----

This function will display information about the three facsimile

queues (task, receive, and log) and the network printer queues

where the users place their jobs to be converted from PCL into

fax images.

spx

---

This function displays brief information about IPX/SPX.  The

IPX driver version and connection count is displayed when

invoked by the user.

status

------

The fax server tracks all utilization of each service that it

provides. This function will display the actual job counts for PCL

printing, OCR routing, MHS messaging, and network service

utilization.

beep

----

The fax server can produce audible beeps on error.

Command         Function

----------      -------------------

beep on         Enable beep on error

beep off        Disable beep on error

Additional Server Features

--------------------------

The Connect�FACSys server also supports the following

additions to the faxlink.ini file:

[macros]

--------

The macros section of the .ini file is optional and permits the pre-

loading and identification of macro files when the fax server

initializes. All macros will be recorded to either expanded

memory (if enough memory is available) or be "played-back"

from disk.  Each line following the [macros] command will be

loaded during initialization.

Several additions have been made to the [FACSys] section.

OCRTimeout=TimeoutValue

-----------------------

The OCR router permits the automatic timeout during a scan for

a routing code on a cover page. As some pages are nearly

illegible, the OCR will spend sometimes in excess of 5 minutes

(on a relatively slow computer system). The timeout value aborts

the OCR scanning and returns with a failure code after the

specified number of seconds has elapsed.

AnswerDisable=Yes

AnswerDisable=No

-----------------

The answer disable function will shutdown all fax devices when

the fax server is shutdown by the console operator and/or remote

workstation. Since the facsimile device is still capable of sending

and receiving after shutdown, this function is utilized to disable

them entirely.

AutoPrintForm=[formtype]

------------------------

Valid formtype arguments are Letter and Legal

The auto-print function has been modified to handle legal size

print-outs. Normally, the letter option will attempt to condense

(vertically) any received fax to fit onto a letter size page. Some

firms may elect to utilize a legal tray for printout instead.

PrintTime=Yes

PrintTime=No

-------------

Autoprint documents can now be date and time stamped upon

receipt. If the feature is enabled, then a small box with the date

and time will be stamped in the lower right hand corner of each

page autoprinted.

FillPage=Yes

FillPage=No

------------

The [engine] section of the initialization file controls fax output

after the PCL engine has converted your printed document into a

fax file. Normally, the PCL image output (raster lines) stops

where the last printed character or bitmap data reside on the

page. Since some fax devices stop faxing where the image ends

(not necessarily an actual full length page), the FillPage option

will "fill" in the entire page with blank raster lines when set to

Yes.

Fax Server Startup

------------------

The fax server will also support the system 'autoexec.fax' startup

batch file. Any valid console commands may be entered into this

file and automatically executed by the  server after the primary

initialization services have been executed.  All commands are

echoed onto the system console.

Technical Reference

===================

This technical reference section describes additional information

on using Connect�FACSys in your environment.  Included is

information on API Shell Services, DOS Utilities, DOS TSRs,

and the Windows Printer Driver.

API Shell Services

------------------

Connect�FACSys includes network shell services which provide

support for the CAS and FaxBios facsimile communications

APIs. By employing such services, Connect�FACSys provides

support for dissimilar applications that adhere to one of the

programming specifications.

The shells operate similarly to that of the NetWare shells with

which you are already familiar; such as NETX.COM.  The

NetWare shell accepts DOS API commands and

interprets/dispatches them to the Novell file server system.

Thus, any application capable of communicating with DOS can

transparently communicate with the file server system.

This is the philosophy for which CASX* (Communicating

Application Specification compatible shell) and FBX* (FaxBios

compatible shell) were designed and implemented. There are two

major functions achieved by the implementation.  First, it

provides a standard form of communications by which

applications and applications programmers can communicate.

Second, it permits dissimilar applications to communicate with

the Connect�FACSys server transparently.

Many applications support one of these facsimile APIs, you may

simply load the shell service of your choice (or both) and operate

the application(s) designed for that (those) API(s). If you're

unsure of your applications' support for one of these fax APIs,

contact the manufacturer of your application.

CASX(TM) Shell

--------------

The CASX shell provides CAS compatibility with your

Connect�FACSys server and requires less than 5K of memory to

operate. There are several parameters which may be used when

invoking the shell.

Use the format CASX [-flags] when invoking the shell.

Flag            Description

-------------   ----------------------------------------------------------

-M              CAS applications communicate via a DOS multiplex

channel which is normally 0xCB (Hex). This option is for

more technical users who wish to develop their own

applications and utilize an alternate channel.

Example: CASX -M=CC

-P              This command parameter allows users to set a default

priority level when transmitting a fax. The priority range

is 0 to 255 with 255 being the highest possible priority

setting.  Since multiple fax servers are supported, this

option permits you to select a specific fax server. If one

is not specified at the command line, a fax server will be

selected for you.

-S              Since multiple fax servers are supported, this option

permits you to select a specific fax server.  If one is not

specified at the command line, a fax server will be

selected for you.

Example: CASX -S=FAX

-T              Since the CAS functions do not offer any type of

login/logout services, the connection with the fax server

is maintained by a watchdog timer. This reduces the

number of connections required by the fax server.  When

any CAS function is invoked, the watchdog determines

whether or not a valid connection exists between the

client/workstation and the fax server.  If not, the

connection is created.  Likewise, after the watchdog

time-out has lapsed due to inactivity within the API

services, the watchdog clears the connection.  The default

watchdog time-out is set to 15 seconds.

Example: CASX -T=60

-U              This command parameter forces the CASX shell to

remove itself from memory.  If other TSRs have

"hooked" interrupts after the shell has loaded, your

workstation will be denied for unloading.

Example: CASX -U

FBX(TM) Shell

-------------

The FBX shell provides FaxBios compatibility with your

Connect�FACSys server and offers two methods of loading static

and dynamic.  During static loading, the shell image is loaded

into the workstation's memory occupying less than 10K.  When

dynamic loading is enforced, a small handler (900 Bytes),

occupies memory and loads the FBX.OVL module when its

services are invoked.  The following parameters may be used

when invoking the shell.

Use the format FBX [-flags] when invoking this shell.

Flag            Description

--------------- ----------------------------------------------------------

-D              Use this option to enable the dynamic loading feature of

the FBX shell driver.  Since some applications allocate

all available memory when in operation, this feature may

not function properly.

Example: FBX -D

-S              Since multiple fax servers are supported, this option will

permit you to select a specific fax server.  If one is not

specified at the command line, the first/nearest fax server

will be selected for you.

Example: FBX -S=FAX

-U              This command parameter will force the FBX shell to

remove itself from memory.  If other TSRs have

"hooked" interrupts after the shell has loaded, your

workstation will be denied for unloading.

Example: FBX -U

Programming Information

-----------------------

If you are planning to develop applications for either of these

programming facsimile interfaces you may contact Optus

Software for further details.  CAS and FaxBios development tool

kits are available from Intel and the FaxBios association.

DOS Utilities

-------------

The Command Line Interface (CLI) offers more experienced

users a convenient way to send facsimile data to the

Connect�FACSys server without utilizing the Connect�FACSys

User Interface Main Menu.

Note: You must have access to the Connect�FACSys program files to

utilize this option.

These commands are Connect�FACSys program files.

FAXQUEUE - Server Queue Application

The FaxQueue application lists all the events associated with the

queue that is given as an argument to the application.

Type: FAXQUEUE -<TRLPS>

Options         Description

T               Task or Send Queue

R               Receive Queue

L               Log/History Queue

P               Personal Log/History Queue

S               Server Name

After initiating this command, you'll see a list of events in

reverse chronological order.

To find all of the Receive events on fax server FAX, for

example,

Type: FAXQUEUE -S=FAX R

FAXSEND - A File Send Utility

FaxSend, one of the most useful of the CLI options, permits you

to send various files to the fax card for immediate transmission.

This process takes just a few seconds and then you're free to

continue other processing while the server handles the data

transmission.

The command FAXSEND requires the use of the syntax:

-[option]=[variable]

for each argument used in the string.

For example, to send the file TEST.DCX to the phone number

4105551212

Type: FAXSEND -F=TEST.DCX -T=4105551212

FAXSEND Options

Option  Description

Q       Quiet mode. No copyright information or immediate

status is given. Users wishing to incorporate this utility

into DOS batch files may wish to use this feature and

check transmission status at a later point

H       High resolution mode is enabled (default).  Your text and

graphics images are sent at 200 x 200 dpi. This feature is

not applicable to binary file transfers.

L       Low Resolution mode. 100 x 200 dpi.

P       Indicates that the sending file is in PCX format.

D       Indicates the DCX format. The DCX format is the format

most popular fax boards use for received fax images. A

DCX file is nothing more than a conglomeration of

smaller PCX pages.

N       Indicates normal text format, 80 columns by 66 lines.

A       Indicates ASCII text.

T       Indicates the phone number.

C       Condensed text format. This option gives an alternate font

of 88 lines per page by 132 columns.

B       The binary flag indicates that the file transfer mode of the

Intel facsimile hardware will be active. When selecting

this option, all other flags become invalidated. The file

transfer option must be utilized with other Connection

CoProcessor or compatible hardware.

S       Select fax server. The variable is the fax server name.

F       Indicates the file name.

I       Indicates Input_file name when sending either multiple

files or to multiple destinations.

To send a file in low resolution mode via fax server FAX, for

example,

Type: FAXSEND -F=[filename] -T=[telephone number] -S=FAX -L

To use an Input file, please use the following format for that file.

You can use a standard text editor to create the following file

(format):

[file]

c:\config.sys

f:\user\tom\pcx\signatur.pcx    g:\faxlink\logo.pcx

[to]

12125559384

0114434029303

GATESEND - File Send Utility via MHS Capable of PCL Conversion

GATESEND is somewhat similar to FAXSEND. The principal

difference is that it allows for the sending and processing of PCL

files.  This occurs because the jobs are sent through the MHS

structure in which Connect�FACSys processes PCL attachments

(as it does for MHS e-mail PCL attachments).  GATESEND uses

the format /[option].

GATESEND Parameters

Parameter       Description

I               Indicates Input_file name when sending

either multiple files or to multiple

destinations.

N               Indicates the file name when only one file

is going to be faxed.

T               Indicates the TO: MHS e-mail address.

F               Indicates the FROM: MHS e-mail address.

S               Indicates the Subject.

Q               Indicates "Quiet" mode; no copyright and status

information will be displayed.

To send the file c:\config.sys from Mary via the

Connect�FACSys gateway which is a valid MHS gateway, type:

GATESEND /n=c:\config.sys /t="FAX @ FACSys {fax:14153872109}" /f="Mary"

To use an Input file, please use the following format.  You can

use a standard text editor to create the following file (format):

[to]

- @ FACSys {fax: 12035552130; to: Kathy Jones}

- @ FACSys {fax: 2903432; to: XYZ Big Company}

[file]

c:\config.sys

f:\faxlink\user\lookup.pcx

h:\wordprc\laser.pcl

FAXLOGO - Logo File Creation Utility

The FAXLOGO utility is used to create logo files for the cover

page of your fax transmissions.

Type: FAXLOGO  <FILENAME>  <Logo.pcx> and press ENTER

Options:        FILENAME

The filename argument to this application must be a valid DCX

file. A logo file will appear on the first three inches of the cover

page. The actual logo file is a standard PCX file. You may create

your own images via applications like PC Paintbrush.  The image

must fit within the size (in pixels) of 1700 wide by 600 high.

Note: You must run at 200 x 200 dpi and 100% scaling on a scanned

PCX image to insure the logo file is correct in size to fit on the

cover page.

As an alternative to a scanner, you may send a facsimile of your

company letterhead/logo to your own fax server. This permits

you to make use of your facsimile machine as a low cost 200 x

200 dpi scanner device. However, intricate logos may not appear

as precise and should be converted with a more appropriate

scanner device.

To use your fax server to make your cover letter logo, do the

following:

1. If you are sending your logo through your fax machine, you

must first receive the file. If you are using a scanner,

proceed to Step 6.

2. From your workstation, enter the Connect�FACSys User

Interface Application of Connect�FACSys and go to the File

Menu. From here you will enter the Receive File function

and find the unread fax in the Current Queue Entries box.

3. Save this fax to a DOS file by placing a destination path in

the destination box, then press Save. Now, exit the

Connect�FACSys Windows Application.

4. To change the file extension from .dcx to .pcx type the

following at your DOS prompt:

Type: FAXLOGO LOGO.DCX LOGO.PCX

Note:  the LOGO.PCX file must be black and white and it

can be no larger than 1728 x 800 pixels.  You can use

DMAN or PC Paintbrush to determine the size.

5. You will be asked "Which page to select:".  Enter 1

6. Now, take your PCX file and copy it to the FAXLINK

directory.

9. From FAXLINK go to the INSTALL

10. Now, under the configuration menu options, type your logo

file name within the logo field provided.

11. Reboot your system. You will now have the company logo

on the head of your facsimiles.

Personal Logo File

------------------

Users can select a personal logo file instead of the system default

logo file. Begin by creating your PCX logo file. Then save the

file in the FAXLINK user directory.

Example:

F:\FAXLINK\USER\USERNAME.PCX

Note: If a GammaFax board is installed, the file must be in TIFF

format.

FAXSIG - Signature Utility

--------------------------

The FAXSIG utility lets you create a copy of your signature to

place in your word processing file. All of your fax transmissions

can then be sent with an authentic signature. The instructions

below will help you to extract your personalized signature from a

fax that you send to your fax server.  You will create and store

your signature as a small graphic image compatible with virtually

any application that supports the PCX file format.

To make your signature graphic image follow the steps below:

1. Sign your name in the upper left corner of an 8.5 x 11 inch

standard page leaving at least a one inch margin between the

edges of the page and your signature.  Use a heavy ball

point or felt-tip marker so that your signature will remain

clear.

2. Send this document through a standard fax machine to your

Connect�FACSys server/gateway.

3. Next, from your workstation, enter the Connect�FACSys

User Interface and go to the File Menu.  From here enter

the Receive File function and find the unread fax in the

Receive Queue from the File Menu.

4. Save this document to a DOS file by placing a destination

path in the destination box, then press Save.  Now, Exit the

Connect�FACSys User Interface.

5. Next, execute the FAXSIG.EXE utility at the DOS prompt

to create a file with your signature in a format compatible

with most word processing applications.  Type this

command:

FAXSIG <FILENAME> <NEWFILENAME.PCX>

The [FILENAME] in this command is the received unread

fax saved as a DOS file, and the NEWFILENAME.PCX is

the signature in a PCX file format for word processing

applications that can support the PCX format for graphic

images.

6. Copy the NEWFILENAME.PCX to your private word

processor work directory so it cannot be used by other LAN

users.

7. You can now load the NEWFILENAME.PCX into your

document. Consult your word processor manual for

information on importing a PCX file into your document.  If

you will use your signature frequently, you may want to

create a macro to simplify this procedure.

FAXEXP - Connect�FACSys Export Utility

--------------------------------------

FAXEXP is an 11k export utility that is capable of exporting

logged events from Connect�FACSys to a CSV (Comma

Separated Value) file.  This can be used in a batch file or called

from within a database application to automatically check a log

queue for transactions from within your own applications.

Type: FAXEXP /F=FILENAME /[options]

Options:

/N      Name

/T      Time

/S      Fax Server

/D      Duration

If a sort option is not specified, TIME will automatically select as

the default option.

Example:

FAXEXP REPORT.CSV /N

Using the FAXEXP utility with this sort option exports the same

logged events that are exported when you enter the QUEUE

MENU, select MANAGEMENT REPORTS, then select Query

Options, and finally, select User Name as the sort option.

Example:

FAXEXP REPORT.CSV /S=FS1 /N

This utility selection sorts and exports the logged events on fax

server FS1 by user name.

FAXLOG - Connect�FACSys Export Utility

--------------------------------------

FAXLOG is similar to FAXEXP.  It exports logged events from

Connect�FACSys to a CSV (Comma Separated Value) file.

FAXLOG, however, tracks the last event exported from

Connect�FACSys to the CSV file. Subsequent utilization of

FAXLOG exports the logged events that were transacted since

the previous export by FAXLOG.

To export and flag logged events by user name on server FS1,

for example,

Type: FAXLOG FILENAME /S=FS1 /N

FAX - DOS User Interface

------------------------

FAX.EXE executes the Connect�FACSys DOS User Interface.

Type: FAX

Options:  None

INDEX

-----

Index.exe has been replaced by PDIR (see PDIR in this section)

FLIST - List Available Fax Servers

----------------------------------

FLIST provides a user a list of all available fax servers and their

related file server.  To view a list of available fax servers,

Type: FLIST

SET PREFERRED FAX SERVER

------------------------

(See "Fax Server Selection" for additional usage)

You may now use the SET environment variable command to

select a particular fax server as the preferred fax server. To set

the environment variable to select fax server FAX as the

preferred fax server, for example,

Type: SET FAXSERVER=FAX

MAKEDCX - Make a DCX file from PCX files

----------------------------------------

The MAKEDCX program helps you create a CAS compatible

DCX image from existing PCX files which have been received

via fax or scanning device.  A DCX image is nothing more than

multiple PCX images stored in a single file.  MAKEDCX creates

one output file from existing PCX files.  To use the MAKEDCX

application, type the following command:

Example:

MAKEDCX MYFILE.DCX FILE1.PCX FILE2.PCX

The above example creates the MYFILE.DCX image containing

two pages composed of the two file arguments passed in the

argument list. You may pass as many arguments as you like and

may repeat the MAKEDCX command as often as you like.  You

cannot exceed 1024 pages.  When creating your .PCX image

files, make certain that your image resolution is in 200 x 200 dpi,

and that your image does not exceed 1728 pixels in width.

Subsequent usage of this program on an existing DCX image will

append the file arguments to the end of the file.  As an option,

you may select the -V(Verbose) mode from the argument list to

display each file name as it is processed.

Once completed, the DCX image may be sent via the FAXSEND

or FACSYS software applications.

BREAKDCX - Break apart DCX files

--------------------------------

The BREAKDCX application helps you break apart DCX files

(received facsimile images saved to a disk file) into PCX

components. Each DCX image file is actually composed of

smaller PCX pages.

When using this utility, you must provide the path/name of the

saved DCX file and press return.

Example: BREAKDCX FILE.DCX

Assuming that the received file (FILE.DCX) was composed of

three facsimile pages, the BREAKDCX utility will create three

smaller .PCX images i.e., FILE1.PCX, FILE2.PCX,

FILE3.PCX.  These PCX image files can be used by a variety of

graphics applications.  When presenting the file argument, the

DCX file extension is not necessary as the utility will assume this

file extension if it is omitted.

As an option, you may select the -V (Verbose) mode from the

argument list which will display each file name as it is processed.

PDIR.EXE - Phone Directory Utility

----------------------------------

The PDIR utility is a phone directory maintenance application

and integrated indexing and conversion utility which can be

invoked in an application mode or in a command line mode.

Upgrade a Phonebook

-------------------

Upgrading your phonebook is a two step procedure.  First, you

must Convert [/C] your existing phonebook and then Import [/I].

Upgrading your phonebook can be performed in application

mode or in command mode.  There are more options available

from the application mode, as identified in the following table.

Application             Command

Mode                      Mode          Function

---------------------   --------------- --------------------------------

Yes                     No              Create phone directories

Yes                     No              Add/delete users

Yes                     No              Add/delete groups

Yes                     No              Maintain group memeberships

Yes                     Yes             Import phone directories from

ASCII files

Yes                     Yes             Export phone directories to

ASCII files

Yes                     Yes             Re-build (index) existing phone

directories

No                      Yes             Convert and export older phone

directories

PDIR Command Line Parameters

----------------------------

When utilizing the PDIR utility in command mode, users and

system administrators can easily perform import/export from

other existing databases utilizing the ASCII CSV (Comma

Separated Value) format. Each of the command line operations

are discussed below.

To utilize PDIR in command mode, enter one of the following

optional switches followed by the phone directory name and

optional import/export ASCII file name.  In all examples, phone

directory extensions of .PBF are assumed if not specified at the

command line.

Option  Description

/C      Convert and export phone directory (Required for Upgrading

Phonebooks)  The convert option is only used for directories

created in earlier versions of FACSys. This is similar to the

export function, but all telephone numbers in "free-form" will

be converted to the newer structured or "Canonical" dialing

format required for Connect�FACSys version 3.40.

Note: Backup your phonebook files to a different

directory before converting them.

Convert Example:  PDIR /C PERSONAL_PHONEBOOK_NAME

Creates a .TXT file of your phonebook. Once completed, and

you have backed up your phonebook files, delete your

PERSONAL_PHONEBOOK_NAME.PBF and

PERSONAL_PHONEBOOK_NAME.IDX files.  Your phone

directory dialing numbers will be converted as follows:

Original                Exported

1-908-271-9572          1,908,2719572

271-9572                ,,2719572

An attempt will be made to decipher all local, long distance,

and inter-country calling parameters (i.e. 01, 011, ...).

The input files for this option may be CAS compatible phone

directories of other file formats support by earlier versions of

the FACSys product.

/I      Import phone directory (Required after running /C to upgrade

phonebooks).  This option will create/import/merge phone

directory information from an ASCII import file.

Import Examples:

PDIR  /I  PERSONAL_PHONEBOOK_NAME  (PERSONAL.TXT assumed)

PDIR  /I  PERSONAL_PHONEBOOK_NAME  OTHER.TXT

This will create a PERSONAL_PHONEBOOK_NAME.PBF

and PERSONAL_PHONEBOOK_NAME.IDX file from the

PERSONAL_PHONEBOOK_NAME.TXT file.

/E      Export phone directory

This option will export an existing phone directory to an

ASCII export file.

Export Examples:

PDIR  /E  PERSONAL_PHONEBOOK_NAME (PERSONAL.TXT assumed)

PDIR  /E  PERSONAL_PHONEBOOK_NAME  MYFILE.TXT

/R      Re-build phone directory index

To rebuild a corrupted phone index, this option may be used

from the command line if index corrupting prohibits one of

your phone maintenance applications from opening a selected

phone directory.

Example:

PDIR  /R  PERSONAL_PHONEBOOK_NAME

ASCII File Format

-----------------

The following fields are used when exporting and importing

phone directory data:

Field Name            Field Length

--------------        ------------

First Name              18

Last Name               18

Company Name            31

Matter Code (Billing)   21

Notes                   31

Fax Country

/Long Distance Code      5

Fax Area Code            5

Fax Local Number        13

Voice Country

/Long Distance Code      5

Voice Area Code          5

Voice Local Number      13

Examples:

Jones,John, ABC Co,12345, Notes re: John, 1, 908, 555-1212, 1, 908,555-1313

Smith,Mary, "Optus Software, Inc.", 12346,,1,908,555-1414,1,908,555-1414

Note: Normally, ASCII text is exported without use of the double quotes

(") except when names contain the comma (,) character.

Adding Groups

-------------

Groups can be created by entering the name of the group (limited

to 31 characters) by placing the name within the square brackets

([ ]). You may optionally add members to the group during

import by adding the BEGIN/END statements immediately

following the group name.

Example 1:

Jones,John,Infinite Technologies,...

Smith,Mary,"Optus Software, Inc."...

[Sales]

[Everyone]

This example creates two person entries and two group entries

(with no members) within the phone directory during import.

Example 2:

Jones,John,Infinite Technologies,...

Smith,Mary,"Optus Software, Inc."...

[Sales]

begin

Jones,John,Infinite Technologies

Smith,Mary,"Optus Software, Inc."

end

[Everyone]

This example create the same entries as Example 1, except the

two person entries are added to the membership list of group

"Sales". While all three fields are not required, the import logic

of the PDIR utility will attempt to identify the record object

created within the phone directory by Last, First, and Company

name. For example, you could omit the company name or person

name if you were certain that no two individuals or company

name entries existed in the phone directory.

All groups should be located after the last person entry in the

import ASCII file.

DOS TSRs

========

TSR Print Redirector

--------------------

The print redirector for DOS based applications is a TSR that

permits workstation users to print directly from their DOS based

applications. (Windows users should use the Windows Printer

Redirector found starting in the next section of this manual) The

print redirector permits DOS applications to seamlessly "re-

direct" the print output to the fax server print queue, or return

printing control to their local/network printing device.

By design, the TSR will "pop-up" when the user either invokes

the TSR via a Hot-Key, or when the user prints from an

application and the TSR has redirected the output to a fax server

assigned print queue, generally PRINTQ_FAX based upon the

user's specific request.

Before Loading the TSR (see Loading The TSR below), we

suggest that you read through the following field descriptions

briefly. Then, select the keystroke combination from the options

list which you would like to use to invoke the TSR, as well as

any other options you would like to use. The default Hot-Key is

ALT-ESC.

Note: You must have created a phonebook, setup the Startup options, and

have at least added (and/or upgraded if using earlier versions of

FACSys) a minimal number of phonebook entries from the DOS or

Windows front-ends as described in this documentation before

using the DOS TSR or Windows Printer Redirector.

Several "controls" on the TSR screen have specific functions and

provide a unique means of operation which are described as

follows:

Phone Directory

---------------

The Phone Directory list box control always lists the current

selected phone directory. (See /P Option, below, for alternate

phonebook choices.)  It displays the names (either Last Name,

First Name, or Company Name, based upon the designation in

the phonebook entry) in alphabetical order with no regard to case.

Upon selection of a phone directory entry, the Person Entry box

(immediately below the Phone Directory list box) displays the

current record from the phone directory database.

Note: You may modify the information within the Person Entry field

prior to adding it to the Destination List.

Destination List

----------------

The Destination List displays the targeted recipients of the fax.

To create a Destination List, select a Phone Directory entry and

press the F1 key. Continue the process until you have completed

your Destination List. You may also manually enter a destination

which is not in the phonebook. TAB to the Person Entry box,

enter the appropriate information, and press the F1 key.

To remove an entry from the Destination List, select the entry

and press the F9 key.

Person Entry

------------

The Person Entry box is composed of several "edit" controls

which permit the user to enter alphanumeric information and

perform minor text editing.

Option          Description

--------------  -------------------------------

Name            The name of the recipient is displayed/entered in Last

Name, First Name order. (Optional field if Company is

entered)

Company         The name of the Company is displayed/entered (Optional

field if Name is entered). First Name Last Name will

appear on the fax.

Fax             The Country Code, Area Code, and full fax number are

displayed (Mandatory field). You will TAB in these three

fields.

Voice           The full Voice number is displayed/entered. (Optional

field)

Billing         The Billing code for that entry is displayed/entered.

(Optional field)

You may modify information within these fields and add the

Person Entry to the destination list and save the record

permanently to disk.

To create a new phonebook person entry, clear the entry within

the Person Entry box by pressing the F9 key.  This will tell the

TSR to create a new phone directory entry.  Enter the

appropriate information and press the F1 key when you are done.

To Save the new Person Entry to the phonebook, press the F2

key.

Note: If you enter a person's last name, first name, and company name,

and would like the company name to show up in the Phone

Directory after saving it to disk, you must select "Use company

name in list" for that entry. This can be selected using either the

DOS or Windows front-end user interface, and editing the phone

book entry from the Phone (from the main title bar), Edit functions.

Note: After saving an entry to disk, you must exit and re-invoke the TSR

to see the new entry in the Phone Directory list. You do NOT have

to unload and re-load the TSR (as you had to with earlier versions

of FACSys).

Scheduler

---------

The Scheduler box is comprised of several controls which permit

the user to schedule the Date and/or Time of the fax event,

Schedule Immediately (the default), and/or include a Cover Page.

Control         Description

--------------  -----------------------------

Date            Is selected by the use of the UP/DOWN arrow

keys. HOME will reset the field to the current

date.

Time            The hour and minute are set in this field by use

of the UP/DOWN arrow keys. HOME will

reset the field to the current time.

Immediately     override the Date and Time and schedules the

fax event for an immediate send. Use the

SPACE bar to toggle the checked state of this

control. This is the default setting.

Send Cover Page Determines whether or not the cover page will be

included with the fax transmission. Use the

SPACE bar to toggle the checked state of this

control.

Cover Page Message

------------------

The multi-line edit control permits the user to enter up to 1024

bytes (characters) of information to be included on the fax Cover

Page area. The Cover Page check box should be checked (see

above) if the cover page is to be included with the transmission.

Pressing the F9 key within this box clears  the contents of the

message box.

Sending a Fax

-------------

To send a fax, the TSR application must capture the print output

and direct it to the fax server print queue.  The user controls this

feature with the F3 key.  The lower right hand corner of the TSR

indicates whether or not fax capturing is enabled.  If the fax

capture is enabled, the TSR application "pops-up" when the user

prints from the DOS based application.  Otherwise, the printed

output resumes its normal path to the selected printer device.

For version 3.30b and earlier users, this replaces the ON/OFF

toggle switch.

When the TSR "pops-up" and you want to send the print job to a

printer instead of as a fax, press F3. The lower right hand corner

of the TSR now indicates PRN. Press ESC and the former print

queue will be restored; jobs will be printed.

Quick Fax - Cover Page Only

---------------------------

When you have scheduled the appropriate destinations and/or

Cover Page Message, you can simply press the F10 key to

transmit the Cover Page with the included text.

Loading The TSR

---------------

There are many different options available to the user when

loading the TSR so that it may be customized for each user's

needs and/or workstation configuration. Any of the following

options may be used in combination with others. To load the TSR

with the options,

Type: F:FACSTSR /option= /option= /option= ...

Option          Description

--------------- ---------------------------

/C              This command option forces the TSR to utilize Conventional

Memory for loading phone directory entries. By default, the

TSR will use XMS, EMS, and then Conventional.

/E              Force the TSR to utilize Expanded Memory.

/K              Control the TSR hot-key sequence. There are many

keystroke combinations which you can define to invoke the

TSR via a Hot-Key. The following arguments are available

when loading the TSR:

Key     Value

A       ALT

L       Left Shift

R       Right Shift

C       CTRL

Example:

FACSTSR /K=AL:ESC  Hot Key = ALT+LSHIFT+ESC

Example:

FACSTSR /K=A:F10  Hot Key = ALT+F10

/L              Specify the local printer port that the TSR will capture. You

may select ports 1-3.

Example: FACSTSR /L=2  Capture LPT2

Note: LPT1 is the default.

/M              Force monochrome color palette. This is useful for

workstations that have LCD displays and don't always

provide flexible shades of gray.

/P              Specify the phone directory used when the TSR is loaded

into memory. The full drive and path should be used to

avoid problems with drive and/or directory changes.

Example:

FACSTSR /P=C:\DATA\SUSAN.PBF

The .PBF extension is not required.

Note: This option is required to load the TSR.

/Q              Specify the print capture queue and optional server name.

Example:

FACSTSR /Q=FS1/PRINTQ_FAX   or FACSTSR /Q=PRINTQ_FAX

Note: If this option is not used, the TSR will attempt to

locate PRINTQ_FAX on the "current" file server by

default.

/U              Unload the TSR from memory.

/W              Monitor all print queue capture changes (i.e.

CAPTURE.EXE) and update print capture when restoring

the print re-direction (i.e. F3 toggle).

Windows Printer Driver

======================

Overview

--------

Note: You will need to set the DOS LOCALAREA environment variable

to fully use Connect�FACSys, (see set LOCALAREA in the

Connect�FACSys User Interface Phone Menu section).

Connect�FACSys supports the use of a printer driver for the

Hewlett Packard Laser Series II.  It mimics the DOS TSR in the

sense that it automatically "pops-up" when a print job that is to

be sent as a fax is invoked.  (See "Using the Connect�FACSys

LaserJet II Printer Driver".)

Note: You may select Use Person Name order preference if you prefer to

sort your phonebook by Company Name and have the Person Name

appear on the fax. The use Company Name in the phone entry must

be selected on an individual basis.

Installing the LaserJet II Printer Driver

-----------------------------------------

The Connect�FACSys LaserJet Series II printer driver is an

unlisted Windows printer driver and should be installed

accordingly.  You may install this driver from your Windows

Control Panel by selecting the Printers icon and clicking on the

Add button.  Then, from the displayed list, choose to "Install an

Unlisted or Updated Printer."  You will be prompted for your

diskette.

1. Highlight the driver, "FACSys HP LaserJet Series II", and

connect it to a local port.  Note: You may attach this port to

any LaserJet type device available on your network.

2. Select Setup.

3. Select Fonts.

4. Select Add Fonts and enter the installation path.

5. Add the FACSys cartridge font information which is found

on your Windows Client Application Disk.

6. Select the FACSys cartridge.

7. Optionally select the "FACSys LaserJet Series II" device as

the default printer.

Using the LaserJet II Printer Driver

------------------------------------

The Connect�FACSys LaserJet II driver is a regular Windows printer

driver in that it successfully meets the objective of translating

Windows GDI calls to PCL4 format. It is different from

conventional printer drivers in one major respect: When faxing,

it dynamically attaches the printer driver to the NetWare print

queue PRINTQ_FAX, and then restores the connection back to

whatever was previously set up. This feature, similar to that of

the DOS TSR, allows a user to conveniently send faxes. The

FACSys LaserJet Series II driver is operated by the following set

of (icon) buttons:

Note: The Connect�FACSys Redirector icon is now automatically

minimized for DDE support. For DDE usage, please consult your

Windows application documentation.

From Field      Enter the name which you would like to have appear in the

FROM: field of the (optional) fax cover page. This name will

remain in the FROM: field on faxes sent from the

Connect�FACSys Redirector until this box is manually changed.

Name,           If you have selected an entry from a phonebook (Destination),

Last Field      text will appear in this field. If you would like to send a

fax to a destination which is not currently saved in a

phonebook, or would like to edit the text, enter the last

name of the person which will show up on the cover page of

the fax.

If you edit the information in this box - or any of these

Destination related data boxes, the Edit box will change to

Update. To save the change, click Update.

Note: Names on the cover page TO: field will appear in First

Name, Last Name order. This is also what will appear in the

CSID line of the fax, unless no First and Last Name is

entered. If no First Name and Last Name is entered, and a

Company Name is entered, then the Company Name will appear in

the CSID line of the fax.

Name,           If you have selected an entry from a phonebook (Destination),

First Field     text will appear in this field. If you want to send a fax

to a destination which is not currently saved in a phonebook, or

would like to edit the text, enter the first name of the

person which will show up on the cover page of the fax.

Company Field   If you have selected an entry from a phonebook (Destination),

text will appear in this field. If you would like to send a fax

to a destination which is not currently saved in a phonebook,

or would like to edit the text, enter the company name which

will show up on the cover page of the fax.

Fax Field       If you have selected an entry from a phonebook, text will

appear in this field, if it was saved in the phonebook record.

If you would like to send a fax to a destination which is not

currently saved in a phonebook, or would like to edit the text,

enter the fax number to which the fax will be sent.

Phonebook

=========

The name of the phonebook which is displayed is the last

phonebook that was used.  Generally, this will either be that of

the user who is using Connect�FACSys, or the System

phonebook, whichever is the one chosen as the default

phonebook (See Startup Options). To select a different

phonebook, click on the Open button.

Add

---

If you would like the new entry added to the

phonebook, select the Edit/Update button (as

described above in Name, Last Field). If you

would like to add destinations to the phonebook

without having entered data in the Destination

related boxes, click on the Add button. This

opens the PhoneBook dialog box. You may then

enter the remaining data such as Billing Field,

Notes Field, and Phone Number Field. When

you are done editing, select the OK button to

save the record and close the dialog box.

Note: We recommend that you first query the

Destination list box before saving a new record

to see if you have the record saved already.  It's

best not to save multiple entries for one

destination such as Optus Software, Optus

Software, Inc., and Optus, which could all be

the same destination.  Multiple entries waste

space in your phonebook.

Edit

----

If you would like to save the modified entry,

click the Edit/Update button. If you have not

made any changes to the data in the Destination

related boxes and would like to change

Destination information, click on the Update

box. This will open the PhoneBook dialog box.

You may then modify the remaining data such as

Billing Field, Notes Field, and Phone Number

Field. When you are done editing, select the

Edit button to save the record and close the

dialog box. For more information, see

Phonebook Editing.

Group

-----

To create, edit, or delete groups or group

member information, click on Group. If you

have highlighted users from the Destination list

and click on Group, you will have the option of

saving that list of users as a new group.

Destination

-----------

This list box displays the saved records in the

phonebook displayed in the PhoneBook box.

You may select a single or multiple

destinations from this list box by highlighting

the entries you need.

Cover Page

----------

You may optionally send a cover page with the

fax. To include a cover page, click on the

Include button. To edit the cover page, click on

the Edit button.

Note:  You have the option to select Save Cover

Page in the preference dialog.  If you do make

this selection, the cover page text remains the

same until edited/changed.  Additionally, you

may Recall previous cover pages by clicking on

the Recall button while in the Cover Page

Message dialog box.

Schedule

--------

If you would like to schedule the fax for a later

transmission time, select the Schedule button.

The schedule dialog box will appear. Edit the

date and time fields to display the date and time

that you would like the fax to be transmitted.

Click OK to select the modified time, or Cancel.

Fax

---

Click on the Fax button to send the fax.

Print

-----

Click on the Print button to print a hard copy of

the intended fax output to a printer. This will

print a hard copy instead of sending it as a fax.

The printer to which the output will be printed is

the one connected (defined/assigned) to the

FACSys driver.

Cancel

------

If you would like to exit the Connect�FACSys

Redirector without sending a fax, select the

Cancel button.

Phonebook Editing

=================

Phonebook

---------

The name of the phonebook displayed is the last phonebook used.

Generally, this is either the user's phonebook or the System

phonebook, whichever is the default. (See Startup Options). To

select a different phonebook, click on the list box which displays

General, Personal or System. Select General to select any

available phonebook. Please note that all saved information is

written back to the FACSYS.INI file.

Person List

-----------

All of the records saved to this phonebook will be

displayed in Last Name, First Name order.

Note:  If you select the "Use company name in list"

option, the company name will appear in the alphabetized

list.

Name, Last      If you have selected an entry from a phonebook (Person

Field           List), text will appear in this field. If there is no

data in this field, enter the last name of the person

to whom you will transmit a fax.

Note: If you edit the information in this box - or any of

these Destination related data boxes, the Edit box will

change to Update.  To save the change, click on Update.

Name, First     If you have selected an entry from a phonebook (Person

Field           List), text appears in this field. If there is no data

in this field, enter the first name of the person to whom

you will transmit a fax.

Company         If you have selected an entry from a phonebook (Person

Field           List), text appears in this field. If there is no data in

this field, enter the company name to which you will

transmit a fax.

Billing         You may optionally enter a billing code for this entry.

This may be used for importing data into an accounting

or chargeback system.

Notes           Enter notes for this entry.

Fax Number      Enter the fax number.

Voice Number    Enter the voice phone number for this entry.

Remove          To delete an entry from the phonebook, select the

Remove button.

Add             If you would like the new entry added to the phonebook,

select the Add button.

Edit            If you edit the information in any of the Destination

related data boxes, the Edit box will change to Update.

To save the change, click Update.

Close           To close the PhoneBook dialog, select the Close button.

Group Entry

===========

Group           The list box displays the last used group, or the group

selected in the Destination list.

To select a different group, press the Down Arrow Key to

the right of the Group box.  All available groups will be

displayed.  Select the desired group.

To add a new group, type in the name of the new group in

the Group box and click on the OK button.

To delete a group, highlight the desired group and click on

the Remove button.  You will then be prompted Yes or

No to confirm the deletion of the group.

Note: All users in the group must be de-selected before

clicking on the Remove button, or the selected users will

be deleted from the group instead.

Group           This list box displays current members of the selected

Members         group.

Available       The list box displays all users saved in the current

Members         phonebook which have not yet been added to the current

group.

OK              To save the name of a new group, click on the OK button.

Cancel          To exit the Group Maintenance dialog, click on the Close

button.

Add             To add Available Members to the group, select the users

desired, and click on the Add button.

Remove          To remove users from a group, select the users from the

Group Members list box, and click on the Remove button.

Troubleshooting Guide

=====================

The following is a list of the most commonly asked questions.

Why can't I initialize my Connect�FACSys server?

* The user name for the fax server is incorrect.

SUPERVISOR equivalence is not required in the CAS

manager description, CASDRV.EXE.

* You must utilize an 80286 (or greater) based computer

workstation with 640KB of system RAM.

* You must correctly install and configure your network

adapter and your fax board with the proper DMA or

interrupt, and I/O Base Address parameters.

* You must load the CAS manager prior to loading the

FAXLINK server application.

CASDRV.EXE

* If you are encountering errors while loading either the CAS

manager or FAXLINK applications, refer to the  Installing

Connect�FACSys Server section of this manual.

* Incomplete or incorrect entries in your FAXLINK.INI file.

Why can't I print my application documents (WordPerfect,

Lotus, etc.) to Connect�FACSys?

* You must load the TSR Hot-Key (i.e. FACSTSR.COM or

FAXTSR.COM) application which handles your print re-

direction and telephone number for sending documents

before loading your application such as WordPerfect and

Lotus.  Hint: Load the TSR from the user's

AUTOEXEC.BAT after logging into the network.

* Your applications must be set up to print to LPT1: or

PRINTQ_FAX using the TSR to insert the phone number in

the print job.  Note:  If you choose to print directly to

queue, this will disable the large TSRs on/off button.

* You must set your print jobs for (PRINTQ_FAX) to BYTE

STREAM. This is necessary under NetWare when sending

fonts and graphics to any network printer.

Why can't my received faxes print through the AUTOPRINT

utility?

* You must enable AUTOPRINT within the FAXLINK.INI

file which resides under the FAXLINK directory.

* You print to a LaserJet II or LaserJet II compatible printer.

The printer requires 1MB since the print job may exceed

750KB per page.

* You do not have the correct server/queue in the

FAXLINK.INI file.

* If the OCR option is selected, you must enable the users

AUTOPRINT option in the Router Table.  See

Manger/supervisor option/user Router Table.

Why do I get a "CAS SUBMIT TASK ERROR" when

transmitting a PCX image through Windows?

* When transmitting a PCX file created on a scanner, be sure

it was made at 200x200 dpi at 100%.

* Also, the image cannot be color.

Why do I get "Connect�FACSys is Unregistered" when I try

to initialize the fax server/gateway?

* Has your Connect�FACSys temporary registration expired?

You must fill out the Connect�FACSys license form and

return it to Infinite Technologies for permanent registration.

Why do I get "Cannot open phonebook" when I load my TSR

personal phonebook with FACSTSR-P?

* You may encounter this error if there were no entries listed

in the phonebook. You may check this under the Phone

Menu.

Why do I get "Cannot open phonebook" when I load the TSR

system phonebook executing FACSTSR?

* You may encounter this error if executing the incorrect

command for the system phonebook, the syntax should be,

FACSTSR -S.  Refer to the manual for additional

information  for using a TSR under graphics mode.

Why does my computer with the VGA screen work under

Windows, yet my EGA does not?

* You must run the Microsoft Windows installation procedure

for the different graphic adapters your LAN currently

supports.

* Is the computer with the EGA adapter operating

Connect�FACSys from within the appropriate directory

where its graphics drivers reside?

Why does my fax server/gateway receive faxes after I set

FAXRING to 0?

* If you rebooted or reinitialized the fax server/gateway since

executing FAXRING to not receive calls, the system will go

back to the default of 1 which will enable it to receive calls.

To be sure the default ring count remains to 0, edit the

FAXLINK.INI file which resides under the FAXLINK

directory and set FAXRING=0.

OCR scan is not working?

* The FAXLINK.INI file which resides under the FAXLINK

directory has not been edited to enable the OCR scanning

capability. Use a text editor and modify the FAXLINK.INI

file to enable the OCR scanning.

* Check the document to insure the characters are surrounded

by double parenthesis ((235)).

* Be sure to have the file CHARACTER.BMP residing in the

FAXLINK directory.

* You must set up a user Router Table.  This is done by going

into the user interfaces.  See the Manager section of this

manual.

Server Errors

-------------

The following is a compilation of Connect�FACSys Server

Errors.

Why do I get PCL Errors at my Connect�FACSys

server/gateway while trying to initialize it?

PROBLEM: If the soft fonts listed in the FAXLINK.INI file are

not in the FAXLINK\FONTS directory, an error code PCL

ERROR 0xFE will appear.  The error will appear for each

missing font.

SOLUTION: These files reside on the INSTALL diskette and

may be manually copied into the FAXLINK directory if they are

not already present.

Why do I get PCL Errors at the Connect�FACSys

server/gateway while print jobs are in PRINTQ_FAX?

PROBLEM: There are different PCL errors to be considered

while the most common are due to an incorrect printer driver

from an application such as 0xCA which is an "illegal escape

sequence".

SOLUTION: Be sure to have a LaserJet II printer driver selected

from within your application before attempting to fax.

Windows Application Errors

--------------------------

Cannot Open Printer

PROBLEM: If your workstation does not have a hard disk,

Windows may generate this error when creating temporary files

while printing.

SOLUTION: Modify the TEMP DOS environment variable to

point to your network hard disk where you have installed

Windows. Example: SET TEMP =F:\WINDOWS You can load

this in the users batch file to the network after the login

command.

Connect�FACSys Server not found

PROBLEM: The workstation could not make a connection with

the Connect�FACSys server.

SOLUTION: Make certain that your Connect�FACSys server is

operational. If further problems continue consult your

Connect�FACSys dealer.

Cannot see a Received Fax

PROBLEM: The user may not have sufficient rights to the

Connect�FACSys application software.

SOLUTION: Be sure the software supplied from Intel is not

resident in any way at the fax server/gateway or on the network

drives.

SOLUTION: Check the Group Information under FAX_ADMIN

to ensure the user is part of the group. This gives the user

managerial rights under FAX_ADMIN. Another way to make a

user see a fax is to manually route it to them through the

Connect�FACSys Application.

Invalid Shell Error

PROBLEM: Windows Version 3.1 requires a network shell

Version 3.X or greater.

SOLUTION: Install Version 3.X network shells or greater to

take advantage of Windows printing and other enhanced features.

Cannot Load NETWARE.DLL

PROBLEM: Windows was installed on a local drive and the

network shell was not resident at the workstation to make

changes to the SYSTEM.INI file for Novell networks.

SOLUTION: Load the shell at the workstation to insure correct

operation in a network environment.

Workstation screen blanks out

PROBLEM: The user is running Windows with a machine that

has a graphics adapter different to the setup parameters of the

Windows software.

SOLUTION: You must run the Microsoft Windows Setup

program for each graphics adaptor your LAN currently supports.

To correct, from the WINDOWS directory, type SETUP/N.

Follow the steps through the Setup process and configure the

parameters for the graphic configurations necessary for that

machine type to operate in its environment.  Example:

T:\WINDOWS\IBMVGA

TSR Hot-Key Application Errors

------------------------------

CAS Submit Error 0282

PROBLEM: This error occurs when data sent to the HP

emulation engine is corrupted. An example is a graphic image in

a document in an application that is not PCL compatible.

SOLUTION: A simple solution is to send a graphics image

included with the application such as WordPerfect graphics and

increase the time out to 20 seconds to verify a successful

transmission.

CAS Submit Error 0281

PROBLEM: Your application could be printing to a print queue

PRINTQ_FAX directly without using the TSR Hot-Key which is

necessary for inserting the phone number to the print job.

SOLUTION: Set the application to print to LPT1: or directly to

PRINTQ_FAX using the TSR Hot-Key for the phone number

function. The TSR Hot-Key must be utilized regardless of where

the application prints. It has 2 primary functions: 1. To insert the

phone number into the print job. 2. To redirect the printing to

either Connect�FACSys, a network printer, or local device.

SOLUTION: The print jobs must be set to BYTE STREAM in

case any fonts or graphics are included in the print job.

SOLUTION: The phone number must be from 4 to 19 digits in

length for a successful transmission.

Print Job Aborted

PROBLEM: You are printing to Connect�FACSys from within

an application (i.e., WordPerfect, Lotus) where the printer driver

is set up for something other than a Hewlett Packard LaserJet II.

SOLUTION: Before printing an application to Connect�FACSys,

be sure to set the printer setup to an HP LaserJet II. Example:

Setting the printer for an HP LaserJet IID or LaserJet III will

cause a PCL Error and not successfully transmit the fax.

File Not Transmitted Successfully

PROBLEM: The file was not found due to incorrect spelling or

path name.

PROBLEM: The FAXSEND.EXE options were issued

incorrectly.

SOLUTION: Check the path name to the file to insure the file is

present.

SOLUTION: The options for FAXSEND.EXE are to be used as

listed:

FAXSEND [FILENAME] [TELEPHONE NUMBER] -QHLPDTNCB]

Why does my TSR keep popping up?

PROBLEM: When printing a large print job containing graphics,

your LPT1 port may time out.

SOLUTION: You can either increase your time out   or print

directly to the queue (PRINTQ_FAX).  Note:  When printing

directly to queue, you will disable the on/off switch in the TSR.

A WordPerfect file was sent by way of FAXSEND and the

output was not in any legible format?

PROBLEM: FAXSEND is a DOS utility to send ASCII, PCX,

DCX, to any Group III facsimile device and Binary to any CAS

compatible fax card.

SOLUTION: In order to send a fax from an application, it first

must be processed by the Hewlett-Packard conversion processor.

This may be accomplished by using the TSR Hot-Key for DOS or

the print filter for Microsoft Windows. Refer to the User's

Manual for further information on the TSR Hot-Key and Print

Filter.