STANDARD MICROSYSTEMS CORPORATION

Novell NetWare 386 v4.x Server Driver

For EtherCard Elite32 Ultra Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide explains how to use the SMC Novell

NetWare 386 v4.x server driver with a Novell NetWare 386 v3.11

server. Specifically, this installation guide describes:

o  How to use EZStart to copy the Novell NetWare 386 version 4.x

server driver from the SuperDisk to a server running Novell

NetWare 386 v3.11.

o  How to load the LAN driver and establish network connection.

o  Novell NetWare 386 v4.x server driver error messages.

o  Novell NetWare 386 v4.x server driver warning messages.

o  Novell NetWare 386 v4.x server driver custom counters.

Note: This driver supports the Novell 32-bit client specification,

which allows the driver to work with Windows NT, UnixWare,

and other 32-bit Intel-based operating systems. Contact

Novell for more information.

COPYING THE DRIVER FILE

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

LOADING THE DRIVER

Installing the LAN driver is part of the NetWare installation

process. Use the following instructions with those

in the NetWare 386 installation manual to install the driver

on a server that is running Novell NetWare 386 v3.11.

Note: When you use any network vendor's 4.x server driver in

a Novell NetWare 3.11 environment, you must use a newer

MONITOR.NLM utility, such as the one provided on your SMC

SuperDisk. Therefore, use the MONITOR.NLM utility provided

on your SuperDisk instead of the one on the Novell NetWare

3.11 diskettes. Otherwise, your server may "lock up."

You will need the SuperDisk during this procedure.

1. Refer to the "Load LAN driver module" section in the Novell

NetWare 386 v4.01 installation manual for instructions on

installing NetWare.

2. Copy the following SuperDisk files to either the SYSTEM

directory on the server or a search path:

o  LSLENH.NLM

o  PATCHMAN.NLM

o  MSM.NLM

o  MSM31X.NLM

o  ETHERTSM.NLM

o  MONITOR.NLM

3. Verify that the SMC driver, SMC8232.LAN, is also located in

either the SYSTEM directory on the server or a search path.

This driver was copied to your Destination Directory when you

copied the Novell NetWare 386 v4.x Server Driver.

4. Follow the instructions in your Novell NetWare documentation

for starting your server.

5. From the command line on the server, type: Load LSLENH.NLM

6. Press the Enter key to load both the LSLENH.NLM and

PATCHMAN.NLM files.

7. From the command line on the server, type: Load MSM31X.NLM

8. Press Enter to load the MSM31X.NLM file.

9. From the command line on the server, type:

Load SMC8232.LAN SLOT=xxx FRAME=nnn

In the above line:

o  xxx = the adapter's slot number, in decimal notation.

o  nnn indicates one of the following frame types:

Ethernet_802.3

Ethernet_802.2

Ethernet_II

Ethernet_SNAP

10. Press the Enter key to load both the SMC driver and

ETHERTSM.NLM files.

11. If you are prompted to specify a slot, select the one

in which your SMC adapter is installed.

Note: A sample .NCF file called 4XON311.NCF is provided

on your SMC SuperDisk. This file handles steps 5

through 8 for you. You can place this .NCF file in

your server's AUTOEXEC.NCF file, so these steps are

performed automatically each time the server starts.

Refer to you NetWare 386 installation manual for

instructions on creating an AUTOEXEC.NCF file.

12. Repeat step 9 and 10 if you are loading a driver for

another LAN adapter or an additional frame type.

Note: The driver has to be bound to start working (refer

to the section "Bind the protocol to the LAN driver"

in the NetWare 386 installation manual).

NOVELL NetWare 386 v4.x SERVER DRIVER ERROR MESSAGES

"Could not find an SMC Ethernet adapter at I/O base

address XXX"

The driver was unable to find an SMC Ethernet LAN adapter

at the specified base I/O address. Check for adapter

conflicts and retry the installation. Check that the adapter

is seated properly in adapter slot.

"Adapter hardware failure detected (initialization)"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test

process due to a hardware problem. Check the adapter

for configuration conflicts.

"Cannot access adapter RAM properly"

The adapter could not read or write the adapter RAM. Verify

that another adapter does not share the same memory space as

the LAN adapter.

"Other system RAM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

other RAM in the system. Reconfigure the adapter to use

a different RAM address and retry the installation.

"Other system ROM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

ROM in the system. Reconfigure the adapter to use a

different RAM address and retry the installation.

"Adapter unable to process a multicast address"

A node address was given on the command line or the board is

configured for a node address that is reserved for multicast

address. Broadcast and multicast addresses are not allowed

as valid node addresses. Use the node address override

with caution as this can cause problems on your network.

"Configuration corrupted; run software setup"

The driver failed to initialize the adapter due to invalid

data in configuration EEROM. Run the software setup utility

and reconfigure the adapter.

NOVELL NetWare 386 v4.x SERVER DRIVER WARNING MESSAGES

"Unneeded Memory Address ignored"

You specified an unnecessary memory address parameter when

loading the driver. A software-specified memory address will

be used instead.

"Unneeded Memory Length ignored"

You specified an unnecessary memory length parameter when

loading the driver. A software-specified memory address will

be used instead.

"Unneeded Interrupt number ignored"

You specified an unnecessary interrupt number parameter when

loading the driver. A software-specified interrupt number will

be used instead.

NOVELL NetWare 386 v4.x SERVER DRIVER CUSTOM COUNTERS

You can use the MONITOR.NLM server utility to view these counters.

TxOKSingleCollision

The number of packets transmitted successfully that en-

countered only one collision during transmission. It is

normal for this counter to increment on busy networks.

TxOKMultipleCollisions

The number of packets transmitted successfully that en-

countered more than one collision during transmission.

It is normal for this counter to increment on busy networks.

TxOKButDeferred

The number of successfully transmitted packets that were

delayed because the medium was busy. This counter may

increment on a busy network.

TxAbortLateCollision

The number of collisions detected after the packet started

transmitting. The packet was not sent successfully due to

a collision during transmit. It is normal for this counter

to increment occasionally due to packet collisions. If

this counter increments frequently, it may indicate a

problem with another NIC adapter on the network.

TxAbortExcessCollisions

The number of packets that were not transmitted because the

maximum collision limit was exceeded (16 collisions). This

counter may occur occasionally if there is high network

traffic.

TxAbortCarrierSense

This counter indicates the number of times the carrier-sense

signal was not-asserted or was deasserted during transmission

of a packet without a collision. This counter may increment

occasionally during periods of high network traffic. If this

counter increments frequently, it may indicate that there is

a cabling problem or a problem with another LAN adapter on

the network.

RxAbortFrameAlignment

The counter indicates the number of packets that were rejected

to an alignment error. The length of the packet did not pass

the Frame Check Sequence (FCS) check. This counter may incre-

ment occasionally due to collisions occurring on a busy net-

work. If the counter increments frequently it may indicate a

problem with another LAN adapter on the network.

AdapterResetCount

The number of times the NIC was reset due to a transmit

timeout error or an unrecoverable error. This counter may

increment occasionally during normal network operation.

If this counter increments frequently, it may indicate that

there is a problem with the LAN adapter.

ReceiveLostPacketCount

This count contains the total number of packets that were

lost due to an overrun of receive buffers. This counter

may increment occasionally during periods of high network

traffic. If this counter increments frequently, it may

indicate a problem with the LAN adapter.

TransmitFIFOUnderrunCount

This counter indicates the total number of times that the

NIC could not access the bus before the FIFO buffer emptied.

Transmission of the packet is aborted when this event

happens. This counter may occasionally increment on a busy

network. If this counter increments frequently, it may

indicate a problem with the LAN adapter.

TransmitTimeoutCount

The number of times a transmit complete interrupt did not

occur within 2 seconds of starting a packet transmission.

When this occurs, the NIC is reset and an attempt is made

to re-transmit the packet. This counter may occasionally

increment on a busy network. If this counter increments

frequently, it may indicate a problem with the network

cabling or the LAN adapter.

AdapterMiscErrorCount

This counter indicates a non-fatal error occurred. This

counter may increment occasionally during normal operation.

If this counter increments frequently, it may indicate a

problem with the LAN adapter.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite32 Ultra, Elite Series, EliteView, EZStart,

SuperDisk, and UltraChip are trademarks of Standard Microsystems

Corporation.  Other products and company names are registered

trademarks or trademarks of their respective companies.