NOVELL 2.0A PATCH

INSTALLATION INSTRUCTIONS

The program included on this diskette modifies Novell Advanced

Netware/286 ver 2.0A so that the new high capacity hard disk drives

from COMPAQ can be used.  These new COMPAQ drive types are:

35  -	130MB, 34 sectors/track ESDI 1:1 interleave

38  -	300MB, 63 sectors/track ESDI 1:1 interleave

45  -	100MB, 33 sectors/track

47  -	 60MB, 25 sectors/track RLL

SYSTEM REQUIREMENTS:

any 286 or 386 based COMPAQ Computer

COMPAQ MS-DOS Version 3 or greater

Novell Advanced Netware 2.0a (backup diskettes)

WHO SHOULD USE:

Use this program if you wish to use Novell Advanced

Netware/286 ver 2.0A with a COMPAQ type 35, 38, 45, or 47 hard disk

drive.

HOW TO USE:

1.  You may copy NOVELL20.EXE to your hard disk (drive C:) or run

it directly from A:.  It is suggested that you at lease make a

backup copy of the patch.

NOTE:  For the following steps, make sure that you are using the

backup diskettes of your Novell Advanced Netware 2.0a, since you

will be modifying several files on different diskettes.

2.  Using the backup diskettes of your Novell Advanced Netware

2.0a, find the diskettes labeled GENOS-2 (GENOS-4 for SFT level II)

and INSTALL.  These will be the two diskettes that the patches will

be installed onto.

3.  Run the patch program with the following syntax:

The syntax of the program is:

[d:][path]NOVELL20 xx

Where:  xx	-   the drive type you wish to modify (eg.  patch 38 for     a type 38 drive).

CAUTION:  The patch must be run on (copies of) your original Novell

diskettes.	Once patched, the files cannot be patched for any other

drive type.  This will show up as 0 patch points found in the

program.

page 1 of 2

COMMENTS:

After running the patch program successfully, you should then

proceed with the normal installation procedure.  This procedure

normally consists of analyzing the hard disk (with COMPSURF.EXE).

If you are installing the 130MB drive or the 300MB drive you must

run the Compaq lowlevel format first.  Then run COMPSURF.EXE and

don't format the drive and don't have it read the bad block table

from the

Next you generate the Operating System (using the GENOS diskettes),

and finally install the system onto the hard disk (using the

INSTALL diskette).

NOTE:  Make sure to use ONLY the patched versions of the software

for these tasks.  The old version of NETWARE WILL NOT WORK PROPERLY.

ALSO, you MUST run the patch BEFORE generating the Operating System

(NET$OS.EXE) using GENOS.BAT.  NET$OS.OBJ is a required file for OS

generation, and it is one of the files that is patched.  A complete

list of the files that are patched follows:

INSTALL DISKETTE

COMPSURF.EXE

DISKED.EXE

LOADER.DAT

INSTALL.EXE

PREPARE.EXE	(For SFT Level I and II ONLY)

GENOS-2 DISKETTE	(For NON SFT level II)

NET$OS.OBJ

GENOS-4 DISKETTE	(For SFT level II ONLY)

NET$OS.OBJ

EFFECTS ON YOUR SYSTEM:

1.  There are no effects on your system.  This program only

patches the named files.

page 2 of 2