COMPAQ Computer Corporation

CPQDA286.DOC     02/08/90

INSTALLATION INSTRUCTIONS

INTRODUCTION:

This document describes the released version (v1.00) of the COMPAQ

device driver to support NetWare 2.1x on the COMPAQ 32-Bit Intelligent

Drive Array (IDA) Controller. This driver supports up to 2 controllers

each with up to 2 logical volumes. The IDA must be configured into the

system, using the COMPAQ EISA Configuration Utility that was

provided with your COMPAQ computer, BEFORE installing the driver.

Please note the interrupt request (IRQ) value for the IDA(s)

as you are configuring your system.

You must configure the IDA as either PRIMARY or SECONDARY controller.

If you disable the IDA controller's Standard Interface,

the CPQDA286 driver will not be able to operate.

Installing CPQDA286, the COMPAQ 32-bit Intelligent Drive Array (IDA)

Controller device driver for NetWare 2.1x (286), is described below.

NOTE: Refer to your Novell NetWare documentation for information about

installing and using Novell NetWare 2.1x

Refer to your COMPAQ 32-bit Intelligent Drive Array Controller

installation guide for configuring the IDA into your EISA system.

SYSTEM REQUIREMENTS:

This device driver requires:

*  Any COMPAQ EISA computer with the COMPAQ 32-bit Intelligent Drive

Array (IDA) controller installed. The system must have been

configured using the COMPAQ EISA Configuration Utility.

*  NetWare 2.1x

WHO SHOULD USE:

This driver replaces the ISADISK driver in NetWare 2.1x running on

any COMPAQ EISA based system with the IDA installed. This driver

provides improved performance over the ISADISK driver.

HOW TO USE:

Installing NetWare 2.1x from Novell NetWare diskettes:

* Format a blank diskette. For information about the MS-DOS FORMAT

command, refer to your documentation. The volume label on this

diskette should be 'DSK_DRV_203'.

* Copy the CPQDA286.OBJ file, found on this distribution diskette,

to the newly formatted diskette.

* Copy the CPQDA286.DSK file, found on this distribution diskette,

to the Novell NetWare AUXGEN diskette.

Installing NetWare 2.1x from another server or fixed disk:

* Copy the CPQDA286.OBJ file, found on this distribution diskette,

to the appropriate NetWare server fixed disk in the

NetWare subdirectory 'DSK_DRV_.203'.

* Copy the CPQDA286.DSK file, found on this diskette, to the

Novell NetWare AUXGEN subdirectory on your server fixed disk.

Proceed with the following installation steps for both diskette and

fixed disk installations of NetWare 2.1x.

* Run the Novell NETGEN program.

* Follow the instructions in the Novell NetWare documentation to

continue the installation. Be sure to select the

COMPAQ 32-Bit INTELLIGENT DRIVE ARRAY CONTROLLER  disk driver

for each IDA controller (channel) in your system.

NOTE: If you selected a fault tolerant option, such as mirroring or

data guarding, using the COMPAQ EISA Configuration Utility,

do not select mirroring under Novell NetWare.

COMPAQ testing indicates that performance improvements are seen

when the IDA provides the mirroring level of fault tolerance.

* When choosing the disk driver configuration, select from the

available options. Novell channels are not required to be

consecutive, but they MUST be configured as either Primary or

Secondary. The I/O Base address and interrupt request (IRQ) value

should match that which was setup during the COMPAQ EISA

Configuration Utility execution. If you do not remember, the driver

will still operate properly, but you may introduce a conflict with

other system components which has previously been resolved by the

COMPAQ EISA Configuration Utility.

* COMPAQ drives are preformatted from the factory.

Running a full pass of COMPSURF should not be required and is

not recommended.

EFFECTS ON YOUR SYSTEM:

The CPQDA286 driver for NetWare 2.1x provides improved performance

over the ISADISK driver.