NWDOSODI.DOC           07/10/91

Copyright 1991, Compaq Computer Corporation.

NETWARE DOS ODI WORKSTATION DRIVER INSTALLATION INSTRUCTIONS

CONTENTS

1. INTRODUCTION

2. FEATURES

3. FILES

4. SYSTEM REQUIREMENTS

5. INSTALLATION

6. INSTALLING NOVELL LAN WORKPLACE v4.0 FOR DOS

7. SOURCE ROUTING SUPPORT

8. NETWIRE

1. INTRODUCTION

This file details the installation of the NetWare DOS ODI driver

for the Compaq 32-Bit DualSpeed Token Ring Controller.

2. FEATURES

1. Provides improved performance over previous releases of the

driver.

2. Source Routing is supported.

3. Microsoft Windows 3.0 is supported

4. Compatable with Novell LAN Workplace 4.0 for DOS.

3. FILES

NWDOSODI.DOC      - This file

CPQTOKNW.COM      - NetWare DOS ODI workstation driver

DOSNET.CFG        - Sample configuration file

4. SYSTEM REQUIREMENTS

This device driver requires:

*   Any COMPAQ EISA computer with the COMPAQ 32-bit DualSpeed

Token Ring controller installed. The system must have

been configured using the COMPAQ EISA Configuration

Utility.

*   NetWare DOS ODI software, included with the NetWare

operating system.

5. INSTALLATION

The DOS ODI workstation driver allows operation of multiple

protocols on a single network board. This allows communication with

a variety of mainframes, workstations, and file servers, without

rebooting your workstation.

In the following procedure, refer to section of your manual on

installing the NetWare ODI Shell for DOS. You must fist create a

master workstation diskette the contains copies of the files

necessary to boot one type of workstation. Then you will either

make individual workstation boot diskettes for use with each

workstation or copy the files from the master workstation diskette

to the hard disk from which a workstation boots.

To install the device driver for the token ring controller,

following these steps:

1.   Create a system diskette by using the MS-DOS

FORMAT command with the /S parameter.

2.   Copy the desired shell file (NET3.COM, for

example) from the root directory of the DOS/DOS

ODI WORKSTATION SERVICES diskette to the master

workstation diskette (the diskette you created

in step 1).

3.   Copy the following files to the master

workstation diskette: LSL.COM and the protocol

stack file (IPXODI.COM, for example).

4.   Copy the CPQTOKNW.COM file to the master

workstation diskette.

5.   Create the AUTOEXEC.BAT file to have the

drivers in the order shown below.  The

following is a sample AUTOEXEC.BAT file:

LSL

CPQTOKNW

IPXODI

NET3

Proceed to the follow steps if you network supports source routing.

Otherwise, you are finished with the installation procedure.

6.   Copy the source routing driver (ROUTE.COM) from

the root directory of the DOS/DOS ODI

WORKSTATION SERVICES diskette to the master

workstation diskette.

7.   Create the AUTOEXEC.BAT file shown below if

your network supports source routing.  Refer

to the IBM Token-ring section of the NetWare

386 Supplements manual for more information.

LSL

CPQTOKNW

ROUTE

IPXODI

NET3

The NET.CFG file is optional for loading the device driver on the

first controller in the system, but it is required for loading

drivers on additional controllers.  Refer to your NetWare

Installation manuals for more information.  The configuration

options used by the CPQTOKNW.COM device driver are as follows:

FRAME frame_type    The variable frame_type can be token-ring or

token-ring_snap.

NODE ADDRESS x      Burn-in node address override.  The variable

x must be 12 hexadecimal digits.

NO EARLYRELEASE     Disables early token release.

PROTOCOL name ID    Specifies which protocol is used with the

driver.

SEND RETRIES x      Maximum number of retries.

SLOT x              Slot override.

The following are the defaults if the keywords are not supplied:

a. Use Burn-In Address.

b. Early token release feature will be enabled at 16Mbps.

c. Protocol is IPX, protocol ID is e0.

d. Frame type is TOKEN-RING.

e. Use the 1st COMPAQ 32-Bit DualSpeed Token Ring Controller

found on the EISA bus.

6. INSTALLING LAN WORKPLACE v4.0 FOR DOS

NET.CFG is an optional file then can be used when installing

Novell DOS ODI Workstation.  To use the TCP/IP protocol stacks

from Novell LAN Workplace v4.0 for DOS with the COMPAQ

DualSpeed Token Ring Controller DOS ODI driver, the

TOKEN-RING_SNAP frame type must be specified in the NET.CFG

file.  The following are examples for using with TCP/IP

protocol stack in the NET.CFG file.

a. IPX/SPX using the TOKEN-RING frame type and TCP/IP using

the TOKEN-RING_SNAP frame type.

This is the recommended method of installing other protocol

stacks other than IPX.

Example:

LINK DRIVER CPQTOKNW

FRAME TOKEN-RING

FRAME TOKEN-RING_SNAP

b. Both IPX/SPX and TCP/IP using the TOKEN-RING_SNAP frame

type.

This method is interoperable with Netware 386 v3.x.  Netware

286 v2.x  does not support the TOKEN-RING_SNAP frame type.

On Netware 386 v3.x, the TOKEN-RING_SNAP frame type must be

loaded before to allow DOS ODI Workstation access to the

server.

Example:

LINK DRIVER CPQTOKNW

FRAME TOKEN-RING_SNAP

7. SOURCE ROUTING SUPPORT

Source routing is a method of routing data across multiple

networks through bridges. To enable the workstation to pass

data through a source routing bridge, the workstation must be

running the source routing program, route.com from Novell.

This program is necessary only if external source routing

bridges are present on the network. The source routing

program, file route.com, is available on the Novell NetWare

DOS Workstation Generation diskette. The Novell source routing

program and installation instructions are also available on

Compuserve NetWire under NOVA forum in library 8 (Novell

Patch/Drv'r) in the TRN050.ZIP file.

See the Novell NetWare documentation for more information on

source routing.

The following is an example of how to load source routing on

the workstation. The following lines should be placed in the

workstation autoexec.bat file.

lsl

cpqtoknw

route

ipxodi

net3

8. NETWIRE

The NetWare DOS ODI driver files for the COMPAQ 32-Bit

DualSpeed Token Ring Controller reside on Compuserve NetWire

under NOVA forum in library 11 (or library 17 for new uploads)

in the CPQTR2.ZIP file.