NAME

ezsetup - configuration program for

o SMC EtheCARD Elite16 ULTRA(8216/8216T/8216C).

o SMC TokenCard EliteSeries (8115T) LAN Adapters.

o SMC EtheCARD PLUS Family LAN Adapters(8003/8013).

SYNOPSIS

ezsetup

ezsetup -io:xxx [node:xxxxxxxxxxxx]

ezsetup -node:xxxxxxxxxxxx

ezsetup -factory

ezsetup -default

ezsetup -newio:xxx

ezsetup -irq:x

ezsetup -ram:xxxx

ezsetup -ramsize:xx

ezsetup -wait:x

ezsetup -romsize:xx

ezsetup -rom:xxxxx

ezsetup -netconn:xxxxx

ezsetup -link_int:x

DESCRIPTION

The Setup Program for DOS is intended to configure boards

that use the WD83C583 or WD83C584 as listed below:

o 8003EB  - 8-bit coaxial adapter configurable by software

only

o 8003W   - 8-bit twisted pair adapter configurable by

software only

o 8003EP  - 8-bit coaxial adapter configurable by software

or jumpers

o 8003WP  - 8-bit twisted pair adapter configurable by

software or jumpers

o 8013EP  - 16-bit coaxial adapter configurable by software

or jumpers

o 8013W   - 16-bit twisted pair adapter configurable by

software or jumpers

o 8013EW  - 16-bit coaxial and twisted pair adapter

configurable by software or jumpers

o 8013EPC - 16-bit coaxial adapter configurable by software

or jumpers

o 8013WC  - 16-bit twisted pair adapter configurable by

software or jumpers

o 8013EWC - 16-bit coaxial and twisted pair adapter

configurable by software or jumpers

o 8115T   - 16-bit UTP/STP unshielded/shielded twisted pair

Token-Ring adapter configurable by software.

o 8216C/T - 16-bit Combo(BNC,10BaseT,AUI), 10BaseT(T), BNC

configurable by software or jumpers.

The ezsetup program can be run only by the 'superuser' due to

dangerous conditions that might occur (See WARNING at the end

of this document).

This Program can support up to 4 boards on the same bus,

assuming that each board uses a 583/584/585, and has a unique

base I/O address.  Upon power on, the contents of EEROM are

loaded into the internal registers on the 583/584.  These

registers occupy the first 16 I/O addresses starting at the

base I/O of a specific board.

There are two modes to execute this program: one to display

the configuration only, and one to change the current

configuration.  Changing the configuration could be done in

two ways:

1. INTERACTIVE METHOD

Type ezsetup and follow the instructions displayed on the screen.

When you have more than one adapter which is supported by this

program installed in your system, then you see all the adapters

listed by their I/O Base addresses.

If you do have more than one adapter, but you see only one I/O

Base address listed, this means that your adapters are probably

configured to the same I/O Base address.  When this happens,

you must remove all the adapters except one from your computer,

then change each adapter's I/O Base address.  After you have

changed each adapter's address, you can add the other adapters and

configure them.

Refer to the adapter's user installation guide for further

installation and configuration instructions.

2. COMMAND LINE METHOD

The command line consists of three components: 1) the command,

ezsetup, 2) the adapter identification, and 3) the configuration

parameters.

You only need to specify the adapter identification when you

have more than one adapter installed.  You can use either

the I/O base address or the node address.  To identify the

adapter, the command for using the node address is -node:x,

where x is the adapter's 12-digit node address.

DISPLAY

1. By typing

"ezsetup"

The program will try to find all 583-based boards (up to four)

on the machine with the range of I/O address 200 - 3E0 (hex),

and display each board's configurations.

2. By typing

"ezsetup -io:xxx [-node:xxxxxxxxxxxx]" or

"ezsetup -node:xxxxxxxxxxxx"

The program will find the board with specified base I/O

address or node address, and display its configuration.

CONFIGURE

1. Interactive mode by typing:

"ezsetup -io:xxx [node:xxxxxxxxxxxx]" or

"ezsetup -node:xxxxxxxxxxxx"

The program will find the specified board, and go through each

options. The current configured option will be shown.  If no

change is to be made, pressing ENTER will leave the option as

is.

2. Command line mode by typing:

"ezsetup -io:xxx [-node:xxxxxxxxxxxx] [..] ..."

"ezsetup -node:xxxxxxxxxxxx [..] ..."

The program will take the options specified, print out the

configuration to be saved, and if confirmed, change the

current configured value.

The following are the list of options with their possible

values:

-?        Displays all ezsetup command line options.

-v	       Displays the program name, version, and copyright

information only.

-factory  Forces the adapter to the default factory-set

values for the SOFT jumper position, ignoring

the specific command options for these values

(I/O base address 240, IRQ 2 Disabled, RAM base

address CE000; For 8013 adapters, CC000).

-default  Forces the adapter to the following settings

when the jumper is set to SOFT: I/O base address

240, IRQ 2, RAM base address CE000 (CC000 for

8013 adapters).

-newio:xxx

Specifies the new base I/O address.

Range : 200 - 3E0

The address xxx will be a hex value, and must be a

multiple of 20 hex.

-node:x   Identifies the node address of the adapter you

want to configure.  x is the 12-digit node address

printed on the NODE ADDRESS label attached to the

adapter.  This command is an alternative to using

the /io command to identify the adapter.  Example:

0000C024D058.

-irq:x    Specifies the interrupt level (IRQ).  Possible

values for x are 2, 3, 4, 5, and 7.  For 8013

adapters, possible values are 2, 3, 4, 5, 7, 10,

11, and 15.  Also enables the IRQ specified.

-ram:xxxxx

Specifies the RAM base address.  Possible values

for x are any hexadecimal address that is a multiple

of 8K between 80000 - FE000.  For 8013 adapters, the

value has to be a multiple of 16K between 80000 -

FFE000. To specify address above 1 Megabytes (above

FC000), 6 digits of address has to be specified. See

your adapter's user installation guide.

-ramsize:xx

Specifies the size of the RAM window in Kbytes.

Acceptable values for xx are 8 or 32.

Note: 32 Kbytes is only available on certain custom

boards.

-rom:xxxxx

Specifies the base address of the boot ROM.

Possible values for x are any hexadecimal address

between 80000 to FC000.

Note: the address must be on the a boundary of

the ROM size.  See your adapter's user installation

guide.

-romsize:xx

Specifies the size of the ROM in Kbytes.  Acceptable

values for xx are 16, 32 or 64.

-wait:x   Specifies whether or not to add wait states.

Possible values for x are y (yes) and n (no).

Most computers work with /wait set to NO.

Computers with fast I/O bus clock rates may

need /wait set to YES.  If the adapter fails

its diagnostic with /wait:n, set /wait to YES.

-netconn:x

Specifies which network connection to use.

Possible values for x are : 1 (for BNC or 10BaseT),

2 (for AUI or 10BaseT), or 3 (for Twisted Pair-

No Link Integrity). This option is valid for

8003WC, 8013WC, 8013EPC, and 8013EWC adapters.

Specifies which network connection to use.

Possible values for x are : 1 (for STP at 4Mbps),

2 (for UTP at 4 Mbps), or 3 (STP at 16Mbps). This

option is valid for 8115T adapters only.

-link_int:x

Specified whether the link integrity is enabled

disabled. Possible values for x are d (disabled)

or e (enabled). This options is valid only for

8013WP and 8013EW adapters.

The following is an example of a properly constructed command

line:

ezsetup -io:240 -newio:280 -irq:5 -ram:C0000 -rom:D0000

The command, ezsetup, starts the program. The adapter identi-

fication, /io:240, tells the system which adapter to

configure, and the remaining information tells the system

how to configure the adapter.  In this example, the I/O base

address is changed to 280, the IRQ level is changed to 5,

the RAM base address is changed to C0000, and the ROM base

address is changed to D0000.

Make sure that the configuration you select for your network

software driver matches what you select in EZSETUP.

Licensed by Standard Microsystems for use by its customers only.

Copyright (c) 1991 Standard Microsystems Corporation

All Rights Reserved

WARNINGS

It is recommended that only run this program while smc network

adapter are off the network, due to the following:

1. Since it is possible that the adapter card uses the

same address as another device does, it might cause

unpredictable results if some other user tries to use the

device.

2. The program uses direct I/O mode to the device and may

behave incorrectly if a SMC/WD LAN adapter is not at the

specified address.

Operating Systems supported:

1. DOS 3.0 or greater

#