TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

Novell NetWare 286/386 Driver Installation

---------------------------------------------------------------------------

This file describes the steps needed for installation of the TIARA NetWare

device driver(s) in the following environments:--

NetWare 286                               Versions 2.12 and 2.15c

NetWare 386                                  Versions 3.0 and 3.1

Please note that these instructions assume that the user is familiar with

the requirements for Server and Work-Station installation per Novell's

instructions for standard NetWare 286/386.

Files which pertain to NetWare 286 Installations:--

Filename                                                   Description

------------   -------------------------------------------------------

ATIARA.OBJ             Object file for Server/Bridge Generation (LANA)

BTIARA.OBJ             Object file for Server/Bridge Generation (LANB)

CTIARA.OBJ             Object file for Server/Bridge Generation (LANC)

DTIARA.OBJ             Object file for Server/Bridge Generation (LAND)

TIARA.LAN              Configuration file for Server/Bridge Generation

STIARA.OBJ                      Object file for Shell (IPX) Generation

STIARA.LAN               Configuration file for Shell (IPX) Generation

Copyright/Trademark Information:--

Novell, NetWare, and SFT are registered trademarks of Novell, Inc.

General Instructions:--

In the instructions which follow, "S:" is used to designate the

(SOURCE) drive containing the TIARA driver distribution diskette,

and "T:" is used to designate the (TARGET) drive to which the

particular item is to be installed.

For example, if the TIARA diskette is in drive "A:" and the user

is installing to drive "B:", then the user should substitute "A:"

for "S:" and "B:" for "T:" as follows:

Original command:   "COPY S:FILENAME T:"

Modified command:   "COPY A:FILENAME B:"

___________________________________________________________________________

Document:  NETWARE.DOC      November 26, 1990                        Page 1

TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

Novell NetWare 286/386 Driver Installation

---------------------------------------------------------------------------

Installation instructions for NetWare version 2.1x.

Note that the TIARA distribution diskette has been given a DOS volume

label of "LAN_DRV_223" which conforms to the requirements of the

ELSGEN/NETGEN/BRGEN/SHGEN utilities.

Floppy Disk Generations:

1.   Proceed according to the NetWare instructions for the

particular type of generation.

2.   At the LAN driver selection menu, for the first (or only)

driver, choose the "Load and Select" option and insert the

TIARA distribution diskette (LAN_DRV_223) when prompted to

do so.

3.   Respond to any informational or warning prompts as they

occur.

In normal circumstances, the only message/prompt which

should appear is the one which informs the user of the Lan

Option Name of the loaded TIARA driver(s).

First-time Hard Disk Generations:

1.   Follow the NetWare instructions pertaining to initial upload

of the NetWare diskette(s).

2.   When asked the question "Upload Additional Diskettes?"

answer yes, and insert the TIARA distribution diskette when

requested to do so.

Pre-existing Hard Disk Generations:

The following instructions assume that the current directory is

set to the NetWare generation directory.

1.   Create the TIARA directory:

MD LAN_DRV_.223

2.   For a NetWare Server or Bridge generation, copy the

following files to the TIARA directory (LAN_DRV_.223):

ATIARA.OBJ, BTIARA.OBJ, CTIARA.OBJ, DTIARA.OBJ, and

TIARA.LAN

3.   For a NetWare Shell (IPX) generation, copy the following

files to the TIARA directory (LAN_DRV_.223):

STIARA.OBJ, and STIARA.LAN

___________________________________________________________________________

Document:  NETWARE.DOC      November 26, 1990                        Page 2

TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

Novell NetWare 286/386 Driver Installation

---------------------------------------------------------------------------

Installation instructions for NetWare 386.

Please Note:  TIARA strongly recommends that all modifications

outlined below be made to DUPLICATE copies of the stated diskettes and

NOT to the originals.

Version 3.0

1.   Copy NETWARE.386\TIARAE.LAN to the Netware 386 "SYSTEM"

diskette.

2.   Follow the instructions in the Novell Installation Manual to

load the driver.

Version 3.1

1.   Copy NETWARE.386\TIARAE.LAN to the Netware 386 "Operating

System-2" diskette.

2.   Follow the instructions in the Novell Installation Manual to

load the driver.

Industry Standard Architecture (ISA) Configuration options supported:

I/O Base Address         300h, 320h, 340h, 380h, 3C0h,

2C0h, 280h, 240h, 200h

Interrupt Request        3, 4, 5, 7, 9, 10, 11, 12, 15

Note that for the LanCard/E*STAR8, interrupt

request jumper 2 is the same as interrupts

request 9 to the driver.

Node ID:                 Configurable.

Protocol ID:             Ethernet_II, Ethernet_802.3 (default).

Micro Channel Architecture (MCA) Configuration options supported:

Slot Number:             1, 2, 3, 4, 5, 6, 7, 8.

I/O Base Address         Obtained from POS configuration information.

Interrupt Request        Obtained from POS configuration information.

Node ID:                 Configurable.

Protocol ID:             Ethernet_II, Ethernet_802.3 (default).

___________________________________________________________________________

Document:  NETWARE.DOC      November 26, 1990                        Page 3