TIARA Computer Systems, Inc.

(C) Copyright 1990. All Rights Reserved.

---------------------------------------------------------------------------

PCNFS Device Driver Installation

---------------------------------------------------------------------------

To install the PC-NFS driver for TIARA adapters please follow the

instructions for installing "other" drivers in the PC-NFS installation

manual. The Tiara PC-NFS driver is called TIARA.SYS.

When using the TIARA.SYS Ethernet PC-NFS driver, additional selection

options are available:

1.   Interrupt Selection:  /inn

Where "nn" indicates the desired interrupt request level.

The default is 3, alternate selection is made by changing jumper

settings on the adapter and by use of this option.

Example:  "Device=TIARA.SYS /i07", this tells the driver to  use

interrupt 7.

2.   Slot Selection (for Micro Channel only):. /sn

Where "n" indicates the slot number required.

If no parameter or "0" is specified, the driver will search from

slot "1" to slot "8" looking for a LanCard/E*MC adapter.

When an adapter is located, that adapter is used by the driver

for PC-NFS.

If more than one Tiara Ethernet adapter is used, this option

permits a particular slot to be specified by the user for use by

PC-NFS.

Example:  "Device=TIARA.SYS /s1", this tells the driver to use

the adapter in slot 1.

Trademark/Copyright information.

NFS, and PC-NFS are trademarks of Sun Microsystems, Inc.

Ethernet is a registered trademark of Xerox Corporation.

___________________________________________________________________________

Document: PCNFS.DOC         November 27, 1990                        Page 1