TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

NDIS Device Driver Installation

- for -

3Com 3+Open Lan Manager Version 1.10

---------------------------------------------------------------------------

Please note that the instructions contained herein assume that the user is

already familiar with the requirements of the particular environment in

terms of workstation setup and software generation.  That is, these

instructions only deal with the basic issue of merging the TIARA device

driver into the particular environment so that workstation generation may

then be performed by the user.

Copyright/Trademark Information:--

3Com and 3+Open are registered trademarks of 3Com Corporation.

OS/2 is a registered trademark of International Business Machines

Corporation.

General Instructions:--

In the instructions which follow, "S:" is used to designate the

(SOURCE) drive containing the TIARA driver distribution diskette,

and "T:" is used to designate the (TARGET) drive to which the

particular item is to be installed.

For example, if the TIARA diskette is in drive "A:" and the user

is installing to drive "B:", then the user should substitute "A:"

for "S:" and "B:" for "T:" as follows:

Original command:   "COPY S:FILENAME T:"

Modified command:   "COPY A:FILENAME B:"

___________________________________________________________________________

Document:  3OPEN.DOC         December 7, 1990                        Page 1

TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

NDIS Device Driver Installation

- for -

3Com 3+Open Lan Manager Version 1.10

---------------------------------------------------------------------------

OS/2 diskette preparation.

1.   Use the DISKCOPY command to make a duplicate of the OS/2 DRIVERS

diskette for use in steps 2, 3, and 4.

2.   Change to the driver directory:

CD T:\3OPEN\OS2DRV

3.   Copy the TIARA driver files to the driver diskette:

COPY S:\MSLANMAN.OS2\DRIVERS\ETHERNET\TNDIS\TNDIS.OS2 T:MAC

COPY S:\3OPEN.110\OS2\TNDIS.NIF T:MAC

4.   Add the PROTOCOL.3CM parameters to the driver diskette.

COPY T:PROTOCOL.3CM T:PROTOCOL.BAK

COPY T:PROTOCOL.BAK + S:\3OPEN.110\OS2\PROTOCOL.3CM

T:PROTOCOL.3CM

DOS diskette preparation.

1.   Use the DISKCOPY command to make a duplicate of the DOS DRIVERS

diskette for use in steps 2, 3, and 4.

2.   Change to the driver directory:

CD T:\3OPEN\DOSDRV

3.   Copy the TIARA driver files to the driver diskette:

COPY S:\MSLANMAN.DOS\DRIVERS\ETHERNET\TNDIS\TNDIS.DOS T:MAC

COPY S:\3OPEN.110\DOS\TNDIS.NIF T:MAC

4.   Add the PROTOCOL.3CM parameters to the driver diskette.

COPY T:PROTOCOL.3CM T:PROTOCOL.BAK

COPY T:PROTOCOL.BAK + S:\3OPEN.110\DOS\PROTOCOL.3CM

T:PROTOCOL.3CM

___________________________________________________________________________

Document:  3OPEN.DOC         December 7, 1990                        Page 2