LanBeep v2.4

Shareware Version

(C) Copyright 1992 Deerfield Computer Solutions, Inc.

All Rights Reserved

Software License Agreement

This program and its documentation are copyrighted and protected by the laws of

the United States and international copyright treaties.  You may not use, copy,

modify, loan, rent, give away, resell for profit, distribute, create derivative

works of, or transfer the program and/or documentation (or any copies thereof)

except as explicitly stated in this license agreement.

This is a single copy license granted by Deerfield Computer Solutions, Inc.,

and entitles you, the end user, to use this product on a single machine with a

single display monitor and a single keyboard.

The software and documentation remain the property of Deerfield Computer

Solutions, Inc.

This license may be terminated at any time by you by destroying all copies of

this program and documentation.  This license may also be terminated by

Deerfield Computer Solutions, Inc. without notice to you if you fail to comply

with the terms specified in this agreement.

You may distribute the shareware version of this program only, in its

unmodified, zipped format, as long as nothing is charged for it.

You may make one copy of this software in machine readable form only, and

provided that the copy is for backup purposes only.

If the software package contains both 5�" and 3�" diskettes, you may only use

one copy of the software (whichever is appropriate or most appropriate for

your single machine).  You may not use the other size diskette(s) on another

machine.

You may not decompile, disassemble, or reverse engineer this software.

If any part of this agreement is found to be unlawful, void, or unenforceable,

then that provision shall be severed from this agreement and will not affect

the validity and enforceability of any of the remaining provisions.

This license may not be modified except by a written document signed by both

parties and stating on its face that it is a modification or amendment of this

contract.  In no case shall a modification be effected by oral or written

representations or advertisements, or by any purchase order or correspondence.

You will not export or transmit this software to Afghanistan, the Peoples

Republic of China, or to any group Q, S, W, Y or Z country specified in the

Export Administration Regulations issued by the U.S. Department of Commerce or

to any country to which such transmission is restricted by applicable

regulations or statutes.

Limited Warranty

This software is licensed "AS IS", and Deerfield Computer Solutions, Inc. makes

no warranty, representation or promise either expressed or implied, except as

set forth in this agreement.  In particular, Deerfield Computer Solutions,

Inc. disclaims all warranties of merchantability or fitness for any particular

purpose, and the end user waives all claims against Deerfield Computer

Solutions, Inc. for special, direct, indirect, lost profits, lost savings, or

other consequential damages arising out of or in connection with the use or

inability to use this product.  Under no circumstances will Deerfield Computer

Solutions, Inc.'s liability exceed the purchase price of this product.

Deerfield Computer Solutions, Inc. does not warrant that the software will meet

your requirements, or that operation of the software will be uninterrupted or

free from errors or defects, or that any errors or defects will be corrected.

The media (diskettes) are warranted to be free from errors and/or defects for a

period of sixty (60) days from the date of original purchase and under normal

use.  Should the media prove defective within this time, Deerfield Computer

Solutions, Inc. will replace them free of charge if they are returned, postage

prepaid, to:

Deerfield Computer Solutions, Inc.

43 Baldwin Terrace

Wayne, NJ 07470

This license constitutes the entire agreement and understanding between the

parties and supersedes any prior agreement or understanding, whether oral or

written, relating to the subject of this license.

This software and documentation are:

(C) Copyright 1992 Deerfield Computer Solutions, Inc.

All Rights Reserved.

Manual revision #0240-960106

LanBeep v2.4                      -Contents-

Program Overview . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  1

Requirements . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

Installation . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  3

Passwords/Access Codes . . . . . . . . . . . . . . . . . . . . . . . . . . .  4

Startup. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  5

The Display Screen . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  6

Beeping. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  7

Configuration. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  8

Monitoring A PC On The Network . . . . . . . . . . . . . . . . . . . . . . . 11

Remote Access. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 12

Commands . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

Sample AUTOEXEC.BAT. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 15

Sample LANBEEP.CFG . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

Beeper Displays. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

Decoding Return Codes. . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

Conversion Table . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

Problems/Possible Solutions. . . . . . . . . . . . . . . . . . . . . . . . . 25

Comments/Suggestions . . . . . . . . . . . . . . . . . . . . . . . . . . . . 26

LanBeep v2.4                 -Program Overview-                         Page  1

LanBeep is a highly reliable network server monitor and also a remote command

center.  With it, you can monitor servers all over the country, and also use it

as a means for giving the PC commands remotely.

The best way to run LanBeep is from AUTOEXEC.BAT (see sample on page 15).

LanBeep must have the network shell loaded before it if it is to scan servers.

LanBeep can also monitor the phone line and if a ring is detected, LanBeep can

answer and carry out a remote command.  LanBeep keeps a log of all of its

activities in a file called LANBEEP.LOG.

If LanBeep determines that a server isn't responding, it initiates a beep which

will display a code that represents the status of all of the servers being

scanned.  It will rebeep at a set time interval until either it receives the

Acknowledgement Access Code, or beeps the maximum number of times.

One of the most common uses of LanBeep is to act as your eyes and ears in a

remote regional site.  Suppose you work at headquarters (housing four file

servers) in New York, and your company has an office in Chicago (housing one

file server).  If you are responsible for the network administration of the

remote office, you probably call someone in that office and have them put their

PC into a remote access program's host mode, waiting for you to dial in.  With

LanBeep, you can call into the LanBeep PC in Chicago, and have it run that

program's host mode for you, waiting for you to dial in, or you can have it run

a BBS, or do anything that you can put into a batch file.  While the PC isn't

being used by you remotely, it is monitoring the file server in Chicago, and it

will beep you if it goes down.  If you have a LanBeep PC running at

headquarters also, you can rest easy knowing that the network is up.  How many

times have you been the first one in to work (an hour before people come in to

start working on the network), only to find that a server had gone down some

time the night before leaving you with one hour to get the server back up, or

face down time.  With LanBeep, you'll get immediate notice of a server going

down, giving you the maximum time possible to correct the problem.  While this

may ruin a perfectly good night's sleep, it's a small price to pay for having

all that extra time to get the network running properly again.

To shut LanBeep down locally, enter the Shutdown Password (see page 4) on the

keyboard. No keystrokes are displayed, and the <Enter> key is not necessary to

signal the end of the password.  If a mistake is made, just start over- no

<Esc> key is necessary.

If you are experiencing any problems, please see Problems/Possible Solutions on

page 25.

LanBeep v2.4                 -Requirements-                             Page  2

A PC with an 80286 processor or higher (if LanBeep is to monitor servers, the

PC must be connected to a Novell(R) network, with the network shell already

loaded).

A Hayes(R) compatible modem (US Robotics(R) Courier(tm) modem necessary for

touch tone recognition- see TONERECOGNITION on page 10).

A beeper/pager (it is possible to use LanBeep without a beeper, but it will

only function as a remote command center- see MINFSERRORCOUNT on page 10).

LanBeep v2.4                     -Installation-                         Page  3

Copy all of the files from the distribution LANBEEP.ZIP file into a

subdirectory of your choice on the hard disk.  To install the LANBEEP.ZIP file

on A: into the C:\LANBEEP subdirectory, enter the following commands:

MD C:\LANBEEP

C:

CD \LANBEEP

PKUNZIP A:\LANBEEP

Edit the LANBEEP.CFG file (see CONFIGURATION on page 8) and then run LanBeep.

LanBeep v2.4                 -Passwords/Access Codes-                   Page  4

LanBeep uses three codes.  The first is the Dial In Access Code.  This is a six

digit number that must be used to issue remote commands to LanBeep.  The second

code is the Acknowledgement Access Code.  This is a six digit number that is

used remotely to tell LanBeep to stop calling the beeper.  The last code is the

Shutdown Password.  This is a 1 - 20 character password used to stop LanBeep's

execution from the keyboard.

If TONERECOGNITION is set to "YES", then the modem will monitor the phone line

for incoming calls.  When a ring is detected, the modem will answer the phone

and await tones from the remote phone.

The Dial In Access Code allows you to enter remote commands (see Commands on

page 14).

The Acknowledgement Access Code signals LanBeep to stop beeping with the

current status.

The Shutdown Password allows termination of LanBeep from the keyboard.  After

entering the Shutdown Password, the modem will be reset, and you will be

returned to the operating system.

The default passwords/access codes are:

Dial In Access Code             111111

Acknowledgement Access Code     222222

Shutdown Password               SHUTDOWN

To change the passwords/access codes, you must use the supplied program

LBPASSWD.COM.  LBPASSWD will ask you to change each code in succession.  No

passwords are displayed on screen.  To change a code, when prompted, enter the

current code being asked for.  This is to prevent the unauthorized changing of

a code by someone who doesn't even know what it currently is.  After entering

the correct code, you will be prompted twice to enter a new code.  The two

entries must match exactly to be flagged for updating.  If a mistake is made

during either entry, the <Esc> key can be hit to bypass the update of that

code.  Once all codes have been processed by you (either changed or left

alone), LBPASSWD will put any changes into effect.

We recommend keeping LBPASSWD.COM separate from the LanBeep program (except

when being used to change codes) because anyone knowing any of the codes can

change them to something else without telling you, in effect, locking you out.

Changing codes is a supervisor's function and thus should be limited to a

select few.

LBPASSWD may be started from any directory by typing:

LBPASSWD<Enter>

as long as LBPASSWD.COM is either in your path or in your current directory.

LANBEEP.COM must be in your current directory.

NOTE:  LBPASSWD modifies LANBEEP.COM so you should use a copy of LANBEEP.COM,

and keep the original as a backup.

LanBeep v2.4                      -Startup-                             Page  5

LanBeep may be started from any directory by typing:

LANBEEP<Enter>

as long as LANBEEP.COM is either in your path or in your current directory.

LANBEEP.CFG must be in your current directory, and LANBEEP.LOG will be created

in your current directory if it doesn't already exist (if it does exist, it

will be appended to).

You may specify a command line option as follows:

LANBEEP /C<Enter>       forces LanBeep to use color video address

LANBEEP /M<Enter>       forces LanBeep to use monochrome video address

LANBEEP /B<Enter>       forces LanBeep into black and white mode

If LanBeep produces no display, try the /C or /M option.  If this doesn't work,

try the other one.  The /B option removes color from the display, leaving only

a black and white screen.  (Depending on your monitor, the screen may actually

be displayed in green and white or amber and white when using the /B option.)

LanBeep v2.4                 -The Display Screen-                       Page  6

The following is a representation of the display screen:

����������������������������������������������������������������������������Ŀ

�    LANBEEP v2.4s  (C) Copyright 1992 Deerfield Computer Solutions, Inc.    �

�                            All Rights Reserved                             �

�    ��������������������������������������������������������������������    �

�                                                                            �

�    ���������������������������Ŀ                  �������������������Ŀ    �

�    � Parameter         Setting �                  � Program Operation �    �

�    �                           �                  � Indicator:   *    �    �

�    � BEEPINTERVAL    =  300    �                  ���������������������    �

�    � MAXBEEPS        =  0      �                ���������������������Ŀ    �

�    � MINFSERRORCOUNT =  10     �                � Scan Indicator:  *  �    �

�    � PORT            =  COM2   �                �����������������������    �

�    � REGION          =  99     �          ���������������������������Ŀ    �

�    � SCANINTERVAL    =  0      �          �       Modem Status:       �    �

�    � TONERECOGNITION =  YES    �          �      Waiting for ring     �    �

�    �����������������������������          �����������������������������    �

�                                                                            �

�                                                                            �

�            ��������������������������������������������������ͻ            �

�            �                 Scanning Server:                 �            �

�            �                       FS1                        �            �

�            ��������������������������������������������������ͼ            �

�                                                                            �

�                                                                            �

�         ENTER SHUTDOWN PASSWORD TO RETURN TO THE OPERATING SYSTEM          �

�                                                                            �

������������������������������������������������������������������������������

The Parameter/Setting box displays the values that each parameter is currently

set at.

The Program Operation Indicator box contains a spinning wheel.  As long as the

wheel is spinning, LanBeep is doing something, and will respond to anything

that needs attention as soon as it has completed its current task.

The Scan Indicator box also contains a spinning wheel.  This wheel may move

rather quickly or hardly at all depending upon how often the servers are being

scanned (see SCANINTERVAL on page 9).

The Modem Status box indicates the current status of the modem/phone line.  If

TONERECOGNITION is set to "NO", this box will display "Phone line not

monitored".

The Scanning Server box displays the name of the server that is currently being

checked.  If MINFSERRORCOUNT is set to 0, this box will display

"None-MINFSERRORCOUNT is set to 0", and the Scan Indicator box will display

"N/A".

To shut LanBeep down locally, enter the Shutdown Password on the keyboard.  No

keystrokes are displayed, and the <Enter> key is not necessary to signal the

end of the password.  If a mistake is made, just start over- no <Esc> key is

necessary.

LanBeep v2.4                 -Beeping-                                  Page  7

When LanBeep determines that it needs to call a beeper, it looks at three

things:

First it looks at MAXBEEPS to see if it has already beeped the maximum

number of times.  If so, it just skips the beep and continues with

whatever it was doing.

Second, it looks for the Acknowledgement Access Code.  Calling the

number that the modem is attached to and entering the Acknowledgement

Access Code will cause LanBeep to discontinue beeping you with the

CURRENT status.  If the status changes again, LanBeep will start

beeping you again with the new status as long as it hasn't already

beeped the maximum number of times.  The only way to reset the beep

count is to either restart LanBeep or dial in (using the Dial In Access

Code) and using command 21 (see Commands on page 14).  Using the

Acknowledgement Access Code will only acknowledge that the beep has

been received, NOT reset the beep count.

Third, LanBeep determines if the time interval for beeping has expired

(see BEEPINTERVAL on page 10).  If so, LanBeep will call the beeper.

LanBeep v2.4                 -Configuration-                            Page  8

LANBEEP.CFG must exist (see sample on page 16) and contain any/all of the

following (keywords are not case sensitive):

Keyword            Keyvalue                        Default   Required

REGION             2 digit number                  n/a       Yes

SERVERNAME         1-48 characters                 n/a       Yes

BEEPER             beeper number to dial           n/a       Yes

OBJECTTYPE         type of server to monitor       0004      No

PORT               any string containing a '1',    COM1      No

'2', '3', or '4'

MAXBEEPS           0-9999                          0         No

SCANINTERVAL       0-3600                          0         No

BEEPINTERVAL       0-3600                          300       No

MINFSERRORCOUNT    0-9999                          10        No

TONERECOGNITION    any string containing a 'Y'     NO        No

The format (using a pure ASCII text editor) is:

keyword=keyvalue

where the keyword starts in column 1, and a separate line is used for each

keyword.  If a semicolon (;) is in column 1, the entire line is treated as a

comment.

Required keywords must be entered.  If they do not apply to your situation, use

any valid value as a dummy keyvalue.

REGION is a 2 digit number which identifies the origination of a beep.

SERVERNAME is a 1-48 character server name to be checked.  Use one statement

for each server to be scanned.

BEEPER is the phone number of the beeper including any necessary extra digits

(e.g., 9).  LanBeep recognizes up to 16 'BEEPER=' statements.  The beeper

numbers specified will be dialed in sequence.  Please note that the

'BEEPINTERVAL=' statement must be set to no less than 60 times the number of

beepers.  I.E., if you are having LanBeep call 4 beepers, set BEEPINTERVAL to

no less than 240.

OBJECTTYPE is the type of server that subsequent SERVERNAMES will refer to.

LanBeep supports definable servers.  OBJECTTYPE is the 1-4 digit hex value that

is recognized by the network's bindery.  Any servers named after this statement

will use this object type as its definition (until another 'OBJECTTYPE='

statement is found).  The default OBJECTTYPE is 0004 (file server), so if no

'OBJECTTYPE=' statement is found, all server names specified on 'SERVERNAME='

statements will be scanned as file servers.

LanBeep v2.4                 -Configuration (Continued)-                Page  9

Some of the known server types are:

0004   File Server                0045   DI3270 Gateway

0005   Job Server                 0047   Advertising Print Server

0006   Gateway                    004B   Btrieve Server

0009   Archive Server             004C   NW SQL Server

0021   NAS/SNA Gateway            004D   XTree Network

0023   NACS Gateway               0050   Btrieve Server

0024   Remote Bridge Server       0098   NW Access Server

0026   Async Bridge Server        0103   Oracle DB Server

0027   TCP/IP Gateway             0107   Rconsole

0028   X.25 Bridge                0771   PC running DCSI'S ADVERTIS.COM

0029   Gateway                                            (see page 11)

002E   Archive Server             6F00   Rabbit Gateway

Note that the above represents an incomplete server type list.  For servers not

listed, contact the product manufacturer to obtain the object type, or you can

use the supplied program LBSCAN.  LBSCAN displays all of the server types that

your network's bindery is aware of, along with the server's name.  These two

fields are to be used, as mentioned previously, in the configuration file.

Simply run LBSCAN.COM with the network shell loaded.  The program will display

objects as they are encountered in the bindery.  If you wish to see a sorted

list, the optional '/S' parameter will display the list in ascending object

type order.  This may take considerably longer than an unsorted list.  To

interrupt the program at any time, hit <ESC>.

Please make sure that the object type that you are scanning is the correct one.

I.E., a file server named FS1 running RCONSOLE will show up twice in LBSCAN's

output- once as type 0004 and once as type 0107.  Using 'OBJECTTYPE=0107' and

'SERVERNAME=FS1' will cause the program to beep you if RCONSOLE is unloaded.

The correct configuration would be to use 'OBJECTTYPE=0004'.  Another example

of misleading object types is print servers- using an object type of 0007 will

report on print servers that EXIST in the bindery regardless of whether or not

they are up and running, whereas object type 0047 will report on the status

(not all print servers advertise).  Please note that the LanBeep PC may need to

be logged in to the file server to 'see' certain bindery objects.

PORT is a string that can contain a '1', '2', '3', or '4'.  (E.G., if a '2' is

found anywhere in this keyvalue, the ports will be set up to use COM2.)  The

default setup is for COM1.

MAXBEEPS is the maximum number of times that LanBeep will dial the beeper until

the count is reset (using command 21).  The default is 0 which means beep until

acknowledged with the Acknowledgement Access Code.

SCANINTERVAL is the number of seconds to wait before checking the server list

again.  LanBeep only validates this for numeric entries, and not the range of

0-3600.  The number will wrap around to 0 somewhere slightly above 3600.  The

default is 0 which means continuous checking.

LanBeep v2.4                 -Configuration (Continued)-                Page 10

BEEPINTERVAL is the number of seconds to wait before dialing the beeper again

if it has not received acknowledgement of the initial beep.  The same

validation holds true for this keyword as for SCANINTERVAL.  The default is

300 which means beep every 5 minutes.  NOTE:  LanBeep delays 40 seconds after

dialing a beeper to allow for the modem  to finish the command.  Setting

BEEPINTERVAL to a value less than 60 may cause a backup which will prevent

local shutdown and possibly remote shutdown.

MINFSERRORCOUNT is the number of times that LanBeep will retry a server that

doesn't respond before considering the server to have a problem.  This

eliminates false beeps caused by sporadic errors that recover automatically.

The default is 10.  If MINFSERRORCOUNT is set to 0, the servers will NOT be

scanned at all, no matter what SCANINTERVAL is set at.  This can be used to

disable the network scanning, and still use LanBeep as a means to shut the PC

down with an errorlevel (e.g., remote shutdown with errorlevel 1 might cause

the PC to jump into ProComm Plus(R) host mode, while remote shutdown with

errorlevel 2 might cause the PC to jump into PC/Anywhere(R) host mode).

TONERECOGNITION is a string that can contain a 'Y'.  If a 'Y' is found anywhere

in this keyvalue, the modem will answer the phone when a ring is detected in

an attempt to receive remote commands.  The default setup is to ignore incoming

calls.

NOTE: When using a US Robotics(R) Courier(tm) modem (required for using this

keyword), the "supervisor chip" needs to have a date of 09/25/91 or later.  To

determine the date of your chip, use a communications package to do the

following:

Go into terminal mode (where you can manually enter modem commands)

Type ATI7<Enter> (the third character is a capital i)

The date of the supervisor chip will be displayed.  If the supervisor chip has

a date before 09/25/91, you can obtain a free upgrade chipset (three chips) by

calling US Robotics(R) Customer Service at (800) 982-5151.  Have your modem's

serial number handy, and ask for the chip upgrade.  It will be free as long as

you return the old chips after you replace them.  An easy to follow instruction

sheet accompanies the chip upgrade package.

LanBeep v2.4            -Monitoring A PC On The Network-                Page 11

ADVERTIS.COM makes any PC on your network visible to LanBeep.  The syntax for

ADVERTIS.COM is:

ADVERTIS /advname

where "advname" is the name that you wish LanBeep to scan for.  For instance,

if you have a machine on your network that acts as an e-mail gateway, you can

use LanBeep to monitor the gateway PC by running ADVERTIS.COM on it before

running the gateway software.  E.G., the gateway's AUTOEXEC.BAT could contain

the following:

@ECHO OFF

CD \LANBEEP

ADVERTIS /EMAILGATEWAY1

CD \EMAIL

RUNMAIL

You can use LanBeep to monitor this PC's status by adding the following to

LANBEEP.CFG on the PC that runs LanBeep:

OBJECTTYPE=0771

SERVERNAME=EMAILGATEWAY1

If the gateway PC fails to respond to the network for three to ten minutes,

LanBeep will issue a call to the designated beeper(s).

Please note the following when using ADVERTIS.COM:

The OBJECTTYPE to scan for (as can be verified by LBSCAN.COM) is 0771h.

It takes up to one minute for the PC to start advertising.

The network can take three to ten minutes to realize that the PC is no

longer active.

The name given to the advertising PC must be a valid server name

between one and 48 characters in length.

Running ADVERTIS.COM without specifying its name on the command line

will result in the PC being known on the network as "A".

Advertising server names must be unique within OBJECTTYPE 0771h.

LanBeep v2.4                 -Remote Access-                            Page 12

To have the LanBeep PC execute commands (via the batch file that started

LanBeep), voice call the PC, and when it picks up, wait about five seconds

before entering digits from your telephone's keypad.  There is no confirmation

from the PC that it has answered, only the lack of rings.  After waiting five

seconds for the PC to set itself up, enter the following on the phone's

keypad:

#Acknowledgement Access Code#

-or-

#Dial In Access Code#command#

where the code is the six digit Dial In Access Code or Acknowledgement Access

Code, and command is a valid command (see Commands on page 14).  The first #

signals the start of the access code, the second # signals the end of the

access code, and also the start of the command, and the third # signals the end

of the command.  Note that the command field is not required when using the

Acknowledgement Access Code (and is ignored if it is supplied).

If an entry has not been completed, and a key is not punched in over the phone

within 20 seconds of the last key punched in, LanBeep will hang up and resume

whatever it was doing before the call came in.

If an incorrect digit is pushed while entering either the access code or

command, you can push '*' to signal that the current FIELD is being re-entered.

If the Dial In Access Code is 123456, and the command is 14, then you'd enter:

#123456#14#

and hang up.  The following examples show possibilities that will result in the

same command being executed as the above example, except that they contain

errors and corrections in the keyed data:

#2*123456#14#           -2 was entered instead of 1 for the first

access code digit, so restart the access code

field by entering a '*'.

#12346*123456#14#       -5 was skipped in the access code, so restart

the access code field by entering a '*'.

#123456#5*16*14#        -5 was entered instead of 1 for the first

command digit, so restart the command field by

entering a '*', then 6 was entered instead of 4

4 for the last command digit, so restart the

command field again.

123456#123456#14#       -all data is ignored before LanBeep recognizes

a '#'.

LanBeep v2.4                 -Remote Access (Continued)-                Page 13

If the Acknowledgement Access Code is 141414, then to stop LanBeep from dialing

the beeper, you'd enter:

#141414#                -Acknowledgement Access Code entered, no

command field.

The following show invalid keyed data which will result in the PC hanging up

the line and resuming whatever it was doing before the call came in:

##14#                   -signal start and end of the access code field

(##), but the access code is actually null

(same effect as if the wrong code had been

entered).

#123456##               -signal start and end of the command field

(##), but the command is actually null (same

effect as if an unknown command had been

entered).  This will, however, still act as an

acknowledgement of a beep (if the

Acknowledgement Access Code was 123456).

#123456#                -timeout will occur 20 seconds after the last

key is hit (unless the Acknowledgement Access

Code was 123456, in which case the beeping

would be acknowledged).

#12                     -timeout will occur 20 seconds after the last

key is hit.

LanBeep v2.4                 -Commands-                                 Page 14

The following are valid commands that LanBeep can react to:

Command         Result

10              exit with errorlevel set to 0

11              exit with errorlevel set to 1

12              exit with errorlevel set to 2

13              exit with errorlevel set to 3

14              exit with errorlevel set to 4

15              exit with errorlevel set to 5

16              exit with errorlevel set to 6

17              exit with errorlevel set to 7

18              exit with errorlevel set to 8

19              exit with errorlevel set to 9

20              beep back with current status

21              reset beep count

NOTES:

Command 20 does not increment the beep count

Command 21 is used to allow LanBeep to start beeping again (if

necessary) after it has reached MAXBEEPS

LanBeep v2.4                 -Sample AUTOEXEC.BAT-                      Page 15

The following is an example AUTOEXEC.BAT that uses the errorlevel exit codes of

LanBeep to determine what to do next:

prompt $p$g                     set the prompt

path=c:\dos;c:\                 set the path

\lan\ipx                        load the network shell (e.g. IPX/NETX)

\lan\netx

cd \lanbeep

lanbeep                         run LanBeep

if errorlevel 9 goto level9     evaluate exit code

if errorlevel 8 goto level8

if errorlevel 7 goto level7

if errorlevel 6 goto level6

if errorlevel 5 goto level5

if errorlevel 4 goto level4

if errorlevel 3 goto level3

if errorlevel 2 goto level2

if errorlevel 1 goto level1

goto done

:level9                         command(s) to execute upon exit code 9

do something

.

.

.

:level2                         commands to execute upon exit code 2

cd \pcplus

pcplus /fhostmode               run Procomm Plus(R) using a host mode

script (see Procomm Plus(R) manual)

cd \                            when host mode is finished,

coldboot                        use a program to reboot the PC

:level1                         commands to execute upon exit code 1

cd \awhost

awhost -m=w                     run PC/Anywhere(R) in host mode

:done

cd \

cls

LanBeep v2.4                 -Sample LANBEEP.CFG-                       Page 16

The following is an example configuration for LanBeep:

;identify this region

REGION=01

;file server object type

OBJECTTYPE=4

;file server names

SERVERNAME=FS1

SERVERNAME=FS2

SERVERNAME=FS3

;access server object type

OBJECTTYPE=98

;access server name

SERVERNAME=ACCESS_SRVR

;beeper number to dial

BEEPER=912015556666

;use com2

PORT=COM2

;beep a maximum of 10 times

MAXBEEPS=10

;scan the server list every 5 seconds

SCANINTERVAL=5

;beep every 15 minutes

BEEPINTERVAL=900

;minimum of 15 errors before considered real

MINFSERRORCOUNT=15

;enable remote command recognition (see REQUIREMENTS on page 2)

TONERECOGNITION=YES

NOTE:  Using this example, if any one of the servers has a problem, LanBeep

will retry that server 15 times in a row (MINFSERRORCOUNT=15).  It will not

consider each 5 second scan (SCANINTERVAL=5) as 1 received error, it will try

up to 15 times within a single scan.

LanBeep v2.4                 -Beeper Displays-                          Page 17

Using the sample configuration file from page 16, the following is the beep

layout:

01ccc000 where:

01      indicates the region code.

ccc     is the count of beeps since last reset (implicitly by

program startup, or explicitly by command 21).

000     is the return code for the servers.

NOTE:  For every group or partial group of 8 servers, there will be 3 digits in

the beep layout for its return code, ranking from right to left corresponding

to the configuration file. If there were 24 servers, the beeper would display:

01ccc222111000 where:

222     is the return code for servers 17-24

111     is the return code for servers 9-16

NOTE:  The beeper may also display 010010 where:

01      indicates the region code.

0010    indicates that the shell isn't loaded.

This is usually followed by a beep with the previously described layout,

indicating that all of the servers are not responding, i.e., you'll get 2

beeps:

010010

01ccc255

NOTE:  A code of 255 means either all of the servers are down, the shell isn't

loaded (which would make it appear to LanBeep as if all of the servers are

down), or that the server to which the Lanbeep PC is attached is down.  For

instance, if you have a LanBeep PC attached to a Hub/MAU which hangs off of

FS4, and FS4 goes down, the PC will no longer be able to see any of the other

servers on the network.  It will get a network error and reboot after three to

ten minutes.  AUTOEXEC.BAT will fail to load the shell, and Lanbeep will beep

you indicating: on the first beep- that the shell isn't loaded, and on every

beep after that- that all of the servers are down.  It is for this reason that

we recommend hanging a LanBeep PC off of a backbone, independent of any one

specific server, when used in a multi-server environment.

LanBeep v2.4                 -Decoding Return Codes-                    Page 18

The return codes are bitmapped from right to left.  To figure out which

server(s) are not responding, convert the digits from decimal to hex to binary.

Assume that LanBeep is scanning servers 1-9.  If the beeper displays

01001001165, it means:

01      = region code.

001     = this is the first beep.

001     = return code for server 9 (9-16, but only 9 exists in this

example).

165     = return code for servers 1-8.

Convert from decimal to hex to binary:

Return code 001 = 01h = 00000001b.

Return code 165 = A5h = 10100101b.

Determine which servers are not responding:

001/01h  � 165/A5h  = status in decimal/hex.

00000001 � 10100101 = status in binary.

xxxxxxx9 � 87654321 = servers in order defined in configuration file,

laid out from right to left.

So FS1, FS3, FS6, FS8 and FS9 are not responding because their corresponding

bits are set to 1.

As previously mentioned, for each group of eight servers, there is a three

digit return code.  If you don't want to do the conversions, see the table on

page 19 which tells you what servers are not responding (within their specific

group of eight).  So, given the previous example of 001165 as the return code

for servers 1-9, after looking at the chart we can see that:

001 = the first server in this group is not responding

165 = server numbers 1, 3, 6 and 8 in this group are not responding

LanBeep v2.4                 -Conversion Table-                         Page 19

Return Code                             Corresponding Server(s)

(Decimal)                               Not Responding (Within Group)

000 ................................... None- all are ok

001 ................................... 1

002 ................................... 2

003 ................................... 1 and 2

004 ................................... 3

005 ................................... 1 and 3

006 ................................... 2 and 3

007 ................................... 1, 2 and 3

008 ................................... 4

009 ................................... 1 and 4

010 ................................... 2 and 4

011 ................................... 1, 2 and 4

012 ................................... 3 and 4

013 ................................... 1, 3 and 4

014 ................................... 2, 3 and 4

015 ................................... 1, 2, 3 and 4

016 ................................... 5

017 ................................... 1 and 5

018 ................................... 2 and 5

019 ................................... 1, 2 and 5

020 ................................... 3 and 5

021 ................................... 1, 3 and 5

022 ................................... 2, 3 and 5

023 ................................... 1, 2, 3 and 5

024 ................................... 4 and 5

025 ................................... 1, 4 and 5

026 ................................... 2, 4 and 5

027 ................................... 1, 2, 4 and 5

028 ................................... 3, 4 and 5

029 ................................... 1, 3, 4 and 5

030 ................................... 2, 3, 4 and 5

031 ................................... 1, 2, 3, 4 and 5

032 ................................... 6

033 ................................... 1 and 6

034 ................................... 2 and 6

035 ................................... 1, 2 and 6

036 ................................... 3 and 6

037 ................................... 1, 3 and 6

038 ................................... 2, 3 and 6

039 ................................... 1, 2, 3 and 6

040 ................................... 4 and 6

041 ................................... 1, 4 and 6

042 ................................... 2, 4 and 6

043 ................................... 1, 2, 4 and 6

044 ................................... 3, 4 and 6

045 ................................... 1, 3, 4 and 6

LanBeep v2.4                 -Conversion Table (Continued)-             Page 20

Return Code                             Corresponding Server(s)

(Decimal)                               Not Responding (Within Group)

046 ................................... 2, 3, 4 and 6

047 ................................... 1, 2, 3, 4 and 6

048 ................................... 5 and 6

049 ................................... 1, 5 and 6

050 ................................... 2, 5 and 6

051 ................................... 1, 2, 5 and 6

052 ................................... 3, 5 and 6

053 ................................... 1, 3, 5 and 6

054 ................................... 2, 3, 5 and 6

055 ................................... 1, 2, 3, 5 and 6

056 ................................... 4, 5 and 6

057 ................................... 1, 4, 5 and 6

058 ................................... 2, 4, 5 and 6

059 ................................... 1, 2, 4, 5 and 6

060 ................................... 3, 4, 5 and 6

061 ................................... 1, 3, 4, 5 and 6

062 ................................... 2, 3, 4, 5 and 6

063 ................................... 1, 2, 3, 4, 5 and 6

064 ................................... 7

065 ................................... 1 and 7

066 ................................... 2 and 7

067 ................................... 1, 2 and 7

068 ................................... 3 and 7

069 ................................... 1, 3 and 7

070 ................................... 2, 3 and 7

071 ................................... 1, 2, 3 and 7

072 ................................... 4 and 7

073 ................................... 1, 4 and 7

074 ................................... 2, 4 and 7

075 ................................... 1, 2, 4 and 7

076 ................................... 3, 4 and 7

077 ................................... 1, 3, 4 and 7

078 ................................... 2, 3, 4 and 7

079 ................................... 1, 2, 3, 4 and 7

080 ................................... 5 and 7

081 ................................... 1, 5 and 7

082 ................................... 2, 5 and 7

083 ................................... 1, 2, 5 and 7

084 ................................... 3, 5 and 7

085 ................................... 1, 3, 5 and 7

086 ................................... 2, 3, 5 and 7

087 ................................... 1, 2, 3, 5 and 7

088 ................................... 4, 5 and 7

089 ................................... 1, 4, 5 and 7

090 ................................... 2, 4, 5 and 7

LanBeep v2.4                 -Conversion Table (Continued)-             Page 21

Return Code                             Corresponding Server(s)

(Decimal)                               Not Responding (Within Group)

091 ................................... 1, 2, 4, 5 and 7

092 ................................... 3, 4, 5 and 7

093 ................................... 1, 3, 4, 5 and 7

094 ................................... 2, 3, 4, 5 and 7

095 ................................... 1, 2, 3, 4, 5 and 7

096 ................................... 6 and 7

097 ................................... 1, 6 and 7

098 ................................... 2, 6 and 7

099 ................................... 1, 2, 6 and 7

100 ................................... 3, 6 and 7

101 ................................... 1, 3, 6 and 7

102 ................................... 2, 3, 6 and 7

103 ................................... 1, 2, 3, 6 and 7

104 ................................... 4, 6 and 7

105 ................................... 1, 4, 6 and 7

106 ................................... 2, 4, 6 and 7

107 ................................... 1, 2, 4, 6 and 7

108 ................................... 3, 4, 6 and 7

109 ................................... 1, 3, 4, 6 and 7

110 ................................... 2, 3, 4, 6 and 7

111 ................................... 1, 2, 3, 4, 6 and 7

112 ................................... 5, 6 and 7

113 ................................... 1, 5, 6 and 7

114 ................................... 2, 5, 6 and 7

115 ................................... 1, 2, 5, 6 and 7

116 ................................... 3, 5, 6 and 7

117 ................................... 1, 3, 5, 6 and 7

118 ................................... 2, 3, 5, 6 and 7

119 ................................... 1, 2, 3, 5, 6 and 7

120 ................................... 4, 5, 6 and 7

121 ................................... 1, 4, 5, 6 and 7

122 ................................... 2, 4, 5, 6 and 7

123 ................................... 1, 2, 4, 5, 6 and 7

124 ................................... 3, 4, 5, 6 and 7

125 ................................... 1, 3, 4, 5, 6 and 7

126 ................................... 2, 3, 4, 5, 6 and 7

127 ................................... 1, 2, 3, 4, 5, 6 and 7

128 ................................... 8

129 ................................... 1 and 8

130 ................................... 2 and 8

131 ................................... 1, 2 and 8

132 ................................... 3 and 8

133 ................................... 1, 3 and 8

134 ................................... 2, 3 and 8

135 ................................... 1, 2, 3 and 8

LanBeep v2.4                 -Conversion Table (Continued)-             Page 22

Return Code                             Corresponding Server(s)

(Decimal)                               Not Responding (Within Group)

136 ................................... 4 and 8

137 ................................... 1, 4 and 8

138 ................................... 2, 4 and 8

139 ................................... 1, 2, 4 and 8

140 ................................... 3, 4 and 8

141 ................................... 1, 3, 4 and 8

142 ................................... 2, 3, 4 and 8

143 ................................... 1, 2, 3, 4 and 8

144 ................................... 5 and 8

145 ................................... 1, 5 and 8

146 ................................... 2, 5 and 8

147 ................................... 1, 2, 5 and 8

148 ................................... 3, 5 and 8

149 ................................... 1, 3, 5 and 8

150 ................................... 2, 3, 5 and 8

151 ................................... 1, 2, 3, 5 and 8

152 ................................... 4, 5 and 8

153 ................................... 1, 4, 5 and 8

154 ................................... 2, 4, 5 and 8

155 ................................... 1, 2, 4, 5 and 8

156 ................................... 3, 4, 5 and 8

157 ................................... 1, 3, 4, 5 and 8

158 ................................... 2, 3, 4, 5 and 8

159 ................................... 1, 2, 3, 4, 5 and 8

160 ................................... 6 and 8

161 ................................... 1, 6 and 8

162 ................................... 2, 6 and 8

163 ................................... 1, 2, 6 and 8

164 ................................... 3, 6 and 8

165 ................................... 1, 3, 6 and 8

166 ................................... 2, 3, 6 and 8

167 ................................... 1, 2, 3, 6 and 8

168 ................................... 4, 6 and 8

169 ................................... 1, 4, 6 and 8

170 ................................... 2, 4, 6 and 8

171 ................................... 1, 2, 4, 6 and 8

172 ................................... 3, 4, 6 and 8

173 ................................... 1, 3, 4, 6 and 8

174 ................................... 2, 3, 4, 6 and 8

175 ................................... 1, 2, 3, 4, 6 and 8

176 ................................... 5, 6 and 8

177 ................................... 1, 5, 6 and 8

178 ................................... 2, 5, 6 and 8

179 ................................... 1, 2, 5, 6 and 8

180 ................................... 3, 5, 6 and 8

LanBeep v2.4                 -Conversion Table (Continued)-             Page 23

Return Code                             Corresponding Server(s)

(Decimal)                               Not Responding (Within Group)

181 ................................... 1, 3, 5, 6 and 8

182 ................................... 2, 3, 5, 6 and 8

183 ................................... 1, 2, 3, 5, 6 and 8

184 ................................... 4, 5, 6 and 8

185 ................................... 1, 4, 5, 6 and 8

186 ................................... 2, 4, 5, 6 and 8

187 ................................... 1, 2, 4, 5, 6 and 8

188 ................................... 3, 4, 5, 6 and 8

189 ................................... 1, 3, 4, 5, 6 and 8

190 ................................... 2, 3, 4, 5, 6 and 8

191 ................................... 1, 2, 3, 4, 5, 6 and 8

192 ................................... 7 and 8

193 ................................... 1, 7 and 8

194 ................................... 2, 7 and 8

195 ................................... 1, 2, 7 and 8

196 ................................... 3, 7 and 8

197 ................................... 1, 3, 7 and 8

198 ................................... 2, 3, 7 and 8

199 ................................... 1, 2, 3, 7 and 8

200 ................................... 4, 7 and 8

201 ................................... 1, 4, 7 and 8

202 ................................... 2, 4, 7 and 8

203 ................................... 1, 2, 4, 7 and 8

204 ................................... 3, 4, 7 and 8

205 ................................... 1, 3, 4, 7 and 8

206 ................................... 2, 3, 4, 7 and 8

207 ................................... 1, 2, 3, 4, 7 and 8

208 ................................... 5, 7 and 8

209 ................................... 1, 5, 7 and 8

210 ................................... 2, 5, 7 and 8

211 ................................... 1, 2, 5, 7 and 8

212 ................................... 3, 5, 7 and 8

213 ................................... 1, 3, 5, 7 and 8

214 ................................... 2, 3, 5, 7 and 8

215 ................................... 1, 2, 3, 5, 7 and 8

216 ................................... 4, 5, 7 and 8

217 ................................... 1, 4, 5, 7 and 8

218 ................................... 2, 4, 5, 7 and 8

219 ................................... 1, 2, 4, 5, 7 and 8

220 ................................... 3, 4, 5, 7 and 8

221 ................................... 1, 3, 4, 5, 7 and 8

222 ................................... 2, 3, 4, 5, 7 and 8

223 ................................... 1, 2, 3, 4, 5, 7 and 8

224 ................................... 6, 7 and 8

225 ................................... 1, 6, 7 and 8

LanBeep v2.4                 -Conversion Table (Continued)-             Page 24

Return Code                             Corresponding Server(s)

(Decimal)                               Not Responding (Within Group)

226 ................................... 2, 6, 7 and 8

227 ................................... 1, 2, 6, 7 and 8

228 ................................... 3, 6, 7 and 8

229 ................................... 1, 3, 6, 7 and 8

230 ................................... 2, 3, 6, 7 and 8

231 ................................... 1, 2, 3, 6, 7 and 8

232 ................................... 4, 6, 7 and 8

233 ................................... 1, 4, 6, 7 and 8

234 ................................... 2, 4, 6, 7 and 8

235 ................................... 1, 2, 4, 6, 7 and 8

236 ................................... 3, 4, 6, 7 and 8

237 ................................... 1, 3, 4, 6, 7 and 8

238 ................................... 2, 3, 4, 6, 7 and 8

239 ................................... 1, 2, 3, 4, 6, 7 and 8

240 ................................... 5, 6, 7 and 8

241 ................................... 1, 5, 6, 7 and 8

242 ................................... 2, 5, 6, 7 and 8

243 ................................... 1, 2, 5, 6, 7 and 8

244 ................................... 3, 5, 6, 7 and 8

245 ................................... 1, 3, 5, 6, 7 and 8

246 ................................... 2, 3, 5, 6, 7 and 8

247 ................................... 1, 2, 3, 5, 6, 7 and 8

248 ................................... 4, 5, 6, 7 and 8

249 ................................... 1, 4, 5, 6, 7 and 8

250 ................................... 2, 4, 5, 6, 7 and 8

251 ................................... 1, 2, 4, 5, 6, 7 and 8

252 ................................... 3, 4, 5, 6, 7 and 8

253 ................................... 1, 3, 4, 5, 6, 7 and 8

254 ................................... 2, 3, 4, 5, 6, 7 and 8

255 ................................... 1, 2, 3, 4, 5, 6, 7 and 8

LanBeep v2.4                 -Problems/Possible Solutions-              Page 25

Problem                         Possible Solution

The PC appears to lock up       Wait up to ten minutes for the shell to

return control to LanBeep.  The PC will

then reboot and report the error.

No display is visible           Use /C command line option

Use /M command line option

Modem doesn't dial beeper       Check beeper string length (it cannot

be larger than your modem's buffer)

Phone isn't being answered      Use TONERECOGNITION=YES in

configuration file

Tones are not recognized from   Use a US Robotics(R) Courier(tm) modem.

remote phone                    Make sure that the supervisor chip's

date is 09/25/91 or later (see

TONERECOGNITION NOTE on page 10).

The PC reboots when you try     When you connect to PC/Anywhere(R), it

to connect to PC/Anywhere(R)    drops back to the next line of the

batch file that called it.  Note how

the sample AUTOEXEC.BAT on page 15

allows PC/Anywhere(R) to change the

current directory and clear the screen.

No reboot will take place until you

exit PC/Anywhere(R) (as long as that

option is set- see the PC/Anywhere(R)

manual).

LanBeep v2.4                 -Comments/Suggestions-                     Page 26

If you have any comments or suggestions, please address any correspondence to:

Deerfield Computer Solutions, Inc.

43 Baldwin Terrace

Wayne, NJ 07470

(908) 874-7730 (voice/fax)

We are also very concerned with putting out accurate, understandable

documentation, and would appreciate any feedback you have to offer.