��������������������������������������������������������������ͻ

�                          LANBENCH                            �

��������������������������������������������������������������Ķ

�                LAN Communications Benchmark                  �

�                       By Dave Frailey                        �

��������������������������������������������������������������ͼ

INTRODUCTION

�������������������������������������������������������������������������������

LANBENCH is a simple utility that applies an almost pure communications test

against a Novell fileserver.  The performance numbers shown are in packets

per second.  They can be used to determine how fast the entire link between

a workstation's application and the file server really is.  All the test is

doing is requesting the date and time from the file server, and counting how

many times per second the workstation can do it.  This tests all of the driver

layers on the workstation (NETX, IPX, etc...) and the workstation card.  If

the workstation's card and drivers are fast enough, the test may actually be

limited by the file server's lan card and driver.

This can be useful for comparing "before and after" changes in workstation

driver software, lan cards, or server cards and drivers.

For example, on my network, I found that Novell's NE3200 card really doesn't

perform that well as a server card - in fact the NE2000 beat it by a sizeable

margin, along with several other types of 16-bit ISA cards (at the expense of

server utilization though, but if you've got CPU utilization to give, for me

it was a smarter move to changeout the NE3200's in the file server).

A fast workstation and server should produce numbers of 1200 and greater.

If you see numbers below 600, start troubleshooting....

Dave Frailey

August 1993

DAC Micro Systems

40941 176th St E

Lancaster, CA  93535