Modem Drivers for NetWare MHS 1.5

---------------------------------

February 18, 1992

**********************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to these

modem drivers, and specifically disclaims any express or implied

warranties of merchantability, title, or fitness for a particular

purpose.

Novell's intentions for these drivers are to provide a temporary

ability to run with modems not specifically supported by the original

NetWare MHS products.  The release of such drivers is typically addressed

in future releases of NetWare MHS.

Distribution of these modem drivers is forbidden without the express

written consent of Novell, Inc.  Novell reserves the right to discontinue

distribution of these modem drivers.

Novell will not be responsible for any data loss that may result from

implementing these modem drivers.  Novell strongly recommends a backup

be made before these are used. Technical support for these drivers is

provided at the discretion of Novell.

**********************************************************************

The following high-speed modems are now available for use with NetWare

MHS 1.5 and NetWare MHS 1.5C:

o Microcom QX4232HS 9600 baud

o Practical Peripherals PM9600SA

o E-Tech BulletModem E9696M (with and without MNP)

o US Robotics COURIER HST (with and without MNP)

o Intel 9600EX

o Hayes V-Series ULTRA Smartmodem 9600

o NEC 9631 and 9632

o MultiTech 932/224

o Codex 3220 and 3260

o Motorola UDS V.3224 and V.3225

o Codex 1133

Updated drivers are available for:

o HAYES-UN, 2400B and 2400BHUB (Hayes Smartmodem 2400 baud)

o 96V32 and 96V32HUB (Hayes Ultra 9600 baud)

o HAYESMAN (Hayes 2400 baud manual dial)

o NULLMDM (The null modem driver)

o DISABLED (The disabled modem driver)

Note: All other drivers shipped with previous versions of NetWare MHS are

obsolete.

In addition to the existing drivers, new drivers are available for:

o Hayes Smartmodem 2400

o A generic 2400 baud driver for Hayes-compatible modems

Two new 2400 baud modem drivers are available for:

o The internal modem in the Compaq LTE

Fax modems supported include:

o Intel SatisFAXtion

o Generic SendFax modems

Others may be added to the list and placed on Netwire when available.

This document describes how to use these drivers.

Contents of this .ZIP file

--------------------------

BULLET96.LOG        BULLET96.DEF

HSTD-HUB.LOG        HSTD-HUB.DEF

FAXMDM.LOG          FAXMDM.DEF

2400B.LOG           2400B.DEF

INTELFAX.LOG        INTELFAX.DEF

HAYESX24.LOG        HAYESX24.DEF

2400BHUB.LOG        2400BHUB.DEF

96V32.LOG           96V32.DEF

HAYESMAN.LOG        HAYESMAN.DEF

INTELV32.LOG        INTELV32.DEF

96V32HUB.LOG        96V32HUB.DEF

DISABLED.LOG        DISABLED.DEF

HSTDNMNP.LOG        HSTDNMNP.DEF

MICROCOM.LOG        MICROCOM.DEF

HAYES-UN.LOG        HAYES-UN.DEF

BULLETEC.LOG        BULLETEC.DEF

PP96-HUB.LOG        PP96-HUB.DEF

NULLMDM.LOG

HULTRA.LOG          HULTRA.DEF

MULTECNR.LOG        MULTECNR.DEF

MULTEC2O.LOG        MULTEC2O.DEF

MULTEC.LOG          MULTEC.DEF

CODX3220.LOG        CODX3220.DEF

UDS322X.LOG         UDS322X.DEF

2400B-CE.LOG        2400B-CE.DEF

HAYESSM.LOG         HAYESSM.DEF

NEC9631.LOG         NEC9631.DEF

NEC9632.LOG         NEC9632.DEF

CMPQLTEB.LOG        CMPQLTEB.DEF

CODX3260.LOG        CODX3260.DEF

PATCH.COM           MODEM.DOC

---------------------------------

Generic Installation Instructions

---------------------------------

Unless otherwise noted, you can add modem drivers by following these

steps.

1) Stop NetWare MHS and exit.

2) If you are running the Network Edition, re-login as a supervisor

equivalent (necessary to change the MHS configuration and copy

files to the MHS directory).

3) Copy all the extracted *.DEF and *.LOG files to your %MV\MHS\SYS

directory (where %MV is the value of your MV environment variable).

4) If you are running MHS 1.5 Rev A, then set MV to the

location of your MHS directory structure, and run the PATCH.COM program

included with the modem drivers.

5) Install your new modem following the manufacturer's instructions.

6) Type "MHS" and press <Enter> twice (to display the main menu)

- Select "Manage Directories"

- Select "Set up this host"

7) Set modem type to the correct driver name (eg 96V32HUB)

- Set Default speed for modem (in this case 9600)

8) Exit to the Directory Manager

- Exit to the main menu

9) Select "Setup Transport Server"

- Set Speed sensing enabled to YES or NO depending on the modem

- Change the modem type to the correct driver name (same as in

step 5)

- Change the modem speed to match that in step 5 (here it's 9600)

10) Make sure that all "hosts which call this host" have a baud rate

compatible with yours in their host definition at this host.

11) Exit MHS, logout, and log MHS back in (for the Personal Edition,

just restart MHS).

General Note concerning high-speed modems:

If you have an older computer or a PC/XT or PC compatible computer -

check the chip used in the serial port. If the chip is an 8250 or 8250B

it will NOT work at high speed. To allow high speed modems to be used,

get an AT serial card that uses a faster version of the 8250 (numbered

as a 82450, 16450, or 16550). An AT serial card WILL work in a PC/XT.

Special Instructions for Each Modem

-----------------------------------

Microcom QX4232HS

-----------------

Modem driver name:     MICROCOM

Default speed:         19200

Speed sensing enabled: NO (Modem does the speed sensing)

Switch Settings:

SW    Front       Rear

1     up          up

2     dn          up

3     up          dn

4     dn          up

5     up          up

6     up          up

7     up          up

8     dn          dn

9     up          --

10     up          --

Practical Peripherals PM9600SA

------------------------------

Modem driver name:     PP96-HUB (can be used both to send and receive

calls)

Default speed:         19200

Speed sensing enabled: NO (Modem does the speed sensing).

NOTE: When using this modem, MHS will sometimes register a "ghost

call" a few minutes after the modem has successfully completed a

call. MHS will report a "failed" call, but there will be no ill

effects. Also, the modem may take up to twenty seconds to shut down the

connection. More recent firmware may have changed this behavior.

E-Tech BulletModem E9696M

-------------------------

Modem driver name:     BULLET96 (with MNP) and BULLETEC (without MNP).

Default speed:         19200

Speed sensing enabled: NO (Modem does the speed sensing).

US Robotics COURIER HST

-----------------------

Modem driver name:     HSTD-HUB (for dual standard and V.32 modems with

MNP) and HSTDNMNP (without MNP).

Default speed:         19200

Speed sensing enabled: NO (Modem does the speed sensing)

NOTE:  If you experience some failures when connecting this modem to

another HST Dual Standard modem, change the default speed to 9600.

When using HSTNMNP, the modem runs without MNP (V.42). This will work

for V.32 modes but not HST modes.  The files can be mixed with HSTD-HUB

to create special conditions.

Incoming Calls        Outgoing Calls

MNP Active      HSTD-HUB.DEF         HSTD-HUB.LOG

MNP InActive    HSTDNMNP.DEF         HSTDNMNP.LOG

The Modem switches should be set as follows:

1  2  3  4  5  6  7  8  9  10

-----------------------------

*  x        *  *  x        x     UP

-----------------------------

x  *           x  x        DOWN

-----------------------------

QUAD UP

x - factory standard

* - change from factory standard

Intel 9600EX

------------

Modem driver name:     INTELV32

Default speed:         19200

Speed sensing enabled: NO (modem does the speed sensing)

NOTE:  If you are using this modem to talk to a 2400 baud modem, add a

space followed by "s1 after the phone number.

NEC 9631

---------

Modem driver name:     NEC9631

Default speed:         19200

Speed sensing enabled: NO (modem does the speed sensing)

Note: For outgoing calls to some 2400 Non-MNP modems you will need to

code an additional command into the dial string in the hosts entry. When

you enter the phone number for the host enter a space and %B3 after the

phone number.

Example:

1,2065551212 %B3

You may also need to change the baud rate in the Host Table for the host

to 2400.

Important Note:  You must use the phone cord supplied with the NEC 9631

modem to ensure proper operation.  Use of a standard phone cord may cause

errors such as: NO DIALTONE or NO ANSWER. Also, do NOT use the NEC9632

modem driver with this modem. Doing so may have unpredictable

consequences.

NEC 9632

---------

Modem driver name:     NEC9632

Default speed:         19200

Speed sensing enabled: NO (modem does the speed sensing)

Notes: For outgoing calls to some 2400 Non-MNP modems you will need to

code an additional command into the dial string in the hosts entry. When

you enter the phone number for the host enter a space and %B3 after the

phone number.

Example:

1,2065551212 %B3

You may also need to change the baud rate in the Host Table for the host

to 2400.

The I9632E Dual Modem is the NEC rack-mount (intelligent chassis). You

must perform the following operations to the chassis itself:

From Command Menu on Controller:

1) Select CONFIGURE <ENTER>

2) Select CHANGE  <ENTER>

3) Select LOCAL <ENTER>

4) Select RACK and SLOT (Up and Down Arrow select field,

Right and Left change value Up and Down)

5) Press <ENTER> when unit to be configured is selected.

There will be a LONG 10 - 30 second pause

(WAIT will be flashing)

From the CHANGE (LOCAL) menu,

1) Enter number of field to change and press <ENTER>

2) Use Right and Left arrows to Change value of field.

Field     New Value

3           AT

10         IDLE

17          ON

18          ON

3) Press <EXEC> when changes are completed.

4) Press <ESC> twice to exit mode.

Important Note: Do NOT use the NEC9631 driver with this modem. Doing so

will cause problems with the 9632 and may cause problems with MHS as

well.

Hayes V-Series ULTRA Smartmodem 9600

------------------------------------

Modem driver name:     96V32 (for sending only), 96V32HUB (for sending

and receiving)

Default speed:         9600

Speed sensing enabled: NO (this driver does not support speed sensing)

NOTE: This driver has been made obsolete by the HULTRA driver. It is

still fully supported by Novell.

Hayes V-Series ULTRA Smartmodem 9600

------------------------------------

Modem driver name:     HULTRA

Default speed:         19200

Speed sensing enabled: NO (the modem does the speed sensing)

Important Note: Hayes V-Series Modems REQUIRE RTS (pin 4) to be connected

when the modem is connected at 9600 or the modem will not TRANSMIT data

to the REMOTE modem.  Some pre-made serial cables do not connect RTS to

the computer.  Please check that your cable connects the following lines

from the modem to the computer: 2, 3, 4, 5, 6, 7, 8, 20, 22.  The pins

these lines connect to on the computer side depends on the style of

computer.  For example:

o    AT 9 Pin Connectors (Modem Pin Number is the FIRST Number)

(Connector on Computer will be a 9 pin MALE (PINS) )

2-3, 3-2, 4-7, 5-8, 6-6, 7-5, 8-1, 20-4, 22-9

o    PC/AT 25 Pin Connectors (Modem Pin Number is the FIRST Number)

(Connector on Computer will be a 25 pin MALE (Pins) )

2-2, 3-3, 4-4, 5-5, 6-6, 7-7, 8-8, 20-20, 22-22

ALL LINES LISTED ARE REQUIRED ON BOTH STYLES.

Also note that A 25 pin FEMALE is a PARALLEL PORT connector.  Connecting

the modem to a PARALLEL PORT *WILL* DAMAGE the PARALLEL PORT.

Hayes Smartmodem 2400

---------------------

Modem driver name:     HAYESSM

Default speed:         2400

Speed sensing enabled: NO (only if all modems you connect to are 2400

baud or higher)

Hayes Smartmodem 2400 for callers only

--------------------------------------

Modem driver name:     2400B

Default speed:         2400

Speed sensing enabled: NO

Hayes Smartmodem 2400 for hubs and callers

------------------------------------------

Modem driver name:     2400BHUB

Default speed:         2400

Speed sensing enabled: NO

Generic 2400 baud driver for Hayes-compatible modems

----------------------------------------------------

Modem driver name:     HAYESX24

Default speed:         2400

Speed sensing enabled: NO (only if all modems you connect to are 2400

baud or higher)

Another generic 2400 baud driver that can be both a hub or a sender

-------------------------------------------------------------------

Modem driver name:     HAYES-UN

Default speed:         2400

Speed sensing enabled: YES

Generic Hayes-compatible 2400 baud driver for manual dial

---------------------------------------------------------

Modem driver name:     HAYESMAN

Default speed:         2400

Speed sensing enabled: NO

Intel SatisFAXtion

------------------

Modem driver name:     INTELFAX

Default speed:         2400

Speed sensing enabled: NO

Generic SendFax Modems

----------------------

Modem driver name:     FAXMDM

Default speed:         2400

Speed sensing enabled: NO (only if all modems you connect to are 2400

baud or higher)

Disabled Modem

--------------

Modem driver name:     DISABLED

Default speed:         Not Applicable

Speed sensing enabled: Not Applicable

Note: Use this driver to prevent MHS from checking the communication

ports if your host does not use a modem.

Null Modem

----------

Modem driver name:     NULLMDM

Default speed:         Any valid setting; limited by the processing

speed of the MHS workstation.

Speed sensing enabled: YES

Multitech 932/224

-----------------

Modem driver name:     MULTEC (does not alter Novram Setup)

MULTECNR (does alter Novram Setup)

MULTEC2O  for Older MultiTech 224 modems (does

not alter Novram)

Default speed:         19200

Speed sensing enabled: NO (modem does the speed sensing)

Notes: You must the default speed for the modem to 19200. The following

switch  settings are required for the V32 (932) Modem:  (***Very Very

Important***)

Switch #1 (8pos)                        (4pos)

1  Up                                  1  Dn

2  Up                                  2  Up

3  Dn                                  3  Dn

4  Up                                  4  Dn

5  Up

6  Up

7  Up

8  Dn

On The 224 make sure that switch one of the 8 position switch pack is UP.

Use the MULTEC2O driver if your MultiTech 224 will not work with the

standard MULTEC/MULTECNR drivers.

Codex 3220

----------

Modem driver name:     CODX3220

Default speed:         19200

Speed sensing enabled: NO (modem does the speed sensing)

Codex 3260

----------

Modem driver name:     CODX3260

Default speed:         19200

Speed sensing enabled: NO (modem does the speed sensing)

Note: There is a smaller probability of success when this modem attempts

to connect to a hub using the Intel EX modem.

Motorola UDS V.3224/3225 and Codex 1133

---------------------------------------

Modem driver name:     UDS322X

Default speed:         19200

Speed sensing enabled: NO (modem does the speed sensing)

NOTE: When using this modem, MHS will sometimes register a "ghost call"

a few minutes after the modem has successfully completed a call. MHS will

report a "failed" call, but there will be no ill effects.

Modems Bundled with the Compaq LTE

----------------------------------

Modem driver name:     CMPQLTEB or 2400B-CE

Default speed:         2400

Speed sensing enabled: YES

Note: There have been two different modems bundled with the Compaq LTE.

Try CMPQLTEB first. If you get port off-line messages or other errors,

try 2400B-CE.

A call from the LTE to a hub with the Codex 3260 modem will have a smaller

chance of success than a call to other high-speed modems using speed sensing.