STANDARD MICROSYSTEMS CORPORATION

Network Address Program

For EtherCard PLUS Family LAN Adapters

And TokenCard Elite Family LAN Adapters

USER GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

This program (NADD.EXE) displays the I/O base address, network

address, and adapter type of every EtherCard PLUS LAN Adapter

and TokenCard EliteSeries LAN Adapter installed in the computer.

To run this program, type

NADD

and press RETURN. Information similar to the following will be

displayed:

NADD Version 5.00

I/O Base  Network Address    Adapter Type

======== =================   ============

280     00 00 C0 00 00 01   8003W

The I/O base address (200 - 3E0) of every installed LAN adapter

will be shown in hexadecimal.  This information may be helpful

in determining how many adapters are installed, and at what

I/O base addresses they are located.

The network address of each adapter, which is a 48-bit binary

number, will be shown as 12 hexadecimal digits.  Some network

programs require this information.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC

and Standard Microsystems are registered trademarks of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.