This README file accompanies  NE386.ZIP. The following files are included for

this revision:

NE2000   LAN    12389   7-26-91   9:36a

NE2      LAN    12310   7-26-91   9:18a

NE3200   LAN    16041   8-09-91   8:21a

NE1000   LAN    11857   7-25-91  12:02p

NE2-32   LAN    12185   7-25-91  11:52a

!NVL0701 CFG     2478   8-20-91  12:13p

@7154    ADF     2657  10-03-88   9:42a

The above files represent the latest drivers available for the NetWare 3.1x

operating system.   These versions have been through software test.

The above server drivers address the following:

1)   These versions have been rewritten to now use the MSM (Media

Support Module).  The MSM is the new tool (module) available to LAN

driver developers from Novell to develop better LAN drivers.

2)   In some environments these new drivers could help solve

communication problems.

3)   The file server has the potential of locking up under a heavy load

if the file server is a 486 machine and you are using either an

NE2000 or an NE/2.  A race condition exists where the last remote

DMA write cycle of the National Semiconductor DP8390 (on the NIC

card) can be prematurely terminated by writing to the Command

Register with bit "RD2" set before the remote DMA write cycle has

completed (refer to a future IMSP technical bulletin for more

detailed information).  These later versions of the drivers address

the issue.

4)   On certain networks with high traffic the adapter would stop

receiving.

NOTE:  When using the NE3200.LAN, interupts must be selected for EISA

configuration.

TO INSTALL:

1)   Copy <filename>.LAN to the same directory as the other NLM files

on your system (SYS:SYSTEM is suggested).

2)    After the system is brought up, type LOAD <filename>

NOTE: <filename.lan> corresponds to the lan card you are using.

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************