ISSUE

NOTE: This is an ASCII version of the document.  It is slightly shorter than the printed version, primarily, because some illustrations can not be reproduced in ASCII.

Introduction

The Novell NetWare Management System (NMS) product family, a distributed network management system and related agents, provides a wide array of network monitoring and management capabilities for multiple vendor Ethernet and token ring networks.  A key feature of NMS is the discovery of network topology and display of that topology on the NMS maps.  The NetExplorer software is the primary component of the NMS discovery process.  NetExplorer discovers network devices and places them in the NMS database.  The NMS network maps are then created from information stored in this database.

This document describes the components of the NetExplorer system, provides a brief theory of operation, gives some tips for planning a NetExplorer discovery session, and describes the system requirements for each of the components and their impact on network performance.

This document does not provide you with operating instructions for running the NetExplorer.  For operating details concerning any of the NMS software, see the NMS documentation, in particular the chapter, "Discovering Your Network" in the NetWare Management System: Getting Started guide.

The NetExplorer Components

--------------------------

A typical NMS installation has three separate NetExplorer components:

- the NMS Console

- the NMS Server

- the LANalyzer Agent server

as described in the sections that follow.

The NMS Console

The NMS Console resides on a Microsoft Windows PC.  The NMS Console contains the NMS database and the NetExplorer Manager components of NetExplorer.  NetExplorer Manager collects discovery packets from the NMS Server, updates the NMS database, and generates discovery alarms.  Each console in an NMS managed network maintains its own separate database.  The network topology, as discovered by the NetExplorer NetWare Loadable Module (NLM), is then displayed on the NMS Console segment and internet maps.  Up to five NMS Consoles can be supported by one NMS Server at any given time.

The NMS Server

An NMS managed network contains at least one NMS Server.  The NMS Server contains the NetExplorer NLM known as NETXPLOR.NLM.  It also includes the NetExplorer discovery NLM.  The discovery NLM software queries the following network devices for information about the inventory of devices on the network:

- NetWare servers

- IP routers

- LANalyzer Agent servers

- LANtern network monitors

- NetExplorer Plus servers

It then saves the information about the discovered objects in a local file (NETXPLOR.DAT) and forwards this information to NetExplorer Manager on the NMS Console.  Each of the following three NetExplorer NLM processes queries the network for a different set of devices, as described in the sections that follow:

- NXPIP.NLM

- NXPIPX.NLM

- NXPLANZ.NLM

These NLM processes communicate through the NetExplorer Inter-Process Communication (IPC) NLM to the NMS Console.  e relationship of the discovery NLM processes, the IPC NLM, and the NMS Console.  "The Discovery Process" section later in this document describes how each of these pieces operates together to discover the contents and topology of a network.

The NetExplorer Software

---------------

NMS Console--| NMS Server   |--IP Discovery NLM

| - NetExplorer|

NMS Console--| - IPC        |--IPX Discovery NLM

| - NLM        |

NMS Console--|              |--LANalyzer Agent Discovery NLM

|              |

NMS Console--|              |

|              |

NMS Console--|              |

---------------

NXPIP.NLM

NXPIP.NLM is installed on the NMS Server.  It uses the Simple Network Management Protocol (SNMP) to discover IP routers.

NXPIPX.NLM

NXPIPX.NLM is installed on the NMS Server.  It uses a variety of NetWare, SNMP, and Internetwork Packet Exchange (IPX) functions, such as SAP and IPX diagnostics, to discover NetWare servers, IPX routers, and IPX workstations.

NXPLANZ.NLM

NXPLANZ.NLM is installed on the NMS Server.  It queries LANalyzer Agent software and LANtern network monitors for information about all the devices that exist on their network segments, regardless of protocol.

The LANalyzer Agent Servers

LANalyzer Agent is a set of NLMs that provide network analysis of a given Ethernet or token ring segment, regardless of the protocol used by stations on the segment, while discovering all nodes on that segment.  Multiple segments can be monitored by putting agents on each segment. NXPLANZ.NLM on the NMS Server queries LANalyzer Agent equipped servers using the IP and IPX protocols for information about each of the devices that resides on their segments.  LANalyzer Agent is available separately and can be installed on any NetWare 3 server or later.  LANalyzer Agent comes with a one user version of NetWare so it can be installed on a dedicated machine as well.

Additional NMS Components in an NMS Network

Operation of the three NetExplorer components already described is typically performed in conjunction with the following network components:

- NetExplorer Plus servers

- LANtern network monitors

- NetWare servers

NetExplorer Plus Servers

NetExplorer Plus servers are any NetWare 3 or later servers with NXPPLUS.NLM installed.  These servers respond to SNMP queries from NXPIPX.NLM with the following information about any workstations that are logged in to the server: LAN type, username, and address.  For best results, it is recommended that you install NetExplorer Plus on every NetWare 3 or later server on your network.

LANtern Network Monitors

Like the LANalyzer Agent software, LANtern network monitors discover all devices on the network segment they are installed on.  This information is then made available to queries from the NXPLANZ.NLM.  A LANtern network monitor operates on a single Ethernet segment and communicates only using the IP protocols.  Like the LANalyzer Agent, it recognizes all devices, regardless of protocol.

NetWare Servers

With their security set to default status, all NetWare servers allow their binderies to be examined in the discovery process.  The NMS Server queries all NetWare servers for information in their workstation tables.

NOTE: For the NetExplorer NLM to discover stations that are attached to NetWare 4 servers, NetExplorer Plus must be installed on the server.

The Discovery Process

---------------------

NetExplorer discovery proceeds to discover the internetwork that it resides on through a process initiated and controlled by the NetExplorer Inter-Process Communication (IPC) NLM (NETXPLOR.NLM).  To identify themselves to the IPC NLM, the discovery NLM sets up an IPC connection.  The IPC NLM then begins the initial discovery cycle starting with IP discovery, followed by IPX discovery, and finally LANalyzer Agent discovery.  Once all three discovery processes have run once, the initial discovery cycle is complete.

Information gathered by the discovery NLM is stored in the NETXPLOR.DAT file on the NMS Server.  Once IP discovery is finished, IPX discovery begins and IP discovery starts over again.  Once the first cycle of IPX discovery is finished, LANalyzer Agent discovery begins and IPX discovery starts its second cycle.  All three of the discovery NLMs run continually to detect changes to the network, unless otherwise directed.  Any changes to the network are saved to the NETXPLOR.DAT file to keep the network database up-to-date.

Each of the discovery NLMs is installed on a NetWare 3 or later server.  A brief description of how each of these NLMs operates is provided in the following sections.

IP Discovery

The first sequence in the NetExplorer discovery cycle involves the discovery of IP routers.  The IP discovery NLM locates its local router using TCP/IP configuration information and queries it for the identity of other routers on the network.  It then queries the MIBs on the routers using SNMP.  The following information is then collected: router IP address, router interface types, MAC address of each router, network adapter, and number of hops required to get to that router from the NMS Server.

The scope of IP discovery is defined by the user.  By default, the IP routing scope is the local IP network.  For example, if your network address is 130.59.0.0, the IP discovery NLM discovers the entire 130.59.0.0 network and its subnets.

IPX Discovery

After the first cycle of IP router discovery, the IPX discovery process begins.  IPX discovery begins at the NMS Server.  IPX discovery uses a series of techniques, including SNMP, RIP, SAP, and IPX and SPX (Sequenced Packet Exchange) diagnostics to discover the attached IPX internetwork.  The IPX discovery NLM (NXPIPX) begins by examining its own server.  Through examination of the bindery, it learns the names of all the IPX devices that it is connected to.  It then queries the binderies of each of these devices and determines what they are attached to.  It then queries the binderies of each of those devices, and so on.  This process continues until all IPX networks reachable from the NMS Server are discovered.

LANalyzer Agent Discovery

LANalyzer Agent monitors every packet on the network segment it is installed on, creating a list in local memory of all the physical addresses and IP addresses of all the devices communicating on the segment.  Once the first pass of the IPX discovery cycle is completed, the NetExplorer LANalyzer discovery NLM (NXPLANZ) uses SNMP to query all LANalyzer Agent servers and LANtern network monitors to read their discovery databases.

The NetExplorer IPC NLM (NETXPLOR.NLM)

The NetExplorer IPC NLM uses Inter-Process Communication to collect discovery data from the discovery NLMs and make it available to the NetExplorer Manager.  This file is kept on the NMS Server, and is made available to the NetExplorer Manager on NMS Consoles through communication between NetExplorer Manager and NETXPLOR.NLM.

As the discovery processes gather information about devices on the network, they forward packets of related data to the NetExplorer IPC NLM.  The IPC NLM process places these packets, along with a transaction number and a time stamp, into the sequential file NETXPLOR.DAT.

NetExplorer Discovery and NETXPLOR.DAT

|                          -----   -------------------------------

|IP Discovery--------------|    |  |Transaction |Time & |Discovery

|                          |    |  |Number      |Date   |Data

|IPX Discovery-------------|    |--|------------|-------|---------

|                          |    |  |____________|_______|_________

|LANalyzer Agent Discovery-|    |  |____________|_______|_________

|                          |    |  |____________|_______|_________

|                           -----

The NetExplorer Manager

A NetExplorer Manager resides on each of the NMS management consoles.

It performs the following tasks:

- Extracts information from the NETXPLOR.DAT file

- Interprets information from the NETXPLOR.DAT file

- Checks to see whether the object has already been discovered

- Uses the rules list to determine any additional attributes of the discovered object

- Generates NetExplorer alarms, and directs discovery information to the NMS database

NetExplorer Manager begins by looking up the name of the NMS Server in its NMS.INI.  This name is entered during installation of the NMS Console.  It then uses the SPX protocol to connect to the NMS Server.  The NetExplorer Manager begins copying NETXPLOR.DAT, beginning with the first record.  The NetExplorer Manager maintains the transaction number of the last transaction processed from NETXPLOR.DAT.  If the connection is broken between the NetExplorer Manager and the NMS Server, the NetExplorer Manager begins copying from NETXPLOR.DAT in the location where it left off.

The NetExplorer Manager receives a transaction record from the NMS Server and begins its activity by searching the database to determine whether the device in the transaction is in any of the database tables.  If it is not, the NetExplorer Manager enters a record of it into an appropriate table.

NetExplorer Manager then looks for a rule in its rules table to help classify the device by matching it one-by-one to a list of predetermined rules.  If the device in the transaction matches one of the rules, NetExplorer Manager adds the attribute of the device to the appropriate location of the database.  This is done to determine additional attributes of the device, such as what icon should be used to display the device on the network maps.  Rules in the NetExplorer Manager rules list are not mutually exclusive, and NetExplorer Manager doesn't stop examining the transaction until it has compared its device to each of the rules on the rules list.  All device information obtained by comparison to the list of rules is forwarded to the NMS database and Alarm Manager.

NetExplorer Manager compares each transaction sequentially to each of the following three lists of rules:

1. Built-in rules

2. Third-party rules

3. MAC address rules

The NetExplorer Manager Rules Table

==================================

(1) Built in NMS Rules

|             ---------------------> To Database

|                |

|                |

|                v

(2) Third Party Rules

|             ---------------------> To Database

|                |

]                v

(3) User Defined Rules

|   MAC Address Mask Rules

|             ---------------------> To Database

|                |

|                |

|                v

|           Classified as an Unknown Object

Built-in rules always apply to objects in the rules table and are supplied with NMS.  The third party rules are added to the NetExplorer rules table by third-party NMS applications to increase the scope of NetExplorer discovery.  MAC address rules define additional attributes for network objects within a MAC address range.  For example, a built-in MAC address rule specifies that a MAC address with a prefix of 00-00-1B is a Novell network adapter.  A large number of MAC address rules are predefined in NMS.  The NXPICON.EXE tool supplied with NMS allows end-users to add MAC address rules.

Network objects that do not correspond to enough of the NetExplorer rules to be fully identified appear as "unknown object" icons on the NMS maps.  To avoid unknown object icons on your maps, a set of MAC address rules should be defined and added using NXPICON.EXE.

You should define any new MAC address rules before running the NetExplorer NLM.  If these rules are not defined before the NetExplorer NLM has been run, you must restart the NLM to cause NetExplorer Manger to update the devices already in the database.  As the new discovery data comes in from the NLM, NetExplorer Manager reviews and updates the database.

What NetExplorer Discovers

The NXPIPX, NXPIP, and NXPLANZ NetExplorer NLM (provided with NMS 2.0) use a variety of techniques to discover the following categories of network devices and present them on the NMS maps:

- NetWare servers

- Workstations that log in to NetWare servers

- IP routers

In most instances, information gathered by NXPIP and NXPIPX is sufficient to place devices on the network maps correctly.  When NXPIP and NXPIPX do not gather enough information however, NXPLANZ.NLM retrieves MAC addresses collected by the LANalyzer Agent and LANtern network monitors.  If NXPIP and NXPIPX have not discovered devices using these MAC addresses, then the new devices are added to the database.  Consequently, all devices will be discovered on segments monitored by LANalyzer Agent or LANtern network monitor.

Details concerning the information that is discovered within each of these categories, and the limitations of the NetExplorer discovery process for discovering them, are discussed in the following sections.

NetWare Servers

NXPIPX.NLM discovers all NetWare servers from the NetWare 2, NetWare 3, and NetWare 4 families.  Certain NetWare server devices that are not equipped with the IPX diagnostic responder are sometimes not discovered or are discovered incompletely.  This limitation is discussed in detail in "What NetExplorer Doesn't Discover."

Workstations

NXPIPX.NLM discovers all stations attached to discovered NetWare 3 and NetWare 4 servers.  NetWare 4 servers must have the NetExplorer Plus software installed for its attached stations to be discovered.  Workstations that are turned off or not attached to any server are not discovered.  For this reason, a NetExplorer cycle run at night or on a weekend may not yield a complete map.

Once a station has been discovered, NXPIPX.NLM queries its IPX diagnostic responder to confirm the discovery and gather more information about it.  If the IPX diagnostic responder is turned off, the station is not placed on the map.

IP Routers

NXPIP.NLM uses primarily, the SNMP protocol, to query all IP routers on the current network and all other specified networks using the community string.  NXPIP assumes a default community string of public, and unless instructed otherwise, will not discover an IP router with a different community string.  Routers with community strings other than public can be discovered by providing a community string file.

If router information such as router name and location are configured in the router's SNMP Management Information Base (MIB), it is discovered and entered into the NMS database and displayed as appropriate on the NMS maps.  This information can be configured into the MIB using any SNMP configuration tool, including the NMS SNMP MIB Browser.  By default, IP discovery only discovers routers using the same network number as the NMS Server.  The exploration can be expanded by specifying additional network numbers in NXPIP.INI.

What NetExplorer Doesn't Discover

There are eight broad categories of network devices that the NetExplorer NLMs may not discover completely or correctly:

- Nodes not equipped with the IPX diagnostic responder

- NetWare SFT III servers

- Source Route Bridged token rings

- Routers that use multiple MAC addresses

- Non NetWare routers

- NetWare MultiProtocol Router software with multiple WAN ports

- Transparent Bridges

A description of how each of these categories of device are discovered by the NetExplorer NLM is discussed in the following sections.

ARCnet Workstations

NOTE:  The ARCnet section of the printed document is incorrect and has therefre, been deleted from the ASCII version.

Nodes Not Equipped with the IPX Diagnostic Responder

The following NetWare server devices are discovered by NXPIPX, but are not necessarily placed correctly on the NMS maps:

- NetWare for UNIX servers

- Portable NetWare servers

- Access servers

- Modem servers

- Print servers

- UnixWare Stations

- Network Printers

Because these devices do not respond to IPX diagnostics, they are unable to answer queries from NXPIPX.NLM.  Consequently, the LAN information required to place them on the maps may not be available.  In this situation, these devices are placed in the DISCOVERY IN PROGRESS segment of the NMS maps.  Further information must be discovered by other NLMs, such as NXPLANZ.NLM, or added manually using the Configure > Selected Object command for these devices to be placed in their correct locations.

NetWare SFT III Servers

SFT III servers are not fully discovered.  The I/O engine portions of an SFT III server are not discovered.  The MS engine server is discovered but not identified as the MS engine portion of an SFT III server, and is placed in the DISCOVERY IN PROGRESS segment as a file server.  It is placed in the database with no LAN information.  Instructions for making SFT III servers appear correctly in the NMS maps is provided in the "Discovering Your Network" chapter in the NetWare Management System: Getting Started manual.

Source Route Bridged Token Rings

The way NMS maps display Source Route Bridged (SRB) token rings depends on whether you have LANalyzer Agent installed on each ring:

- If you do not have LANalyzer Agent installed on each SRB ring in your network, NMS discovers the network but consolidates all groups of SRB token rings into the same network number and displays them as a single segment.

- If you do have LANalyzer Agent installed on each SRB ring, each LANalyzer Agent discovers its own ring (segment) and every device on it.  NMS displays the ring as an unconnected segment in the internet map.

- If you have LANalyzer Agent installed on an SRB ring connected to a router, the NMS internet map shows the correct connections.  However, if two networks each have several rings and only one ring in each network is connected to a router, the NMS internet map shows the correct connections only of the rings that are directly connected to the router.  The other Source Route Bridged rings in each network are displayed as disconnected segments in the internet map.

If bridge information is not discovered, NetExplorer treats each interface of a bridge as a separate device on the network.  One icon appears on the NMS maps for each interface of the bridge.

When you have LANalyzer Agent installed on one server on each ring of an IPX Source Route Bridged network, the segment names displayed on the NMS internet map consist of the IPX network number followed by the MAC address of that server's interface to the ring.  If LANalyzer Agent is monitoring more than one interface, the address shown for a ring is the MAC address of the interface monitoring that ring.

Routers That Use Duplicate MAC Addresses

The NetExplorer discovery NLM can experience difficulties correctly characterizing some NetWare routers because of the method the routers use to identify their adapters.  In some cases, all the network adapters on a router have the same MAC address.  In these cases, it appears to NXPIPX that one adapter is connected to multiple segments.  Unless otherwise specified, NXPIPX interprets that the multiple adapters are one adapter.  Therefore, the multiple segments that the adapters are connected to are seen as one segment and NetExplorer mistakenly consolidates the multiple segments.  Special conditions can be set in the NetExplorer section of NMS.INI to describe these conditions.

For example, NMS has a special rule to determine when DECnet routers are in use.

An example of a DECnet router configuration that has three interfaces with the same MAC address (AA0004001234).

|_________________|__________|____________|

|

|

DECnet Router

/\

AA-00-04-0012-34     /  \   AA-00-04-0012-34

In the NetExplorer section of NMS.INI, AA0004 is designated as a duplicate MAC address (DupMac1=AA0004).  This tells NetExplorer Manager that any MAC address beginning with AA-00-04 is allowed to be duplicated.  Hence, NetExplorer Manager will know not to treat the address as unique, causing all interfaces and attached networks to be consolidated.  A similar adjustment exists for Wellfleet routers in NMS.INI.  You can make similar adjustments to NMS.INI for other router types.  If your network routes the XNS protocol, you must enter the MAC address of the routers in NMS.INI.

Non NetWare Routers

NXPIP discovers IP bound interfaces only.  In a case where IP isn't being run on a router, the IPX-bound interfaces are discovered under the following conditions:

- Each individual interface of the router is discovered if it is connected to a NetWare server.

- Interfaces that are discovered are not placed in the same router box on the NMS maps.  Consequently, the interconnections are not correct in the internet map and the router box appears in multiple places with different interfaces.

NetWare MultiProtocol Router with Multiple WAN Ports

The NetExplorer discovery NLM has difficulty discovering WAN connected NetWare MultiProtocol Router correctly.  This is because all WAN ports on these routers effectively have the same port address.

Example

100186080000 WAN Port #1\

|                         \             LAN Connection

|                           > MPR -----------------------

|                          /

100186080000 WAN Port #2 /

The NetExplorer discovery NLM determines the network addresses of WAN ports 1 and 2 using the internal network number of the router and adding zeros.  This results in an address for both WAN port #1 and WAN port #2 of 100186080000.  To prevent NetExplorer Manager from consolidating both of these ports as a single port, the address of the WAN connection must be entered in the NetExplorer section of NMS.INI as a duplicate address.

Transparent Bridges

NetExplorer cannot fully discover transparent bridges.  NMS consolidates groups of transparently bridged segments running the same network number into a single segment in the maps.

NetExplorer Operation

Through experience operating the NetExplorer software on real-life networks, several techniques have been learned for accurately and efficiently using the NetExplorer software for building a network database.  This includes setting up the components with the correct hardware and software, determining the load impact on your network, planning how to implement the process, and tips for running the NetExplorer NLMs.  Each of these topics are thoroughly considered in the following sections.

Hardware and Software Requirements for Running the NetExplorer Components

Figure 14 (shown only in Postscript printed version) shows each of the NetExplorer components as they are deployed in a typical NetWare network.

The software and hardware requirements for each of the components in Figure 14 are detailed below.

NMS Console Requirements

|System Component  Requirement for Operation

|----------------  -------------------------

|CPU               386 or 486 based PC

|Display           VGA or Super VGA

|Mouse             MS Windows 3.1 supported

|Adapter           Novell certified network adapter w/ODI driver

|RAM               12MB, inc. 510KB lower RAM free before starting Windows

|Disk Space        Approximately 60MB free, depending on network size

|                   - Small (up to 100 nodes): 10MB

|                   - Medium (up to 1,000 nodes): 30MB

|                   - Large (up to 10,000 nodes) 50MB

|                   - 30MB of additional free disk space is required:

|                      - 20 MB for NMS Software

|                      - 10 MB for NetWare Expert help

Operating System Software

DRDOS 6.0, Novell DOS 7.0 or later, or MSDOS 5.0 or later;

MS Windows 3.1 running in enhanced mode

Additional Software Requirements

NetWare Client 4.01 must be installed before NMS installation.

SHARE.EXE must be loaded and running

NetExplorer Server Requirements

System Component   Requirement for Operation

----------------   -------------------------

CPU                386, 486 or Pentium based PC

RAM                8MB dedicated to NetExplorer, in addition to memory required by the server & other applications (minimum 8MB on NetWare 3, 10MB on NetWare 4 servers)

Disk Space         20 MB available

Operating System   NetWare 3.11 or later

Server Running NXPPLUS.NLM Software Requirements

================================================

System Component      Requirement for Operation

----------------      -------------------------

Operating System      NetWare 3.11 or later

NetWare Management Agent Operating System Requirement

=====================================================

NetWare 3.11 or later

NetWare LANalyzer Agent Requirements

====================================

System Component    Requirement for Operation

----------------    --------------------------

Operating System    NetWare 3.11 or later

LAN Adapter         Ethernet or Token Ring w/promiscuous mode driver

Load Considerations for Running NetExplorer

To help you plan the impact that NetExplorer software will have on your network, data from discovery sessions involving the following two sample networks is provided:

- A small 31 node network

- A large 4862 node internetwork

The following equipment was used to operate the NetExplorer software in these sample discovery sessions:

- The NMS Server was a Compaq 486/33.  It was running only the NLMs required for an NMS Server configuration.

- The NMS Console was a Compaq 486/33 running only the NMS Console software.

These examples provide load statistics for the NMS Console, the NMS Server, and the network that is being discovered.

The following table describes each of the statistics obtained in the tests:

Key to Test Statistics

======================

- Average Utilization

Describes the average utilization of the NMS Server during the course of the cycle being tested.  This statistic is an average; percent utilization will spike up to a higher percentage for periods of time during operation.

- Response Time (to IP Ping)

Describes the time it takes the server being tested to respond to an IP ping from a workstation on its LAN.  This statistic gives an indication of the server's responsiveness to external demands.

- Cache Buffers Used

Indicates the number of cache buffers used by the discovery process, during specific operations of the test.  Statistics are given for cache buffer use without NetExplorer running, during the discovery cycle, and while the NMS Server is transmitting to the NMS Console.  The number of cache buffers used without NetExplorer running is always given as 0 (zero) for use as a baseline.

- Average Packet Rate

Indicates the average packet rate in packets per second on the effected network during the discovery cycle, and while the NMS Server is transmitting to the NMS Console.

Total # of Packets (Transmitted and Received)

Indicates the total number of packets transmitted to and from the NMS Server.  This number is given for the length of the discovery cycle and for the length of time that the NMS Server is transmitting to the NMS Console.

- Average Packet Size

Indicates the average size of a packet being received or transmitted from the NMS Server during the specified period.  This statistic is given for the discovery cycle and for the length of time that the NMS Server is transmitting to the NMS Console.

- Average % Load on T1 Link

Indicates what the load on a T1 (1.544 Mbps) remote link would be if the entire network being discovered were on the other side of the remote link from the NMS Server.  This statistic is the maximum load that a remote link would carry to discover the network in question.  It was computed from the average packet rate assuming that every packet would be crossing the T1 link.  To obtain an estimate of the increase in utilization that the T1 link would experience during a more realistic discovery process, multiply the percentage of the network that you expect would be on the other side of the link times the statistic given.

For example, if your network is in two locations with a T1 link connecting them, and the number of stations in each location is approximately equal, you can estimate that the increase in utilization of the T1 link due to NetExplorer will be about half the value given in the table

- Average % Load on Ethernet LAN

Measures the average % network utilization as computed during the specified process.  This statistic is given for the discovery cycle and for the length of time that the NMS Server is transmitting to the NMS Console.

- Console Performance Characterization

Indicates the impact on performance of the NMS Console during a specified NetExplorer process.  This characterization of console performance is given for the initial discovery cycle and also for later cycles.

- Maximum Packet Rate

Indicates the maximum packet rate observed during the discovery cycle of the large network.

- Maximum % Load on T1 Link

Indicates the maximum % load computed from the maximum rate for a T1 link (1.544 Mbps) during the discovery cycle of the large network.

- Maximum % Load on Ethernet LAN

Indicates the maximum % load for the Ethernet LAN computed from the maximum packet rate during the discovery cycle of the large network

Small 31 Node Network Example

In this example a small network with 31 nodes was discovered, to measure the impact on the NMS Server, the network being discovered, and the NMS Console.  As clearly demonstrated by the statistics, the impact of operating the NetExplorer discovery NLM is minimal on network, server, and remote link performance.

Table 7 provides a brief description of these performance impacts.

Table 7 Summary of Test Results from Small 31-Node Example

==========================================================

- NetExplorer Component = NMS Server

Impact on Performance while Running NetExplorer NLM

---------------------------------------------------

The NMS Server showed a minimal increase in average utilization (from 1% to 4%) during operation of the discovery NLM and no measurable increase during the period of time that the NMS Server was transmitting to the NMS Console.

Response time of the NMS server to an IP Ping was shown not to be affected during either operation of the discovery NLM or during the period of time that the NMS Server was transmitting to the NMS Console.

A modest 285 cache buffers were used during operation of discovery NLM and only 74 were used during the period of time that the NMS Server was transmitting to the NMS Console.

- NetExplorer Component = Network Being Discovered

Impact on Performance while Running NetExplorer NLM

---------------------------------------------------

A minimal increase of 7 packets/s was added to the average packet rate during operation of the discovery NLM.  An increase of only 1

packet/s was observed during the period of time that the NMS Server was transmitting to the NMS Console.  Average utilization of the Ethernet LAN was measured as a low .2%

- NetExplorer Component = Remote Link

Impact on Performance while Running NetExplorer NLM

---------------------------------------------------

Even if the entire sample network were located on the other side of a T1 link, the NetExplorer NLM would only consume 0.4% of the available bandwidth.

Similarly, with both NMS Server and NMS Console located on opposite sides of a T1 link, the increase of bandwidth utilization would be .04% while the NMS Console was transmitting to the NMS Server.

- NetExplorer Component = NMS Console

Impact on Performance while Running NetExplorer NLM

---------------------------------------------------

During the initial discovery cycle, when the NMS maps were being created, NMS Console performance was impacted significantly.  However, during subsequent discovery cycles, the NMS Console only experienced occasional periods when performance was impacted to a noticeable extent.

Table 8 through Table 10 detail the statistics used to determine the summary described in Table 7.  Table 8 contains the results of testing the average utilization, response time, and cache buffer utilization of the NMS Server.

Table 8  NMS Server Load on Small 31 Node Network

=================================================

|                  |Average    |Response Time| Cache

|                  |Utilization|(to IP Ping) | Buffers

|                  |           |             | Used

------------------------------------------------------

Without NetExplorer|           |             |

Running            |   1%      |   .4ms      | 0

|                  |           |             |

During the         |           |             |

Discovery Cycle    |   4%      |   .4ms      | 285

|                  |           |             |

While NMS Server   |           |             |

is transmitting to |           |             |

the NMS Console    |   1%      |   .3ms      | 74

Table 9 contains load statistics for the small network during operation of the discovery NLMs.  The results of this test include average packet rate, total number of packets transmitted and received, average packet size, and average percent load on the local Ethernet LAN and on the T1 remote link (if the entire network were on the other side of the remote link from the NMS Server).  These statistics are provided for the period of time while the NLMs are discovering the network (during the discovery cycle), and also for when the NMS Server is transmitting the results of its discovery process to the NMS Console.

Table 9 NetExplorer Impact on Small 31 Node Network

===================================================

|             During the| While the NMS Server

|             Discovery | is Transmitting to

|             Cycle     | the NMS Console

----------------------------------------------

Average     |           |

Packet Rate | 7 packets | 1 packet

|           |           |

Total#      |           |

of Packets  |           |

(Transmitted|           |

& Received) | 9000      | 500 packets

|           |           |

Average     |           |

Packet Size | 120 bytes | 110 bytes

|           |           |

Average %   |           |

Load on T1  |           |

Link        | .4%       | .04%

|           |           |

Average %   |           |

Load on     |           |

Ethernet LAN| .06%      | .007%

It took 10 minutes and 29 seconds for the NMS Server to transmit its discovery data to the NMS Console.

Table 10

NMS Console Performance on Small 31 Node Network During Discovery

=================================================================

During the Initial Discovery Cycle

----------------------------------

Console performance is very sluggish

During Later Discovery Cycles

-----------------------------

Console performance has occasional periods of slight sluggishness

Large 4862 Node Internetwork Example

In this example a large internetwork with 4862 nodes was discovered, to measure the impact on the NMS Server, the network being discovered, and the NMS Console.  As clearly demonstrated by the statistics, the impact of operating the NetExplorer discovery NLM is minimal on network, server, and remote link performance.

Table 11 Summary of Test Results from Large 4862-Node Example

=============================================================

Description of Impact on Performance while Running NetExplorer NLM

NMS Server

The NMS Server showed a minimal increase in average utilization (from 8% to 10%) during operation of the discovery NLM, and showed no measurable increase during the period of time that the NMS Server was transmitting to the NMS Console.

Response time of the NMS Server to an IP Ping was shown not to be significantly affected during operation of the discovery NLM.  During the period of time that the NMS Server was transmitting to the NMS Console, the IP Ping response time showed an increase from .6 ms to .9 ms.  This increase is not judged to be a serious impediment to performance.

715 cache buffers were used during operation of discovery NLM and only 258 were used during the period of time that the NMS Server was transmitting to the NMS Console.

The maximum observed packet rate, % T1 load, and % Ethernet LAN load are included to make you aware of spikes in demand that occur during operation of the NetExplorer process.  These spikes are only occasional and are not a cause for concern.

Network Being Discovered

A small increase of 20 packets/s was added to the average packet rate during operation of the discovery NLM.  An increase of only 1 packet/s was observed during the period of time that the NMS Server was transmitting to the NMS Console.  Average utilization of the Ethernet LAN by the discovery NLM was measured as a low .2%.

Remote Link

Even if the entire sample network were located on the other side of a T1 link, the NetExplorer NLM would only consume 1% of the available bandwidth.

Similarly, with both NMS Server and NMS Console located on opposite sides of a T1 link, the increase of bandwidth utilization was .07% while the NMS Console was transmitting to the NMS Server.

NMS Console

During the initial discovery cycle, when the NMS maps were being created, NMS Console performance was impacted significantly.  However, during subsequent discovery cycles, the NMS Console only experienced occasional periods when performance was impacted to a noticeable extent.

Table 12 through Table 15 detail the statistics used to determine the summary described in Table 11.

Table 12 NMS Server Load on Large 4862-Node Internetwork

========================================================

|                  |Average    |Response Time| Cache

|                  |Utilization|(to IP Ping) | Buffers

|                  |           |             | Used

------------------------------------------------------

Without NetExplorer|           |             |

Running            |   8%      |   .6ms      | 0

|                  |           |             |

During the         |           |             |

Discovery Cycle    |  10%      |   .5ms      | 715

|                  |           |             |

While NMS Server   |           |             |

is transmitting to |           |             |

the NMS Console    |   8%      |   .9ms      | 258

Table 13 contains load statistics for the large 4862 node internetwork, during operation of the discovery NLM.  The results of this test include average packet rate, total number of packets transmitted and received, average packet size, and average percentage load on the local Ethernet LAN and on the T1 remote link.  These statistics are provided for the period of time while the NLMs are discovering the network (during the initial discovery cycle), and also for when the NMS Server is transmitting the results of its discovery process to the NMS Console.

Table 13

NetExplorer Software Impact on Large 4862 Node Test Internetwork

================================================================

|             During the | While the NMS Server

|             Discovery  | is Transmitting to

|             Cycle      | the NMS Console

----------------------------------------------

Average     |            |

Packet Rate | 20 packets | 1 packet

|           |            |

Total#      |            |

of Packets  |            |

(Transmitted|            |

& Received) | 4,000,000  | 65,500 packets

|           |            |

Average     |            |

Packet Size | 120 bytes  | 130 bytes

|           |            |

Average %   |            |

Load on T1  |            |

Link        | .1%        | .07%

|           |            |

Average %   |            |

Load on     |            |

Ethernet LAN| .2%        | .01%

Table 14 contains statistics for the maximum load that was observed during the large network testing.  It describes maximum packet rate, maximum percentage load on the T1 remote link, and maximum percentage load on the Ethernet LAN for the network during the initial and later discovery cycles.

Table 14. Maximum Load, NetExplorer on Large 4862-Node Internetwork

===================================================================

Maximum Packet Rate During Discovery Cycle = 250 packets/s

Maximum % Load on T1 Link During Discovery Cycle = 14.8%

Maximum % Load on Ethernet LAN During Discovery Cycle = 2.3%

It took 19 hours, 20 minutes, and 10 seconds for the NMS Server to transmit its discovery data to the NMS Console.

Table 15. NMS Console Performance on Large 4862-Node Internetwork

================================================================

During the Initial Discovery Cycle

----------------------------------

Console performance is very sluggish

During Later Discovery Cycles

-----------------------------

Console performance has occasional periods of slight sluggishness

Tips for Planning the Discovery Process

Before operating the discovery NLM, you should determine the protocols operating on your network and plan the use of the discovery NLM accordingly.  As discussed earlier, the NetExplorer network discovery process proceeds through the use of three server NLMs: the IP discovery NLM file (NXPIP.NLM), the IPX discovery NLM (NXPIPX.NLM), and the NetWare LANalyzer Agent NLM (NXPLANZ.NLM).  Depending on the composition of your network, you will run one or more of these NLMs, as described in Table 16.

Table 16

When to Run Discovery NLM

=========================

Run NXPIP.NLM if you have IP routers on the network you are discovering.

Run NXPIPX.NLM if you have IPX devices, such as NetWare servers, on your network.

Run NXPLANZ.NLM if you have LANalyzer Agent software installed on your network.

Planning for IP Discovery

The network that you want discovered must meet the following requirements to be prepared for accurate IP discovery:

-- All routers should use the community name "public" or a comprehensive list of community names must be provided in a community name file.

-- SNMP must be enabled on all IP routers to be discovered.

-- For router information such as router name and location to be discovered by NetExplorer and placed in the NMS database, the information must be configured in the router's MIB.

-- IP forwarding must be enabled for a router to be discovered as a router.

Planning for IPX Discovery

The network that you want discovered must meet the following requirements to be prepared for accurate IPX discovery:

-- To ensure discovery of the workstations on your network, you should install NetExplorer Plus on all NetWare 3.11 or later servers.

-- Workstations that are only logged in to NetWare 4 servers that do not have NetExplorer Plus installed will not be discovered.  For this reason, it is highly recommended that you install NetExplorer Plus on all NetWare 4 servers.

-- It is also highly recommend that NetExplorer Plus be installed on at least one server on every network segment.  If this is not done, the network media type will not be discovered.

-- It is recommended that you install NetExplorer Plus on all servers configured at a higher than default level of security and on servers that most workstations log in to.

-- Workstations must have diagnostic support enabled to be fully discovered.

Planning for LANalyzer Agent Discovery

It is recommended that LANalyzer Agent software be deployed on each segment of your network.  Deploying LANalyzer Agent software on your network enhances discovery by

- Accurately depicting all token ring segments in the maps

- Discovering all IP devices

- Discovering non-IP or IPX devices on your network

Operating the Discovery Software

The three discovery NLMs operate sequentially.  For the best possible results with the initial discovery of complex networks, you should run each NLM until the results are satisfactory before running the next NLM.  This process is only necessary the first time that the discovery NLM is run.  Once the configuration problems detected by this process are corrected, the discovery NLM can run unattended.  In the following sections, a procedure is described for obtaining accurate results using each of the discovery NLMs.

Running NXPIP.NLM

The following procedure describes the best method for using NXPIP.NLM to discover IP devices on your network:

1. Run NXPIP.NLM.

2. Discover the network map.

3. Check the number of routers discovered from the screen titled "NXPIP" on the NMS Server.

Verify that number against your knowledge of the number of IP routers on the discovered network.

4. Select the View > All Routers command from the NMS menu.

Compare the information contained in this window with information that you have about the routers on the discovered network.  Data from the Routers window can be exported to a file or printed to make your examination easier.

5. Use the troubleshooting steps contained in the product documentation on undiscovered routers to determine why they were not discovered.

6. Correct the configuration problems identified by the previous step.

7. Rerun NXPIP.NLM.

8. Discover the map again.

NOTE: Configuration changes that occur while IP discovery is in process are not detected.  Because of this, and also to ensure the highest degree of accuracy, it is recommended that you run NXPIP.NLM at least two or three times and compare the results from each cycle.

Running NXPIPX.NLM

The following procedure describes the best method for using NXPIPX.NLM to discover IPX devices on your network:

1. Run NXPIPX.NLM after you are satisfied with the database created by the NXPIP.NLM.

2. Discover the network map.

3. Check the number of routers, servers, and workstations discovered from the screen titled "NXPIPX" on the NMS Server.  Compare that number to your knowledge of the discovered internetwork.

4. Determine what segments or types of devices, if any, were not discovered.

5. Use the troubleshooting steps contained in the product documentation on undiscovered segments and stations to determine why they were not discovered.

6. Correct the configuration problems identified by the previous step.

7. Rerun NXPIPX.NLM.

8. Discover the map again.

Running NXPLANZ.NLM

The following procedure describes the best method for using NXPLANZ.NLM to discover additional devices on your network:

1. Run NXPLANZ.NLM after you are satisfied with the database created by the NXPIP.NLM and the NXPIPX.NLM.

2. Discover the network map.

3. Check the number of segments discovered from the screen titled "NXPLANZ" on the NMS Server.

Compare that number to your knowledge of the number of segments containing either LANalyzer Agent or a LANtern network monitor on the discovered internetwork.

4. Determine which segments, if any, were not discovered.

5. Use the troubleshooting steps contained in the product documentation on undiscovered segments to determine why they were not discovered.

6. Correct the configuration problems identified by the previous step.

7. Rerun NXLANZ.NLM.

8. Discover the map again.

-----------------------------------------------------------------

Any trademarks referenced in this document are the property of their respective owners.  Consult your product manuals for complete trademark information.

-----------------------------------------------------------------