Network Technologies - Basics

I.	Networking Basics

A.	OSI model layers

1.	Physical (bits)

a.	Defines the mechanical & electrical specifications of the network medium, the network interface hardware, & how they connect to one another. Specifies the number & functions of pins in a network connector, types of transmission medium, & how bits are encoded on the transmission medium.

b.	Topics

i.	Analog & Digital Signalling - How the signal is placed on the transmission medium

ii.	Multiplexing - How several lower speed signals are placed on one high speed connection

iii.	Connection Timing - How computers communicating over the network determine when pieces of data begin & end

iv.	Bit Synchronization - How computers on a network identify individual bits of data

c.	Devices - network cards, active & passive hubs, connectors, repeaters

2.	Data Link (frames)

a.	Organizes the Physical layer’s 1s & 0s (bits) into frames (A frame is a contiguous series of data with an independent logical meaning). This layer concentrates on the individual links between a pair of computers on a network within the area of one network segment (A portion of the network with no internetworking device on it). Typically divided into media access & logical link sublayers

b.	Topics

i.	Media Access - how computers access the transmission medium; what decides who gets to use the network at any given moment (Media Access sublayer)

ii.	Logical Topology - how the network works logically (Media Access sublayer)

iii.	Transmission Synchronization - how computers on the network know when a frame of data begins & ends (Media Access sublayer)

iv.	Addressing - identifying a computer on a given network segment (Logical Link sublayer)

v.	Connection Services - How information is accounted for on the link; is each frame acknowledged, a set of frames ackowledged at once, no acknowledgement; are the frames associated with a particular connection or just sent to a given network entity with no relation to frames previous or following (Data Link sublayer)

c.	Devices -  intelligent hubs, network interface boards, & bridges

3.	Network (datagrams)

a.	Moves information across a network made up of multiple network segments, an internet. Identifies upper layer protocols through service addresses.

b.	Topics

i.	Addressing - Assigns logical addresses to network entities so they may be identified regardless of their physical location on the network.

ii.	Switching - Uses the physical layer addresses to determine which segment a datagram belongs on & sends it there; datagram, each packet may take a different route through the network, or virtual circuit, all packets take the same route through the network.

iii.	Route Selection - Uses the logical address of the network entity to determine the segment the datagram belongs to & forwards it there. If the datagram belongs on a segment the internetworking device is attached to, it forwards the datagram to the correct physical address.

iv.	Route Discovery - Methods to find routes between computers on an internetwork.

v.	Gateway Services - Translates between different data link protocols of attached network segments.

vi.	Connection Services - Provides higher reliability connection oriented services using lower level connectionless services.

4.	Transport (segments)

a.	Ensures end-to-end reliable data delivery & flow control, often compensating for a lack of reliability in the lower layers, using Transport Layer acknowledgements.

b.	Topics

i.	Address/Name Resolution - Maps between physical addresses & logical names (or addresses)  of computers on the network

ii.	Addressing - Provides service names to identify upper layer processes, allowing service requestors to communicate with service providers.

iii.	Segment Developement - Breaks large messages up & combines smaller messages to make efficient use of the transmission medium; reorders out-of-sequence segments so upper layers receive information in the correct order.

iv.	Multiplexing

v.	Connection Services - Provides higher reliability connection oriented services using lower level connectionless services.

5.	Session

a.	Establishes, maintains, synchronizes, & manages dialog between two applications across a network; controls dialog between service requestors & providers. Coordinates/verifies numbering scehemes & transmission procedures; Remote Procedure Calls (RPCs) are at this level.

b.	Topics

i.	Dialogue Control

ii.	Session Administration

Ÿ	Connection Establishment - including deciding which entity begins a conversation, verifying user names & passwords, establishing connection identification numbers, & agreeing on which services are needed & for what duration

Ÿ	Data Transfer - includes data transfer & acknowledging data receipt

Ÿ	Connection Release

c.	Handles upper layer problems, such as the paper being out on a printer or inadequate disk space.

6.	Presentation

a.	Formats & translates data into a mutually agreed upon format

b.	Topics

i.	Encryption - to prevent unauthorized access to data; public or private key

ii.	Translation - bit order, byte order (big/little endian), character code translation, file syntax translation

7.	Application

a.	Manages user interface into the network & communications between local applications.

b.	Topics

i.	Network Services

ii.	Service Advertisement - Active: service sends out broadcasts to all stations attached to the network periodically; Passive: services register with a directory service, which clients then query

iii.	Service Use - OS Call Interception, in which the network intercepts calls to the operating system & translates them into network calls (the operating system does not know about the network software); Remote Operation, in which the operating system makes calls directly to the network software (the network software does not know about the operating system); Collaberative, in which the network software & the operating system work together to fill client requests.

c.	Implementations

i.	Message - Stores, access, & delivers binary, graphic, digitized video, & audio data; EMail, Integrated EMail/Voice Mail, Object Oriented Applications, Workflow & linked object workgroup applications

ii.	Database - Provide server based database storage/retrieval which allow network clients to manipulate data; typically evaluate incoming requests & return data. Distributed - Database management system is responsible for making the database appear as one logical until while coordinating changes that occur on various computer systems. Replication creates & synchronizes multiple copies of a database over the network

iii.	File - network applications designed to effenciently store, retrieve, & move data files; provide file services for storing print queues

Ÿ	Online - immediately available to user

Ÿ	Nearline - not immediately available, but requires no human intervention to access

Ÿ	Offline - requires human intervention to access

iv.	Print - queues print jobs & routes incoming faxes to the desktop

v.	Application Services - network services which run software for network clients allowing computers to share processing power, not just data; use specialized hardware & software to increase processing speed & security

Memory trick - All People Seem To Need Data Processing

B.	Management

1.	accounting- implementing a chargeback system for network usage

2.	performance - evaluating network cost, predicting next years hardware requirements

3.	security - evaluating integrity of file system, identifying secure means of transport, network auditing tools

4.	configuration - documenting & tracking service agreements & inventories

5.	fault - running backup software

C.	Models

1.	Centralized - one computer does all of the processing & distributes data to terminals; provides optimized hardware utilization, increased file security, & fast relocation of central resources

2.	Distributed - various computers do the processing sharing one set of data

3.	Cooperative - a number of computers cooperate in storing data & processing

D.	Types of Networks

1.	Local Area Network

a.	 Less than 10s of km

b.	One type of transmission medium

c.	contained within a building or campus environment

2.	Metropolitan Area Network

a.	Less than 1000s of km

b.	larger then a LAN

c.	covers the area of a city

3.	Wide Area Network

a.	More Than 100s of km

b.	larger than a Metropolitan Area Network

c.	interconnects LANs

d.	Enterprise - connects all the networks of a single enterprise

e.	Global - crosses international borders & may connect several organizations

E.	All Networks Require

1.	At least 2 individuals

2.	A method of contact (a pathway - or wire)

3.	Rules used to communicate (protocols)

II.	Standards Organizations & Standards Setters

A.	International Standards Organization (ISO)

1.	issues OSI standards

2.	HDLC (Data Link)

B.	CCIT

1.	ISO is a member

2.	ISDN

3.	CCIT V.24 similar to EIA232/RS232 (physical layer)

4.	CCIT X.21 (physical layer)

5.	LAPB (data link)

6.	CCIT X.25 (network)

C.	IEEE

1.	issues 802.x series - physical & data link layer standards

2.	World’s largest professional organization

3.	Standards/Groups

a.	802.1 - Overview

i.	overview, system management, & internetworking

ii.	Spanning Tree Algorithm

iii.	only IEEE standard involving physical, datalink, network, & transport layers

b.	802.2 - Logical Link

i.	Logical Link; interface between media access layers  (802.3, 802.4, 802.5) & network layers

ii.	Part of OSI data link layer

c.	802.3 - CSMA/CD

i.	CSMA/CD (Ethernet type) media access

ii.	Naming convention - first part data rate in MB/s, second part modulation mode, third part special designator

d.	802.3u - 100MB/s ‘Fast Ethernet’

e.	802.4 - Token Bus

i.	Token bus media access

ii.	Used primarily by the Manufacturing Automation Protocol group (MAP) in factories

f.	802.5 - Token Ring, FDDI

g.	802.6 - Distributed Queue Dual Bus (DQDB)

i.	Metropolitan Area Network (MAN)

ii.	Fast dual counter-rotating rings of fiber

iii.	allocates bandwidth to voice, data, & other needs dynamically

iv.	can be migrated to ISDN

h.	802.7 - broadband signaling

i.	802.8 - Fiber Optic Technology

j.	802.9 - integrated voice & data technology; ISDN

k.	802.10 - network security/encryption

l.	802.11 - unbounded/wireless networking technologies

m.	802.12 - 100BaseVG AnyLAN

D.	American National Standards Institute (ANSI)

1.	ANSI X3.4 - similar to ASCII

2.	ADCCP (data link)

3.	ANSI X3T9.5 - FDDI

E.	IBM

1.	EBCDIC character encoding system

2.	16MB/s TokenRing

F.	AT&T - T1

G.	ARCNet Trade Association - ARCNet

H.	Apple

1.	AppleTalk

2.	LocalTalk

3.	EtherTalk

4.	TokenTalk

III.	Vocabulary

A.	Segment

1.	any portion of a network with no internetwork device on it, such as a bridge, router, or gateway

2.	Any electrically continuous section of a network

B.	VTAM - Virtual Telecom Access Method; IBM/SNA for providing access to shared info

C.	VMS - Virtual Memory System; DEC’s VAX operating system

D.	de jure - legislated standard

E.	de facto - commonly accepted standard

F.	Null Modem - any cable which directly connects the transmit & receive signals of two circuits

G.	Non-proprietary - developed by standards body

H.	NetBIOS - de facto session layer interface

I.	Bandwidth - the capacity of a transmission medium in MHz

Lower Layer Protocols

I.	Physical Layer Basics

A.	Physical Media (computer networking transmission media)

1.	Unbounded

a.	Microwave

i.	easy to intercept, prone to interferance by weather (fog, rain, snow), complicated to install & maintain

ii.	Terrestrial

Ÿ	typically used to link buildings

Ÿ	uses 4-6 & 21-23 GHz

iii.	Satellite - bounces off of satellite, 11-14 GHz

b.	Spread Spectrum - uses entire band simultaneously

i.	direct sequence

Ÿ	spreads encoded data called chips across a subset of radio frequencies (uses multiple frequencies simultaneously)

Ÿ	supports up to 6MB/sec

ii.	frequency hopping quickly switches between predetermined frequencies

c.	Low Power Single Frequency Radio - supports up to 10MB/sec

d.	Infrared

i.	point to point or broadcast

ii.	line of sight only (can bounce off of walls or other objects)

iii.	100GHz - 1000THz

iv.	prone to interferance in high intensity light environments

e.	Cellular Phone Digital Packet - uses ‘extra space’ in voice cellular to transmit data packets

2.	Bounded

a.	Coax - inner core surrounded by outer copper braid (sheild), then coated with plastic shell

i.	Advantages

Ÿ	low probablity of electrical interferance

Ÿ	mature standards

Ÿ	high bandwidth

Ÿ	physically sturdy

ii.	types

Ÿ	RG8 & RG11 - Thicknet, Ethernet (50ohm)

Ÿ	RG58 - Thinnet (50ohm)

Ÿ	RG59 - TV, 10Broad36 (75ohm)

Ÿ	RG62 - ARCNet (93ohm)

b.	Fiber

i.	most resistance to electromagnetic interference

ii.	uses glass or plastic fibers

iii.	uses outer coating to reflect light back into fiber channel

iv.	expensive; requires complicated connections

v.	does not require electrical ground for installation

vi.	multi-mode

Ÿ	allows many different paths for light to travel through the fiber

Ÿ	uses LEDs for light source

Ÿ	shorter distance

vii.	single mode

Ÿ	optimized for one light path through fiber

Ÿ	uses lasers for a light source

c.	Twisted Pair

i.	advantages

Ÿ	cheap to install - most buildings already have some form of TP installed

Ÿ	technology stable & mature

Ÿ	twist in cable reduces crosstalk, the transfer of signals between two copper wires placed close together

ii.	disadvantages

Ÿ	highly susceptible to electrical interference

Ÿ	does not resist tapping

iii.	Unshielded Twisted Pair Catagories

Ÿ	Catagories 1 & 2 - voice, low speed data (less than 4MB/s)

Ÿ	Catagory 3 - Data 10 to 16MB/s (100MB/s possible)

Ÿ	Catagory 5 - high speed data (greater than 100MB/s), uses RJ45 connectors

iv.	Shielded Twisted Pair capable of 500MB/s

B.	Physical Layer Devices

1.	Concentrators

2.	Hubs

a.	Passive - simply connects all attached stations; each segment may only extend half the maximum allowable network length for the medium

b.	Active - regenerates (repeats) the signal; each segment may extend the full length of the maximum allowable network length for the medium

3.	Repeaters

4.	Media Connectors

5.	Modems

6.	Codecs

C.	Physical Topologies

1.	Mesh - point to point connections between each entity on the network; easy to troubleshoot

2.	Star

a.	central point facilitates troubleshooting

b.	can be logical multipoint, polling, or star

3.	Ring - a single break affects all stations in the loop

4.	Bus

a.	easy to install; difficult to troubleshoot

b.	modern implementation of a multipoint system

c.	most bus topologies require termination

5.	Hybrid - like the Internet

6.	Cellular

a.	combines point to point with multipoint

b.	devices within a cell (or given area) communicate with a central station or hub

D.	Modulation

1.	Devices

a.	Modem - Data Communications Device (DCE) which modulates digital signals onto an analog (broadband) line

b.	Codec - DCE which places an analog signal onto a digital (baseband) line

2.	Types

a.	Baseband (direct digital)

i.	uses direct digital signaling (can have only one of a limited range of values)

ii.	signal occupies entire bandwidth of cable (channel); transitions instantly between states

iii.	baud - the number of times in a second the signal state changes on a signal line (roughly analogous to frequency)

b.	Broadband (analog)

i.	advantages/disadvantages/basics

Ÿ	signal does not take entire bandwidth of cable (channel)

Ÿ	less sucseptable to distortion

Ÿ	can cover longer distances with no regeneration

Ÿ	requires radio frequency engineering

Ÿ	described using frequency, amplitude, & phase

ii.	basic configurations

Ÿ	two separate cables (tx & rx) with ‘passive headends’ to transfer data between them

Ÿ	one cable with tx & rx on separate carrier frequencies, ‘active headends’ repeat the tx signals at the rx frequency

E.	Data Encoding

1.	Coding Systems

a.	ASCII (ANSI X3.4)

i.	7 bit character code with 1 parity bit

ii.	128 characters

b.	Extended ASCII

i.	8 bit character code

ii.	256 characters

c.	EBCDIC

i.	8 bit data code designed by IBM

ii.	255 possible characters

iii.	derived from one of the earliest data encoding schemes, Binary Coded Decimal (BCD)

F.	Analog Signalling Types

1.	Amplitude Shift Keying (AM)

a.	varies signal amplitude between two or more levels

b.	uses current state to indicate data

2.	Frequency Shift Keying (FSK or FM)

a.	varies frequency of signal between two outer limits

b.	uses current state to indicate data

3.	Phase Shift Keying

a.	varies the phase of the signal against a reference

b.	uses state tranisition to indicate data

c.	often combined with FSK in high speed modems

G.	Digital Signalling Types

1.	Unipolar - current state; not self clocking

a.	only a positive or a negative voltage (but not both) represents the data - +3 volts may be a 0, & 0 volts may be a 1

b.	typically used in teletype interfaces & PC interfaces that are TTY compatible.

c.	separate clock (not self clocking)

d.	sucseptible to noise corruption

e.	level of signal denotes signal or state/current state

2.	Polar - current state; not self clocking

e.

a.	positive & negative voltages used to represent data, rather than a single voltage & ground

b.	not self clocking

c.	current state

3.	Bipolar/Alternate Mark Inversion - state transition; not self clocking

c.

a.	two voltage levels & ground used; positive or negative inidicates 1, ground indicates 0

b.	easiest to detect noise induced errors on

c.	all connected devices do not need to be in the same polarity; immune to cross wiring errors

d.	Alternate Mask Inversion is a popular type

e.	state transistion indicates data - not self clocking

4.	Return to Zero - current state; self clocking

e.

a.	Polar encoding with a transition to ground in the middle of each time frame (bit)

b.	self clocking - clocks from mid-bit transition

c.	level indicates data/current state

5.	Non-return to Zero - state transition; not self clocking

c.

a.	signal transitions for ones, remains stable for zeros

b.	not self clocking

c.	state transition indicates data

d.	simple & inexpensive to design & build

e.	used in RS232 & other serial transmission methods

6.	Manchester - state transition; self clocking

e.

a.	Used in Ethernet & 802.3

b.	mid-bit transition represents data (transition strategy) - low to high = 1, high to low = 0

c.	self clocking

7.	Differential Manchester - transition state; self clocking

c.

a.	Used in Token Ring

b.	mid-bit transition for clocking

c.	start of bit transitions indicate data (transition strategy) - no transition = 1, transition = 0

d.	self clocking

8.	Biphase Space (FM-0) - self clocking

d.

a.	requires at least one transition per time slot

b.	self clocking

H.	Multiplexing - used to transmit several low speed signals over one high speed line (interleave)

1.	Frequency Division Multiplexing

a.	uses different frequencies on the same cable to transmit different signals

b.	broadband multiplexing scheme

c.	can be used to seperate different direction traffic on a single cable

2.	Time Division Multiplexing - basbadn scheme; channel is divided into fixed slots of time, with each channel always taking the same slot

3.	Statistical Time Division Multiplexing - channel is divided into fixed slots of time, which are then given to different channels on a statistical basis (StatDM)

I.	Connection Basics

1.	Bit Synchronization - baud rate must be locked for synchronization

a.	Synchronous

i.	signals which have the same frequency & are sourced from the same clock

ii.	transmission mode in which the receiver & transmitter are ‘synchronized,’ or working from the same clock or timing

iii.	faster than asynchronous

b.	Asynchronous

i.	signals sourced from different clocks & having different frequencies &/or phase relationships

ii.	used for randomly generated information

iii.	character oriented, using extra synchronization bits/information

iv.	transmitter sends a single start bit, receiver uses an internal clock to know how frequently to measure the signal

c.	Isonchronous - signals dependant on a seperate ‘uniform timing signal’ or which carry timing information in the data packet

2.	Data flow

a.	Simplex - one direction communication

b.	Half Duplex - one direction at a time (polite conversation, walkie talkie)

c.	Full Duplex - data flows both directions at once  (like telephone)

3.	Types

a.	Point-to-point - direct connection between two points

b.	Multipoint

i.	connect three or more points on a shared channel

ii.	requires addressing

iii.	bus is modern implementation

4.	Basic data rates

a.	V.32 4.8KB/s or 9.6KB/s

b.	V.32bis 14.4KB/s

c.	V.34 28.8KB/s

d.	ISDN Basic Rate 128KB/s

e.	ISDN Primary Rate 1.54MB/s

f.	T-1 1.544MB/s

g.	OC-3 155.52MB/s

h.	OC-12 622.08MB/s

i.	OC-48 2.488GB/s

II.	EIA232 (RS232) - similar to CCIT V.24

A.	Basics

1.	Covers electrical, mechanical, functional & procedural characteristics of a serial interface

2.	de jure standard

B.	Standards

1.	Maximum 50ft cable

2.	25 pin connector with specific pin out

3.	<-3v = 1; >+3v is 0

4.	Maximum rate of 20kbps

C.	Pins

1.	pin 1: protective ground

2.	pin 2: Transmit Data (TD)

3.	pin 3: Receive Data (RD)

4.	pin 4: Request To Send (RTS)

5.	pin 5: Clear to Send (CTS)

6.	pin 6: Data Set Ready (DSR)

7.	pin 7: signal ground

8.	pin 15: transmitter clock

9.	pin 17: receiver clock

10.	pin 20: Data Terminal Ready (DTR) - first signal in startup sequence

11.	pin 23: data rate select

12.	pin 24: external clock

D.	Sequence of operation

1.	DTR (pin 20) true with computer/terminal (DTE) power on

2.	DSR (pin 6) true with modem (DCE) power on

3.	computer/terminal (DTE) request permission to TX by placing RTS (pin 4) true; when finished, computer (DTE) drops RTS

4.	modem (DCE) places CTS true & places ‘binary one’ synch signal on analog line

5.	receiving modem (DCE) detects mark tone & places CD true to attached computer/terminal (DTE)

6.	Originating computer/terminal (DTE) places signal on TD (pin 2) to it’s modem (DCE)

7.	Receiving modem (DCE) places data on RD (pin 3) to attached computer/terminal (DTE)

III.	EIA449 (RS449)

A.	provides RS232 signals plus dedicated return signals

B.	provides greater data speed than RS232

C.	provides greater cable length than RS232

IV.	V series of specifications

A.	CCIT (de juer standard)

B.	V.24 is similar to RS232

C.	deal with physical layer specifications

V.	SONET

A.	2.5gb/sec over fiber

VI.	T1

A.	By AT&T (de facto standard)

B.	uses time division multiplexing to place 24 voice channels on one line

C.	1.544mbps digital line

VII.	Public Switched Telephone Network

A.	General

1.	Largest Circuit Switched network in the world

2.	Not originally deigned for data transmissions

B.	Four classes of components

1.	subscriber equipment

2.	local loop (1000ohm)

3.	trunks & switching offices - switching offices responsible for dial tone

4.	long distance carrier facilities

VIII.	ISDN (Integrated Services Digital Network)

A.	General

1.	CCIT (de jure) standard

2.	designed to carry voice, data, & fax

3.	designed to transform the current telephone network into a digital network

B.	A high speed line multiplexed into several channels

1.	A Channel - 4KHz analog/voice

2.	B Channel - 64kbps digital

3.	C Channel - 8 or 16kbps digital for out of band signaling

4.	D Channel - 8 or 16kbps digital for out of band signaling

a.	s - subchannel for call setup signaling

b.	t - subchannel for utility use & telemetry

c.	p - subchannel for low bandwidth packet data

5.	E Channel - 64kbps for internal signaling

6.	H Channel - 384, 1536, or 1920 kbps digital

C.	Basic Rate - 2 B & 1 D/16kbps channel

D.	Primary Rate

1.	US/Japan - 1 D/16kbps & 23 B/64kbps for a 1.544MB T1 link

2.	Europe - 1 D/16kbps & 30 B/64kbps for CCIT 2.048MB modem system

IX.	Asynchronous Transfer Mode (ATM)

A.	General

1.	Based on Broadband ISDN (B-ISDN)

2.	2.2gb/sec

3.	Fixed frame size (53 octects)

B.	Classes of service

1.	Class 1: constant bit rate

2.	Class 2: variable bit rate

3.	Class 3: connection oriented

4.	Class 4: connectionless

X.	CCIT X.21

A.	Physical layer for X.25

B.	Based on point to point connections over a hybrid mesh topology

C.	Synchronous

XI.	IEEE 802.3/Carrier Sense Multiple Access/Collision Detect (CSMA/CD) & Ethernet

A.	General

1.	transmitter checks channel for activity before transmitting, detects & enforces collisions, & retransmits collided packets

2.	probabilistic

3.	good for bursty traffic

4.	uses SQE (hearbeat) to perform periodic check of collision detection circuits

B.	Differences

1.	Ethernet developed by Xerox

2.	Ethernet does not use or support SQE (Heartbeat) to check collision detection circuitry

3.	replaces length of data field with type of packet field

4.	replaces 802.3 MAU with tranceiver

5.	Ethernet only supports 10MB/s signaling; 802.3 supports 1-10MB/s signaling

C.	Types

1.	10Base5

a.	original Ethernet

b.	500 meter bus/10MBps

c.	thick coax (RG8 & RG11)

d.	uses Network Interface Card (NIC), Attachment Unit Interface (AUI), & Medium Attachment Unit (MAU)

2.	10Base2

a.	200 meter bus/10MBps

b.	thin coax (RG58)

c.	uses NIC, AUI, & MAU

3.	10BaseT - Twisted Pair star topology, 10MBps

4.	1Base5 - Star topology, Up to five repeating hubs, 1 MBps

5.	10Broad36 - 2 1800 meter segments with active headends

6.	10BaseF - fiber

a.	10BaseFL - fiber link

b.	10BaseFB - fiber backbone

c.	10BaseFP - fiber passive

D.	Frame

1.	Preamble - 7 bytes

2.	Start of Frame Delimiter (SFD) - syncs receivers, 1 byte

3.	Destination Address - 6 bytes

4.	Source Address - 6 bytes

5.	Length of Data - 2 bytes, does not include length of padding (type of packet for Ethernet)

6.	Optional 802.2 fields (see frame type below)

a.	destination service access point (DSAP)

b.	source service access point (SSAP)

c.	control fields

d.	information (optional within 802.2)

7.	Data & Padding - 46 to 1500 bytes

8.	Frame Check Sequence -  CRC, 4 bytes

E.	802.3/Ethernet Frame Types

1.	Ethernet 802.3 (Raw)

a.	Developed by Novell ahead of the ratification of the 802.2 standard

b.	Does not include 802.2 flow control information

c.	64 to 1518 bytes

d.	Only used on Novell networks; default for Netware 2.2 & 3.11

2.	Ethernet 802.2

a.	fully IEEE compliant

b.	contains three more one-byte fields than 802.3 (Raw), providing flow control, error control, & reliability

c.	64 to 1518 bytes

d.	default frame type for Netware 3.12 & 4.x servers

3.	Ethernet SNAP

a.	uses subnetwork addresses

b.	has a few different fields than 802.2, using constant values in some

c.	limiting to higher level applications

d.	developed for use in TCP/IP & UNIX; preferred Appletalk frame type

4.	Ethernet II

a.	Xerox

b.	original Ethernet packet type

c.	identifies type, rather than length, of packet in length field

d.	required to support TCP/IP on a Netware 3.1x server

XII.	IEEE 802.5 & IBM Token Ring

A.	General

1.	Shielded Twisted Pair (260 stations) or Unshielded Twisted Pair (72 stations)

2.	Deterministic ring configuration

3.	Each station acts as a unidirectional repeater, passing the signal to the next station

4.	Early token release support means releasing a new token as soon as the transmitting station is finished receiving a frame

5.	Active monitor, usually the fastest station, performs ring maintenance

6.	token is 3 bytes long

B.	Differences

1.	IEEE 802.5 does not specify media types

2.	IEEE 802.5 signals at 1 or 4MB/s, IBM at 4 or 16MB/s

C.	Frame

1.	Start Delimiter - 1 byte

a.	violates differential Manchester Encoding

b.	synchronizes receiver to transmitter

2.	Access Control Byte - 1 byte

a.	Controls frame’s priority

b.	Contains 3 priority & 3 reservation bits

c.	Station with higher priority than indicated in priority of free token can seize it for transmission

d.	Station can reserve token by setting the reserve bits to it’s priority. If the token comes around free with the station’s priority, it seizes it for transmission

e.	Stations which raise the priority of a token are responsible for reducing it back to the level they found it once finished transmitting

3.	Frame Control - 1 byte

a.	Access Control Token Bit - 0: token, 1: data/command

b.	Access Control Monitor Bit

i.	keeps ring free of inactive tokens

ii.	when 0, active monitor sets to 1

iii.	if 1, monitor removes token

4.	Destination Address - 2 or 6 bytes

5.	Source Address - 2 or 6 bytes

6.	Optional Routing Information Fields for source routing- <=18 bytes

7.	Optional 802.2 Fields

a.	Destination Service Access Point (DSAP) - 1 byte

b.	Source Service Access Point (SSAP) - 1 byte

c.	Control Fields - 1 or 2 bytes

d.	Information - optional within 802.2

8.	Data Field

9.	Frame Check Sequence (CRC) - 1 byte

10.	End Delimiter - violates Differential Manchester Encoding; 4 bytes

11.	Frame Status - information sent back to originating station; 1 byte

a.	Address Resolution - if receiver gets the frame, its sets this to 1, implying it is on the ring

b.	Frame Copied - if the receiver accepts the frame, it sets this to 1, saying it has the data

D.	Beaconing - node reporting problem to downstream neighbor

XIII.	ARCNet

A.	General

1.	Managed by the ARCNet Trade Association

2.	Deterministic

3.	Propagation delay cannot exceed 31 microseconds between any two stations

4.	Token Passing System - highest numbered node completes a logical ring by passing tokens to the lowest numbered node

5.	Character-oriented protocol

6.	0 is reserved for broadcast

B.	Media Types - twisted pair, coax, & fiber

C.	Topologies

1.	Star Topology uses active or passive hubs

a.	Active hubs can link up to 2000ft

b.	Passive Hubs can link up to 100ft

2.	Bus can be a link off of a hub or daisy chained

D.	Autoreconfiguration

1.	highest ID node attempts to transmit to next lowest numbered node, progressing down until it is found; this chains down until all nodes are found

2.	occurs after any error, including removal & addition of ring members

E.	Frame Types

1.	ITT - invitation to transmit

2.	FBE - free buffer inquiry

3.	ACK - acknowledge

4.	NAK - negative acknowledge

5.	PAC - data

F.	Frame Parts - not all frames have all sections

1.	Alert - string of six 1’s

2.	EOT - end of TX

3.	DID - destination identifier

4.	ENQ - inquire

5.	ACK - acknowledge

6.	NAK - negative acknowledge

7.	SID - sending identifier

8.	Count - indicates number of bytes in information field

9.	CRC - frame check sequence

10.	SQH - start of header

XIV.	LocalTalk

A.	General

1.	LocalTalk Link Access Protocol

2.	Baseband bus topology

3.	Interface specified by RS449

4.	Biphase encoding (sometimes called FM-0)

5.	Shielded twisted pair cabling

6.	300 meters/32 devices maximum/230.4KBps

B.	Uses dynamic address assignment

1.	Node picks an address & broadcast a request to use it

2.	If already used, it picks another & tries again

3.	Client addresses are always between 1 & 127

4.	Server addresses are always between 128 & 254

5.	A broadcast message uses an address of 255

C.	Carrier Sense Multiple Access/Collision Avoidance

1.	Transmitter senses for carrier (channel in use), then transmits an RTS frame to notify other staions of impending transmission

2.	transmitter waits 200ms (minimum interframe gap, or IFG)

3.	receiver transmits CTS frame

4.	after receiving CTS, transmitter waits another IFG

5.	transmitter sends data frames, each separated by an IFG

6.	If transmitter’s RTS is not acknowledged, a collision is assumed

D.	Frame

1.	Flag - uses bit stuffing to prevent duplication

2.	Destination Node ID

3.	Source Node ID

4.	Type - control or data

5.	Data Length

6.	Data Field

7.	Frame Check Sequence

a.	16 bit CRC

b.	covers all fields

8.	Trailer flag

9.	Abort Sequence

XV.	Distributed Queue Dual Bus (IEEE 802.6)

A.	Fiber based

B.	Dual Bused - two counter rotating unidirectional rings

C.	Supports synchronous & asynchronous signals, including voice, data, & video

XVI.	FDDI - Covers physical & data link layers, station management

A.	General

1.	Championed by IBM, AMD, & Cisco

2.	Assumes upper layers access FDDI through 802.2 LLC

3.	ANSI X3T9.5

B.	Physical Media

1.	Two counter-rotating rings of fiber - one primary, the other secondary; secondary is typically idle, unless there is a failure in the primary, in which case stations on either side of the failure direct traffic onto the secondary

2.	500 stations/100KM of cable maximum

3.	Repeater or bridge required every 2KM or less

4.	LED/Multimode with a core of 62.5 micrometers

5.	Class A station connects to both rings, Class B only to primary; this is only possible in star configuration

6.	Encoding

a.	4B/5B (four of five bit) encoding - encodes each four bits of data into one symbol for 80% network bandwidth use

b.	100MBps requires a 125MHz signal

c.	Uses NRZ-1 encoding (non-returning to zero - 1) - 1 is indicated by transition at beginning edge, 0 by no transition

C.	Data Link

1.	Token passing

a.	tokens are passed around the ring continuously

b.	as a station transmits data, it absorbs one token & releases another

c.	when a receiving station gets a token, it sets a state flag & passes it back to the transmitting station, which pulls it off of the ring

d.	uses 802.5 token passing methods (same as token ring)

D.	Signal Types

1.	Can be used to transmit synch or asynchronous data

2.	A station may issue a ‘restricting token,’ preventing any other station from transmitting asynchronous data

3.	Uses beaconing, like Token Ring

E.	Frame - cannot be over 4500 bytes

1.	Preamble & short delimiter

2.	Frame Control indicates

a.	frame is synch/asynch data

b.	uses 16 or 48 bit address

c.	data or command frame

d.	type of command

3.	Destination Address

4.	Source Address

5.	Information

6.	Frame Check Sequence (CRC)

7.	End Delimiter

8.	Frame Status

XVII.	X.25 - specifies physical through network layers

A.	CCIT - X series deals with packet packaging & sending

B.	Components:

1.	X.1 - X.9: terminal types, interfaces, services, & facilities

2.	X.21: physical layer

3.	LAPB: data link layer

4.	X.25: network layer, can also run on 802.2 services

5.	X.40 - X.199: network architectures, signalling, etc.

Data Link Layer

I.	Data Link Layer Basics

A.	Media Access

1.	Contentious

a.	Probablistic

b.	Each station transmits at will

c.	Must have some form of collision detection/avoidance

d.	Types:

i.	CSMA/CD - Carrier Sense Multiple Access/Collision Detect (802.3)

ii.	CSMA/CA - Carrier Sense Multiple Access/Collision Avoidance (AppleTalk)

2.	Non-Contentious

a.	Deterministic

b.	Each station waits its turn or asks for permission to transmit

c.	Types:

i.	Token Ring (802.5)

ii.	Polling - takes up a lot of bandwidth for notices & acknowledgements

B.	Connection Services

1.	Unacknowleged Connectionless - sends & receives frames with no flow, error, or packet sequence control

2.	Connection Oriented

a.	provides flow, error, & packet sequence control

b.	uses logical link layer (LLC) static window, dynamic windows, or guaranteed rate flow control

c.	uses LLC error control, which notifies upper layers of lost or damaged packets; uses checksums or CRCs; activiated most of the time by NAKs

3.	Acknowledged Connectionless - provides flow & error control for point-to-point connections

C.	Data Link Layer Devices

1.	Bridge

a.	Standard

i.	Learns the addresses of devices on both sides & forwards packets as needed

ii.	acts as a repeater, retiming & regenerating signals

b.	Source Routing

i.	forwards packet to next bridge specified in the packet - all routing information is specified by the sending station

ii.	used in IBM Token Ring environments

2.	Intelligent Hub - sends packets only to that segment on which the addressed node exists (like a multiport bridge)

II.	SDLC

A.	General

1.	bit-synchronous

2.	Developed by IBM as a data link method for SNA

3.	Basis for ADCCP, HDLC, LAP, & LAPB

4.	Uses bit-stuffing to prevent duplication of the flag bit

5.	Uses transmit & receive sequence numbers along with a poll/flag bit to control data flow

6.	Uses Normal Response Mode; primary stations send commands, secondaries send responses

B.	Configurations

1.	Point-to-Point - one primary, one secondary station

2.	Multipoint - one primary, several secondary stations in a star configuration

3.	Loop - one primary, several secondary stations in a ring configuration

C.	Frame

1.	Start of Frame (SOF)/Flag - 01111110

2.	Source Address

3.	Destination Address

4.	Admin/Control or Unnumbered Flag

a.	Information - contains frame sequence number

b.	Supervisor - receiver ready, not ready, or reject

c.	Unnumbered - used for data link maintenance

5.	Checksum

6.	End Of Frame (EOF) flag

III.	HDLC/ADCCP

A.	General

1.	HDLC set by ISO; based on SDLC

2.	ADCCP set by ANSI; based on SDLC

3.	Does not have individual, group, or broadcast addresses

4.	Does not have loop or go-ahead configurations of SDLC

5.	Uses point-to-point connections only

6.	Three transfer modes instead of one

a.	Normal response mode

i.	one primary, one secondary

ii.	polling system

b.	Asynchronous response mode

i.	one primary, one secondary

ii.	secondary communicates freely with primary

c.	Asynchronous balanced mode

i.	two or more ‘combined’ (both primary & secondary) stations

IV.	LAPB

A.	By CCIT

B.	Similar to SDLC/Subset of HDLC

C.	Uses only Asynchronous Balanced Mode

D.	No information is allowed in the response frames

E.	circuits are set up by DTE/DCE

F.	bit-synchronous

G.	primary link access for X.25 networks

V.	IEEE 802.2 Logical Link Layer/Logical Link Control (LLC)

A.	Modeled after HDLC

B.	LLC frames are often called Protocol Data Units (PDUs)

C.	Accessed by network layers through Service Access Points (SAPs)

D.	Four Fields

1.	Destination Service Access Point

2.	Source Service Access Point

3.	Control Field - specifies service requested

4.	Information Field - optional

E.	Error Recovery

1.	ensures integrity on a single physical link - not end-to-end

2.	does not perform data integrity checks, only finds ‘dropped frames’

3.	resets connections

F.	Flow Control - can use either stop & wait or sliding window

G.	Link Types

1.	Type 1/Unacknowledged Connectionless

a.	most commonly used

b.	does not provide for packet acknowledgment

2.	Type 2/Connection Oriented

a.	sets up virtual connection

b.	Uses sliding window flow control

3.	Type 3/Acknowledged Connectionless

VI.	Switched Multimegabit Data Service (SMDS)

A.	Designed by Bell in 1991

B.	Uses a fixed cell size

C.	Uses packet switching over virtual circuits

D.	Isonchronous

E.	connectionless

F.	runs over SONET & DQDB

G.	1.544MB/s to 45MB/s

Upper Layer Protocols

I.	Network Layer Basics

A.	Switching Techniques

1.	Circuit switching

a.	directly connects sender & receiver in unbroken path

b.	telephone system

c.	virtually no channel access delay

2.	Message Switching (store & forward)

a.	packets are not broken up or fragmented; relies on data link layer to make certain packets are the right size

b.	destination address is appended (or prepended) to each packet

c.	each switch forwards the packet to a switch closer to the destination

d.	all packets between two nodes do not always take the same path between them

e.	packets may be stored temporarily by switches in the network

3.	Packet Switching (stores packets in memory)

a.	most common switching technique used in networks today

b.	packets broken up into smaller units by PAD (packet assembler/disassemblers)

c.	address & sequence number appended to each packet

d.	Datagram - packets take various routes through the network

e.	Virtual Circuit - Sender & receiver agree upon a standard route for their packets through the network, establishing a logical connection

B.	Addressing

1.	Logical Network - identifies logical entities on the network

2.	Service - identifies a service within a logical entity

C.	Route Discovery

1.	Distance Vector

a.	routers send routing tables to all routers attached to 'their' segment

b.	routers build a router table by combining information from all the routing tables they receive

c.	each router sends entire table each time period - not just updates

2.	Link-State

a.	routers send full router table on startup & at set intervals

b.	routers send change only information

c.	builds routing tables by combining information from nearby routers

D.	Route Selection

1.	Dynamic - route is determined as each packet/message traverses the network

2.	Static - routes between networks are predetermined

E.	Network Layer Devices - Routers

1.	Network Layer address is final node address

2.	Physical layer address is the router’s address - each router in the network changes the physical address to the next router’s address in the link to the destination node

II.	Internet Protocl - Upper Level

2.

A.	Control & Origin

1.	de facto standard

2.	Developed by Stanford University & Bolt, Baranek, & Newman with funding from the DOD Advanced Research Projects Agency (DARPA)

3.	The Internet Activities Board (IAB) currently organizes Internet research

4.	RFCs are maintained by the SRI Network Information Center in Menlo Park, CA

5.	addresses are distributed/controlled by SRI

B.	Network Level Protocols

1.	Internet Protocol (IP)

a.	connectionless, no acknowledgment, packet switched

2.	Frame

a.	Header

i.	Version

ii.	IP Header Length (IHL)

iii.	Type of Service - precedence, low delay, high throughput, & reliability flags

b.	Length

c.	Identification

d.	Flags - controls if the datagram may be fragmented

e.	Fragment Offset

f.	Time to Live

g.	Protocol

h.	Header Checksum

i.	Source & Destination Addresses

i.	Class A - 1 byte net

ii.	Class B - 2 bytes net

iii.	Class C - 3 bytes net

iv.	Subnets

3.	Internet Control Message Protocol (ICMP)

a.	provides error & other control information

b.	Time Exceeded, destination unreachable, source quench, redirect, echo request & reply messages

C.	Transport/Session Protocols

1.	Transmission Control Protocol (TCP)

a.	Provides full duplex, acknowledged, flow controlled, connection-oriented transport for upper level protocols

b.	Supports both computer (logical) address & process destination address

c.	Frame

i.	Source Port

ii.	Destination Port

iii.	Sequence Number

iv.	Acknowledgment Number

v.	Data Offset - indicates number of 32 bit words in the TCP header

vi.	Reserved

vii.	Flag

Ÿ	URGent; urgent data is often control data

Ÿ	ACKnowledge

Ÿ	PUSH - if set instructs sending process to immediately deliver all data on this circuit to the lower layers for transmission; receive process to deliver to the upper layers immediately; setting this flag indicates their desire to establish & synchronize a virtual connection

Ÿ	RESET - reset connection

Ÿ	SYNchronize - sets up virtual connection

Ÿ	FINish - no more data

viii.	Window  (16 bits) - number of data bits sender is willing to accept

ix.	Checksum (16 bits) - used to determine if header is damaged in transport

x.	Urgent Pointer (16 bits) - Points to byte immediately following last byte of urgent data

xi.	Options (variable length) - contains various information, such as maximum segment size

2.	User Datagram Protocol (UDP)

a.	connectionless

b.	no acknowledgments

c.	Header

i.	length (16 bits)

ii.	source port (16 bits)

iii.	destination port (16 bits)

iv.	checksum (16 bits)

d.	Faster than TCP

D.	Upper Level Protocols

1.	Routing Information Protocol (RIP) - Application/Presentation layers

a.	provides routing & reachability information to IP processes throughout the network

b.	broadcast information to all nodes on a regular basis - distance vector routing system

c.	not good on large, complex networks

2.	Open Shortest Path First (OSPF) - Application/Presentation layers

a.	Link state route discovery protocol

b.	Provides better performance than RIP in a large network environment

3.	File Transport Protocol (FTP) - Application/Presentation layers

a.	process level protocol enabling copying of files across a network

4.	Simple Mail Transfer Protocol (SMTP) - Application/Presentation layers

a.	Uses UDP to route mail between hosts

b.	Does not provide a local user interface

5.	Remote Terminal Emulation (TELNET) -Application/Presentation layers

a.	allows PCs to access host based applications as terminals

6.	Remote Procedure Calls (RPC) - Session layer

7.	Network File System (NFS) - application & presentation layers

a.	Developed by SUN Microsystems as part of its Open Network Computing platform

b.	Application/Presentation layers

c.	Includes:

i.	Network File System (NFS)

ii.	External Data Representation (XDR - at the presentation layer)

Ÿ	encodes data into a machine independent format

Ÿ	A ‘C’ language API

iii.	Remote Procedure Call (RPC - session layer)

Ÿ	local function calls are passed over the network & executed by the host

Ÿ	virtually all modern networks are based on RPCs

Ÿ	basis for client/server computing

III.	Netware

| ServerOSI |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STREAMSNetwareApplicationWorkstation Core Protocols |  |  |  |  |  |  |  |  |
| PresentationApplication | NetBIOS Emulator |  | Shell (VLM) |  |  |  |  |  |
| RPCSession |  |  |  |  |  |  |  |  |
| Transport | PEP |  | LSL/ODI |  |  |  | SAP |  |
|  |  |  | IPX/SPX |  |  |  |  |  |
| Network |  |  |  |  |  |  |  |  |
| Data Link | Standard IEEE LLC/Physical |  |  |  |  |  |  |  |
| Physical |  |  |  |  |  |  |  |  |

A.	Transport Layer Interfaces

1.	Streams - enables Netware to run on a number of different transport protocols

2.	Transport Level Interface (TLI) - uses Streams

3.	Link Support Layer/Open Datalink Interface (LSL/ODI)

B.	Transport & Network Layers

1.	Internet Packet Exchange (IPX)

a.	Based on XNSs Internetwork Datagram Protocol (IDP)

b.	Connectionless - no packet numbering

c.	Routes on information from Routing Information Protocol (RIP)

d.	Frame:

i.	Checksum (16 bits)

ii.	Length (16 bits) - complete datagram length; minimum 30, maximum 576 if routed

iii.	Transport Control (8 bits)

Ÿ	Initialized at 0 by sender

Ÿ	Incremented by each router the packet passes through

Ÿ	Packet is discarded when the count reaches 16

Ÿ	Similar to ‘Time to Live’ in IP

iv.	Packet Type (8 bits)

Ÿ	0 - not known

Ÿ	4 - Packet Exchange Protocol

Ÿ	5 - Sequenced Packet Exchange

Ÿ	17 - Netware Core Protocol

v.	Destination Network (32 bits) - if both nodes are on the same network, set to 0

vi.	Destination Host (48 bits) - if broadcast, all 1s

vii.	Destination Socket (16 bits)

Ÿ	0451H - file service

Ÿ	0452H - Service Advertisement

Ÿ	0453H - Novell Routing Information Protocol (RIP)

Ÿ	0455H - NetBIOS

Ÿ	0456H - Diagnostics

viii.	Source Network (32 bits)

ix.	Source Host (48 bits)

x.	Source Socket (16 bits)

xi.	Data

2.	Sequenced Packet Exchange (SPX) - garaunteed delivery and path selection

a.	Provides reliable data transport to upper level services, primarily RConsole & SNA services

b.	Uses end-to-end flow control connection service

c.	Provides file context & bit translation services

d.	Frame:

i.	IPX Header (30 bytes)

ii.	Connection Control (8 bits) - controls data flow

iii.	Data Stream Type (8 bits) - type of data in packet

iv.	Source & Destination Connection IDs (16 bits each) - identifies virtual circuits

v.	Sequence Number (16 bits)

vi.	Acknowledgment Number (16 bits) - provides next packet receiver expects

vii.	Allocation Number (16 bits) - provides end-to-end flow control & indicates number of receive buffers

viii.	Data

C.	Upper Level Protocols/Layers

1.	NetBios - based on Packet Exchange Protocol (PEP)

2.	Netware Core Protocols (NCP)

a.	provides file, print, file locking & bindery services

b.	uses file syntax & character code translation

3.	Netware Shell - Intercepts application request & determines if network access is required

4.	Netware Remote Procedure Calls (RPC) - Session layer

5.	Message Handling Service (MHS)

6.	BTrieve - database access system (NLM at server)

7.	Service Advertisement Protocol (SAP) - used by servers to advertise services available to clients

8.	Netware Link Support Protocol (NLSP) - used by NetWare to find & determine the state of network links; Link State routing protocol

IV.	Open Systems Interconnect (OSI)

| Application | VT | DS | FTAM | CMIP/CMIS | MHS | ASN.1 |
| --- | --- | --- | --- | --- | --- | --- |
|  | ASCE/RTSE/ROSE |  |  |  |  |  |
| Presentation | OSI Presentation Services |  |  |  |  |  |
| Session | OSI Session Services |  |  |  |  |  |
| Transport | TP0/TP1/TP2/TP3/TP4 |  |  |  |  |  |
| Network | ES-IS/IS-IS |  |  |  |  |  |
| CLNP |  |  |  |  |  |  |
| HDLS/LAPBData LinkX.25 PLP |  |  |  |  |  |  |
| IEEE 802.2 | IEEE 802.3/802.4/802.5 FDDI |  |  |  |  |  |
| Physical |  |  |  | RS-232/RS-449 X.21/V.35/ISDN |  |  |

A.	Overall design & standards

1.	Designed/maintained by the International Standards Organization (ISO)

2.	Built for pre-existing physical & data link protocols

3.	Created to provide multinational protocol standards

4.	Service Access Points (SAPs) are used for interlayer communications

5.	End Systems (ES) are the ultimate source/destination for data

6.	Intermediate Systems (IS) are usually routers

B.	Network Layer Protocols

1.	CCIT X.25 Packet Level Protocol (PLP)

a.	Most widely used transport layer protocol

b.	Used with Connection Oriented Network Services (CONS)

c.	Uses 802.2 type 2 connections

2.	Connectionless Network Protocol (CLNP)

a.	Uses 802.2 type 1 connections

3.	End System - Intermediate System (ES-IS) Routing Protocol - routes data from end systems to intermediate systems & ISs to ESs within a local area network

4.	Intermediate System - Intermediate System (IS-IS) Routing Protocol - makes complex routing decisions based on changing network conditions

C.	Transport Layer Protocols

1.	TP0

a.	Only runs over connection oriented services

b.	Fragments & reassembles packets

2.	TP1

a.	Only runs over connection oriented services

b.	Fragments & reassembles packets

c.	Numbers packets & provides error control

3.	TP2

a.	Only runs over connection oriented services

b.	performs packet fragmentation & reassembly

c.	does not provide error recovery

d.	controls data flow

e.	multiplexes

4.	TP3

a.	Only runs over connection oriented services

b.	Performs packet fragmentation & reassembly

c.	Numbers packets & provides error recovery

d.	Controls data flow

e.	Multiplexes

5.	TP4 - similar to TCP in Internet Suite

a.	Fully Reliable/Connection Oriented

b.	Runs over connectionless services

c.	Best known OSI transport protocol

D.	OSI Session Layer

1.	Turns data into a session with ‘activities,’ self contained events

2.	Adds control mechanisms (Session Management/Dialogue Control)

a.	Uses tokens to control data flow & priority

b.	Dialogue Control handles errors which occur at upper levels

c.	Can ‘roll back’ all communications to a ‘checkpoint’

E.	OSI Presentation Layer

1.	OSI - Typically a pass-through layer with facilities for establishing connections, sending data, controlling dialogs, & synchronizing activities

2.	Allows two computers to communicate although they may use different encoding rules

3.	ASN.1 - Abstract Syntax Notation One

a.	Based on CCIT X.409

b.	Provides machine independent data transfer

c.	Uses Basic Encoding Rules (BER) to encode data

d.	BER Frame:

i.	TAG - identifies data type

ii.	LENGTH - length of data

iii.	Value

F.	OSI Application Layer

1.	Based on Application Service Elements (ASE)

2.	Association Control Service Elements (ASCE) - controls association of elements between application elements

3.	Reliable Transfer Service Element (RTSE)

a.	Provides reliable transport

b.	Makes checkpoints & session activities easy to use

c.	Provides notification of successful or unsuccessful delivery

4.	Remote Operations Service Element (ROSE)

a.	Manages remote operations via control of the request-reply mechanism

b.	Similar to RPCs

c.	Either side can be a server (performer)

5.	Message Handling Systems (MHS)

a.	Provides underlying transport mechanism for electronic mail

b.	derived from CCIT X.400

c.	Uses the Message Oriented Text Interchange System (MOTIS)

d.	Communications scheme - Users communicate with a User Agent (UA), which transmits/receives messages from the Message Transfer System (MTS), which in turn communicates to other MTSs through Message Transfer Agents (MTAs).

e.	Administrative Domains:

i.	Administrative Management Domains (ADMDS) - controlled by a Post, Telephone, & Telegraph (PTT) organization

ii.	Private Message Domains (PRMDs)

6.	File Transfer, Access, & Management (FTAM)

a.	uses virtual filestore (offering a universally understood file system) to exchange files between different systems

b.	similar to NFS & FTP combined

7.	Directory Services

a.	Derived from CCIT X.500

b.	Stored in Directory Info Base (DIB)

c.	Accessed through Directory User Agent (DUA)

d.	Retrieved by Directory System Agent (DSA)

8.	Virtual Terminal (VT)

a.	similar to Telnet

b.	Based on a shared data structure called a Conceptual Communications Area

i.	Conceptual Datastore - 3 dimensional array from agreed upon character set determined in initial session setup

ii.	Control Object - controls actions

iii.	Device Object - interacts with input/output devices

iv.	Token Ownership Object - used in synchronous mode

v.	Parameters Object - keeps track of session & device parameters

9.	Common Management Information Protocol (CMIP)

a.	Uses CMIS service

b.	Addresses accounting, performance, security, fault, & configuration information

c.	Uses Management Information Bases (MIBs)

d.	Uses Agents & Managers to get & set managed device information

V.	Systems Network Architecture (SNA)

| Application | Transaction Services |  |
| --- | --- | --- |
|  | Presentation Services | Network Addressable Units |
| Presentation |  |  |
| Session |  |  |
|  | Data Flow Control |  |
| Transport |  |  |
|  | Transmit Control |  |
| Network |  |  |
|  | Path Control | Path Control Network |
| Data Link |  |  |
|  | IEEE 802.2/SDLC |  |
| Physical | IEEE 802.3/802.4/802.5 |  |

A.	Path Control Network

1.	Implemented on a mainframe by the Advanced Communication Functions/Network Control Program (ACF/NCP)

2.	Provides routing, header construction, boundary functions (translating local addresses into network addresses), & fragmenting/reassembling datagrams

B.	Network Addressable Units

1.	Implemented on the mainframe by the Advanced Communications Functions/Virtual Telecommunications Access Method (VTAM)

2.	Provides transmission control & data flow control

C.	Basic Hierarchical Node Types

1.	Host (Type 5)

a.	Used in S/370 systems running VTAM

b.	Controls a domain

2.	Communications Controller (Type 4)

a.	Also called a ‘Front End Processor’

b.	Routes & controls data flow in a network

3.	Peripheral (Type 2) - Cluster controllers, terminals, etc

D.	Hierarchical Network Addressable Units

1.	System Services Control Point (SSCP)

a.	VTAM

b.	types

i.	Type 5 - manages configurations, coordinates requests, provides session services

ii.	Type 2.1 - smaller scale control point which concentrates on peer-to-peer connections

2.	Physical Unit (PU) - hardware, firmware, & software that monitors & manages node resources

a.	Type 1 - terminal

b.	Type 2

i.	cluster controllers, terminals, & printers

ii.	only communicate to a mainframe

c.	Type 2.1

i.	cluster controllers, mincomputers, workstations, gateways

ii.	communicate to mainframes & other type 2.1 devices

d.	Type 4 - communication controllers

e.	Type 5 - host computers

3.	Logical Unit - entry point into the network

a.	Options: Time Sharing, Customer Information Control, Information Management System, & Conversational

b.	Types:

i.	type 0

Ÿ	program to device/general purpose

Ÿ	batch terminals, card readers, printers

ii.	type 1 - program to device; master/slave relationship using SNA character string data

iii.	type 2 - program to device; master/slave relationship using 3270 CRT data stream

iv.	type 3 - program to device; master/slave relationship using 3270 printer stream data

v.	type 4 - program to program or program to device; master/slave or peer/peer relationship using character string printer data stream & 6670 connections

vi.	types 6 & 6.1 - program to program; peer/peer host to host CICS or IMS

vii.	type 6.2

Ÿ	program to program

Ÿ	peer/peer general data stream

Ÿ	Basis of APPN

viii.	type 7 - 5250 data stream used by AS/400, System 36, & System 38

E.	SNA Session Layer

1.	Provides logical connection between two Network Addressable Units

2.	Primary Logical Unit is responsible for error correction

3.	Session initiates when Primary Logical Unit sends a ‘bind’ message to the Secondary Logical Unit

F.	SNADS

1.	Provides file & messaging services

G.	SNA Advanced Peer-to-Peer Networking (APPN)

1.	Developed in 1984

2.	Each node has a control point that handles network communications

3.	Each Physical Unit (PU) is type 2.1 (Peripheral Nodes)

4.	PUs broadcast their presence to the network

5.	Uses Logical Unit type 6.2 (program to program/peer to peer)

6.	Reduces data stream size by not transmitting terminal screen information

7.	Provides better error recovery & security

8.	Allows an SNA network to run without a mainframe

H.	SNA NetView - SNA compatible network management system

1.	Problem Management - handles network errors through problem determination, bypass/recovery, resolution, & tracking/control

2.	Performace/Accounting - monitors network & records status information

3.	Configuration Management - records physical identification & logical relationships between resources

4.	Change Management

a.	Records changes in hardware, firmware, & software

b.	Helps with planning, & can institute some network changes

VI.	Digital Network Architecture (DNA)

| Application | OSI ASCE, ROSE, MOTIS, MHS, FTAM, & VT | DNA Data Access Protocol |
| --- | --- | --- |
| Presentation | OSI Presentation Services |  |
| Session | OSI Session, DNA Session Control |  |
| Transport | ISO 8073, NSP |  |
| Network | OSI CLNP/CLNS/ES-IS/IS-IS, X.25 PLP |  |
| Data Link | 802.2, FDDI, X.25, HDLC, LAPB | DDCMP |
| Physical | 802.3, FDDI, RS-232, RS-449 |  |

A.	DNA Origin/Control

1.	Proprietary network architecture

2.	designed/maintained by DEC

3.	currently in Phase V; completely compliant with OSI

B.	DNA Physical Layer

1.	IEEE 802.3

2.	FDDI

3.	RS-232

4.	RS-449

C.	DNA Data Link Layer

1.	Standard synchronous - LABP & HDLC

2.	Standard Local Area Network - IEEE 802.2 & FDDI

3.	Digital Data Communications Message Protocol (DDCMP)

a.	Original DNA protocol

b.	Can run asynchronous or synchronous

c.	Uses byte counts

d.	Uses polling on multi-access lines

e.	Uses positive ACKs

f.	Provides ‘pipelining’ - multiple ACKs piggybacked onto data

D.	DNA Network Layer - uses standard protocols

E.	DNA Transport Layer - uses standard protocols

F.	DNA Session Layer

1.	OSI Session Services

2.	DNA Session Control

a.	Connection Control - sets up, maintains, & tears down transport connections

b.	Address Resolution/Naming Service

c.	Address Selection Services

G.	DNA Application Layer

1.	Uses OSI ASCE/ROSE/MOTIS/MHS/FTAM/VT

2.	Data Access Protocol (DAP) for file transfer

3.	Network Virtual Terminal Service - like Telnet

4.	Mail II

5.	SNA Gateway services - remote job entry, 3270 emulation, data transfer, & user written programs

6.	VMS suite for MS-DOS

7.	VideoTex - allows access & storage from central databases

8.	Enterprise Management Architecture - provides network management

a.	Uses Common Management Information Protocol from OSI

b.	Entities - a device with an agent

c.	Directors - gather information about managed devices & displays

9.	Other - including Time Services, Distributed Job Queuing, & Remote Systems Management

VII.	Appletalk

| Application | Appleshare File Server AppleShare Print Server AppleShare PC |
| --- | --- |
| Presentation | AppleTalk Filing Protocol |
| Session | Printer Access Protocol AppleTalk Session Protocol Zone Information Protocol |
| Transport | AppleTalk Transport Protocol AppleTalk Data Stream Protocol |
| Network | Datagram Delivery Protocol Routing Table Maintenance Protocol Name Binding Protocol |
| Data Link | AppleTalk Address Resolution Protocol |
| Physical | Ethertalk (802.3) TokenTalk (Token Ring) LocalTalk |

A.	Physical Layer

1.	EtherTalk - AppleTalk over Ethernet; an ‘extended network architecture’

2.	TokenTalk - AppleTalk over Token Ring; an ‘extended network architecture’

3.	LocalTalk - Twisted Pair connections at 230.4KBps

B.	Data Link Layer - AppleTalk Address Resolution Protocol (AARP)

1.	Translates between upper layers & link layer

2.	associates link layer addresses with physical layer addresses

3.	used to arbitrate/assign link level addresses

C.	Network Layer

1.	Datagram Delivery Protocol

a.	Provides connectionless services between sockets

b.	Static sockets number 1 through 127

c.	Dynamically allocated sockets number 128 through 254

d.	Network address consist of 16 bit network number (for phase II only - on ‘extended network architectures’), 8 bit node ID, & 8 bit process number

2.	Routing Table Maintenance Protocol

a.	Routing tables have five entries for each node - network, range, distance, next intermediate router, & entry state

b.	Four packet types - data, request, route data request, & response

3.	Name Binding Protocol (NBP)

a.	Translates between AppleTalk names & physical network addresses

b.	Binds physical addresses to Network Visible Entities

c.	Registers, confirms, looks up, & deletes names

D.	Transport Layer

1.	AppleTalk Transport Protocol (ATP)

a.	Provide reliable transport

b.	Based on transaction system

c.	Fragments & reassembles packets

d.	Limited to 8 packets of no more than 578 bytes

2.	AppleTalk Data Stream Protocol (ADSP) - Similar to TCP in Internet Suite

a.	Full duplex, flow controlled, reliable, connection oriented

b.	Uses sliding window flow control mechanism

c.	establishes logical relationships between two sockets

E.	Session Layer

1.	Printer Access Protocol (PAP)

2.	AppleTalk Session Protocol (ASP) - establishes, maintains, & terminates sessions

3.	Zone Information Protocol

a.	maintains network number to zone name mappings in Zone Information Tables

b.	allows transmissions to a logically related group of stations

F.	Presentation Layer - AppleTalk Filing Protocol (AFP)

1.	Provides transparent access to remote files

2.	Provides file system security through user authentication, volume passwords,  directory access control, & file access control

3.	Supports multiple volumes

4.	Uses file context translation

G.	Application Layer

1.	AppleShare File Server - uses AFP

2.	AppleShare Print Server - uses NBP & PAP

3.	AppleShare PC