Networking Technologies Quick Test Questions

1.	The Physical Layer is related to:

}	datagrams

}	frames

}	bits

}	segments

}	none

v	bits

2.	The Data Link Layer is related to:

}	datagrams

}	frames

}	bits

}	segments

}	none

v	frames

3.	The Network Layer is related to:

}	datagrams

}	frames

}	bits

}	segments

}	none

v	datagrams

4.	The Transport Layer is related to:

}	datagrams

}	frames

}	bits

}	segments

}	none

v	segments

5.	The Session Layer is related to:

}	datagrams

}	frames

}	bits

}	segments

}	none

v	none

6.	The Presentation Layer is related to:

}	datagrams

}	frames

}	bits

}	segments

}	none

v	none

7.	The Application Layer is related to:

}	datagrams

}	frames

}	bits

}	segments

}	none

v	none

8.	Various computers share the processing and one set of data

}	centralized

}	coorperative

}	distributed

v	distributed

9.	One computer completes all processing and distributes data to terminals

}	centralized

}	coorperative

}	distributed

v	centralized

10.	Various computers share in both processing and storing the data

}	centralized

}	coorperative

}	distributed

v	coorperative

11.	Defines the mechanical and electrical specifications of the network:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

12.	Includes number and functions of pins in network connectors:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

13.	Moves information across a network made up of multiple network segments:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

14.	Organizes data into logical groupings of data called frames:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	data link

15.	Determines the types of cables which may be used:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

16.	Examines the network layer address to send data to the next location in an internet:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

17.	Formats and translates data into a mutually agreed upon format:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	presentation

18.	Ensures reliable end-to-end data transmission and provides flow control:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	transport

19.	May compress and encrypt data:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	presentation

20.	Handles message fragmentation and reassembly:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

21.	Handles upper layer problems, such as paper out conditions or lack of storage space:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	session

22.	Manages the dialog between two network applications:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	session

23.	Manages user interface into the network:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	application

24.	Manages communications between applications on a workstation:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	application

25.	Verifies user ids and passwords:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	session

26.	Coordinates and verifies transmission schemes and procedures:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	session

27.	Workstation operating system is not aware of network:

}	OS Call Interception

}	Remote Operation

}	Collaberative

v	OS Call Interception

28.	Operating System and network software work together to fill client requests:

}	OS Call Interception

}	Remote Operation

}	Collaberative

v	collaberative

29.	Workstation operating system sends calls directly to the network:

}	OS Call Interception

}	Remote Operation

}	Collaberative

v	remote operation

30.	Network software intercepts calls to the workstation operating system:

}	OS Call Interception

}	Remote Operation

}	Collaberative

v	OS Call Interception

31.	Unbounded (multiple answers)

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Microwave, Spread Spectrum, Infared, CPDP

32.	Bounded Media (multiple answers):

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Coax, Fiber, Twisted Pair

33.	Easy to intercept; prone to weather interfearance:

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Microwave

34.	Low Probablility of electrical interfearance (multiple answers):

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Coax, Infared

35.	Most resistant to electromagnetic interfearance:

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Fiber

36.	Complicated to install and maintain:

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Microwave

37.	Physically Sturdy:

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Coax

38.	Mature Standards (multiple answers):

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Coax, Twisted Pair

39.	Line of Sight only:

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Infared

40.	Terrestial or Satellite:

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Microwave

41.	High bandwidth:

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Coax

42.	Multi-mode or Single Mode:

}	Microwave

}	Coax

}	Spread Spectrum

}	Infared

}	Fiber

}	CPDP

}	Twisted Pair

v	Fiber

43.	Probablistic:

}	contentious

}	non-contentious

v	contentious

44.	Token Ring or Polling:

}	contentious

}	non-contentious

v	non-contentious

45.	Stations transmit at will:

}	contentious

}	non-contentious

v	contentious

46.	Must have some sort of collision detection/avoidance:

}	contentious

}	non-contentious

v	contentious

47.	Each station waits its turn to transmit:

}	contentious

}	non-contentious

v	non-contentious

48.	CSMA/CD and CSMA/CA:

}	contentious

}	non-contentious

v	contentious

49.	Modulates digital signal onto an analog line:

}	Modem

}	Codec

v	Modem

50.	Places analog signal onto a digital line:

}	Modem

}	Codec

v	Codec

51.	Uses phase shifts to indicate information:

}	amplitude shift keying

}	frequency shift keying

}	phase shift keying

v	phase shift keying

52.	Uses changes in the signal level to indicate information:

}	amplitude shift keying

}	frequency shift keying

}	phase shift keying

v	amplitude shift keying

53.	Often combined with frequency shift keying in high speed modems:

}	amplitude shift keying

}	frequency shift keying

}	phase shift keying

v	phase shift keying

54.	Uses the frequency of the signal to indicate information:

}	amplitude shift keying

}	frequency shift keying

}	phase shift keying

v	frequency shift keying

55.	Uses direct digital signalling on the transmission medium:

}	baseband

}	broadband

v	baseband

56.	Signal does not take entire bandwidth of transmission medium:

}	baseband

}	broadband

v	broadband

57.	Uses analog signalling:

}	baseband

}	broadband

v	broadband

58.	Uses digital signalling:

}	baseband

}	broadband

v	baseband

59.	Less likely to distort signal:

}	baseband

}	broadband

v	broadband

60.	Covers longer distances with no regeneration:

}	baseband

}	broadband

v	broadband

61.	Uses entire channel:

}	baseband

}	broadband

v	baseband

62.	Uses passive or active headends:

}	baseband

}	broadband

v	broadband

63.	Described using frequency, amplitude, and phase:

}	baseband

}	broadband

v	broadband

64.	Described using baud, or rates of signal changes:

}	baseband

}	broadband

v	baseband

65.	8 bit character code (multiple answers):

}	ASCII

}	Extended ASCII

}	EBCDIC

v	Extended ASCII, EBCDIC

66.	Derived from Binary Coded Decimal:

}	ASCII

}	Extended ASCII

}	EBCDIC

v	EBCDIC

67.	7 bit character code with one bit for parity or extra characters:

}	ASCII

}	Extended ASCII

}	EBCDIC

v	ASCII

68.	128 Characters:

}	ASCII

}	Extended ASCII

}	EBCDIC

v	ASCII

69.	255 Characters:

}	ASCII

}	Extended ASCII

}	EBCDIC

v	EBCDIC

70.	256 Characters:

}	ASCII

}	Extended ASCII

}	EBCDIC

v	Extended ASCII

71.	Self Clocking (multiple answers):

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	Return to Zero, Manchester, Differential Manchester, BiPhase

72.	Only a positive or negative represents data - not both:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	unipolar

73.	Two voltage levels and ground:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	bipolar

74.	Signal indicated by state or level:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	unipolar, polar, bipolar, non-return to zero

75.	Used in teletype interfaces:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	unipolar

76.	Positive and negative represent data, not ground:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	polar

77.	Clocks from mid-bit transition:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	return to zero, manchester, differential manchester, biphase

78.	Alternate Mask Inversion:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	bipolar

79.	RS232:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	non-return to zero

80.	transition indicates data:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	non-return to zero, manchester, differential manchester

81.	Immune to cross wiring errors:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	bipolar

82.	Ethernet

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	manchester

83.	Easiest to detect noise and errors on:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	bipolar

84.	Token Ring:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	differential manchester

85.	Low to high transition = 1, high to low = 0:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	manchester

86.	Start of bit transition = 0, no transition = 1:

}	Unipolar

}	Polar

}	Bipolar

}	Return to Zero

}	Non-Return to Zero

}	Manchester

}	Differential Manchester

}	Biphase

v	differential manchester

87.	Used to transmit several lower speed signals over one high speed line:

}	frequency division multiplexing

}	time division multiplexing

v	both

88.	Places different signals in different time slots:

}	frequency division multiplexing

}	time division multiplexing

v	time division

89.	Places different signals on different frequencies:

}	frequency division multiplexing

}	time division multiplexing

v	frequency division

90.	Faster than asynchronous:

}	synchronous

}	asynchronous

}	isonchronous

v	synchronous

91.	Signals sourced from different clocks, frequencies, and/or phase relationships:

}	synchronous

}	asynchronous

}	isonchronous

v	asynchronous

92.	Used for randomly generated information:

}	synchronous

}	asynchronous

}	isonchronous

v	asynchronous

93.	Carried timing information in the packet:

}	synchronous

}	asynchronous

}	isonchronous

v	isonchronous

94.	Have the same frequency or are sourced from the same clock:

}	synchronous

}	asynchronous

}	isonchronous

v	synchronous

95.	Character oriented, using extra synchronization bits:

}	synchronous

}	asynchronous

}	isonchronous

v	asynchronous

96.	One direction communication:

}	simplex

}	half duplex

}	full duplex

v	simplex

97.	Polite conversation:

}	simplex

}	half duplex

}	full duplex

v	half duplex

98.	One direction at a time:

}	simplex

}	half duplex

}	full duplex

v	half duplex

99.	Telephone:

}	simplex

}	half duplex

}	full duplex

v	full duplex

100.	Walkie talkies:

}	simplex

}	half duplex

}	full duplex

v	half duplex

101.	Both directions at once:

}	simplex

}	half duplex

}	full duplex

v	full duplex

102.	Point to point connections between each entity on the network:

}	point to point

}	multipoint

}	mesh

}	star

}	ring

}	hybrid

v	mesh

103.	Direct connection between two points:

}	point to point

}	multipoint

}	mesh

}	star

}	ring

}	hybrid

v	point to point

104.	Connects three or more points on a shared channel:

}	point to point

}	multipoint

}	mesh

}	star

}	ring

}	hybrid

v	multipoint

105.	Central point facilitates easy troubleshooting:

}	point to point

}	multipoint

}	mesh

}	star

}	ring

}	hybrid

v	star

106.	Like the Internet:

}	point to point

}	multipoint

}	mesh

}	star

}	ring

}	hybrid

v	hybrid

107.	Requires adressing schemes:

}	point to point

}	multipoint

}	mesh

}	star

}	ring

}	hybrid

v	multipoint

108.	Can be logical multipoint, polling, or star:

}	point to point

}	multipoint

}	mesh

}	star

}	ring

}	hybrid

v	star

109.	Bus is modern implementation:

}	point to point

}	multipoint

}	mesh

}	star

}	ring

}	hybrid

v	multipoint

110.	Directly connects sender and receiver in unbroken path:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	circuit switching

111.	Most common used method in networks today:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	packet

112.	Telephone System:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	circuit

113.	Packets broken up by PADs:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	packet

114.	Relies on data link layer to fragment packets:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	message

115.	Virtually no channel access delay:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	circuit

116.	Packets may be stored in switches before being forwarded:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	message, packet

117.	Store and Forward:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	message

118.	Stores packets in memory:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	packet

119.	Datagram or Virtual Circuit modes:

}	Circuit Switching

}	Message Switching

}	Packet Switching

v	packet

120.	SDLC:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	data link

121.	IP:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

122.	EIA232:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physcial

123.	Internet Control Message Protocol:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

124.	CCIT V.24:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

125.	EIA449:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

126.	TCP

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	transport, session

127.	CCIT V series:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

128.	EtherTalk

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

129.	UDP:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	transport, session

130.	TokenTalk

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physcial

131.	HDLC:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	data link

132.	LocalTalk

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physcial

133.	Routing Information Protocol:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	application, presentation

134.	Apple Talk Address Resolution Protocol (AARP)

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	data link

135.	SONET:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

136.	Apple Datagram Delivery Protocol

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

137.	T1:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

138.	Apple Name Binding Protocol

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

139.	File Transport Protocol (FTP):

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	application, presentation

140.	Apple Talk Transport Prtocol (ATP)

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	transport

141.	ADCCP:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	data link

142.	Apple Talk Data Stream Protocol

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	transport

143.	Simple Mail Transfer Protocol (SMTP):

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	application, presentation

144.	ISDN:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

145.	Apple Printer Access Protocol (PAP)

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	session

146.	Telnet:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	application, presentation

147.	ATM:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

148.	Remote Procedure Calls (RPCs):

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	session

149.	LAPB:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	data link

150.	Network File System:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	application, presentation

151.	CCIT X.21:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

152.	Netware Streams:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network, transport, session, presentation

153.	Switch Multimegabit Data Service (SMDS):

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	data link

154.	IPX:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

155.	IEEE 802.3:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical, lower data link

156.	SPX:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	transport

157.	Ethernet:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical, data link

158.	ASN.1:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	session

159.	Appletalk:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	all

160.	Repeater:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical

161.	CCIT X.25:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical, data link, network

162.	IEEE 802.5:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical, lower data link

163.	Bridge:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical, data link

164.	IEEE 802.2:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	data link upper half (logical link)

165.	ARCNet:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical, data link

166.	Gateway:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	transport, session, presentation, application

167.	FDDI:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical, lower data link

168.	IEEE 802.6:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	physical, lower data link

169.	Router:

}	physical

}	data link

}	network

}	transport

}	session

}	presentation

}	application

v	network

170.	HDLC:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	ISO

171.	802.x

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	IEEE

172.	ADCCP:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	ANSI

173.	ARCNet:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	ATA

174.	V.24:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	CCIT

175.	X.21, X.25:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	CCIT

176.	OSI:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	ISO

177.	LAPB:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	CCIT

178.	ISDN:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	CCIT

179.	EBCDIC:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	IBM

180.	T1:

}	ISO

}	CCIT

}	IEEE

}	AT&T

}	ANSI

}	IBM

}	ATA

v	AT&T

181.	NetWare VLMs

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	session, presentation

182.	EIA232

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical

183.	NetWare LSL/ODI

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	transport

184.	IP

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	network

185.	NetWare Core Protocols

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	session, presentation, application

186.	SDLC

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	data link

187.	NetWare Service Advertisement Protocol (SAP)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	application

188.	RS232

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical

189.	NetWare Link Support Protocol (NLSP)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	presentation, application

190.	ICMP

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	network

191.	SNA Path Control

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	data link, network

192.	EIA449

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical

193.	SNA Advanced Communication Functions/Network Control Program (ACF/NCP)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	data link, network

194.	HDLC/ADCCP

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	data link

195.	SNA Network Addressable Units

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	network, transport, session, presentation, application

196.	TCP

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	transport, session

197.	SNA Transmit Control

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	network, transport

198.	SONET

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical

199.	SNA Data Flow Control

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	transport, session

200.	LAPB

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	data link

201.	Digital Data Communications Message Protocol (DDCMP)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link

202.	UDP

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	transport, session

203.	Digital Data Access Protocol

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	presentation, application

204.	T1

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical

205.	Digital VideoTex

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	application

206.	Routing Information Protocol (RIP)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	presentation, application

207.	ISDN

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link

208.	ATM

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link, network

209.	Open Shortest Path First (OSPF)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	presentation, application

210.	802.2

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	data link

211.	X.21

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical

212.	File Transfer Protocol (FTP)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	presentation, application

213.	802.3

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link

214.	802.5

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link

215.	Switched Multimegabit Data Service (SMDS)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	data link

216.	Simple Mail Transfer Protocol (SMTP)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	presentation, application

217.	ARCNet

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link

218.	Remote Terminal Emulation (TELNET)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	presentation, application

219.	LocalTalk

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link

220.	Remote Procedure Calls (RPC)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	session

221.	Network File System (NFS)

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	presentation, application

222.	802.6

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link

223.	FDDI

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link

224.	NetBIOS

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	session, presentation

225.	X.25

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	physical, data link, network

226.	IPX

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	network, transport

227.	SPX

}	Physical

}	Data Link

}	Network

}	Transport

}	Session

}	Presentation

}	Application

v	network, transport