# Service & Support

I.	Basics & Overview

A.	Problem Types

1.	physical

a.	temperature

i.	the temperature inside a computer may be up to 40 degrees higher than room temperature

ii.	continuous change in temperature can

Ÿ	cause chip creep

Ÿ	reduce component life

iii.	prevention - keep computer temperatures constant

Ÿ	let computers settle to room temperature before turning on

Ÿ	make certain there is adequate ventilation to prevent overheating

Ÿ	computers should be left on constantly to reduce changes in temperature (reduce on/off cycling)

b.	magnetism

c.	air quality - computers should have a filtered air supply

2.	electrical

a.	crosstalk

i.	interference between the magnetic fields of two cables

ii.	use shielded cable to reduce crosstalk

b.	static (electrostatic discharge)

i.	basics

Ÿ	sudden discharges of static electricity can cause damage

Ÿ	voltage discharged must be beyond 2,000 to 3,000 volts for you to feel it, only 20 to 30 volts to damage electronics

Ÿ	about 90% of the time ESD causes damage which does not cause the equipment to fail immediately

ii.	avoiding

Ÿ	always ground yourself when working on equipment

Ÿ	never let anyone touch you while working on equipment

Ÿ	never touch component leads

Ÿ	always use static bags

Ÿ	keep ambient humidity between 70 & 90%

c.	transients (spikes)

i.	sudden high voltage burst of electricity

ii.	prevention

Ÿ	put computer equipment on a separate circuit

Ÿ	ensure the power is properly grounded

Ÿ	consider using a surge protector

d.	line noise

i.	cables between devices are the most likely place for the introduction of noise and data corruption

ii.	noise distortion prevents a system from detecting thresholds

iii.	use shielded cable

iv.	make certain power lines are properly grounded

v.	don’t run power or other lines close to sources of interference.

Ÿ	radio frequency interference (RFI) is caused by radios, microwaves, ovens, & other appliances

Ÿ	electromagnetic interference (EMI) is caused by lights, radar, & industrial tools

3.	Electronic Data Processing (EDP) security

a.	protection of the entire data processing environment

i.	ensuring dialup lines have security measures

ii.	off site backup storage

iii.	encryption schemes

iv.	passwords & frequent password changes

b.	four threats

i.	destruction

ii.	corruption

iii.	disclosure

iv.	interruption

c.	example solutions

i.	restricting login times

ii.	requiring password changes

iii.	requiring unique passwords

iv.	turning off modems when not in use

v.	building redundancy into servers

vi.	encryption

Ÿ	set allow unencrypted passwords to off

Ÿ	when considering encryption, ask - what system resources & how much will it require, is it possible to defeat, is it the most current system available, & how much does it cost

4.	viruses

a.	types

i.	virus

Ÿ	programs which disrupt the normal flow of data processing

Ÿ	alter stored files or configurations

ii.	worm

Ÿ	fulfills a specific purpose at a specific location

Ÿ	leaves no trace of it’s travels through a network

iii.	trojan horse - a harmless looking program which causes damage when run

b.	removal

i.	viruses should be removed using appropriate software; formatting the hard drive of the infected machine will not always remove the virus, as it often continues to reside in memory

ii.	FDISK /MBR will sometimes clean up the master boot record of an infected DOS drive

c.	prevention

i.	educate users

ii.	regular backups

iii.	routinely scan for viruses

iv.	flag executable files execute only and/or read only

v.	grant only read & file scan rights in the PUBLIC & LOGIN directories

vi.	check all diskettes for viruses before use

vii.	discourage downloading of software from online services

viii.	be prepared for action

ix.	always boot file servers from the same source

d.	NetWare, by design, frustrates the spread of most viruses

B.	Troubleshooting

1.	gather information (ask questions)

a.	has it ever worked?

b.	when did it last work?

c.	what has changed?

2.	develop an attack plan (four step plan)

a.	rule out user error

b.	check physical stuff

i.	turn power off & on

ii.	check cable connections

c.	back up data

d.	eliminate overhead/simplify the system

3.	eliminate possibilities one by one

4.	record/log results

5.	explain the problem & solution to the user

C.	Documentation

1.	the purpose of system documentation is to assist in the maintenance & troubleshooting of the network

2.	system baseline

a.	tools used to create a system baseline

i.	automated record keeping tools

ii.	LANalyzer

b.	a measurement of typical network performance

3.	types

a.	LAN records

b.	LAN history

c.	resources available

d.	detailed map

D.	Resources

1.	NSEPro (NetWare Support Encyclopedia Professional)

a.	basics

i.	only comes in ‘professional’ version

ii.	installation

Ÿ	DOS - SETUP /M (minimum) /F (folio  software only - default)  /I (copy infobase) /A (copy all files) <filespec> (where to put the files)

Ÿ	Windows - SETUP; Setup option for CDROM only, Install option to copy program files to the local HD, Stand Alone option to copy all files to the local HD

Ÿ	to ensure correct downloading of files from the CD, you must set the NSE_DOWNLOAD environment variable

Ÿ	requires 170MB for a full install

b.	searching NSEPro

i.	spaces between words in a search equate to a logical and

ii.	search screen

Ÿ	Words - supplies listing of all indexed words in NSEPro

Ÿ	Query For

-	area to type in search words

-	search is performed as you type

Ÿ	Records with Hits - current search results

Ÿ	Scope - books included in the search

c.	Modules

i.	What’s New - quick summary of what’s new

ii.	Service & Support

Ÿ	technical information documents

Ÿ	files, patches, & fixes

Ÿ	third party files

Ÿ	Novell Application Notes - text only (no graphics)

Ÿ	Novell Professional Development builders

Ÿ	Novell Labs Bulletins

Ÿ	training

Ÿ	Service Provider’s Guide

Ÿ	top issues

Ÿ	decision trees

iii.	NetWare Updates - does not have the latest version of updates; lags behind NetWire by at least one month

iv.	Product Manuals

Ÿ	all manuals included in their entirety

Ÿ	includes older manuals (to NetWare 2.15)

v.	Sales & Marketing - press releases & marketing information

vi.	Novell Programs - CNE, reseller partnetships, etc

vii.	About NetWire - information on NetWire

viii.	New User Information - information on using NSEPro and getting newer copies

2.	Microhouse Technical Library (MHTL)

a.	basics

i.	requires you run both install & setup to install properly; install copies files, setup creates Windows icons

ii.	includes jumper settings, configuration information, and diagrams of equipment appearance

iii.	updated every three months

iv.	up to ten keywords may be used in a keyword search

v.	requires a hardware key

b.	main databases

i.	Encyclopedia of Hard Drives

Ÿ	interface types and installation

Ÿ	directory of manufacturers

Ÿ	hard drive specifications

Ÿ	BIOS drive type tables

Ÿ	solutions to common problems

Ÿ	glossary

Ÿ	IBM BIOS error codes

Ÿ	recommended reading

ii.	Encyclopedia of Main Boards

Ÿ	directory of manufacturers

Ÿ	using the encyclopedia

Ÿ	Intel CPUs

Ÿ	Intel NPUs

Ÿ	chipsets

Ÿ	CMOS setup guidelines

Ÿ	system and cache memory

Ÿ	bus architecture

Ÿ	misc components

Ÿ	system BIOS

Ÿ	bibliography

Ÿ	pin out reference

Ÿ	component identifier

Ÿ	glossary

Ÿ	acronyms and abbreviations

Ÿ	memory speed guidelines

Ÿ	conversion tables

iii.	Encyclopedia of Network Cards

Ÿ	topology

Ÿ	LAN access protocols

Ÿ	cable types

Ÿ	directory of manufacturers

Ÿ	jumper settings

Ÿ	glossary

Ÿ	node id quick reference

Ÿ	hardware device settings

Ÿ	common pin assignments

iv.	Encyclopedia of I/O Cards

3.	NetWire

a.	accessed through Compuserve

b.	updated daily

c.	menu choices

i.	What’s New

ii.	Technical Services

iii.	File Updates

iv.	Sales & Marketing

v.	Novell Programs

vi.	New User Information

d.	various forums through which you can ask for help & do research

i.	section 1 - introduction

ii.	section 2 - Novell System Product Message Forums

Ÿ	NetWare 2.x (NETW2X)

Ÿ	NetWare 3.x (NETW3X)

Ÿ	NetWare 4.x (NETW4X)

Ÿ	UnixWare (UNIXWARE)

Ÿ	Novell Client forum (NOVCLIENT) - IPXODI, NETX, VLM, ODINSUP, NetBios, & NetWare/Windows issues

Ÿ	Novell Desktop Systems Group Forum (NOVDESKTOP/NDSG) - NetWare DOS, Programming, Dataclub, NetWare Lite, NetWare NT Client

Ÿ	NetWare OS/2 forum (NOVOS2)

Ÿ	connectivity forum (NCONNECT) - NACS, NW Connect, AS/400 connectivity, host printing, SNA links, LAN/LAN links (multiprotocol router), NetWare Macintosh, NetWare/DEC connectivity, Portable NetWare, NetWare IP & NFS, MHS/EMail, LAN Workplace/Workgroup

Ÿ	NetWare management forum (NOVMAN) - ManageWise, NetWare for SAA management, Navigator

Ÿ	Novell Hardware forum (NOVHW) - Token Ring, Ethernet, backups, cabling, remote boot

iii.	section 3 - groupware

Ÿ	GroupWise forum (GROUPWISE)

Ÿ	SoftSolutions forum (SOFTSOL)

Ÿ	InForms forum (INFORMS)

iv.	section 4 - business/home products

Ÿ	Perfect Office forum (POFFICE)

Ÿ	WordPerfect for Windows (WPWIN)

Ÿ	WordPerfect for DOS (WPDOS)

Ÿ	WordPerfect for Macintosh (WPMAC)

Ÿ	WordPerfect for UNIX (WPUNIX)

Ÿ	Quattro Pro (QPRO)

Ÿ	Presentations (PRESENTATIONS)

Ÿ	PerfectHome (PERFHOME)

Ÿ	Electronic Publishing Tools (ENVOY, ELECPUB)

v.	section 5 - file areas

Ÿ	Technical Solutions Database (NTID)

Ÿ	Files Library (NOVLIB) - contains latest patches & updates

Ÿ	Files Area (NOVFILES) - most commonly downloaded files on NetWire

Ÿ	Applications Solutions Database (GWTID/APPTID)

Ÿ	Applications Library (GWFILES/WPFILES)

vi.	other

Ÿ	NOVUSER - user files

Ÿ	NOVBULL - Novell bulletins

Ÿ	NOVBG - Novell Buyer’s Guide

Ÿ	NDEVINFO - developer product information

Ÿ	NDEVSUPP - developer support

Ÿ	NOVLABS - Novell Labs

Ÿ	NSP - NetWare Service Provider library

Ÿ	NPRESS - NetWare Press Releases

Ÿ	LANUTILS - Avanti Technology

Ÿ	NVENA - vendor support forum A; Blue Lance, Concurrent Controls, CONUG, Knozall, On Technology, LAN Support Group, Multi-User DOS, Podell Systems, Tricord Systems

Ÿ	NVENB - vendor support forum B; Firefox, NetWorth, Ontrack Data Recovery, Simware

Ÿ	NOVVEN - vendor support forum; Dell

Ÿ	XWARE - Extensions Software

Ÿ	FOLIO - Folio Corporation

Ÿ	INFINITE - Infinite Technologies

Ÿ	MDOS - Intelligent MicroSoftware

Ÿ	MOUNTAIN - Mountain

Ÿ	NOTEWORK - Notework Corporation

Ÿ	ROSEWARE - RoseWare

Ÿ	SYNOPTICS - SynOptics

Ÿ	UTGROUP - UT Group

Ÿ	VINCA - Vinca Corporation

4.	AppNotes

a.	available as a subscription from Novell

b.	paper copy is the only source with the full graphics; NetWire & NSEPro only contain the text versions

E.	tools

1.	LANAlyzer for Windows

a.	basics

i.	allows you to monitor your network at the packet level

ii.	gives you the ability to analyze specific problems occurring on the network

iii.	statistics

Ÿ	Current network statistics can be viewed via the "dashboard," which provides a broad analysis of network performance, including packets/sec, utilization %, and errors/sec

Ÿ	short & long term network statistics can be displayed in graphs & tables

b.	monitoring

i.	station filtering

Ÿ	Between two selected stations on a network

Ÿ	From one selected station to another selected station

Ÿ	From one selected station to all other stations

Ÿ	To a selected station from all other stations on the network

ii.	protocol filtering - lets you filter network packets of a specified protocol

c.	requires

i.	requires network cards which operate in 16 or 32 bit promiscuous mode

ii.	works over ODI drivers

iii.	runs over either NETX 3.26 or later or VLMs

iv.	does not require NETX or VLMs, but some features are lost if not running them

d.	will only see packets on the current segment

2.	Time Domain Reflectometer - used to find problems in network cabling

F.	NetWare Protocols

1.	SAP - used by servers to advertise services

2.	NetWare Core Protocol

a.	regulates how connection control information is passed

b.	assigns packets sequence number to determine when packets are lost

c.	provides file, print, file locking & bindery services

d.	uses file syntax & character code translation

3.	IPX

a.	provides routing, packet fragmentation, & packet reassembly

b.	at the workstation, acts as the communication bridge between the NIC & the communications link

4.	SPX - provides reliable transport to upper level services

5.	NetBIOS - based on packet exchange protocol (PEP)

6.	NetWare Shell - Intercepts application request & determines if network access is required

7.	NetWare Remote Procedure Calls (RPC) - Session layer

8.	Message Handling Service (MHS)

9.	NetWare Link Support Protocol (NLSP) - used by NetWare to find & determine the state of network links; Link State routing protocol

G.	odds & ends

1.	‘hardened’ computers are especially built for noisy or problematic areas where there is extra physical or electrical stress on hardware

2.	third party software is the best way to obtain information on a system’s hardware

II.	Adapter Cards & Cabling

A.	Basics & Overview

1.	possible configuration conflicts

a.	interrupt

b.	base i/o

c.	memory address

d.	page memory range

e.	dma channel

f.	node address

2.	items to consider

a.	cabling specifications & requirements

b.	hard drive specifications & requirements

c.	software driver certification for the operating system

3.	odds & ends

a.	1 meter = 3.28 feet

b.	8 bit cards have 1 bank of connectors, 16 bit cards have 2

c.	physical addresses are assigned by board manufacturers, logical addresses by network administrators

d.	jumpers are used to close electrical circuits by ‘jumping’ a pair of wires rising from a circuit board

B.	ARCNet

1.	Basics

a.	Attached Resource Computer Network

b.	funded by Datapoint

c.	Novell calls this RXNET or TRXNET for the turbo version

d.	operates at 2.5MB/s

e.	uses a token passing scheme

f.	can support up to 255 nodes on a segment, although systems of this size are not practical

g.	disadvantage - slow

h.	frame size - 4202 bytes

i.	ANSI 878.1

2.	adapter cards

a.	node number

i.	1-255

ii.	should be grouped according to physical location

b.	base i/o

c.	memory address

d.	interrupt

e.	network timeout

3.	cabling types & adapters

a.	coax

i.	RG62/U

ii.	93 ohm terminators

iii.	BNC connectors (bayonet navy connectors)

b.	unshielded twisted pair

i.	24 or 26 gauge stranded wire

ii.	22, 24, or 26 gauge solid core wire

iii.	100 ohm terminators

c.	hubs

i.	active

Ÿ	regenerates signal

Ÿ	can be connected to active or passive devices

ii.	passive

Ÿ	does not regenerate signal

Ÿ	can only connect to active devices

Ÿ	open ports must be terminated

4.	cabling rules

a.	31 microsecond maximum delay

b.	never connect 2 passive devices

c.	never connect 2 active hubs through a passive hub

d.	terminate unused ports on all hubs; passive hubs require termination, but it is recommended for all open ports

e.	UTP in bus configuration can have no more than 10 NICs on the segment

f.	maximum distances

i.	total - 20,000ft

ii.	node to node - 2,000ft

iii.	active hub to active hub - 2,000ft

iv.	active hub to passive hub - 100ft

v.	passive hub to passive hub - do not do this

vi.	node to passive hub - 100ft

5.	Troubleshooting - things to look for

a.	no more than one node may have the same address; one of the two nodes will lose its connection to the network

b.	missing terminators cause data retransmission, network errors, or timeouts

c.	terminator must be of the correct value

d.	NICs must all be of the same impedance value

e.	failed NICs

f.	failed active hubs or ports

g.	exceeded cable lengths

h.	coax connectors not built correctly, not grounded correctly, or not crimped on correctly

C.	Ethernet

1.	Basics

a.	originally developed by Xerox, IBM, & Digital in 1970

b.	also known as spanning tree topology

c.	uses CSMA/CD

d.	max throughput of 10MB/s

e.	frame size - 1514 bytes

f.	CRC used to find corrupted frames

2.	Cards

a.	settings

i.	shared memory - most cards do not require

ii.	i/o address

iii.	interrupt

iv.	active connector selection

v.	boot PROM socket

b.	connectors

i.	BNC for coax

ii.	UTP  for RJ45

iii.	DIX for AUI

3.	Cabling

a.	10Base5 (thicknet)

i.	uses separate transceiver & aui (tap is transceiver)

ii.	cabling

Ÿ	minimum distance between taps is 2.5 meters

Ÿ	maximum segment length 500 meters

Ÿ	maximum network length 2,500 meters

Ÿ	one terminator must be grounded

Ÿ	drop cable maximum length 50 meters

Ÿ	all cable segments must be from the same spool of wire

Ÿ	maximum taps/segment is 100

Ÿ	5 segments maximum			]

Ÿ	4  repeaters maximum			]  5/4/3 rule

Ÿ	3 populated segments maximum	]

iii.	disadvantages

Ÿ	large size

Ÿ	high cost

Ÿ	awkward connections

b.	10Base2

i.	basics

Ÿ	thinnet (RG58/u or RG58c/u) with 50 ohm terminators & t-connectors

Ÿ	onboard transceiver

Ÿ	bnc connectors

ii.	cabling

Ÿ	minimum cable length is  .5 meters

Ÿ	t-connector must be directly attached to the NIC

Ÿ	maximum segment length is 185 meters

Ÿ	entire network cannot be over 925 meters

Ÿ	30 nodes maximum on one segment

Ÿ	5 segments maximum			]

Ÿ	4  repeaters maximum			]  5/4/3 rule

Ÿ	3 populated segments maximum	]

iii.	disadvantages

Ÿ	high cost compared to 10BaseT

Ÿ	unreliable configuration - one cable break brings down the entire network

c.	10BaseT

i.	basics

Ÿ	easy to troubleshoot

Ÿ	flexible scalability

Ÿ	inexpensive

ii.	cabling

Ÿ	uses unshielded twisted pair in a start topology with a logical bus

Ÿ	cat 4 or 5 - 22, 24, or 26 AWG wire

Ÿ	cables connected together in a hub or concentrator

Ÿ	wire rated at 85 to 115 ohm impedance at 10MHz

Ÿ	512 maximum nodes

Ÿ	100 meters maximum segment length

Ÿ	1024 maximum segments

Ÿ	100 meters maximum, .6 meters minumum hub to workstation

4.	frame types

a.	common attributes

i.	preamble in first line of header

ii.	start of frame delimiter - 1 byte

iii.	destination address - 6 bytes

iv.	origination address - 6 bytes

v.	length - 2 bytes

vi.	data - 46 to 1500 bytes

vii.	CRC - 4 bytes (optional)

b.	802.3

i.	used by NetWare ahead of IEEE ratification

ii.	default for NetWare 3.11 and below

c.	802.2 additions

i.	3 one byte logical link control fields

ii.	ANSI/IEEE/ISO standard

iii.	supports IPX checksums for greater data protection

iv.	default for NetWare 3.12 and above

d.	SNAP additions

i.	2 of the 3 802.2 LLC fields have fixed values

ii.	type of packet field

iii.	Ethernet_SNAP frame format is used in Token Ring & Appletalk Phase 2 environments

e.	Ethernet II

i.	replace length with type

ii.	start frame delimiter & preamble are combined

iii.	not an international standard

5.	troubleshooting

a.	twisted pair specific

Ÿ	make certain twisted pair cable has the correct number of twists & meets the correct data grade

b.	coax specific

Ÿ	if excess shielding is exposed on coax cable, make certain it is not grounding the connector

Ÿ	do not coil coax tightly

Ÿ	check for missing terminators

c.	check for electrical interference - fluorescent lights, electric motors, etc

d.	make certain connectors are properly crimped

e.	check cable lengths

f.	make certain topology rules are followed

g.	test cable continuity

h.	if Fileserver Not Found is error

Ÿ	check for proper/matching frame types

Ÿ	try a different network board

Ÿ	use comcheck.exe

Ÿ	double check the media type in the card configuration

i.	run NIC diagnostics

j.	remove all cards except NIC & retry

k.	make certain all cables are connected properly

D.	Token Ring

1.	basics

a.	IEEE 802.5

b.	physical star, logical ring

c.	each node connected to a multistation access unit (MSAU or MAU)

d.	runs at 4 or 16 MB/s

i.	16MB Token Ring allows a station to release a free token upon the completion of transmitting data

e.	each station acts as a repeater, sending packets to its nearest active downstream neighbor (NADN)

f.	token is 3 bytes; frame is up to 4202 bytes

g.	in a NetWare file server, the first board must be set to A20, & the second to A24

h.	in a NetWare environment, the file server usually ends up being the active monitor

2.	advantages/disadvantages

a.	shows no degradation under heavy load

b.	built in diagnostics and error recovery make it more reliable

c.	easier to connect to mainframes

d.	single card can be attached to 2 MAUs so the ring is rebuilt automatically if a cable breaks (ring wrap)

e.	more expensive

f.	can be difficult to troubleshoot

3.	cabling (UTP ?)

a.	type 1

i.	braided shield surrounding solid core 22AWG twisted pair

ii.	connects terminals, distribution panels, & different wiring closets in the same building

iii.	best choice for noisy environments

b.	type 2

i.	braided shield surrounding solid core 22AWG twisted pair; 2 unshielded twisted pairs for voice

ii.	connects terminals, distribution panels, & different wiring closets in the same building

c.	type 3

i.	4 unshielded twisted pairs of 24 AWG solid core wire; 2 twists per inch

ii.	4MB/s data, voice

iii.	long, low grade transmissions within walls

d.	type 5

i.	fiber-optic

ii.	2/100um or 140um fibers in one jacket

iii.	used only in the main ring of token ring networks

e.	type 6

i.	braided shield around 2 twisted pairs of 26AWG stranded pairs

ii.	more flexible than type 1

iii.	used for short runs, such as patch & extension cables

f.	type 8

i.	shielded 28AWG stranded twisted pair

ii.	designed for under carpet use

g.	type 9

i.	fire resistant for use in plenums

ii.	shielded 26AWG solid core twisted pair

h.	patch

i.	IBM connectors on both ends

ii.	connects MAUs, repeaters, & other token ring devices

i.	adapter

i.	IBM data connector on one end, DB9 on other end

ii.	connects workstations to the network

j.	16MB/s token ring is also run over level 5 UTP with RJ45 connectors & media filters

4.	types of Token Ring systems

a.	small moveable

i.	96 clients/servers

ii.	12 MAUs

iii.	type 6 cable

b.	large non-moveable

i.	basics

Ÿ	260 clients/servers (IBM)

Ÿ	250 clients/servers (IEEE)

Ÿ	33 MAUs

Ÿ	types 1 & 2 cable

Ÿ	involves punch panels, etc

ii.	cabling restrictions

Ÿ	min patch cable length - 8ft

Ÿ	maximum patch cable length - 150ft

Ÿ	standard patch cables - 8, 30, 75, & 150ft

Ÿ	max total patch cable length - 400ft

Ÿ	max adapter cable length - 100 meters

5.	MAUs/MSAUs

a.	central cabling component

b.	no intelligence

c.	8228 original MAU

i.	10 connectors - 8 workstation, ring in, ring out

ii.	when first set up, a set up tool must be inserted into each port to initialize it

iii.	mechanical device which uses relays to connect & disconnect ports

6.	CAU (Controlled Access Unit)

a.	‘power concentrator’ which can take a role in physical network management

b.	has a power plug & intelligence

7.	troubleshooting

a.	adapter cards

i.	base i/o, dma, shared memory, & irq conflicts

ii.	compatible drivers & boards

iii.	failed NIC

iv.	incorrect card speeds

v.	duplicate node addresses

b.	MAUs

i.	ring in/out ports connected correctly

ii.	bypass the MAU for testing

iii.	different manufacturer’s MAUs may not interoperate

iv.	MAU power supply problems

v.	bad port

c.	cables

i.	correct cable attachments

ii.	wrong type of cable for the network speed

iii.	bent or broken pins on connectors

d.	beaconing

i.	first station powered up becomes the active monitor, all others are standby monitors

ii.	active monitor sends out an announcement to it’s nearest active downstream neighbor (NADN) telling it to send an announcement to it’s NADN

iii.	as announcement is passed around the ring, each station discovers it’s nearest active upstream neighbor (NAUN)

iv.	if a station does not receive an announcement from it’s NAUN, it sends out a beacon telling the network of the situation; beacon contains node address, it’s NAUN, & type of problem

v.	NAUN Notification continues throughout operation every 7 seconds

| Property | ARCnet | 10Base5 | 10Base2 | 10BaseT | Token Ring |
| --- | --- | --- | --- | --- | --- |
| Total Length | 20,000ft | 8,200ft 2,500m | 3,035ft |  |  |
| Segment Length | active/other 2000ft passive/other 100ft | 1,640ft 500m | 607ft 185m | 328ft 100m |  |
| Minumum Length |  | 8ft/2.5m between taps | 1.5ft/.5m | 1.5ft/.5m | 8ft |
| Total Nodes | 256 |  |  | 512 | 260/250 |
| Nodes/Segment |  | 100 taps | 30 | 512 | 260/250 |
| Total Segments |  | 5 total 3 populated | 5 total 3 populated |  |  |
| Physical Addresses | manual | NIC maker | NIC maker | NIC maker | manual |
| Other | 31 us max delay | 4 repeaters maximum, 50m max drop cable | 4 repeaters maximum |  | 33 mau’s max, 150ft max patch, 400ft total max patch, 150ft max adapter cable |
| Cable | RG62/u coax utp | RG8/u coax | RG58/u coax | cat 4 utp | type 1 - type 9, |
| Physical Topology | bus/star | bus | bus | star | star |
| Logical Topology | ring | bus | bus | bus | ring |

E.	FDDI

1.	basics

a.	uses 802.5 token passing methods

b.	uses glass pulled into thin fibers, which are then bundled into cores (to carry the signal) & wrapped in a glass cladding (to contain the signal)

c.	100 MB/s

d.	uses 2 counter rotating rings

i.	can compensate for broken cables by redirecting traffic onto the second ring

ii.	class B stations only attach to one ring at time (single attached station)

iii.	class A stations attach to both rings (dual attached station)

e.	allows multiple tokens on the network at once

f.	tokens can bypass low priority stations to provide faster service to the higher priority ones

2.	advantages

a.	fast

b.	provides fair & timely access to the network for all attached stations

c.	difficult to wiretap (information security)

d.	more resistant to cable break (physical security)

e.	not susceptible to electrical interference (electrical security)

f.	uses multiple tokens

g.	ability to prioritize workstation access

h.	fiber cables are lighter than comparable copper cables

i.	FDDI signals can cover longer distances

j.	built in management

i.	ring - finds & resolves network ring problems

ii.	connection - controls stations inserting & removing themselves from the network

iii.	station - allows high level software to monitor the ring

3.	disadvantages

a.	demands great expertise

b.	while cable cost is comparable to UTP, concentrators are expensive

4.	troubleshooting

a.	incorrect cable type for internode distance

i.	plastic cable may not work well over 50 meters or 10MB/s

ii.	check on multimode vs single mode

b.	dirt on connectors - use alcohol to clean

c.	delay - up to 4ms is normal; if there are problems with this, use packet burst mode

d.	check on path efficiency

i.	replace bridges with routers where needed

ii.	FDDI translation bridges are preferred

e.	bad cable or open cable section

f.	cable breaks

i.	bad breaks can be found with a flashlight

ii.	minor breaks require an OTDR

F.	Which topology is right?

1.	ethernet is good for light to medium loads

2.	token ring is good for heavy loads

3.	consider

a.	types of applications & % of use for each

b.	flexibility required

c.	cost

d.	knowledge of various systems

e.	availability of replacements & add-ons

4.	steps in planning

a.	understand the user’s job

b.	create a list of requirements

c.	create a list of components to fulfill the requirements

III.	Storage Devices

A.	Hard Drives

1.	basics

a.	device which enables communication between hard drive & rest of computer is the controller

b.	time it takes for

i.	the head to find the correct track is the seek time

ii.	the sector with the correct information to rotate under the head is the drive latency

iii.	the drive to get to data is access time

c.	data transfer rate is the rate at which the drive transfers data to the rest of the system

d.	a cluster is the minimum amount of useable storage space by DOS

e.	a block is the minimum amount of useable storage space by NetWare

i.	when setting up a NetWare server, you choose the block size and the number of blocks on each volume, rather than choosing the volume size

2.	interfaces

a.	ST506

i.	originally designed for 5MB drives

ii.	uses MFM or RLL encoding

iii.	RLL stores more information

b.	IDE (integrated drive electronics)

i.	AT type interface

ii.	unless a special BIOS is used, 528MB is maximum size

iii.	does not support drive command overlapping or multitasking i/o

iv.	does not support optical or tape drives

v.	does not support bus mastering

vi.	do not low level format IDE drives

c.	ESDI (enhanced small device interface)

i.	uses floppy, hd, & data cables

ii.	higher data rate & storage capacity than ST506

iii.	can store information about bad tracks on the drive

iv.	can support tape drives

d.	SCSI (small computer systems interface)

i.	basics

Ÿ	supports up to 7 devices on each bus

Ÿ	arbitration system allows fast data transfer

Ÿ	SCSI controller has its own BIOS

Ÿ	an advanced SCSI programming interface (ASPI) driver is needed to coordinate communications between different types of devices

Ÿ	SCSI bus must be terminated at both ends

Ÿ	each attached device must have a unique id number (logical unit number - LUN)

Ÿ	12 inch minumum cable length

Ÿ	18 inch maximum cable length

ii.	types

Ÿ	SCSI - 50 pin, single ended signaling

Ÿ	SCSI2 - normally 4MB/s, fast up to 10MB/s; reliably handles all types of SCSI devices

Ÿ	WIDE SCSI - fast up to 20MB/s; differential signaling

iii.	troubleshooting/setting up a SCSI environment

Ÿ	do not mix SCSI & SCSI2 devices unless the controller is deigned to handle this

Ÿ	do not mix differential & single ended signaling

Ÿ	make certain all driver revision levels are current

Ÿ	use SCSI2 for all new installs

Ÿ	use a SCSI adapter which supports software configuration

Ÿ	make certain you use the proper connectors & cables

Ÿ	check for conflicts with IRQs, base I/Os, & DMAs

Ÿ	do not mix models of SCSI controllers in one machine

Ÿ	keep device models consistent

Ÿ	erratic performance often means the terminators are not correct

Ÿ	most common SCSI problems are caused by incorrect termination or LUNs in conflict

-	make certain each device has a unique LUN

-	make certain all devices are supplying the correct terminating current

-	make certain both ends of the SCSI cable are terminated

e.	disk coprocessor boards (DCB or HBA)

i.	controller cards which offload i/o from the cpu

ii.	also referred to as host bus adapters

iii.	a disk channel consist of one dcb & it’s attached drives

iv.	a file server can handle up to 4 dcb channels

v.	each dcb channel can handle up to 8 SCSI controllers

3.	setting up hard drives

a.	jumpers

i.	IDE

Ÿ	ACT - lights a light on the drive if it is active

Ÿ	C/D or DS - selects whether this is the C or D drive

Ÿ	DSP - indicates is cable is being shared with another drive & which drive is master

ii.	SCSI

Ÿ	ACT - lights a light on the drive if it is active

Ÿ	address - determines LUN

iii.	ESDI, MFM, & RLL

Ÿ	drive select - set depending on number of drives in the system & cable twists

b.	cables

i.	colored stripe goes to pin one

ii.	MFM/RLL/ESDI cables

Ÿ	twisted in the higher numbered pins & have 5 twisted lines

Ÿ	require separate control cable

iii.	floppy cables are twisted in the lower numbered pins & have 7 twisted lines

iv.	SCSI may have 25 or 50 pin cables,& both ends must be terminated

v.	IDE - cable may be no longer than 18 inches; 40 pins

c.	steps

i.	enter CMOS information

Ÿ	ESDI is type one; ESDI BIOS overwrites with the correct information on boot

Ÿ	SCSI is none installed or type 0

Ÿ	otherwise choose a type which has equal or fewer sectors than physically present

Ÿ	most commonly used drive type is 47, which allows custom drive parameters

Ÿ	if you use a custom drive type, make certain you write down the parameters you used & stick them to the hard drive

ii.	low level format

Ÿ	usually done by the factory

Ÿ	destructive

Ÿ	sets interleave - slows down hard drive so slower processors can keep up with data transferred

-	1:1 for 10MHz or faster servers, 20MHz or faster workstations

-	2:1 for 10Mhz or faster workstations

-	3:1 for 6-8MHz workstations

-	4:1 for 4.77MHz workstations

iii.	establish partitions

Ÿ	FDISK for DOS partition

-	should be at least 10MB on a NetWare server

-	 DOS 3.3 cannot handle a partition greater than 32MB

Ÿ	INSTALL.NLM for NetWare

iv.	high level format

Ÿ	FORMAT.COM for DOS

Ÿ	/S for transferring system files

Ÿ	sets up boot sector & FAT

Ÿ	creates blank root directory

Ÿ	scans disk & notes bad sectors

d.	NetWare drive information

i.	NetWare calculates a unique device number for each drive

Ÿ	first two digits are the driver identifier (such as Adaptec, DCB, Storage Demensions, etc)

Ÿ	third and fourth digits are the load instance of this driver (first instance is 0)

Ÿ	fifth and sizth digits are the disk number

Ÿ	seventh and eighth digits are the controller number; usually represents the SCSI controller ID

ii.	ISADISK is used for MFM, RLL, & IDE drives

iii.	IDE.DSK is preferred for IDE drives

iv.	SCSI drivers provided by controller manufacturer

v.	NetWare partitions

Ÿ	each partition has a unique partition number

Ÿ	a logical partition can be part of a physical partition or contain several physical partitions or parts of physical partitions

Ÿ	created using INSTALL.NLM

vi.	up to eight segments on a hard drive & 32 segments in a volume

vii.	up to 64 volumes on a NetWare 3.11 server

4.	RAID - a disk subsystem architecture which combines two or more standard physical drives in order to achieve data redundancy

a.	RAID 0

i.	stripes data across all drives in blocks

ii.	provides no data redundancy or fault tolerance

b.	RAID 1

i.	mirrored drives

ii.	can be duplexed

c.	RAID 2

i.	data is written across each drive one bit at a time

ii.	faulty bits are isolated using checksums

iii.	drives transfer data in parallel, enabling faster reads

d.	RAID 3

i.	data is written across all drives one bit at a time

ii.	separate drive for parity information

iii.	drives transfer data in parallel, enabling faster reads

iv.	checksum computation slows writes

v.	not ideal for NetWare, as NetWare’s blocks are too small to take advantage of RAID 3’s extended read capabilities

e.	RAID 4

i.	same as RAID 3, except data is written across all drives in blocks

f.	RAID 5

i.	data is written across all drives in blocks

ii.	parity information is written across all drives

iii.	efficient in handling small blocks

iv.	slower than RAID 0 or RAID 3

v.	efficiency goes up as the number of drives increases

vi.	hot spares can be mounted in the cabinet

g.	RAID 10

i.	data duplicated across two identical RAID 0 arrays

ii.	same concept as NetWare SFT III

h.	Choosing a RAID system

i.	importance of data & cost of downtime

ii.	number of users & amount of drive capacity needed

iii.	size of the data blocks

iv.	proportion of reads to writes

5.	Disk Mirroring & Duplexing

a.	Mirroring

i.	two drives attached to the same channel

ii.	information is written to both drives at the same time, so the backup is a mirror or the primary

b.	Duplexing

i.	two drives attached to two separate channels

ii.	information is written to both channels at the same time, so the backup channel is identical to the primary

iii.	protects against controller card & cable failure

iv.	can handle split seeks, in which a seek is issued to both channels at the same time, & the drive reaching the information first returns it; increases performance

c.	installing under 3.x

i.	install hardware

ii.	bring the server up & go to the console (or RCONSOLE)

iii.	load INSTALL.NLM

iv.	from the installation menu, select disk options

v.	choose server mirroring

vi.	select the primary drive

vii.	press insert to add the mirror

viii.	choose the drive to mirror to

d.	tips

i.	mirroring or duplexing should not take the place of doing backups

ii.	always load the disk drivers in the same order

iii.	NetWare does no differentiate between mirroring & duplexing in system messages

iv.	periodically check the status of mirrored & duplexed drives

v.	keep a record of the drive device codes

vi.	IDE drives should not be used as mirrored drives because the controllers form a master/slave relationship

6.	troubleshooting

a.	keep good backups

b.	2 common mistakes when installing hard drives are using the wrong cable and putting the cable on backwards

c.	VREPAIR

i.	volume must be dismounted before use

ii.	last resort only; exhaust all other options first

d.	if you have data corruption while using caching controllers, try turning the caching off

B.	CDROMs

1.	advantages

a.	large volumes of information

b.	standard file formats

c.	media is lightweight, easy to store, & cheap to duplicate

2.	disadvantages

a.	players are expensive

b.	slower than hard drives

3.	loading

a.	load device driver for adapter (HBA)

b.	load ASPICD or CDNASPI unless your are using a Future Domain or Logic Bus adapter

c.	load CDROM.NLM

4.	tips

a.	check for incompatibilities

b.	make certain you use caddies if needed

c.	do not install a CDROM directly above a hard drive; the magnetic fields can destroy the data on the hard drive

d.	Invalid drive spec means MSCDEX is not loaded properly

e.	Disc not High Sierra Format means your MSCDEX is not a current revision

C.	Magneto-Optical Drives

1.	basics

a.	uses - audio capture, video capture, archiving, backups, storage of large databases or information including CAD/CAM, etc

b.	should not be confused with WORM - data can be rewritten

c.	operation based on lasers & magnetics

d.	setup is similar to tape backup devices

2.	advantages

a.	removable media

b.	capacity - 650MB to 1,000GB; can be loaded into jukeboxes

c.	random access

d.	durability

e.	backup

3.	disadvantages

a.	costs

b.	speed

c.	size

IV.	The DOS Workstation

A.	basics

1.	resources

a.	interrupts

i.	a way of interrupting the processor when information or a process must be handled

ii.	cannot be shared among devices

iii.	common interrupts

Ÿ	0 - timer

Ÿ	1 - keyboard

Ÿ	2 - chained

Ÿ	3 - com2

Ÿ	4 - com1

Ÿ	5 - lpt2

Ÿ	6 - floppy

Ÿ	7 - lpt1

Ÿ	8 - clock

Ÿ	9 - redirected irq2

Ÿ	10 - reserved

Ÿ	11 - reserved

Ÿ	12 - reserved

Ÿ	13 - math coprocessor

Ÿ	14 - fixed disk

Ÿ	15 - reserved

b.	DMA - allows devices to write to system memory without the CPU

c.	base I/O  - memory reserved by the CPU to send/retrieve information to/from a device

d.	base address - memory reserved by a device for ROM or other code

2.	configuration

a.	standard systems generally use some keystroke combination to enter setup at boot

b.	EISA & Microchannel systems use configuration utilities

c.	configuration information is stored in CMOS; if this information is lost, most likely cause is the battery

3.	memory

a.	conventional (base) - everything that is available within the first 640K

b.	upper memory - following 384K used by adapter cards, video , & serial ports

c.	expanded memory - memory outside of the lower 1MB which can be swapped into & out of a frame in the lower 1MB in pages

d.	extended memory - everything above 1MB

e.	high memory

i.	a subset of extended memory

ii.	the first 64K of extended memory

4.	loading software into high memory

a.	MSDOS - LH in front of command

b.	DRDOS - HILOAD in front of command

5.	busses

a.	way of transferring data through the computer

b.	types

i.	ISA

Ÿ	industry standard architecture

Ÿ	runs at 8MHz

ii.	EISA

Ÿ	extended industry standard architecture

Ÿ	runs at 8MHz

Ÿ	accepts ISA or EISA boards

iii.	Microchannel

Ÿ	MCA

Ÿ	only accepts microchannel boards

iv.	Local Bus

Ÿ	fastest

Ÿ	runs at the speed of the processor oe a factor of the speed of the processor

6.	CPU modes

a.	real mode is when a processor emulates an 8086

b.	protected mode is when all memory must be accessed through the operating system

B.	workstation connection

1.	components

a.	IPX.COM

i.	functions as the go-between for the NIC card & the communications link

ii.	responsible for the communications functions & management of the sockets used in the workstation

iii.	ascertains the network segment address, network number, & the node address

iv.	earlier versions of NetWare (3.11 & before)

v.	to check settings, run IPX /I

b.	ODI

i.	a specification of how the datalink layer works in the OSI model

ii.	developed in 1989

iii.	provides for multiple protocols on the same network interface with one driver

iv.	includes LSL, MLID, & IPXODI

c.	NETX/VLM

i.	NETX is NetWare Shell; VLM.EXE is VLM manager

ii.	establishes, maintains, & terminates the connection between the workstation & the server

iii.	watchdog timer- automatically logs workstations out of the network after a certain period of inactivity

d.	NET.CFG

i.	contains information about the NIC card settings, protocols, & printer options

ii.	first frame type listed in the [Link Drivers] section is the frame type used to communicate to the server

iii.	PB BUFFERS

Ÿ	0 disables packet burst

Ÿ	range is 0 through 10

Ÿ	2 or 3 generally provides best performance

2.	remote boot (diskless workstations)

a.	boot prom on NIC connects to network & maps to boot image

b.	boot image

i.	DOSGEN used to create the boot image

ii.	appears to diskless workstation as a virtual drive

iii.	needs CONFIG.SYS, AUTOEXEC.BAT, network drivers

c.	advantages

i.	cost

ii.	convenience

iii.	security

iv.	speed

d.	NET$DOS.SYS is placed in a working directory; it must be flagged as sharable

e.	Remote booting DOS 5 requires RPLFIX be run against NET$DOS.SYS, which changes the size of COMMAND.COM

C.	troubleshooting the workstation connection

1.	basics

a.	verify proper seating of the NIC & cable connections

b.	check configuration of NIC & confirm with IPX.COM or in NET.CFG

c.	check for conflicts with board resources

d.	make certain the LAN driver is bound to IPX or IPXODI

e.	ODI drivers can be unloaded by using a command line switch (u)

2.	using TRACK to see a workstation log on

a.	set TRACK ON at the server console

b.	log on

c.	the workstation should issue:

i.	get nearest server

ii.	give nearest server server_name

iii.	route request

D.	DOS Commands

1.	MODE - used to configure serial ports for printer use

2.	SETVER - MS-DOS command

a.	used to report various version numbers back to software

b.	output would be a list of files (EXE, COM, DLL, etc) & the version number to report

3.	LH/LOADHIGH - MS-DOS command to load a driver into upper memory

4.	HILOAD - DR-DOS command to load a driver into upper memory

5.	SETUP - DR-DOS command which gives user a full screen utility to configure DOS

6.	SMARTDRV - MS-DOS command to create a local disk cache

7.	RAMDRIVE.SYS

a.	loaded in CONFIG.SYS to create a virtual or RAM drive

b.	if your disk caching software fails to load after creating a virtual drive, reduce the size of the drive

8.	DiskMax - DR-DOS - compression, optimization, caching, and undeleting

9.	DOSBook - DR-DOS - online DOS help

10.	TaskMax - DR-DOS - task switching utility

11.	HIDOS.SYS - DR-DOS - extended memory manager

V.	Network Printing

A.	NetWare print utilities

1.	CAPTURE	 - Redirects workstation output to a network printer or file (OS/2 compatible).

2.	NETUSER

a.	captures workstation output to a network printer

b.	prints files from outside of an application

c.	monitors print queues

d.	sends, modifies, or deletes print jobs.

3.	NPRINT - Prints files from outside of an application (OS/2 compatible).

4.	NPRINTER - Loads NetWare printer driver (NLM on the NetWare server, EXE on the workstation).

5.	PCONSOLE

a.	creates, modifies, deletes, and monitors print queues, print servers, and printers

b.	sends, modifies, pauses, and deletes print jobs. Quickly installs basic print services

6.	PRINTCON

a.	creates, modifies, and deletes print job configurations

b.	simplifies using CAPTURE and NPRINT commands.

7.	PRINTDEF

a.	defines printer forms for use in CAPTURE, NPRINT, and print job configurations

b.	defines print devices in a database for use in print job configurations

8.	PSC - Monitors and controls printer and print server status from the command line (OS/2 compatible).

9.	PSERVER - Loads print server software (NLM).

10.	PUPGRADE - Upgrades NetWare v3.11 printing objects, print job	configurations, and printer definitions (NLM on the NetWare server).

11.	RPRINTER

a.	you should use a batch file to load RPRINTER, so errors cause an automatic retry

b.	if RPRINTER reports the printer is already in use after a system reboot, wait until the SPX session times out

B.	creation/configuration

1.	create print queue

2.	create print server

3.	configure printer

4.	assign queue

5.	run PSERVER

C.	print job order

1.	application sends data to port, CAPTURE picks it up

2.	when the application is finished, CAPTURE sends the data to the print queue

3.	print server takes jobs from the top of the queue and sends them to the port driver

4.	port driver (NPRINTER) sends the job to the printer

5.	printer prints the job

D.	printer connections

1.	serial

a.	settings

i.	baud rate

Ÿ	1200 to 57600

Ÿ	speed at which data is transmitted to the printer

ii.	parity

Ÿ	whether or not extra bits are added to the information to check for errors

Ÿ	even, odd, mark, space, or none

iii.	data bits - 7 or 8 data bits can be transmitted in each character, usually 7

iv.	stop bits

Ÿ	how many bits are used to represent the end of a character

Ÿ	ranges from 0 to 2

v.	flow control

Ÿ	software - XON/XOFF

Ÿ	hardware - DTR/RTS

Ÿ	none

b.	cabling - max length is 50ft, although longer ones may work (up to 500ft)

2.	parallel

a.	faster than serial

b.	cabling - max length 15ft officially, although longer may work (20ft)

c.	can be extended using special parallel cable extenders

E.	troubleshooting

1.	process

a.	run the printer’s self test

b.	if the self test fails, turn the printer off & on, then rerun the self test

c.	if the self test fails again, the problem is with the printer

d.	if the self test works, print a screen

e.	if the screen print doesn’t work, the problem is with the printer or cable

f.	if the screen print does work, use PCONSOLE to make certain jobs are entering the queue

g.	if jobs are entering the queue, the problem is with the print server or the printer

h.	if jobs are not entering the queue, the problem is with the capture; use CAPTURE /SH to find out where the jobs are printing

2.	general troubleshooting

a.	check the paper path

b.	clean the printer

c.	check the environment - high heat, humidity, or cold can cause problems

d.	don’t lubricate unless the owner’s manual tells you to

3.	dot matrix printers

a.	general

i.	make certain the printer is not covered and is well ventilated to disperse the heat generated by the print head

ii.	use high quality ribbons

iii.	don’t touch the platen knob while the printer is on

iv.	be certain you are using the right paper

v.	make certain the paper feeds easily

b.	troubleshooting

i.	check the dip switches

Ÿ	emulation setting

Ÿ	carriage return/line feed setting

ii.	check for correct driver

iii.	make certain the MODE of the serial port is set correctly

iv.	poor quality

Ÿ	replace the ribbon

Ÿ	adjust print head to platen distance

Ÿ	clean the print head

Ÿ	replace the print head

v.	printing stops in the middle of a job

Ÿ	check for jammed paper/ribbon

Ÿ	check for overheating

4.	ink jet

a.	general

i.	replace cartridges - do not try to prolong their life

ii.	avoid refilling cartridges

iii.	make certain the printer is level

iv.	use high quality paper

b.	troubleshooting

i.	print fades/starts & stops

Ÿ	replace the ink cartridge

Ÿ	level the printer

Ÿ	clean the printer

ii.	printer stops

Ÿ	replace the ink cartridge

Ÿ	check for paper jams

5.	laser

a.	general

i.	clean the inside of the printer when replacing the toner

ii.	be careful around the corona wires

iii.	replace the fuser bar regularly

iv.	keep in a well ventilated area

v.	do not refill a cartridge more than once

vi.	use the correct paper, envelopes, and other supplies

b.	troubleshooting

i.	streaks or fading - replace the toner

ii.	clean the corona

iii.	if regular spots, check for a damaged drum

iv.	if output is black, check for broken corona wire or bad cartridge

v.	shake toner

vi.	if the output is stretched of compressed, the motor may be bad

vii.	if the toner flakes off the page, replace the fuser

6.	PostScript

a.	general

i.	make certain you are using the correct drivers

ii.	make certain the printer is in PostScript mode

iii.	use /NFF, /NB, and /NT when capturing

7.	print queues

a.	jobs not entering the queue

i.	check CAPTURE

ii.	make certain application is sending data to the correct place

iii.	try setting the autoendcap (/EC)

iv.	check disk space on queue’s drive

b.	jobs not leaving the queue

i.	check the printer

ii.	make certain PSERVER.NLM is running

iii.	make certain the port driver is running

8.	print servers and printer objects

a.	make certain PSERVER.NLM is running on the server

b.	be certain the print server is running from PCONSOLE

c.	down & up the print server using PCONSOLE

d.	make certain the Print Server object is created and configured correctly

e.	make certain users have rights to the Print Server object

f.	check server RAM

g.	make certain the correct queues are being serviced

h.	make certain the printer does not need a real-time dialog with the computer, such as plotters

i.	if behavior is inconsistent, try running DSREPAIR

9.	PSERVER.EXE

a.	NET.CFG on workstation must have SPX CONNECTIONS = 60

VI.	Optimization & Disaster Recovery

A.	Basics

1.	establish a dependable backup

a.	check tape logs for confirmation of backups

b.	establish a good tape rotation plan

c.	back up the NDS database as well as the data

2.	recovering data

a.	VREPAIR

i.	may recover data if problem is not hardware related

ii.	should not be used for major disk problems

iii.	may be useful for

Ÿ	power failure data corruptions

Ÿ	mirroring errors

Ÿ	volume won’t mount

iv.	volume must be dismounted when VREPAIR is run

b.	professional data recovery services

c.	third party utilities

i.	NETUTILS3 - collection of utilities for NetWare 3.x

ii.	Check-It Pro

iii.	On Track Data Recovery

iv.	STAC

3.	patches

a.	basics

i.	the PATCHES command can be used from the console of a file server to check the current revision level of all patches applied to the server

ii.	all patches should be stored in the SYS:SYSTEM directory

iii.	PATCHMAN

Ÿ	should be located in the SYS:SYSTEM directory

Ÿ	should be loaded early in the AUTOEXEC.NCF file, so patches stored in the SYS:SYSTEM directory are automatically loaded when the server is booted

b.	types

i.	static

Ÿ	simple

Ÿ	include a DOS program for modifying SERVER.EXE

ii.	semi-static

Ÿ	loaded with PATCHMAN

Ÿ	server must be down during load

iii.	dynamic

Ÿ	can be loaded and unloaded using PATCHMAN

4.	MONITOR

a.	activating processor utilization display is done by adding -p to the command line

b.	CRC/Alignment Error (?)

c.	main screen

i.	server up time

ii.	utilization

Ÿ	can be misleading

Ÿ	if this is consistently 60 to 100%, you should consider server upgrades

Ÿ	generally because of not enough server memory

iii.	original cache buffers

iv.	total cache buffers

v.	dirty cache buffers

Ÿ	if dirty cache buffers consistently stays near the total cache buffers, you should start checking the server for problems or optimizing it

5.	Packet Burst Protocol

a.	allows up to 64K of outstanding packets before acknowledgment

b.	bad packets are sent after initial burst individually

c.	automatically enabled on server

d.	automatically enabled on VLM clients

6.	Large Internet Packet

a.	allows NetWare to use larger packets to communicate more efficiently

b.	automatically enabled at both the server and VLM workstations

B.	SET parameters

1.	communications

a.	Console Display Watchdog Timeouts

i.	if on, a message is displayed at the console each time a connection is cleared

ii.	default OFF

b.	New Packet Receive Buffer Wait Time

i.	determines how long system will wait after receiving a request for another packet receive buffer before allocating one

ii.	default .1 seconds, range .1 to 20 seconds

c.	Maximum Physical Receive Packet

i.	sets maximum size of packets the system can receive

ii.	default 4202, range 618 to 24,682

iii.	configured in STARTUP.NCF

d.	Maximum Packet Receive Buffers

i.	default 100

ii.	range 50 to 4,000

iii.	baseline configuration

Ÿ	use MONITOR to determine the number of packet receive buffers & service processes your server is using

Ÿ	increase until each EISA or Microchannel NIC has 5, or until each workstation has 10

e.	Minimum Packet Receive Buffers

i.	default 50, range 10 to 2,000

f.	Number of Watchdog Packets

i.	number of watchdog packets a server will send without a reply before disconnecting the workstation

ii.	default 10, range 5 to 100

g.	Delay Between Watchdog Packets

i.	determines the amount of time between sending watchdog packets

ii.	default 59.3 seconds, range 1 to 626.2 seconds

h.	Delay Before First Watchdog Packet

i.	amount of time the server will wait after receiving a packet from the workstation before sending the first watchdog packet

ii.	default 240 seconds, range 15.7 to 172.3 seconds

i.	Maximum Service Processes

i.	default 40, range 5 to 400

2.	memory parameters

a.	Cache Buffer Size

i.	should be set to the same size as the disk block

ii.	default 4096 bytes, range 4096, 8192, or 16384 bytes

b.	Maximum Alloc Short-Term Memory

i.	sets the maximum amount of memory for drive mappings, loadable module tables, queued broadcast messages, open & locked files, service request buffers, & user connection information

ii.	default 2,097,152, range 50,000 to 16,777,216

c.	Auto Register Memory above 16MB

i.	default ON

ii.	some EISA boards with 24 bit address lines will cause only 16MB of memory to be useable

d.	Garbage Collection Interval

i.	how often garbage collection is done

ii.	default 15, range 1 to 60 minutes

e.	Number of Frees for Garbage Collection

i.	performs garbage collection when this number of calls to free memory have been executed

ii.	default 5,000, range 100 to 100,000

f.	Minimum Free for Garbage Collection

i.	number of bytes which must be in system pool for successful garbage collection

ii.	default 8,000; range 1,000 to 1,000,000

g.	Allow Invalid Pointers

i.	allows an application to define a pointer to an undefined block of memory

ii.	default OFF, if ON displays message at console rather than ABENDing

h.	Read Fault Emulation

i.	allows NetWare to handle a memory read fault without ABENDing

ii.	default is OFF, if ON displays a message at the console

i.	Read Fault Notification

i.	default is ON, console displays a message for each read fault

j.	Write Fault Emulation

k.	Write Fault Notification

l.	Alloc Memory Check Flag

i.	allows server to constantly check for memory corruption

ii.	default OFF

m.	Reserved Buffers Below 16MB

i.	default 16, range 8 to 300

3.	file caching parameters

a.	Maximum Concurrent Disk Cache Writes

i.	default 50, range 10 to 4,000

ii.	increasing this increases write performance

iii.	decreasing this increases read performance

iv.	if dirty cache buffers is over 70%, increase this

b.	Dirty Disk Cache Delay Time

i.	amount of time NetWare holds a write request before writing the information to the disk

ii.	default 3.3, range .1 to 10 seconds

c.	Minimum File Cache Report Threshold

i.	sets the level at which the server sends a warning about cache buffers

ii.	default 20, range 0 to 1000

d.	Minimum File Cache Buffers

i.	sets the minimum number of cache buffers NetWare will allocate

ii.	default 20, range 20 to 100

e.	Read Ahead Enabled

i.	controls server read ahead

ii.	default ON

f.	Read Ahead LRU Sitting Time Threshold

i.	amount of time read ahead area must be unused before overwriting

ii.	default 10 seconds, range 0 seconds to 1 hour

4.	disk compression

a.	Compression Daily Check Stop Hour

i.	default 6 (6AM)

ii.	military time

b.	Compression Daily Check Starting Hour

i.	default 0 (12AM)

ii.	military time

c.	Minimum Compression Percentage Gain

d.	Enable File Compression

i.	default ON

e.	Maximum Concurrent Compressions

i.	default 2

f.	Convert Compressed to Uncompressed Option

i.	0 - keep file compressed

ii.	1 - keep file compressed the first time only

iii.	2 - leave file uncompressed

iv.	default 1

g.	Uncompress Percent Disk Space Free to Allow Commit

i.	default 10%, range from 0 to 100

h.	Uncompress Free Space Warning Interval

i.	set in minutes; 0 to disable

i.	Deleted File Compression Option

i.	controls compression on deleted files

ii.	0 - never compresses deleted files

iii.	1 - compresses one day after deletion

iv.	2 - compresses immediately after deletion

j.	Days Untouched Before Compression

i.	default 7

5.	directory caching parameters

a.	Dirty Directory Delay Time

i.	default .5, range 0 to 10 seconds

ii.	amount of time NetWare will wait between a directory write request & writing the information to disk

b.	Maximum Concurrent Directory Cache Writes

i.	default 10, range 5 to 50

ii.	increasing this increases write performance

iii.	decreasing this increases read performance

c.	Directory Cache Allocation Wait Time

i.	how long NetWare waits between allocating allocation buffers

ii.	default 2.2, range .5 to 120 seconds

iii.	if DIR or NDIRs run slow, try reducing this

d.	Directory Cache Buffer Nonreferenced Delay

i.	amount of time NetWare waits before overwriting an unused directory cache buffer

ii.	default 5.5, range 1 to 120 seconds

iii.	increasing this increases allocated directory cache buffers & increases performance

e.	Maximum Directory Cache Buffers

i.	determines how large the directory cache can be

ii.	default 500, range 20 to 4000

iii.	permanent until the server is rebooted

f.	Minimum Directory Cache Buffers

i.	determines the smallest possible size of the directory cache

ii.	default 20m range 20 to 4000

6.	file system parameters

a.	Maximum Extended Attributes per File or Path

i.	default 32, range 4 to 512

ii.	applies to all volumes on the server

b.	Immediate Purge of Deleted Files

i.	default OFF

c.	Maximum Subdirectory Tree Depth

i.	default 25, range 10 to 100

d.	Volume Low Warn All Users

i.	default ON

e.	Volume Low Warning Reset Threshold

i.	determines the amount of space which must be free to stop sending volume low warning messages once they have begun

ii.	default 256, range 0 to 100,000

f.	Volume Low Warning Threshold

i.	default 256, range 0 to 100,000

g.	Turbo FAT Re-Use Wait Time

i.	how long after a file is closed to wait before destroying a turbo FAT

ii.	default 329.6 seconds, range .3 seconds to 1 hour 5 minutes 54.6 seconds

h.	Minimum File Delete Wait Time

i.	amount of time before a deleted file can be purged

ii.	default 65.9 seconds, range 0 to 7 days

i.	File Delete Wait Time

i.	amount of time to wait before a file is marked as purgeable

ii.	default 329.6 seconds, range 0 to 7 days

j.	NCP File Commit

i.	allows files to flush all pending writes to disk

ii.	default is ON

k.	Maximum Amount of Volume Used By Directory

i.	how much of the volume space may be used by directory entries

ii.	default 13, range 5 to 50

l.	Maximum Amount of Volume Space Allowed for Extended Attributes

i.	only addressed when volume is being mounted

ii.	default 10, range 5 to 50

7.	lock parameters

a.	Maximum Record Locks per Connection

i.	how many records can be locked for each connection

ii.	default 500, range 10 to 10,000

b.	Maximum File Locks per Connection

i.	default 250, range 10 to 1,000

c.	Maximum Record Locks

i.	maximum number of record locks for the entire server

ii.	default 20,000, range 100 to 200,000

d.	Maximum File Locks

i.	default 10,000, range 100 to 100,000

8.	transaction tracking parameters

a.	Auto TTS Backout Flag

i.	On allows the system to back out of any incomplete transactions without asking

ii.	default OFF

b.	TTS Abort Dump Flag

i.	if On, creates a log file which records transactions which have been backed out of

ii.	default OFF

c.	Maximum Transactions

i.	default 10,000, range 100 to 10,000

d.	TTS Underwritten Cache Wait Time

i.	maximum amount of time a TTS block may wait in memory before being written to disk

ii.	default 65.9, range 11 to 659.1 seconds

e.	TTS Backout File Truncation Wait Time

i.	how long allocated blocks are available to the TTS backout file

ii.	default 59 minutes 19.2 seconds, range 65.9 seconds to 1 day 2 hours 21 minutes 51.3 seconds

9.	miscellaneous parameters

a.	Enable Disk Read After Write Verify

i.	default ON

b.	Maximum Outstanding NCP Searches

i.	maximum number of simultaneous NetWare Core Protocol directory searches

ii.	default 51, range 10 to 1,000

c.	Allow Unencrypted Passwords

i.	default OFF

d.	New Service Process Wait Time

i.	how long NetWare waits after receiving a request for a new service before it actually allocates one

ii.	default 2.2, range .3 to 20 seconds

e.	Pseudo Preemption Time

i.	default 2,000, range 1,000 to 10,000

f.	Display Spurious Interrupt Alerts

i.	default ON

g.	Display Lost Interrupt Alerts

i.	default ON

h.	Display Disk Device Alerts

i.	default ON

i.	Display Relinquish Control Alerts

i.	if ON, an alarm is shown on the system console if an NLM holds the processor for more than .4 seconds

ii.	default OFF

j.	Display Old API Names

i.	shows if any old API calls are being used

ii.	default OFF

k.	Maximum Service Processes

i.	default 20, range 5 to 40

C.	Performance Considerations

1.	name spaces

a.	try to use only one name space on each volume

b.	if multiple name spaces are used on one volume, increase the Minimum Directory Cache Buffers to mount the volume more rapidly

c.	install all name spaces on the volume at initial installation to promote efficient use of the Disk Directory Entry Table Blocks

2.	Turbo FAT

3.	Burst Mode

D.	internetworking devices

1.	repeater

a.	physical layer

b.	expense very low, speed very high, functionality very low

2.	bridge

a.	data link layer

b.	expense low, speed high, functionality low

3.	router

a.	network layer

b.	expense medium, speed medium, functionality medium

4.	gateway

a.	all upper layers

b.	expense high, speed low, functionality high

E.	Odds & Ends

1.	the correct order to load & bind LAN drivers in is:

a.	LOAD NE2000 port=300 int=3 frame=ETHERNET_802.3 NAME=ETH3

b.	BIND ETH3 to IPX net=CA9BC11

2.	having more memory than needed on a NetWare file server is recommended; this guards against memory fragmentation

3.	you should divide a network segment when utilization reaches 70% of max