STANDARD MICROSYSTEMS CORPORATION

SMC USL svr4.0.T, svr4.2, UNIXWARE 1.1 SuperDisk

For SMC Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Available drivers.

o  Procedures for preparing a "pkgadd" diskette.

o  Procedures for installing the driver with a "pkgadd" diskette.

o  Special Notes PCI Configuration Space Access Mechanism.

AVAILABLE DRIVERS

The USL superdisk, smc.usl, is designed for the following SMC adapters:

sme drivers for SMC 80xx/8216/8416 Ethernet ISA adapters,

sebm drivers for SMC 82M32 Ethernet EISA adapter, and

smpw drivers for SMC 8432 EtherPower PCI adapter.

The smc.usl superdisk includes:

o  nine sets of drivers:

sme42L -- sme driver for USL svr4.2 (Lachman)

sme42W -- sme driver for USL svr4.2 (WIN TCP/IP)

smeUW11 - sme driver for Unixware 1.1

sebm42L -- sebm driver for USL svr4.2 (Lachman)

sebm42W -- sebm driver for USL svr4.2 (WIN TCP/IP)

sebmUW11 - sebm driver for Unixware 1.1

smpw42L -- smpw driver for USL svr4.2 (Lachman)

smpw42W -- smpw driver for USL svr4.2 (WIN TCP/IP)

smpwUW11 - smpw driver for Unixware 1.1

o  ezsetup ---- a utility to configure the setup of SMC 80xx/8216/8416

Ethernet ISA adapters.

(check install.ezs for more information)

o  smcmkflop -- a utility to convert a tar file into an installable

file system for "pkgadd".

PREPARING A "pkgadd" DISKETTE

The following procedures describes how to prepare a "pkgadd" diskette.

In this procedure, variables are denoted by upper-case characters. In

addition, <CR> is used to denote pressing the Enter (or carriage

return) key.

1. Log in as root and enter single user mode.

2. Change to the /tmp subdirectory by entering the following command:

cd /tmp <CR>

3. Insert the DOS diskette that contains DRIVER.usl (for example,

smc.usl) into the floppy drive. Then use the following doscp

command to copy the DRIVER.usl file to the /tmp subdirectory:

doscp X:DRIVER.usl DRIVER.usl <CR>

In the above command: X is floppy drive A or B.

DRIVER is the driver name, such as smc.

Example: doscp a:smc.usl smc.usl <CR>

4. Extract the tar file, DRIVER.usl, using the following command:

tar xvf DRIVER.usl <CR>

In the above command: DRIVER is the driver name, such as smc.

Example: tar xvf smc.usl <CR>

5. Run "smcmkflop" to create the driver-installation disk using the

following command:

./smcmklop DRIVER <CR>

In the above command: DRIVER is the driver name, such as smc.

Example: ./smcmkflop smc <CR>

INSTALLING THE DRIVER WITH A "pkgadd" DISKETTE

To install the SMC driver using a pkgadd diskette:

1. Type the following command to start the pkgadd process:

pkgadd -d diskette1 <CR>

2. Follow pkgadd's on-screen instructions.

3. Run "/etc/confnet.d/configure -i" and follow the instructions.

4. Restart the computer, UNIXWARE will rebuild the kernal upon boot up

to make the changes effective.

SPECIAL NOTES FOR PCI CONFIGURATION SPACE ACCESS MECHANISM

User can configure how the driver access the PCI Configuration Space.

This is done by modifying the initialized value of the variable,

smpw_confm, in /etc/conf/pack.d/smpw0/space.c file. The possible

values are :

SMPW_CM_AUTO, SMPW_CM_MECH1, SMPW_CM_MECH2 and SMPW_CM_BIOS32.

where SMPW_CM_AUTO --- autodetermine between Mechanism 1

and Mechanism 2.

SMPW_CM_MECH1 -- Mechanism 1.

SMPW_CM_MECH2 -- Mechanism 2.

SMPW_CM_BIOS32 - use 32 bit BIOS services.

(The 32 bit BIOS services must be

coded as the PCI BIOS specification

ver 2.1 specifies.)

The default value is SMPW_CM_AUTO.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite32 Ultra, Elite Series, EliteView, EZStart,

SuperDisk, and UltraChip are trademarks of Standard Microsystems

Corporation.  Other products and company names are registered

trademarks or trademarks of their respective companies.