STANDARD MICROSYSTEMS CORPORATION

NDIS2 Driver

For EtherPower, EtherPower^2, and EtherPower 10/100

PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  An overview of the NDIS2 driver.

o  Preinstallation hardware and software requirements

o  Supplied files.

o  Adapter installation notes.

o  Basic driver installation (for any Network Operating System).

o  Installing multiple adapters.

o  PROTOCOL.INI parameters.

o  Error messages.

NOTE: Refer also to the installation document for the particular

network operating system for which you are installing the

NDIS2 driver. For example, if you are installing the DOS NDIS2

driver for DEC PATHWORKS support, refer to the DEC41DOS.DOC

installation document located on the SuperDisk at \NDIS2\DEC_PATH

DUAL CHANNEL SUPPORT (EtherPower^2 Only)

The EtherPower^2 Dual Channel PCI Ethernet adapter conforms to the

PCI-to-PCI Bridge Architecture Specification, Revision 1.0, April 5,

1994. Networking environments that support dual channel operation

include Novell server (32-bit ODI driver), Windows NT, and SCO. Other

networking environmnets do not support the unique dual channel capability

of the EtherPower^2 adapter. These environments include Novell client

(DOS ODI driver), Windows for Workgroups v3.11, and networking

environments that use DOS or OS/2 NDIS2 drivers.

IMPORTANT PROTOCOL.INI PARAMETERS

Two important PROTOCOL.INI parameters are SIA_MODE, which

defines the serial interface connection (including full-duplex

or half-duplex operation), and SLOT, which specifies the slot

number location of the adapter. Refer to the section in this

document titled PROTOCOL.INI PARAMETERS for more details.

ETHERPOWER 10/100 FAST ETHERNET USERS ONLY:

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. The EtherPower 10/100 adapter is default configured

to operate at 100 Mbps.

Configuration information is stored in the PROTOCOL.INI file.

It is important to set the adapter at the correct line speed

(10/100 Mbps) using the PROTOCOL.INI parameter DATA_RATE.

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

OVERVIEW

This driver conforms to the Network Driver Interface Specification

(NDIS) v2.0.1 defined by Microsoft/3Com. The driver has been

tested with Microsoft LAN Manager, 3Com 3+Open, IBM LAN Server,

IBM PC LAN Program, DEC Pathworks, and other NDIS-compatible network

operating systems and protocol stacks.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

- An EtherPower, EtherPower^2, or EtherPower 10/100

PCI adapter for that bus.

Software Requirements:

- MS-DOS 5.0 or later with one of the following:

o Microsoft LAN Manager 1.3 or 2.x

o Microsoft Windows for Workgroups v3.1

o PATHWORKS for DOS v4.1

OR

- OS/2 V1.3 or V2.x with Lan Manager

(the driver should work with Lan Server, but no

installation support is provided in this release;

the driver has not yet been tested with Lan Server)

SUPPLIED FILES

General files

o  RELEASE.DOC      Release notes for this version of the driver.

o  NDIS.DOC         This file.

Files to install the driver on Pathworks and Windows for Workgroups

o  SMCPWR.DOS       NDIS2 DOS driver

o  PROTOCOL.INI     Includes driver binding information and

operational parameters.

o  OEMSETUP.INF     Network information file (Windows for Workgroups)

Files to install the driver on LAN Manager under DOS

o  SMCPWR.NIF       Network information file for MS LAN Manager

o  SMCPWR.DOS       NDIS2 DOS driver

o  PROTOCOL.INI     Includes driver binding information and

operational parameters.

Files to install the driver on LAN Manager under OS/2

o  SMCPWR.NIF       Network information file for MS LAN Manager

o  SMCPWR.OS2       NDIS2 OS/2 driver

o  PROTOCOL.INI     Includes driver binding information and

operational parameters.

ADAPTER INSTALLATION NOTES

Refer to the user guide for complete adapter installation instructions.

When installing an SMC PCI adapter in a PCI machine, make sure the

slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using

the appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's setup menu and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

BASIC DRIVER INSTALLATION

Various network operating systems (Lan Manager, Windows for

Workgroups, Pathworks) provide utility programs for installing

drivers.

The installation diskette provides support for these utility

programs, FOR THE FIRST BOARD INSTALLED ONLY.  If multiple boards

are installed, the second and subsequent boards must be installed

manually. (See 'Installing Multiple Boards').

Basic driver installation (for any Network Operating System):

1. Add the following line to CONFIG.SYS:

DEVICE=<drive>:[pathname]\SMCPWR.DOS (or SMCPWR.OS2)

2. If the driver is installed in DOS HIGH memory use the following

line:

DEVICEHIGH=<drive>:[pathname]\SMCPWR.DOS

Adapter configuration information is displayed when the system

is rebooted, including SLOT, IRQ, SIA_MODE, and DATA_RATE.

Where:

- SLOT is the PCI slot location of the adapter.

- IRQ is the interrupt request level.

- SIA_Mode is the serial interface mode.

- DATA_RATE is the ethernet speed: 100 Mb or 10 Mb.

INSTALLING MULTIPLE ADAPTERS

To install more than one board, follow the manual installation

procedures for your operating environment, adding '2', '3' etc.

to the device driver's name in PROTOCOL.INI (so that each driver

name and section is unique). In other words, the drivers are

SMCPWR$, SMCPWR2$, SMCPWR3$, etc.

SLOT must be specified for multiple boards.

For example, here is the PROTOCOL.INI for two adapters installed

under Windows for Workgroups, using Twisted Pair:

[network.setup]

version=0x3100

netcard=SMCPWR,1,SMCPWR

netcard=SMCPWR2,1,SMCPWR2

transport=ms$netbeui,MS$NETBEUI

lana0=SMCPWR,1,ms$netbeui

lana1=SMCPWR2,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=SMCPWR,SMCPWR2

LANABASE=0

[SMCPWR]

DriverName=SMCPWR$

SLOT=16

SIA_Mode=AUTODETECT

DATA_RATE=MB100

[SMCPWR2]

DriverName=SMCPWR2$

SLOT=17

SIA_Mode=AUTODETECT

DATA_RATE=MB10

PROTOCOL.INI PARAMETERS

The PROTOCOL.INI file stores configuration and binding information.

It consists of a series of named sections, where the section name is

in fact the module name from a module characteristics table.

The following sections describe the PROTOCOL.INI settings used for

the SMCPWR driver, their default values, the range of values that

can be given, and value units.  A stub version of this file, listing

the supported parameters, can be found in the root directory of the

installation diskette.

1.  Required Keyword

DRIVERNAME -- specifies the network device driver.

DRIVERNAME=SMCPWRn$ , where n is nothing for the first board,

2 for the second, etc.

units:[text]

range:[SMCPWR$ - SMCPWR8$]

default:[SMCPWR$]

2.  Optional Keywords in the SMCPWR section:

a. SLOT -- specifies where the SMCPWR adapter is placed.

o A number from 1 to 15 specifies a slot number on the

EISA bus

o A number from 16 to 26 specifies the relative position

of an adapter on the PCI bus. 16 specifies the first

adapter on the PCI bus, 17 the second such adapter, and

so on.

If the slot value is not specified or is specified as 0, the

driver scans for an EISA adapter; if no EISA adapter is found,

it then scans for a PCI adapter.

Each PCI adapter installed in your PC is assigned a logical slot

number based on the number of PCI devices installed. For example,

if there is only one PCI device installed in your computer, it is

assigned PCI slot number 16. If your PC already contains a single

PCI device and you then install another PCI adapter, the PCI

device already installed is assigned PCI slot number 16 and the

newly-installed adapter is assigned PCI slot number 17. Subsequently

installed PCI devices will be assigned slot numbers beginning from

18.

Slot

units:[integer]

range:[1-26]

default:[16]

b. SIA_MODE  -- Defines the Serial Interface connection.

o AUTODETECT executes an algorithm which detects automatically

the serial interface connection type.

o AUI selects the AUI connector for external transceiver

Some boards may also require a jumper on the board to

be set to distinguish between AUI and BNC.

o BNC selects the BNC (thin wire) connector for internal

transceiver.

o TP selects the RJ-45 connector for the Twisted-Pair

on-board transceiver for connection to a Twisted Pair hub.

o TP_FULL_DUPLEX specifies Full Duplex operation via the

Twisted-Pair transceiver. The adapter should be connected

to another Ethernet station with a cable suitable for Full

Duplex mode.

Sia_Mode

units:[text]

range:[BNC,AUI,TP,TP_FULL_DUPLEX,AUTODETECT]

default:[AUTODETECT]

c. DATA_RATE  -- Defines the data rate at which the LAN controller

works.

o MB10 specifies a data rate of 10 Mbps (Suitable for standard

Ethernet, EtherPower adapters - 8432 board types).

o MB100 specifies a data rate of 100 Mbps (suitable for fast

Ethernet, EtherPower 10/100 adapters  - 9332 board types).

The keyword is valid only if the physical 100 Mbps port is

connected to the network.

NOTE: The default is MB10 (10 Mbps) for the EtherPower adapter

and MB100 (100 Mbps) for the EtherPower 10/100 adapter.

d. SOFTWARE_CRC -- controls whether packet CRCs are calculated by

software or appended by SMCPWR

o ENABLED means that the CRC will always be calculated by

software, for packets greater than 144 bytes.

o DISABLED means that the CRC will never be calculated by

software (always appended by SMCPWR).

o <absent> If this keyword does not appear in PROTOCOL.INI,

packet CRCs will be calculated by software for chip

revisions A-H (for packets longer than 144 bytes), and

will be appended by the SMCPWR for chip revision J and later.

SOFTWARE_CRC

units:[text]

range:[ENABLED, DISABLED, <absent>]

default:[<absent>]

ERROR MESSAGES

o Could not locate adapter

No EtherPower-series adapter present in system.

o Could not locate adapter at specified SLOT

The adapter was not found in the slot specified in the PROTOCOL.INI

file.

o Driver instance limit exceeded

Too many EtherPower-series adapters are in the system.

o Could not open Protocol Manager

The protocol manager, which coordinates different networking

software, could not be opened and therefore the driver could not be

initialized.

o Could not register driver with Protocol Manager

The driver could not be registered with the protocol manager.

Probably due to an initialization problem.

o Invalid keyword in PROTOCOL.INI

One of the entries in the PROTOCOL.INI file in the EtherPower

section is invalid.

o PROTOCOL.INI keyword has too many parameters

One of the entries in the EtherPower-series section of PROTOCOL.INI

has too many parameters.

o PROTOCOL.INI parameter too low, ignored

One of the entries in the EtherPower-series section of PROTOCOL.INI

has a value too low.

o PROTOCOL.INI parameter too high, ignored

One of the entries in the EtherPower-series section of PROTOCOL.INI

has a value too high.

o PROTOCOL.INI string parameter too long, ignored

One of the entries in the EtherPower-series section of PROTOCOL.INI

has a string value with too many characters.

o Invalid PROTOCOL.INI string parameter, ignored

One of the entries in the EtherPower-series section of PROTOCOL.INI

has a string value which is invalid.

o PCI BIOS failed to enable I/O operation

The adapter cannot access the machine's I/O space and is

therefore not operational.

o PCI BIOS configured I/O base illegal for Intel platform

The adapter cannot access the machine's I/O space due to wrong

address mapping and is therefore not operational.

o PCI BIOS configured IRQ illegal for Intel platform

The adapter interrupt pin is hooked to an illegal interrupt line

in the machine and therefore not operational.

o Could not allocate GDTs

The OS/2 driver was unable to allocate descriptor tables and is

therefore not operational.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of Standard Microsystems Corporation. Other product and company names

are registered trademarks or trademarks of their respective holders.