SROUTE.DOC

This SROUTE.DOC file accompanies SROUTE.ZIP containing IBM Token-Ring Source

Routing drivers for NetWare V2.x and NetWare V3.x.  The following files are

included in this revision:

SROUTE   DOC     (this file)

SRCONFIG COM     3086  10-10-90   8:51a    (certified ships w/v2.2/v3.11)

ROUTE    COM     4450   5-01-91   8:28a    (certified by Software Test)

ROUTE    NLM     4584   5-01-91   8:14a    (certified by Software Test)

ROUTE    SYS    33200   6-25-91   9:53a    (certified by Software Test)

ROUTE    VP0     3896   5-01-91   1:53p    (certified by Software Test)

TEMPRT   ZIP                               (temporary - not certified)

The above files are the latest source routing drivers and they address all

known problems.  NetWare v2.2 and v3.11 manuals now contain instructions on

how to implement these drivers.  SROUTE.DOC also documents how to implement

these drivers plus any new features that are added to the drivers.

**************************************************************************

GENERAL INFORMATION -----

**************************************************************************

ROUTE.SYS - This ROUTE.SYS fixes a problem with the direction bit being

set incorrectly, causing incorrectly source routed packets.  ROUTE.SYS

enables source routing for two OS/2 v1.3 drivers:

TOKEN.SYS and CMGRLAN.SYS.  ROUTE.SYS should appear in the OS/2

config.sys IMMEDIATELY FOLLOWING either TOKEN.SYS or CMGRLAN.SYS.  To

implement ROUTE.SYS, refer to the OS/2 Requester v1.3 documentation to

implement this driver.

ROUTE.COM - This is the Source Routing Driver for the DOS Workstation.

It can be used with EITHER the DOSODI LANSUP.COM or TOKEN.COM, or with

IPX.COM.  (IPX.COM can be either IPX.OBJ linked with SLANSUP.OBJ, or IPX.OBJ

linked with STOKEN.OBJ) It should be loaded AFTER IPX and BEFORE the SHELL.

Type ROUTE ? for a HELP Panel.  To implement ROUTE.COM, refer to the section

below titled: INSTALLING ROUTE.COM.

ROUTE.NLM - This is the Source Routing Driver for NetWare 386.  It fixes

ABEND: GPPE when loading route a second time to over-ride previous route

settings as shown: LOAD ROUTE RSP=AR.  It should be loaded AFTER TOKEN.LAN.

To implement ROUTE.NLM, refer to the section below titled:  INSTALLING

ROUTE.NLM.

ROUTE.VP0 - This is the Source Routing VAP for the NetWare 286 OS.  It

needs to be put on the SYS:SYSTEM directory of the OS.  After it is loaded

on the OS, type ROUTE ? for a description of its parameters. It is named

ROUTE.VP0 because it should be the FIRST VAP that is loaded.  To implement

ROUTE.VP0, refer to section below titled: INSTALLING ROUTE.VP0.

TEMPRT.ZIP - This zip file is to be used to facilitate the upgrade

process when upgrading Token-Ring drivers from pre-TRN050.zip drivers to the

latest Token-Ring drivers.  For instructions to implement these drivers,

unzip this file and refer to TEMPRT.DOC.  Old source routing drivers were

contained in TRNSRB.ARC which has been removed from NetWire.

SRCONFIG.COM - This is the Configuration utility for ROUTE.vp0.  It is

menu driven.  Just type SRCONFIG and it will help you along.

Source Routing is supported on the server/external router side through

a VAP or NLM; on the DOS workstation side it is supported using ROUTE.COM;

on the OS/2 workstation side it is supported using ROUTE.SYS.

----------------------------------------------------------------------------

1)   On a NetWare 386 server, LOAD the ROUTE.NLM.

2)   On a v2.x server, copy the ROUTE.VP0 and SRCONFIG.COM into the

SYS:SYSTEM directory on your server. Run SRCONFIG to set up the VAP

for source routing.   Type SRCONFIG <enter> to see the settings and

related defaults.  Load the VAP as you normally would.

3)   On DEDICATED IPX or ODI workstation clients, run ROUTE.COM after

IPX.COM   (generated during Workstation Client Driver Installation)

is loaded and before the SHELL (ie: NETx.COM, EMSNETx.EXE, or

EMSNETx.EXE) is loaded.

4)   On OS/2 workstation clients, the ROUTE.SYS driver is specified in

the workstation's   config.sys and must appear immediately after

TOKEN.SYS (for OS/2 Standard Edition) or after CMGRLAN.SYS (for

OS/2 Extended Edition).  Refer to the OS/2 requestor manuals to

implement all command arguments related to ROUTE.SYS.

*****************************************************************************

GENERAL INFORMATION ON HOW NETWARE SOURCE ROUTING DRIVERS WORK

*****************************************************************************

This text explains the use of the IBM Token-Ring Source Routing Driver on

NetWare networks.

The IBM Token-Ring Source Routing Driver enables NetWare communication across

IBM Token-Ring network bridges.

Guidelines for using the Source Routing Driver

You should understand the following general guidelines before attempting to

use the IBM Token-Ring Source Routing Driver on your NetWare network.

o   All stations that will access a server connected through an IBM

Token-Ring network bridge must use the IBM Token-Ring Source Routing Driver.

o   All Novell File Servers and External Routers connected through an IBM

Token-Ring network bridge must use the same Novell network number on both

sides of the IBM bridge.   The IBM Token-Ring network bridge is transparent

to NetWare's IPX protocol.  Rings that are linked together through IBM

bridges (no matter what the number of rings and IBM bridges) are viewed as

one ring with only one network number by Novell Routers.

NOTE:  Do not confuse the ring number with the network address used when

generating NetWare software.  Network addresses for servers and bridges

linked through IBM bridges must be the same, but each ring number must be

unique.  Ring numbers are assigned to PRI and ALT Token-Ring adapters only

when configuring IBM Bridge software.

Using the Source Routing Driver with NetWare 386

ROUTE.NLM is the driver that performs Source Routing on a NetWare 386

file server.  ROUTE must be loaded on the file server before you set up

workstations for source routing.  Refer to the NOVELL manuals and to the

INSTALLING ROUTE.NLM section in this document for instructions on loading

ROUTE and setting parameters.

**************************************************************************

INSTALLING ROUTE.NLM

**************************************************************************

Using the ROUTE Loadable Module

Use ROUTE.NLM to allow NetWare to pass frames (packets) through IBM bridges

on a Token-Ring cabling system.  You must load the TOKEN.LAN driver before

you load ROUTE.

All NetWare workstations must use the new Token-Ring workstation

drivers.

The following tasks are described in this section:

Set up the file server for source routing

Change a source routing parameter

Command format

LOAD [path]ROUTE [parameter...]

Command parameters

Replace path with a full directory path to the directory containing the

module; begin with a DOS drive letter or a NetWare volume name.  If you do

not specify a path and the SYS volume is mounted, the operating system

assumes that the module is located in the SYS:SYSTEM directory.  (Use

SEARCH to set up additional paths for automatic searching.)

Replace parameter with one or more of the following:

BOARD = number          REMOVE = number

CLEAR                   RSP = value

DEF                     TIME = number

GBR                     UNLOAD

MBR

Each parameter is described below.

Parameters

All the parameters described on the following pages are optional.  ROUTE

can be loaded re-entrantly (a second time) with a specified parameter to

change ROUTE's configuration.

Most of the parameters have default values that should work with simple

configurations for IBM bridges.  If you have installed parallel IBM bridges,

you can change some of the parameters to reduce traffic on some of the paths.

For more information about source routing and broadcast frames, see the

IBM Token-Ring Network Architecture Reference produced by IBM.

BOARD = number

Use this parameter if the Token-Ring driver is not the first driver

loaded.  The operating system numbers the boards, starting with the first

driver loaded.  That board is board number one.  (Check the order in the

AUTOEXEC.NCF file if you have loaded more than one driver.)  Use this

parameter to specify which board you want to change parameters on or load

ROUTE for.

CLEAR

Use this parameter to manually clear the Source Routing table.  This

parameter will force a dynamic rebuilding of the table by sending a default

frame to each specific node in the network.  Use this parameter when an IBM

bridge on the network has gone down and an alternate route is available.

DEF

Use this parameter to prevent frames that have unknown (DEFault)

destination addresses from being sent across Single Route IBM bridges.

If this parameter is specified, all frames that have addresses not

currently in the file server's Source Routing table will be forwarded

as All Routes Broadcast frames.

If this parameter is not specified, all frames that have addresses not

currently in the file server's Source Routing table will be forwarded

as Single Route Broadcast frames.

If ROUTE has already been loaded with the DEF parameter, reloading ROUTE

with this parameter broadcasts all subsequent Single Route Broadcast

frames as All Routes Broadcast frames.

GBR

Use this parameter to specify that all General BRoadcast frames are to

be sent as All Routes Broadcast frames.

If this parameter is not specified when ROUTE is loaded, all General

BRoadcast frames are broadcast as Single Route Broadcast frames.

If ROUTE has already been loaded with the GBR parameter, reloading ROUTE

with this parameter broadcasts all subsequent General BRoadcast frames

as All Routes Broadcast frames.

MBR

Use this parameter to specify that all Multicast BRoadcast frames are

to be sent as All Routes Broadcast frames.

If the parameter is not specified when ROUTE is loaded, all Multicast

BRoadcast frames are broadcast as Single Route Broadcast frames.

If ROUTE has already been loaded with the MBR parameter, reloading ROUTE

with this parameter broadcasts all subsequent Multicast BRoadcast frames

as All Routes Broadcast frames.

REMOVE = number

Use this parameter to remove a specified node address from the file

server's Source Routing table.  Use the parameter when a bridge has gone

down.  When you remove the node from the Source Routing table, you force

the file server to determine an alternate route.

Replace number with a 12-digit (06-byte) hexadecimal number.  If less

than 9 digits are entered, ROUTE prefixes the address with 4000h. For

example, REMOVE=2 is interpreted as REMOVE=400000000002

RSP = value

Use this parameter to specify how the file server should ReSPond to a

broadcast request.

Replace value with one of the following.

NR   Specifies that all Broadcast Requests are to be responded to

directly; a Broadcast Response is Not Required.  This is the

default value.

AR   Specifies that all Broadcast Requests are to be responded to with

an All Routes Broadcast frame.

SR   Specifies that all Broadcast Requests are to be responded to with

a Single Route Broadcast frame.

TIME = number

Use this parameter to specify how often the Source Routing table should

be updated.  This parameter forces the table to be updated with a new

route if the current route has not been used during the specified time

period.  This parameter enables ROUTE to dynamically determine alternate

routes when an IBM bridge goes down.  Replace number with a value from

03 to 255 seconds.  The default is 03 seconds.

UNLOAD

Use this parameter after the board parameter to specify which board you

want to remove source routing from.  To unload ROUTE.NLM from the file

server, use UNLOAD ROUTE.

Additional information

As frames pass through an IBM bridge, source routing information is

added to the frame header at the Media Access Control (MAC) layer.

NetWare's routing is done above this layer.  ROUTE.NLM enables NetWare:

To keep track of the source routing information in the frames;

To configure the source routing information in the frames according to

the parameters that ROUTE is loaded with.

ROUTE can be loaded re-entrantly.

If you have two Token-Ring boards in the file server, ROUTE should be

loaded twice.  The second time it is loaded, you must use the BOARD

parameter to specify a board number.

If you need to change the configuration, load ROUTE with the parameter

that needs to be changed.

Set up the file server for source routing

To set up the file server for source routing, complete the following

steps.

Load the TOKEN.NLM LAN driver.

Load ROUTE.  Type

LOAD ROUTE  <Enter>

Change a source routing parameter

To change a parameter in ROUTE after ROUTE has been loaded, select the

example that fits your situation.

One Token-Ring network board

To clear the Source Routing table when only one driver has been

loaded, type

LOAD ROUTE CLEAR

Two Token-Ring network boards

To change a parameter when more than one driver has been loaded,

complete the following steps.  This example shows how to change the

update time for the Source Routing table from the default of 3 seconds

to 10 seconds.

To change the TIME parameter for the first driver loaded, type

LOAD ROUTE BOARD=01 TIME=10  <Enter>

To change the TIME parameter for the second driver, type

LOAD ROUTE BOARD=02 TIME=10  <Enter>

**************************************************************************

INSTALLING ROUTE.COM

**************************************************************************

Using ROUTE.COM for the workstation

To install source routing on workstations, complete the following steps.

1.   Create a workstation boot diskette with the ODI TOKEN.COM or LANSUP.COM

drivers or with the dedicated DOS drivers SLANSUP.OBJ or STOKEN.OBJ

appropriately linked with IPX.OBJ.

2.   Copy the ROUTE.COM file to the boot diskette.

3.   Add the command to load ROUTE.COM to the AUTOEXEC.BAT file.

NOTE:     Steps 2 and 3 should have been completed when you prepared your

Token-Ring boot diskettes.

4.   Set the parameters for ROUTE.COM.  See "Additional Information" on the

next page for the parameters that ROUTE.COM can be loaded with.  Add

the parameter in the AUTOEXEC.BAT file.

Additional Information

The following parameters can be entered when ROUTE.COM is first loaded.

BOARD = number

MBR

CLEAR

NODES = number

DEF

REMOVE = number

GBR

TRO=#

The parameters can also be changed by loading ROUTE.COM a second time.  For

on-line help, type

ROUTE ?  <Enter>

Most of the parameters have default values that should work with simple

configurations for IBM bridges.  If you have installed parallel IBM bridges,

you can change some of the parameters to reduce traffic on some of the paths.

Each parameter is described below.

BOARD = number

Use this parameter to specify a board number for the Token-Ring network

board.

o    If this parameter is not specified, the default of 01 is used.

o    If you have loaded more than one LAN driver in the workstation, the

board number is determined by the order the LAN drivers are loaded (the

first board is 1).  See your AUTOEXEC.BAT file for the order you have

specified.

CLEAR

Use this parameter to manually clear the Source Routing table and force a

dynamic rebuilding of the table by sending a default frame to each specific

node in the network.  Use this parameter when an IBM bridge on the network

has gone down and an alternate route is available.

DEF

Use this parameter to prevent frames that have unknown (DEFault) destination

addresses from being sent across Single Route IBM bridges.

o    If this parameter IS specified, ALL FRAMES that have addresses not

currently in the workstation's Source Routing table will be forwarded

as All Routes Broadcast FRAMES.

o    If this parameter is NOT specified, all frames that have addresses not

currently in the workstation's Source Routing table will be forwarded

as Single Route Broadcast frames.

o    If ROUTE.COM has ALREADY been loaded with the DEF parameter, reloading

ROUTE.COM with this parameter broadcasts all subsequent Single Route

Broadcast frames as All Routes Broadcast frames.

GBR

Use this parameter to specify that all General BRoadcast frames are to be

sent as All Routes Broadcast frames.

o    If this parameter is NOT specified when ROUTE is loaded, all General

Broadcast frames are broadcasted as Single Route Broadcast frames.

o    If ROUTE has ALREADY been loaded with the GBR parameter, reloading ROUTE

with this parameter broadcasts all subsequent General Broadcast frames

as All Routes Broadcast frames.

MBR

Use this parameter to specify that all Multicast BRoadcast frames are to be

sent as All Routes Broadcast frames.

o    If the parameter is NOT specified when ROUTE is loaded, all Multicast

Broadcast frames are broadcast as Single Route Broadcast frames.

o    If ROUTE has ALREADY been loaded with the MBR parameter, reloading ROUTE

with this parameter broadcasts all subsequent Multicast Broadcast frames

as All Routes Broadcast frames.

NODES = number

Use this parameter to specify the number of table entries in the Source

Routing table.  This parameter must be entered the first time ROUTE.COM is

loaded.  The parameter cannot be changed by reloading ROUTE.COM.

Replace "number" with a value from 8 to 255.  The default is 16.

REMOVE = number

Use this parameter to remove a specified node address from the workstation's

Source Routing table.  Use the parameter when a bridge has gone down.

Removing the node from the Source Routing table forces the workstation to

determine an alternate path.

Replace "number" with a 12-digit (6-byte) hexadecimal number.  If less than

9 digits are entered, ROUTE.COM prefixes the address with 4000h.  For

example, REMOVE=2 is interpreted as REMOVE=400000000002.

TRO = number

ROUTE.COM contains a new feature allowing workstations to broadcast to the

local ring segment only (non-source routed), and if no response is obtained

from the local ring, the broad-cast will then go to the entire network

(source routed). This feature is implemented by using a new command argument:

"TRO=#."   TRO stands for "This Ring Only," with "#" being a number from 0

to 255;  the default is 0.  "TRO=#" must be less than the IPX retry count for

this to work properly.  If SHELL.CFG or NET.CFG does not specify an IPX retry

count, then IPX's retry count is modified to eight (8) by SLANSUP.OBJ or

STOKEN.OBJ (from TOKENB.ZIP).  The override takes place at IPX load time.

(The ability to over-ride the IPX retry count to eight (8) retries, first

became available with version 2.61 of SLANSUP.OBJ and STOKEN.OBJ).

***************************************************************************

INSTALLING ROUTE.VP0

***************************************************************************

Setting up the NetWare 2.X file server or External Router

for using the ROUTE.VP0 Source Routing VAP.

If you have NetWare 286 file servers or bridges on your network and want to

use Source Routing, you must configure them for Source Routing.  The Source

Routing file for NetWare 286 file servers and bridges is a VAP, and the VAP

loads as the file server or bridge is booted.  On a file server, NetWare 286

searches for VAPs in the SYS:SYSTEM directory.  On a bridge, NetWare checks

the current drive for VAPs.  VAPs with a numerical extension (.VP0, VP1,

etc.) are loaded in numerical order.  ROUTE.VP0 has a zero as its last number

in the extension because it should be loaded first.

To set up a NetWare 286 file server or bridge for Source Routing, you will

need to complete the following tasks.

o    Configure ROUTE.VP0 for the appropriate LAN driver.

o    Copy ROUTE.VP0 to SYS:SYSTEM for a server or to the root directory for

a bridge.

o    Down the file server or bridge and then reboot.

To complete these tasks, complete the following steps.

1.   If you want to change configuration parameters, use SRCONFIG.  SRCONFIG

is not required for most External Router applications, since ROUTE.VP0

is configured for most External Router applications.

SRCONFIG  <Enter>

Follow the prompts.

2.   Copy the ROUTE.VP0 file

o    For a NetWare 2.x file server, copy it to the SYS:SYSTEM directory

o    For a NetWare External Router, copy it to the boot directory.

3.   Down the file server or the bridge.

4.   Reboot the file server or bridge.  Answer the prompts so that the VAP

loads.

5.   If needed, configure the VAP with the specialized parameters.  ROUTE.VP0

loads a console command, ROUTE.  With ROUTE, you can configure the

NetWare 286 file server or router.  Most of the ROUTE.NLM parameters

can be used, with the following exceptions.

UNLOAD: This parameter cannot be used with the VAP.  You will need to

reconfigure the VAP with SRCONFIG to remove Source Routing from a LAN

driver.  To remove Source Routing from LAN A but retain it for LAN B,

use the following command.

SRCONFIG  <Enter>

BOARD:  Board number is determined by the LAN.  LAN A is board 1; LAN

B, 2; etc.  LAN A is the default if no board number is specified in a

command.

Use the following format to enter a console command.

ROUTE TIME=10  <Enter>

This would change the TIME parameter on LAN A.

To change the TIME parameter on LAN B, use the following format.

ROUTE BOARD=B TIME=10  <Enter>

NOTE:     If you need to permanently configure the VAP with one of these

parameters, use SRCONFIG with the following format.

SRCONFIG  <Enter>  (From here, you will be prompted for all input.)

********** MODIFICATION HISTORY TO: SROUTE.ZIP & SROUTE.DOC ***************

04-29-91: Initial release of SROUTE.ZIP & SROUTE.DOC.

11-11-91  Reformated readme.

05-27-92  Updated DOC to reflect TRO parameter on ROUTE.COM.

**********************  END OF MODIFICATION HISTORY  ***********************

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************