386OS.DOC

This 386OS.DOC accompanies 386OS.ZIP which contains IBM Token-Ring drivers

for the NetWare v3.1x Operating System.  The following files are included in

386OS.ZIP:

TOKEN    LAN    10324   9-09-91  11:30a  (certified Version 3.16)

TOKENDMA LAN     9302   3-25-92   8:16a  (certified Version 3.13)

FIRMLOAD COM     1628   1-04-91   8:57a  (certified & ships w/V3.11)

386OS    DOC         (this file)

The above files represent the latest drivers available for the NetWare 3.1x

operating system.  A discription of each file follows:

HISTORY:

TOKEN.LAN

Version 3.15:

It MUST be use with Novell's Communications Server NLM on a

v3.11 NetWare server.  It corrects lost connection problems

when a workstation is attached to a Host via the COM-Server.

It also corrects a problem that version 3.13 (the version of

the driver that currently ships with NetWare 3.11) had with

GENERAL PROTECTION PROCESSOR EXCEPTION (GPPE).  This GPPE can

be caused by beaconing out on the ring.  TOKEN.LAN is

compatible with ALL IBM Token-Ring cards except for the IBM

24-bit busmaster Token-Ring card.   Refer to the following

when installing TOKEN.LAN:

Version 3.16:

Fixes a problem not parsing the NODE override correctly.

1.   Replace the current Token-Ring driver located in the SYS:SYSTEM

directory with this file.

2.   Unload the TOKEN.LAN driver at the server console.

3.   Reload and bind the TOKEN.LAN driver.

TOKENDMA.LAN - This is version 3.11 rev A driver for the 24 bit IBM Token-

Ring 16/4 Busmaster Server Adapter/A.   TOKENDMA.LAN was initially

distributed on NetWire inside of TRNDMA.ZIP.  This file has been certified

ships with NetWare v3.11.  To implement TOKENDMA.LAN, refer to the section

in this document titled:  TOKENDMA.LAN INSTALLATION.

FIRMLOAD.COM - FIRMLOAD.COM was formerly known as DOWNLOAD.COM inside of

TRNDMA.ZIP on NetWire.  It is certified and ships with Netware v3.11.  This

is v1.00, also found on the Netware 3.11 System-2 diskette.  This program

downloads firmware files to the IBM TOKEN-RING BUSMASTER card.  To implement

FIRMLOAD.COM, refer the section in this document titled: DOWNLOADING THE

MICROCODE.

NOTE: To implement TOKENDMA.LAN refer to the 3.11 System Administration

manual sections titled: LOAD LAN driver and BIND.  To implement TOKEN.LAN,

refer to the v3.10/v3.11 System Administration manual sections

titled: LOAD LAN driver and BIND.

***************************************************************************

TOKENDMA.LAN INSTALLATION

***************************************************************************

The IBM Token-Ring Network 16/4 Busmaster Server Adapter/A is a 24-bit

micro-channel adapter.  You can use up to eight busmaster server adapters in

a micro-channel file server running NetWare v3.10.  Although the adapter

provides optimum performance installed in a 32-bit expansion slot, it can

also be inserted into a 16-bit slot if no 32-bit slots are available.

HISTORY:

TOKENDMA.LAN

Version 3.10 rev A

Version 3.10 did not support an adapter in a file server with

more than 16MB of memory and more than 8GB of disk storage.

TOKENDMA.LAN dated 9-16-91

Version 3.11

Version 3.11 supports more than 16 MB of memory in the server.

This was the first version with this capability.

TOKENDMA LAN     9302  3-25-92   8:16a

Version 3.13

This released version replaces all previous beta and released

versions. This driver supports more than 16 meg of memory in

the server.

1. Added parameters LOAD TOKENDMA NBR=3 NBT=3  (Number Recieve

Buffers & Number of recieve buffers).  This is only used with

more that 16 meg of ram.  This allows the double buffering to

occur.  From a hardware standpoint this is required because

the adapter is not 32-bit AND is busmastering.  The defaults

have already been set to optimal settings so it isn't

necessary to manipulate with these parameters although they

exist.

2. Now if frequent beaconing conditions exist on the ring, the

following error message may appear on the server console:

"Lan Receive buffer limit reached.  Possible Causes are:  disk

channel bottleneck, or buffers lost."   This error is caused

by Event Control Blocks (ECBs) backing up in the driver's

send queue due to TOKENDMA.LAN pausing execution until

beaconing stops.

3. Updated to address a problem with the Monterey adapter not

being fully opened by the driver when TOKENDMA.LAN loads from

the autoexec.ncf and is the first adapter to insert onto the

ring.   More time is required for an "adapter-open" to

complete when the adapter is the first node to insert onto the

ring.  This, coupled with the server having over 16 megs of

ram, results in LSL ECBs maxing out.

4. Modified from using priority-1 bits to priority-0 bits.

5. Automatically adjusts NBR and NBT values to accomodate

varied amounts of ram installed in the server.

NOTE: It may be necessary to increase the following setable

parmeter at the console if multiple TOKENDMA adapters are

being used:

SET RESERVED BUFFERS BELOW 16 MEG: 16 (8-200)

(Setable only in STARTUP.NCF)

This sets the number of cache buffers to be kept for device

drivers unable to access memory above 16 meg.

*********** End of modification history *****************************

The IBM Token-Ring 16/4 Busmaster Server Adapter/A is functionally different

from the IBM Token-Ring 16/4Adapter/A in two ways:

o    Busmaster capability

Busmastering allows the IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A to transfer between the network and the server's

memory without the aid of the server's CPU.  The server's CPU can

then process other network tasks more quickly than it otherwise

could.

o    Downloadable microcode

Downloading microcode allows you to upgrade the IBM Token-Ring

Network 16/4 Busmaster Server Adapter/A's firmware without

replacing any chips on the adapter.  FIRMLOAD.COM (supplied by

Novell) allows you to download the firmware (supplied by IBM) to

the adapter.

Before you install this adapter in a v3.1x server, you should have designed

your network and assembled the necessary hardware.  If you have any questions

about Token-Ring hardware or network layout, see Section 1 of the NetWare

Installation Supplement for IBM Token-Ring manual for further information.

INSTALLING and SETTING the Adapter

For information concerning installation and configuration of the 16/4

Busmaster card please refer to the "IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A Installation and Testing Instructions" guide that

accompanies the lan card.

Additional information concerning the setable parameters found in "change

configuration" selection in the Reference program:

o    Data rate

The default data rate for the adapter is 4 Mbps.

The other valid data rate is 16Mbps.

Important: You must set all IBM Token-Ring 16/4 adapters on a ring to use the

same data rate (either 16Mbps or 4Mbps).  Failure to do so will cause network

errors.

o    Interrupt level

The default is Interrupt 2.

Other valid levels include Interrupts 3, 4, 5, 7, 10, 11, and 12.

o    I/O address

The default I/O address is 86A0-86AF.

Each valid address consists of a 16-byte block of RAM within the

microchannel machine.  There are seven additional valid I/O

addresses:  96A0, A6A0, B6A0, C6A0, D6A0, E6A0, and F6A0.

Each adapter installed in the machine must have a unique I/O

address.

o    Arbitration level

The default arbitration level is E.

Other valid arbitration levels are 0, 1, 2, 3, 5, 6, 7, 8, 9, A,

B, C, and D.

Each busmaster adapter requires a unique arbitration level for

negotiating with other busmaster adapters within the same machine

for control of the microchannel bus.

o    Fairness

The default is to operate with fairness ON.

The other valid option is to operate with fairness OFF.

Fairness determines how a microchannel busmaster adapter negotiates

with other busmaster adapters within the same machine for control

of the microchannel bus.

For more information about arbitration and fairness, read the IBM

publication, PS/2 Hardware Technical Reference (part# S68X-2330).

o    Bus release

The default is "Maximum."

The other valid setting is "Immediately."

The bus release setting determines how long a microchannel

busmaster adapter has to relinquish the bus when another adapter

preempts it during a DMA operation.

A setting of "Immediately" causes the adapter to relinquish control

of the microchannel bus upon receiving the preempt signal.

A setting of "Maximum" allows the adapter time to finish critical

DMA operations, resulting in better overall performance.

o    Early Token Release (ETR)

When the data rate is 16Mbps, the default is to operate with ETR.

The other valid option is NO ETR.

Operation with ETR is not valid for Token-Rings with a data rate

of 4 Mbps.

ETR can increase network performance by causing the adapter to

release the token immediately upon de-encapsulating the data

packet, rather than after it has successfully

transferred the de-encapsulated data to the next higher network

protocol.

***************************************************************************

DOWNLOADING the Microcode

***************************************************************************

****************************IMPORTANT**************************************

The Microcode must be downloaded to the Busmaster Server Adapter/A each time

the machine is powered up.  This can be accomplished manually (from the

command line) or can be automated in a batch file.

**************************************************************************

The IBM Token-Ring 16/4 Busmaster Server Adapter/A requires microcode that

is not resident on the adapter.  The microcode is contained in two files

included on the adapter's OPTION diskette (supplied by IBM):

o    MONT400.BIN

o    WRTRAM.BIN.

To download MONT400.BIN and WRTRAM.BIN to the adapter, you must use

FIRMLOAD.COM, a utility supplied by Novell.

When you have located these three files (including FIRMLOAD.COM) complete the

following steps:

1.   Copy all three files--MONT400.BIN, WRTRAM.BIN, and FIRMLOAD.COM--to

the same diskette (or to the same directory to which you have

mapped a drive).

2.   Insert the diskette containing MONT400.BIN, WRTRAM.BIN, and

FIRMLOAD.COM into drive A.

3.   Change to drive A.

4.   Then type

FIRMLOAD  <Enter>

A message similar to the following appears:

Novell Token-Ring DownLoad Utility for IBM DMA Bus Master v1.00

(901129)(C) Copyright    1990 Novell Inc.  All Rights Reserved.

IBM Token-Ring DMA Bus Master FOUND in Slot 2;

Data Rate = 04 MBps         DMA Level = 0E;  Interrupt Level=02

Node Address = 1000 5A4F 00AE;    Port = 86A0

IBM Token-Ring DMA Bus Master DownLoad SUCCESSFUL

******************** 386OS.DOC MODIFICATION HISTORY *************************

4-29-91   Initial release of 386OS.DOC.

11-11-91  Upgraded to a newer TOKENDMA.LAN dated 9-16-91 that supports more

16 meg of memory.

Upgraded to a newer TOKEN.LAN (minor changes).

5-27-92   Upgraded to a newer TOKENDMA.LAN dated 3-25-92 and added detail on

history of TOKENDMA.LAN.

******************** END OF MODIFICATION HISTORY ****************************

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************