RPL.DOC

This RPL.DOC file accompanies RPL.ZIP which contains Remote Program Load

files for NetWare 286 v2.15 rev C and NetWare 386 v3.10a.  The following

files are included for this revision:

RPL      DOC     (this file)

RPL      VP1     1856   5-03-91   2:31p   (certified by Software Test)

RPCONFIG COM     2726   6-08-90   5:15p   (certified ships w/v2.2/v3.11)

TOKEN    RPL    13085  11-01-90  10:21a   (certified ships w/v2.2/v3.11)

TOKENRPL NLM    16823   1-21-91  11:29a   (certified ships w/v3.11)

ETHERRPL NLM    15749   4-12-91  11:38a   (certified by Software Test)

ETHER    RPL    12035   4-12-91  11:28a   (certified by Software Test)

The above drivers provide the ability for a workstation (with IBM Token-Ring

or Ethernet adapters with RPL Proms installed) to remote boot to a Novell

V2.x or V3.1x server.  For more information on how Remote Program Load works,

refer to the section in this document titled:  GENERAL EXPLANATION OF REMOTE

PROGRAM LOAD ---(RPL).

RPL.VP1 - This is the Remote Program Load VAP for the NetWare V2.x OS or

External Router.  To load properly, AppleTalk Phase-II Lan drivers  must be

linked into the 2.x Server or External Router.   RPL.VP1 must be copied into

the SYS:SYSTEM directory on a server, or, for external routers, the vap may

be copied onto the same directory where ROUTER.EXE resides.  To implement

RPL.VP1, refer to the section in this document  titled:  INSTALLING RPL.VP1.

Note: Rpl.vp1 dated 8-08-90 has been replaced by Rpl.vp1 dated 5-03-91

which addresses problems with GPI errors.

RPCONFIG.COM -  This utility configures two parameters on RPL.VP1: (1)

To enable RPL on lan A,B,C, or D, or any combination of these.  Default: RPL

is enabled on A,B,C, and D.  (2)  To set the name of the RPL file that is

downloaded by the RPL vap to the workstation.  This file is NOT the

NET$DOS.SYS file.  It is the Remote Program Load file (e.g. TOKEN.RPL).  The

default name is:  *.RPL.

RPCONFIG.COM is menu driven.  Just type RPCONFIG and it will help you

along.  ("*.rpl" refers to ANY file with a .rpl extention.)

TOKEN.RPL - This is the file that RPL.VP1 downloads to a workstation

when the RPL boot ROM requests to be loaded.  This file loads in ram with the

vap, and waits to be down-loaded to a workstation.  When a server or external

router boots, RPL.VP1 loads and creates its own RAM AREA in the server's or

external router's ram.  Then, RPL.VP1 searches for TOKEN.RPL and loads it

into this RAM AREA.  TOKEN.RPL must reside in the SYS:LOGIN directory! If

loading on a server, RPL.VP1 connects internally to this server to search for

SYS:LOGIN\TOKEN.RPL. But, if loading on an external router, RPL.VP1 connects

externally (out on the wire) to a server to find TOKEN.RPL.  In the latter

case, the first server on the wire to respond to RPL.VP1's connect request,

must have TOKEN.RPL in the SYS:LOGIN directory.  The server can EITHER be

NetWare V3.1x or V2.x and TOKENRPL.NLM or RPL.VP1 need not be loaded on

either of these servers for the external router to successfully locate and

upload TOKEN.RPL from them.

TOKENRPL.NLM - This file performs the same functions respectively that

both TOKEN.RPL and RPL.VP1 perform together.  TOKEN.RPL is pre-linked inside

of TOKENRPL.NLM, so that when the NLM loads and binds at the V3.1x server,

TOKEN.RPL also loads.  There is no need for TOKEN.RPL to be in SYS:LOGIN,

unless a local external router (with the RPL vap) needs it.  When a

workstation RPL prom requests a load file, the NLM down-loads TOKEN.RPL from

the v3.1x server's ram.   To implement TOKENRPL.NLM, refer to the section in

this document titled: INSTALLING TOKENRPL.NLM.

ETHER.RPL - This file is used for IBM Ethernet Remote Program Load.  It

functions the same way respectively for IBM Ethernet RPL as TOKEN.RPL

functions for IBM Token-Ring RPL.  It must be copied into the SYS:LOGIN

directory and it is accessed by RPL.VP1 in exactly the same manner as for

TOKEN.RPL.  To implement ETHER.RPL, refer to the section in this document

titled:  INSTALLING RPL.VP1.

ETHERRPL.NLM -  This file works in a similar manner as TOKENRPL.NLM.  It

containes an embedded pre-linked ETHER.RPL file which loads into a v3.1x

server's ram and is downloaded to a workstation when the RPL prom requests

a load file.  To implement ETHERRPL.NLM, refer to the section in this

document titled:  INSTALLING ETHERRPL.NLM.

***************************************************************************

GENERAL EXPLANATION OF REMOTE PROGRAM LOAD --- (RPL)

***************************************************************************

NOVELL RPL VAP & NLM

Unlike NOVELL Boot ROMs, the IBM Token Ring RPL PROM is not NetWare

specific.  In order to be able to boot from various systems, it uses a

"Staged Boot" idea.  That means it sends out a generic "Find Frame" packet,

and expects whichever kind of system it is on to be able to interpret and

respond.  Then it asks for a file to be downloaded.  This file (typically

called TOKEN.RPL, or PCN2.RPL, or ETHER.RPL, etc.) will be the NetWare

specific file that contains code to boot from a NOVELL network.

In the past, the only way to get NetWare to respond to the RPL packets

was to make the driver capable of recognizing those packets, and responding

accordingly.  This was mostly because the Find Frame is sent to a multicast

address, and NetWare 286 didn't support multicast addressing.  With the

advent of AppleTalk Phase II drivers, a VAP can receive and respond to

multicast addresses, making this the preferred way of doing things.

****************************************************************************

INSTALLING RPL.VP1

****************************************************************************

RPL.VP1 must be loaded after ROUTE.VP0, if source routing is to be

supported. It supports several .RPL download files:  TOKEN.RPL, ETHER.RPL,

AND PCN2L.RPL.

In order to work with multiple LAN drivers, or multiple .RPL files, the

RPL VAP must be configured.  This is done by running RPCONFIG.  RPCONFIG

prints out a help screen and prompts for inputs.

When the RPL VAP comes up, it will load specified .RPL files, and

connect them with specified LAN drivers.  Then, when the VAP receives a Find

Frame, and subsequent Send File requests from a given RPL PROM, it will

respond with the proper .RPL file.

Since the VAP attaches without logging in, it only has read rights in

the SYS:LOGIN subdirectory.  Therefore, THE .RPL FILES ARE EXPECTED TO BE

LOCATED IN SYS:LOGIN.  This is not the same as in the previous system, where

they were expected to be in SYS:SYSTEM.

If the RPL VAP hasn't been configured by RPCONFIG, it will default when

it comes up.  It will go out to SYS:LOGIN, find the first .RPL file it can,

and connect it with the first LAN driver available.  This is generally not

reliable for more than one .RPL file, or more than one LAN driver.  If in

doubt, run RPCONFIG.

The server that responds to the Find Frame, and downloads the .RPL file,

is not guaranteed to be the server that the workstation will attach to during

the RPL sequence.  This means that the BOOTCONF.SYS files, and DOS Boot Disk

Image Files (NET$DOS.SYS, etc.) will still need to be in SYS:LOGIN of every

server that could respond to the Get Nearest Server request.  Not just the

server with the RPL VAP loaded.

*****************************************************************************

INSTALLING TOKENRPL.NLM

*****************************************************************************

Use TOKENRPL.NLM to enable remote booting of workstations (mostly

diskless) that have Token-Ring network boards with RPL PROMS installed.

Command format

LOAD [path]TOKENRPL

Command parameters

Replace path with a full directory path to the directory containing the

module; begin with a DOS drive letter or a NetWare volume name.  If you do

not specify a path and the SYS volume is mounted, the operating system

assumes that the loadable module is located in the SYS:SYSTEM directory.

(Use SEARCH to set up additional paths for automatic searching.)

Additional information

To remotely boot a diskless Token-Ring workstation, you must complete the

following:

Configure the workstation boot files for remote booting.  Follow the

DOSGEN instructions in   Appendix D of the NetWare 386 Installation

manual.

Load ETHERRPL.

Bind the ETHERRPL (ETHRPL is the old driver syntax, upgrade ETHERRPL.NLM

if this   syntax works) protocol to the network boards in the file server.

To load ETHERRPL, complete the following steps.

1.   Type

LOAD ETHERRPL  <Enter>

2.   Bind the ETHERRPL protocol to the driver for the Ethernet network

board(s) in the file server. Type the following:

BIND ETHERRPL LAN_driver  <Enter>  (if "BIND ETHRPL LAN_driver" works,)

(then upgrade ETHERRPL.NLM. ETHRPL)

(Was proper syntax for the old)

( ETHERRPL.NLM. )

Replace LAN_driver with the name of the driver you are using.  If you

have loaded the driver   more than once, you will need to repeat this

step.

3.   Edit the AUTOEXEC.NCF file.  Add the following lines to load and bind

ETHERRPL:

LOAD ETHERRPL

BIND ETHERRPL LAN_driver driver_parameters

Replace LAN_driver with the name of the driver you used in Step 2.

Replace driver_parameters with the parameters you used to load the LAN

driver.

***************************************************************************

EXAMPLE LOAD AND BIND SEQUENCE FOR IBM 802.2 ETHERNET NETWORKS

***************************************************************************

LOAD NE2000

BIND IPX TO NE2000

LOAD NE2000 FRAME=ETHERNET_802.2

LOAD ETHERRPL

BIND ETHERRPL TO NE2000 FRAME=ETHERNET_802.2

***************** END OF EXAMPLE *****************************************

***************** RPL.ZIP & RPL.DOC MODIFICATION HISTORY *******************

4-29-91   Initial release of RPL.ZIP & RPL.DOC.

5-30-91  Updated the following file(s):

ETHERRPL NLM    14415   1-21-91  11:12a   (certified ships w/v3.11)

ETHER    RPL    10673  10-03-90  12:00p   (certified ships w/IBM ORG OPT Dsk)

to the following certified file(s):

ETHERRPL NLM    15749   4-12-91  11:38a   (certified by Software Test)

ETHER    RPL    12035   4-12-91  11:28a   (certified by Software Test)

7-26-91  Updated the following file(s):

RPL      VP1     1856   5-03-91   2:31p   (certified by Software Test)

11-11-91 Removed Documentation RPLDR2.DOC - They were old beta solutions.

Removed RPLDR2.ZIP with the below files.

RPLDR2   SYS      671   2-04-91   8:45a  (beta passed field test)

TOKENRPL NLM    16854   2-04-91  11:25a  (beta passed field test)

RPLDR2   ZIP    15591   4-21-91   1:40a  (beta passed field test)

TOKEN    RPL    13085   2-04-91  10:43a  (beta passed field test)

*************** END OF MODIFICATION HISTORY  ********************************

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************