OS2WS.DOC

This OS2WS.DOC file accompanies OS2WS.ZIP which contains IBM Token-Ring

drivers for NetWare Requestor v1.21 to v1.3c.  The following files are

present in this revision:

CMGRLAN  SYS    42976   3-26-91   9:32a   (certified ships w/v1.3c Requester)

TOKEN    SYS    26896   4-17-92   9:30a   (certified)

TOKENEE  SYS    38224   8-08-90  10:58a   (certified ships w/v1.3  Requester)

OS2WS    DOC      (this file)

NOTE: SEE NSD004.ZIP or later on Netwire for latest requestor 1.3 update.

Below is a DESCRIPTION of EACH file in this kit:

TOKEN.SYS - This driver is to be used without the communication manager.

If implementing Source Routing, ROUTE.SYS must immediately follow the

device=TOKEN.SYS in the config.sys file. (1=source is no longer needed in the

config.sys for source routing.)

TOKENEE.SYS - This driver is to be used with the communication manager,

in conjunction with Novell's Requester for OS/2 versions 1.3 and below,

extended editions only.  (Don't use this driver for version 1.3c of Novell's

Requester for OS/2.)  If implementing source routing with this driver, you

must include 1=SOURCE immediately following the device= TOKENEE.SYS statement

in the config.sys file.

CMGRLAN.SYS -  This driver is to be used with the communication manager

when using versions 1.21 and above of Novell's Requester for OS/2, extended

edition only.  (This driver replaces TOKENEE.SYS and was released with

Novell's Requester for OS/2 version 1.3C.)  If you are using source routing

with this driver, you must include ROUTE.SYS immediately following the

device=CMGRLAN.SYS statement in the config.sys file.

Note:  If you are using source routing, refer to SROUTE.DOC.

Instructions for implementing the new driver:

1.   Change to the directory where the lan drivers are kept.

2.   If the driver you are replacing, matches the current driver name,

then rename the existing driver.

Example:  Ren token.sys token.old

3.   Copy the new driver into the driver directory.

4.   Reboot the machine.

Note:  If you are adding the driver to your configuration for the first

time, you will have to make any necessary changes to your config.sys and

net.cfg files.

***************** OS2WS.DOC & OS2WS.ZIP MODIFICATION HISTORY ***************

4-29-91   Initial release of OS2WS.DOC & OS2WS.ZIP.

11-11-91  Updated TOKEN.SYS to the latest certified revision.

5-27-92   Updated TOKEN.SYS to the latest certified revision.

******************** END OF MODIFICATION HISTORY *************************

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************