CONFIGURING the IBM TOKEN-RING 16/4 Busmaster Server Adapter/A

This text explains the features, configuration, installation, and testing

procedures for the IBM Token-Ring 16/4 Busmaster Server Adapter/A.

The IBM Token-Ring Network 16/4 Busmaster Server Adapter/A is a 24-bit

microchannel adapter.  You can use up to eight busmaster server adapters in

a microchannel file server running NetWare v3.10.  Although the adapter

provides optimum performance installed in a 32-bit expansion slot, it can

also be inserted into a 16-bit slot if no 32-bit slots are available.

Important: Do not use this adapter in a file server with more than 16MB of

memory and more than 8GB of disk storage.

The IBM Token-Ring 16/4 Busmaster Server Adapter/A is functionally different

from the IBM Token-Ring 16/4 Adapter/A in two ways:

o    Busmaster capability

Busmastering allows the IBM Token-Ring Network 16/4 Busmaster Server

Adapter/A to transfer between the network and the server's memory

without the aid of the server's CPU.  The server's CPU can then process

other network tasks more quickly than it otherwise could.

o    Downloadable microcode

Downloading microcode allows you to upgrade the IBM Token-Ring Network

16/4 Busmaster Server Adapter/A's firmware without replacing any chips

on the adapter.  DOWNLOAD.COM (supplied by Novell) allows you to

download the firmware (supplied by IBM) to the adapter.

Before you install this adapter in a v3.1 server, you should have designed

your network and assembled the necessary hardware.  If you have any questions

about Token-Ring hardware or network layout, see Section 1 of the NetWare

Installation Supplement for IBM Token-Ring manual for further information.

INSTALLING and SETTING the Adapter

The IBM Token-Ring 16/4 Busmaster Server Adapter/A is set using the setup

program on the REFERENCE (or similarly named) diskette that accompanied the

microchannel system.

PREPARING the REFERENCE program

The REFERENCE program requires the configuration information and other

software on the OPTION diskette that came with the IBM Token-Ring 16/4

Busmaster Server Adapter/A.  You should use a working copy of the REFERENCE

diskette.

1.   If you have not already made a working copy of the REFERENCE diskette,

follow the instructions in the Quick Reference documentation that

accompanied the machine and make one.

Note:      If you have previously added or removed a network adapter,

you may see a configuration error message.  You will also see

a prompt to automatically configure the system.  Answer "No"

to the prompt.

2.   When you are still in the REFERENCE program, select the menu option to

copy the OPTION diskette to the working copy of the REFERENCE diskette.

Follow the instructions in the Quick Reference documentation that

accompanied the machine.

Note:      Do not use the DOS COPY command to update the REFERENCE

diskette.  Use "Copy an option diskette" in the REFERENCE

program.

INSERTING the Adapter into the Server

To insert the IBM Token-Ring 16/4 Busmaster Server Adapter/A into the server,

complete the following steps (using the manufacturer's documentation, if

necessary):

1.   Turn off the power switches and unplug the power cords of the server and

peripherals.

2.   Disconnect all cables from the back of the server.  (Label the cables so

that you will know where to reconnect them.)

3.   Remove the server's cover according to the manufacturer's instructions,

taking precautions against the discharge of static electricity.

4.   Insert the adapter into a 32-bit expansion slot according to the

manufacturer's instructions.  If you do not have an available 32-bit

slot, you can use a 16-bit slot.

SETTING the Adapter

Now that the configuration files and other software have been copied to the

working copy of the REFERENCE diskette and the adapter has been inserted into

the server, you are ready to set the adapter.

The REFERENCE program displays the default settings for the IBM Token-Ring

16/4 Busmaster Server Adapter/A and provides you with access to valid

optional settings.

For instructions on running the REFERENCE program, refer to the Quick

Reference documentation that accompanied the microchannel system.  Complete

the following steps:

1.   Select the Option Identification Label for your language and record the

settings as you run the setup program.

2.   Insert the working copy of the REFERENCE diskette into drive A and boot

the microchannel machine.

3.   Run the REFERENCE program.

Select "Set configuration" from the main menu and then "Change

configuration".  Scroll to the appropriate slot number, and make the

following settings for each IBM Token-Ring 16/4 Busmaster Server

Adapter/A:

o     Data rate

The default data rate for the adapter is 4 Mbps.

The other valid data rate is 16Mbps.

Important: You must set all IBM Token-Ring 16/4 adapters on a ring

to use the same data rate (either 16Mbps or 4Mbps).

Failure to do so will cause network errors.

o     Interrupt level

The default is Interrupt 2.

Other valid levels include Interrupts 3, 4, 5, 7, 10, 11, and 12.

o     I/O address

The default I/O address is 86A0-86AF.

Each valid address consists of a 16-byte block of RAM within the

microchannel machine.  There are seven additional valid I/O

addresses:  96A0, A6A0, B6A0, C6A0, D6A0, E6A0, and F6A0.

Each adapter installed in the machine must have a unique I/O

address.

o     Arbitration level

The default arbitration level is E.

Other valid arbitration levels are 0, 1, 2, 3, 5, 6, 7, 8, 9, A,

B, C, and D.

Each busmaster adapter requires a unique arbitration level for

negotiating with other busmaster adapters within the same machine

for control of the microchannel bus.

o     Fairness

The default is to operate with fairness ON.

The other valid option is to operate with fairness OFF.

Fairness determines how a microchannel busmaster adapter negotiates

with other busmaster adapters within the same machine for control

of the microchannel bus.

For more information about arbitration fairness, read the IBM

publication, PS/2 Hardware Technical Reference (part# S68X-2330).

o     Bus release

The default is "Maximum."

The other valid setting is "Immediately."

The bus release setting determines how long a microchannel busmaster

adapter has to relinquish the bus when another adapter preempts it

during a DMA operation.

A setting of "Immediately" causes the adapter to relinquish control

of the microchannel bus upon receiving the preempt signal.

A setting of "Maximum" allows the adapter time to finish critical

DMA operations, resulting in better overall performance.

o     Early Token Release (ETR)

When the data rate is 16Mbps, the default is to operate with ETR.

The other valid option is no ETR.

Operation with ETR is not valid for Token-Rings with a data rate of

4 Mbps.

ETR can increase network performance by causing the adapter to

release the token immediately upon de-encapsulating the data packet,

rather than after it has successfully transferred the

de-encapsulated data to the next higher network protocol.

4.   With the machine turned off, attach the Option Identification label

(where you have recorded the adapter's configuration settings) to the

adapter by adhering it to the flat portion of the blue plastic handle,

located at the top of the adapter.

5.   Replace the machine's cover, reconnect all cables, and restart the

machine.

DOWNLOADING the Microcode

The IBM Token-Ring 16/4 Busmaster Server Adapter/A requires microcode that

is not resident on the adapter.  The microcode is contained in two files

included on the adapter's OPTION diskette:

o    MONT400.BIN

o    WRTRAM.BIN.

To download MONT400.BIN and WRTRAM.BIN to the adapter, you must use

DOWNLOAD.COM, a utility supplied by Novell.

When you have located these three files (including DOWNLOAD.COM), complete

the following steps:

1.   Copy all three files--MONT400.BIN, WRTRAM.BIN, and DOWNLOAD.COM--to the

same diskette (or to the same directory to which you have mapped a

drive).

2.   Insert the diskette containing MONT400.BIN, WRTRAM.BIN, and DOWNLOAD.COM

into drive A.

3.   Change to drive A.

4.   Then type

DOWNLOAD  <Enter>

A message similar to the following appears:

Novell Token-Ring DownLoad Utility for IBM DMA Bus Master  v1.00 (901129)

(C) Copyright 1990 Novell Inc.  All Rights Reserved.

IBM Token-Ring DMA Bus Master FOUND in Slot 2;

Data Rate = 04 MBps         DMA Level = 0E;  Interrupt Level = 02

Node Address = 1000 5A4F 00AE;    Port = 86A0

IBM Token-Ring DMA Bus Master DownLoad SUCCESSFUL

You are now ready to test the adapter.

TESTING the Adapter

The working copy of the REFERENCE diskette contains software for testing all

components of the machine as well as the configuration files from the IBM

Token-Ring 16/4 Busmaster Server Adapter/A's OPTION diskette.

The testing program performs a series of diagnostic routines and also asks

for information from the user.

Note:      Before you begin testing, make sure you know the type of cable that

is connected to the adapter, as well as the data rate (4 Mbps or

16 Mbps) of the LAN segment to which the adapter is connected.

To test the adapter, complete the following steps:

1.   Insert the working copy of the REFERENCE diskette into drive A.

2.   Reboot the machine.

3.   Test the adapter, following the prompts and the instructions for testing

in the Quick Reference documentation that accompanied your microchannel

system.

Where to GO from Here

When you have configured the IBM Token-Ring 16/4 Busmaster Server Adapter/A

and have the necessary microcode on board, you are ready to load the NetWare

v3.1 server driver.

TOKENDMA.LAN is the server driver for the IBM Token-Ring 16/4 Busmaster

Server Adapter/A.  For information about loading and binding the TOKENDMA.LAN

driver, see LOAD LAN driver and BIND in NetWare v3.1 System Administration.

The TOKENDMA.LAN parameters are similar to TOKEN.LAN parameters discussed in

LOAD LAN driver:

FRAME=name            Same default and supported values as TOKEN

NAME=name             Same default and supported values as TOKEN

NODE=number           Same default and supported values as TOKEN

PORT=number           Specify the I/O address you selected when you

configured the board.  The default and supported

values are listed on page 6 of this text.  You

can specify PORT rather than SLOT if you want to.

SLOT=number           Specify the slot that holds the adapter.

For information about using the NetWare source routing driver with the

Token-Ring 16/4 Busmaster Adapter/A, see Appendix D of the NetWare

Installation Supplement for IBM Token-Ring.