This README file of TRXNET.ZIP accompanies the TRXNET.LAN for NetWare v3.1x.

The following files are included for this revision:

TRXNET.LAN            11,691        8-19-91        9:02a

Problem:

This version of TRXNET.LAN v2.00 fixes a bottleneck problem on a LAN with

mutilple file servers.  If a server goes down on the LAN, workstations trying

to communicate with the server through a bridge will retry, and "clog" the

network.  Symptoms of this problem include Abort Retries at the workstations.

Additionally, users may be unable to login because the Packet Receive

Buffers have reached the Maximum limit.

Solution:

This new TRXNET.LAN changes the MaxRetries from a default of 10 to 0. There is

little need to keep retrying a send to a node that does not respond in

any way to the Free Buffer Inquiry.  While the driver/card is retrying the

packet, all other sends to the driver are being queued.  Thus, the Maximum

Receive Buffer count may be driven up on busy networks where packets are being

routed internally to the ARCNET card. This may cause a bottleneck in the

system.

Installation:

1)     Copy TRXNET.LAN to your NetWare system start-up area (SYS:SYSTEM and/or

DOS partition). Be sure to rename the old version of the TRXNET driver.

2)     Load the driver as you normally would on your system.

Note: If needed, the Max Retry default may be changed during load time.

***************************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to any

NetWare software, and specifically disclaims any express or implied

warranties of merchantability, title, or fitness for a particular purpose.

Distribution of any NetWare software is forbidden without the express written

consent of Novell, Inc.  Further, Novell reserves the right to discontinue

distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use of the

software, loss of data, costs of re-creating lost data, the cost of any

substitute equipment or program, or claims by any party other than you.

Novell strongly recommends a backup be made before any software is installed.

Technical support for this software may be provided at the

discretion of Novell.

***************************************************************************