External Drivers pack for FX Uucico

===================================

WATTCP TCP/IP

=============

Preliminary TCP/IP driver for FX Uucico, based on Waterloo TCP/IP.

NOTE: This is not the definitive TCP/IP solution for FX Uucico. It

is a quick, dirty, cumbersome, akward, and ugly if you want,

workaround. But it works, and it works quite well.

In the first place, you need a Packet Driver. You can't do anything

without the appropiate packet driver for you hardware device. Yes,

you can use a Slip or PPP packet driver.

No dialer or script executer is provided. You would need one for calling,

and loggin in to you server on dial-up IP connections. There are several

ones widely available, and you can probably also use FX AT.

This driver is not a TSR. It establishes a connection to the remote host,

and then it invokes FX Uucico. When Uucico exits, the driver closes the

connection and return to the DOS prompt unloading itself from memory.

Type syntax is as following:

FXWTCP host uucico [ parameters]\r\n");

Where <host> is the IP address or the remote host name. Any resolvable

syntax is valid.

<uucico> is the name of the FX Uucico executable. If it is not in

the DOS 'PATH', you need to specify the full path.

<parameter> are any additional options to be passed to FX Uucico.

Before running FXWTCP, a packet driver must be loaded. And for dial-up

connections, the SLIP or PPP link should already be established through

some external dialer.

FX Uucico must be configured for a Fossil driver. No, it can't be used

as a generic TCP/IP Fossil driver. You also need a "WATTCP.CFG" file

unless you are running BOOTP.

There are several limitations in this release:

-   Client mode only, can't receive (listen) to incoming uucp calls.

-   Works only with the standard, TCP uucpd port (540). You can't drive

a Telnet connection.

-   You must specify the destination host in the driver's command line.

Can't be done at the 'systems' file.

-   There is no support for retries, or invoking more than one session.

And the more important drawback, you will get very limited support from

me. I didn't wrote the TCP/IP stack, nor know very much about it. For

question as "Configuring the WATTCP protocol", "Running a packet driver

under Windows or Desqview", you will have better luck asking somewhere

else. comp.os.ms-dos.mail-news is a good place to ask such questions.

WATTCP is the same stack used by the NNTP version of Waffle (but a much

older version though), you can probably share the same configuration.

Copyright (c) 1994 by Jorge Cwik

<jorge@satlink.net>

Portions of FXWTCP are Copyright (C) 1989, 1990, 1991, University of Waterloo