INSTALLATION INSTRUCTIONS FOR

WESTERN DIGITAL LAN ADAPTER BOARD DRIVERS

FOR USE WITH NOVELL ADVANCED NETWARE 2.1

This diskette provides drivers for the WD8000

families of Western Digital LAN adapter boards:

DRIVERS         WESTERN DIGITAL ADAPTER BOARD TYPES

WD8000            StarCard  (WD8000SL)

StarLink  (WD8000SH)

StarLink2 (WD8000SH2)

Included on this diskette are the following files:

INSTALL.DOC			This File.

RELEASE.DOC                  Release notes file.

WD8000.LAN			The LAN driver descriptor file.

WD8000AD.OBJ			The Server A-Driver object file.

WD8000BD.OBJ			The Server B-Driver object file.

WD8000CD.OBJ			The Server C-Driver object file.

WD8000DD.OBJ			The Server D-Driver object file.

WD8000SH.OBJ			The Shell Driver object file.

To install the WD drivers on the Novell NetWare 2.1 installation diskettes

copy the following files onto the specified Novell diskettes.

WD FILE      Copy To    Novell Diskette

-------      -------    ---------------

WD8000.LAN -----------> AUXGEN & SHGEN-1

WD8000AD.OBJ\

WD8000BD.OBJ \

WD8000CD.OBJ  >-------> LAN_DRV_001

WD8000DD.OBJ /

WD8000SH.OBJ/

Once the WD drivers are installed on the Novell diskettes proceed with the

normal NETGEN and SHGEN procedures. At the point in the procedures where you

are directed to select the LAN drivers the WD8000 drivers will appear in the

list of choices as follows:

WD StarCard, StarLink v2.1A