WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD SHELL DRIVER

FOR USE WITH

NOVELL ADVANCED NetWare, SFT NetWare, AND ELS NetWare LEVEL II

v2.1 AND LATER VERSIONS

INSTALLATION GUIDE

This disk provides a single shell driver for the entire "PLUS"

(WD8003, WD8013) family of Western Digital LAN adapter boards

for XT/AT and Micro Channel bus systems.

Included in this subdirectory are the following files:

INSTALL.DOC     This File.

RELEASE.DOC     Version number and specific

release information.

Included in the root directory are the actual driver files:

WDPLUSSH.LAN                 The LAN shell driver descriptor

WDPLUSSH.OBJ                 The shell driver object file.

NOTE

NetWare installation procedures require the driver files

listed above to be located in the root directory of a

floppy disk whose volume label is LAN_DRV_200.  If you

copy these files to another floppy disk, be sure that

you set the volume label of the new disk to LAN_DRV_200.

Refer to the NetWare Installation manual and follow the instruc-

tions for loading a new LAN driver from a floppy disk.

During software generation, you must choose "Load and Select

Item" from the "LAN Driver Options" menu.  At the point in the

procedures where you are directed to select the LAN drivers, the

WDPLUS drivers will appear in the list of choices as follows:

Western Digital Star/EtherCard PLUS

RAM BUFFER SIZE CONFIGURATION INFORMATION

When generating a shell for XT or AT bus adapters, there are

various configurations that can be selected, one of the para-

meters is the RAM buffer size. There are configuration selections

designating RAM sizes of 8, 16, and 32 Kbytes of RAM.  During

shell initialization the driver adjusts to the adapter once the

adapter's type and RAM size are determined.

Adapter's with 8 Kbytes of RAM can be used with configuration

selections which specify 8, 16, or 32 Kbytes of RAM.

Adapter's with 32 Kbytes of RAM must be used with configuration

selections which specify 32 Kbytes of RAM.

WD's AT bus 16 bit adapters can function in two modes, 16 bit

mode and 8 bit mode. In 16 bit mode, the adapter performs 16 bit

data transfers and utilizes the full 16 Kbytes of RAM. In 8 bit

mode, the adapter performs 8 bit data transfers and utilizes only

8 Kbytes of RAM.

To operate the 16 bit adapter in 16 bit mode, place the adapter

in a 16 bit bus slot and configure the driver to a configuration

selection which specifies 16 or 32 Kbytes of RAM.

To operate the 16 bit adapter in 8 bit mode, configure the driver

to a configuration selection which specifies 8 Kbytes of RAM or

place the adapter in an 8 bit bus slot and specify any driver

configuration.

NOTE

The drivers "Default" configuration specifies 8 Kbytes RAM and

therefore will cause the 16 bit adapter to function in 8 bit

mode.

SHELL DRIVER CONFIGURATION

This Western Digital shell driver has been designed to limit the

need to reconfigure or regenerate the IPX.COM file even though a

variety of WD adapters and adapter configurations may be present

in your workstations.

For adapters that utilize onboard jumpers to select I/O base

address, interrupt request level (IRQ) and RAM base address,

choose configuration selections which specify hardware settings

that meets your needs. Then configure your adapters to match the

hardware settings selected.

Software configured adapters include all Micro Channel adapters

and XT/AT bus adapters which are configured with the SETUP

program. The shell driver automatically detects these adapters

and reads the configuration information directly from the

adapter, even if the configuration entry selected specified a

different I/O base, IRQ, and RAM base.

Therefore, an IPX.COM file generated for a specific hardware

configuration will function with all adapters set to that

particular hardware setting. Plus it will function with all Micro

Channel and XT/AT bus software configured adapters because the

shell driver reads the configuration information directly

from the those adapter.

To create a driver which will function on software configured

adapters only, select the configuration entry for "First Software

Configured Adapter".

If your workstation has more than one software configured

adapter, select "First", "Second", "Third", or "Fourth Software

Configured Adapter" based on which adapter you want IPX.COM to

use. With XT/AT bus adapters the order (first, second, third,

fourth) is determined by the adapter's I/O Base Address, from

lowest to highest. With Micro Channel adapters order is deter-

mined by the Micro Channel bus slot number from lowest to

highest.

CUSTOM DRIVER CONFIGURATIONS

If an XT/AT bus workstation requires some combination of inter-

rupt, I/O Base and RAM address that is not offered in one of the

configuration choices, you can use the NWPATCH1 program or the

Novell utility JUMPERS to create a custom configuration to meet

your needs. NWPATCH1 is distributed with this driver, JUMPERS is

distrubuted by Novell.

STARLAN CONFIGURABILITY

This shell driver when used with WD StarCard PLUS adapter family

is default configured to use the same packet protocol used by the

Novell StarLAN driver for AT&T StarLAN adapters. This maintains

compatibility in a network with AT&T adapters using the Novell

driver.  This packet protocol limits the maximum packet size to

512 bytes and will not function if packets are routed across a

non-Novell bridge.

The drivers may be reconfigured using the NWPATCH1 program or the

Novell utility JUMPERS, to use the standard 802.3 packet protocol

with a maximum packet size of 1024 bytes. If your network

requires that you route packets across a non-Novell bridge you

MUST reconfigure your drivers to the 802.3 packet protocol.  If

your network uses only WD StarLAN adapters you can reconfigure

your drivers to take advantage of increased driver performance

due to the increased maximum packet size. Remember, when changing

the packet protocol the workstation drivers must be reconfigured.