PLPHPLP.DOC

----------

Protected mode P386 ADI LongPlot driver for Hewlett Packard Plotters

Supported plotters include:

Hewlett Packard 7220, 7470, 7475, 7550, 7580, 7585, 7586,

ColorPro, DraftPro, DraftPro DXL, DraftPro EXL, and

DraftMaster I HP-GL pen plotters.

Hewlett Packard 7586B & 7596A roll feed, long axis capable plotters.

PLPHPLP.EXP V1.00 for AutoCAD 386 (R)

Copyright 1990-91, Autodesk, Inc.

Revision 1.3

Mar. 16, 1991

/******************************************************************************

*                                                                             *

*                  Copyright (C) 1990-91 by Autodesk, Inc.                    *

*                                                                             *

*       Permission to use, copy, modify, and distribute this software and     *

*       its documentation for the purpose of creating applications for        *

*       AutoCAD, is hereby granted in accordance with the terms of the        *

*       License Agreement accompanying this product.                          *

*                                                                             *

*       Autodesk makes no warranties, express or implied, as to the           *

*       correctness of this code or any derivative works which incorporate    *

*       it.  Autodesk provides the code on an ''as-is'' basis and             *

*       explicitly disclaims any liability, express or implied, for           *

*       errors, omissions, and other problems in the code, including          *

*       consequential and incidental damages.                                 *

*                                                                             *

*       Use, duplication, or disclosure by the U.S.  Government is            *

*       subject to restrictions set forth in FAR 52.227-19 (Commercial        *

*       Computer Software - Restricted Rights) and DFAR 252.227-7013 (c)      *

*       (1) (ii) (Rights in Technical Data and Computer Software, as          *

*       applicable.                                                           *

*                                                                             *

******************************************************************************/

INTRODUCTION

This document explains the installation and use of PLPHPLP.EXP, a protected

mode Autodesk Device Interface (ADI) driver designed to run

Hewlett-Packard plotters (listed above) with AutoCAD 386.

Please read the AutoCAD 386 Installation and Performance Guide concerning

plotting and printer plotting with Hewlett-Packard plotters and printers,

the AutoCAD 386 Reference Manual section on plotting, and your plotter

manual.  This driver was developed by Autodesk, Inc. and does not represent

an endorsement of any particular product.

ADI Drivers -

This driver is a protected mode ADI plotter driver.  ADI is an acronym for the

Autodesk Device Interface specification that was used to create this

driver.  This driver was tested using AutoCAD Release 11.

****************************************************************************

COPYING THE DRIVER SOFTWARE

Here's how to copy the driver software onto your hard disk.

The example assumes an AutoCAD directory is on drive C and named

\ACAD and the distribution disk for the driver is in drive A:

1.  Change to the directory where you store AutoCAD

C:> CD ACAD [Return]

2.  Insert the ADI Drivers update diskette in drive A.

3.  Copy the driver files to your hard disk:

C:> COPY A:\PLPHPLP\PLPHPLP.*  [Return]

CONFIGURING AUTOCAD 386 FOR THE PLPHPLP PLOTTER DRIVER

--------------------------------------------------

In AutoCAD 386, Release 11, only a single ADI driver will be

recognized at configuration time.  The configuration menu for the

plotter devices will always display a P386 ADI driver as a choice.

The driver that is selected is determined through the use of the

PLPADI environment variable and/or the ADIPLOT.EXP magic file name.

If the user picks the P386 ADI driver choice, AutoCAD will search,

first for the PLPADI environment variable which names the driver (with

either a fully qualified path name or just a file name) and, failing

that, searches for the ADIPLOT.EXP magic file name.  Both searches

start in the current default directory.  If the driver is not found,

AutoCAD then searches the following directories for the filename in

the order given:  ACADCFG configuration, ACAD execution and, AutoCAD

command line execution prefix.  If the driver is still not found,

AutoCAD will ask you which disk drive and directory it should search

to find the driver.

The preferred means for AutoCAD to use this ADI driver are:

1) Use the set command with the environment variable PLPADI and a complete

path name, for example if plphplp.exp is in the \acad\device directory

you would type:

SET PLPADI=\ACAD\DEVICE\PLPHPLP.EXP

You may want to add a similar line to your autoexec.bat file.

or

2)  Copy plphplp.exp to the file name adiplot.exp.  This file name is

unique and will be used when AutoCAD is loaded if you have selected a

P386 ADI driver during configuration.

COPY PLPHPLP.EXP ADIPLOT.EXP

Both of these methods require the .exp file to be located in a directory

accessible to AutoCAD.  Please see the first paragraph of this section.

It is necessary to configure AutoCAD 386 for a protected mode ADI DRIVER.

You will need to do this before you use the PLPHPLP ADI driver for the first

time.  AutoCAD will save these settings until you change them.  Refer

to your AutoCAD Installation and Performance Guide for a more

in-depth explanation of the steps below.

To configure AutoCAD for an P386 ADI plotter driver, select from the

AutoCAD Main Menu:

Option 5 - Configure AutoCAD

Your current configuration will be displayed.  Press RETURN.

Then select from the Configuration Menu:

Option 5 - Configure plotter

Select ADI P386 Plotter from the list of 10 or more plotters.

You will be shown the following table of supported plotters.

1.  7220

2.  7470

3.  7475

4.  7550

5.  7580

6.  7585

7.  7586

8.  ColorPro

9.  DraftPro

10.  DraftPro DXL

11.  DraftPro EXL

12.  DraftMaster I

13.  7586B & 7596A Roll Fed

Enter selection, 1 to 13 <1>:

Note: There are no late model Hewlett Packard plotters shown.  Those

devices, the HP 7600, the HP DraftMaster RX, SX or MX, the HP PaintJet XL w/

HP-GL/2 cartridge, or the HP LaserJet III, are supported by the ADI

P386 HP-GL/2 driver PLPHPGL2.EXP.  If you have one of those devices,

please use the PLPHPGL2.EXP driver available from Autodesk.

Select your plotter from the list.  If you want to do long axis plotting,

you need to select:

13) 7586B & 7596A Roll Fed

You will be then asked two questions:

This device can do long axis plots of from 44" to 600" long.

If you wish produce 44" or longer plots , an additional question will

be asked at run time configuration:

Do you wish to do long axis plotting? <N>.  Answer Y(es) or N(o).

If you answer yes, you can do plots of up to 50' (600").  Otherwise you

will only be able to do plots up to 44" long.  You may reconfigure

at any time and select long axis plotting.

You will then go through the standard AutoCAD configuration dialogue.

NOTE:  Calibration and Long Axis plotting:  We have limited the

amount of axis calibration to 5% during a long axis plot.  Or

one-half inch (0.5") inches in ten inches (10").  If your plotter

is more than 5% out of calibration we cannot assure accurate

long plots.  Please have it re-calibrated by your dealer.

Finally, if you selected long axis plots, you will be asked the

width of the roll of paper you are using.  The message:

Roll widths:

1) 11"

2) 24"

3) 36"

How wide is your roll, 1 to 3 <1>:

will be displayed.  Enter the number that matches the width of

your paper roll.

If you have any questions see the Configuration section of the

Installation & Performance Guide to AutoCAD.  Also see the AutoCAD

Reference Manual for AutoCAD details, and your plotter's User

guide for plotter details.

AutoCAD will now be configured to plot using the P386 ADI driver.

PLOTTING ON A STANDARD SIZE PAPER (Sizes A-E)

---------------------------------------------

From the drawing editor issue the Plot command or from AutoCAD's

Main Menu select 3 - Plot a drawing.  Make sure your plotter is cabled

correctly, is turned on, and is On-Line.

PLOTTING A LONG AXIS PLOT (UP TO 50 feet)

-----------------------------------------

Note:  You must have answered yes to the long plotting question

asked during the plotter configuration dialogue.

**********************************************************************

AutoCAD doesn't directly support Longplots. AutoCAD can plot only

65,535 plotter steps in each direction, which is about 64" for Hewlett

Packard plotters which use 1016 steps per inch.  Longplots are done by

rescaling the plot data sent to the driver, magnifing the data sent to

the plotter at a small cost in plot resolution.

***********************************************************************

Long axis plots require that you must have a black 0.3mm

fiber-tip pen (for plotter paper) or a 0.35mm drafting pen

(for vellum or polyester film) in pen stall #8.  Only these pens enable

the plotter to detect the registration marks used for frame-to-frame

alignment.

The double matte polyester film is the only media type that will guarantee

precise registration between the multiple frames used to produce long axis

plots.  This is because non-polyester media will expand and contract due to

changes in temperature and humidity. Part of the media will be

exposed to the air and part of it will be rolled up.  This means that

slight gaps may appear between one frame and the next if you use paper,

vellum or other non-polyester films.  But polyester film is expensive and

we recommend as Hewlett Packard does, using it only for archive storage

or high accuracy applications.  Please see your Hewlett Packard user guide

for more details.

Start your plot in the normal manner, from AutoCAD's drawing editor,

issue the plot command or select the plot option (#3) from AutoCAD's

main menu.

Specify the part of the drawing you wish to plot by answering the prompt:

What to plot -- Display, Extents, limits, View, or Window <D>:

AutoCAD displays the basic plot specifications and asks if you want to

change any of them:

Do you want to change anything? <N>  -- Answer <Y>es

The following prompt will appear with the currently selected X-axis

length.

X-axis paper length for this long plot? (Inches) <XX">

If you want a different value than XX", enter it at this time.  We

only allow inches here.  If you have entered a new value we will post

the following reminder;

Don't forget to change the Width for Plotting Size to YY".

where YY" is the length of the new long axis plot width.

Note: that you must enter a legal value here, otherwise the prompt will

be repeated.  A legal value is numeric, greater than 0 and less than

600".  The driver does this here so that it can lie to AutoCAD.  It would

be to late if we waited for the run time configuration to be completed.

When you come to the question below you MUST enter the x-axis paper-length

for long plotting you entered  earlier as the first value.

Enter the Size or Width, Height (in units) <default>:

We must change the width here to what we've entered above, because

that width displayed is used to calculate how much to lie to AutoCAD.

The Height you enter here size of the paper roll mounted in

your plotter.  It will be 35" or less.

For example, you have a 500' roll of 35" wide paper in your plotter.  You

want a long plot 300 inches long.  You will enter the plot length of 300"

twice:  once at the question of x-axis page length and secondly at the size

question -- Enter the Size or Width, Height (in units) < >.  Here

300",35" would be correct.

Then, when you come to this question:

Specify scale by entering:

Plotted units=Drawing units or Fit or ? <default>:

Enter your standard scale factor as you always do.  If you have a 300'

drawing and are plotting to 300"'s it is 1=12.

AutoCAD will then prompt you to ready your hardware and the vectors will

be output.  In this case, all the vectors are sent to a file that is

resent for the number of frames the plot may require.  The driver

will post a message:

"Is the device ready for the Long Axis output?" <Y>

The user can cancel the long axis plot at this point.  If the plot is not

cancelled, the driver will start sending data to the plotter.  We will

post a message updating the plot output.  A typical message for a 4 frame

plot would look like:

Doing Frame #1 of 4 frames.

Doing Frame #2 of 4 frames.

Doing Frame #3 of 4 frames.

Doing Frame #4 of 4 frames.

This message tells the user which frame is currently being plotted and

how many frames will make up the long axis plot.  Long plots can take a

long time, be patient.

IMPORTANT!! - SPECIAL PLOT SCALING REQUIREMENTS FOR LONG PLOTS

In order to create long-plots, the driver has to magnify the plot

sent from AutoCAD. This question (asked by AutoCAD) must be answered

with a scale factor to get accurate long plots, otherwise the resulting

plot is may be incorrect.

Let's start by considering the relatively simple situation where you

want the plotted size to be the same as the AutoCAD drawing editor

size you selected.  We'll assume you would want to plot a scale of 1:1

if AutoCAD were able to create long plots directly.

You have a drawing that is 100" long and you want a 100" plot.  Your

scale factor would be 1"=1".  But what if drawing is 300 yards long and

you want the plot to be 300".  Your scale would be 1"=3'.  For every

inch of Plotted units you need 3' of Drawing units.

The ADI driver will automatically scale your drawing to

fit within the desired length. This length will also control how

far the plotter advances the paper at the end of the plot.

The driver will send all the vectors off to the computers

hard disk and then when AutoCAD has finished the plot, the driver will

reread the file and the plotter will automatically advance the page to

plot each frame.

For those who are curious, the driver actually will figure the

magnification factor, M, by which to scale plot data by (assuming inches)

using the equation:

M = (X-Axis length * plotter steps / AutoCAD's plot-space) + 1.

The AutoCAD plot-space is 65,535 steps and the plotter steps is 1016

steps/inch. for all long plot devices.

If you want a 300 inch long plot the factor M would be:

M = (300 * 1016 / 65,535) + 1

M = 5.65 or

M = 5 after truncation.

The driver modifies the plotter units (in steps/inch) by dividing the

default units (1016) by M (5) in this case.  Then the

plotter scales up all plot data by multiplying by 5. The plot is magnified

by this factor of 5 in both X and Y directions.

STOPPING THE PLOT

-----------------

If you wish to stop the long axis drawing while AutoCAD is sending

vectors to the plotter, abort it in the normal manner (Control C).

NOTES ON OPERATION

------------------

Here are some tips for using the PLPHPLP P386 ADI driver with AutoCAD:

1.  The plotter must be connected, turned on, and on line before

you plot using the PLPHPLP P386 ADI driver.

2.  Do not turn the plotter on and off in the middle of a

plotting session.  An error message will be displayed if you try to

use the PLPHPLP P386 ADI driver while the plotter is turned off.

3.   To send a plot file to the plotter, type the following at the

DOS command line:

copy 'filename' lpt(n):     OR  copy 'filename' com(n):

where filename is the name of the file you wish to plot and lpt(n) or

com(n) represents the port you have connected to your plotter.  If using

the serial (com) port, you will first have to issue a mode statement to

match the serial configuration of your plotter.

If you are using serial port 1 (com1:) and

the DOS copy command as shown above, the mode command is:

MODE COM1:9600,e,7,1

or 9600 baud, even parity, 7 data bits, and 1 stop bit.  Please see your

DOS manual for more information on the use of the mode command.

4.  Disk Space Considerations: If you wish to plot to file,

please insure that you have sufficient disk space to do so.

File sizes are not dependent on the size of AutoCAD's DWG file, rather

they depend on the contents of that file.  CHROMA.DWG, while only

12,359 bytes as distributed with AutoCAD, expands to almost 240k bytes

when plotted to file.  This is due to the amount of fill the

plotter must do to plot this file.  Each solid filled object

in the drawing requires that the driver send pen movement information

sufficient to fill the object.  Simple line drawings will require much

less disk space.

PLOTTER SPECIFIC INFORMATION

----------------------------

Long plots require a roll of paper to be mounted in your Hewlett

Packard plotter.  Please see the Hewlett Packard manual that came with

your device for more information.  You may want to obtain a copy of the

Hewlett Packard Plotter Notes brochure entitled "Long Axis Plotting with the

HP 7586B".

Hardware Requirements:

----------------------

The hardware requirements specific to using the PLPHPLP plotter P386 ADI

driver with AutoCAD.

o A serial card and cable is required for plotting to all these

plotters.  A custom cable may be required but all the needed

hardware information is in the AutoCAD Release 11 386 Installation

& Performance Guide (AutoCAD R11 386 IG) in the plotter chapter's

subsection on the use of Hewlett-Packard plotters.

AutoCAD, AutoSketch and ADI are registered trademarks of Autodesk, Inc.

IBM, PC, XT, AT are trademarks of International Business Machines Corporation.

HP is a trademark of Hewlett-Packard Corporation.