========================================================================

ADIHDG.DOC

The document file for Real mode ADI driver(ADIHDG.COM / Ver 3.0)

========================================================================

The file ADIHDG.COM is a memory resident program that allows a

Hitachi tablet listed below to interface with AutoDesk programs  such as

AutoCAD, AutoSketch, AutoShade, or any other AutoDesk application software

that can use a real-mode ADI digitizer driver.

HDG-1111C

HDG-1212D

HDG-1217D

HDG-1515D

HDG-1717D

HDG-2222/2222S/2222T

HDG-2436/2436S/2436T

HDG-3648/3648S/3648T

HDG-4460/4460S/4460T

HDG-1212E

To install the ADIHDG driver, you need to set the functions of Tablet

by DIP swiches or Menu sheets.

1. For HDG-1111C with Pen/4-B cursor

DSW1 - 1, 3 and 4 ; ON

DSW2 - 3 and 5    ; ON

DSW3 - All        ; OFF

If 12-B cursor is used, DSW3-4 must be ON.

(Note) The HDG-1111C will be set as;

0.025mm, REMOTE mode, Lower-left origin,

9600bps, 7-data, Even parity, 1-stop

ASCII format(Hitachi)

2. For HDG-1212D, 1515D, 1717D and 1217D

DSW1 - all  ; OFF

(Note) The D-series tablet will be set as;

0.025mm, REMOTE mode, Lower-left origin,

9600bps, 7-data, Even parity, 1-stop

ASCII format(Hitachi)

3. For HDG-2222 - HDG-4460 with DP004 I/F board

MODE/RATE(DIP switch)   - all  ; OFF

BAUDRATE(DIP switch)    - 8    ; ON

Jumper pin on the board - 1S and ASYNC(2) ; ON

(Note) The large tablets with I/F board(DP004) will be set as;

0.025mm, REMOTE mode, Lower-left origin,

9600bps, 7-data, Even parity, 1-stop

ASCII fotmat(Hitachi)

4. For HDG-2222S - HDG-4460S with DP401 I/F board

DSW1 - all     ; OFF

DSW2 - 2 and 4 ; ON

DSW3 - 1 and 2 ; ON

(Note) The large tablets with I/F board(DP401) will be set as;

0.025mm, REMOTE mode, Lower-left origin,

9600bps, 7-data, Even parity, 1-stop

ASCII fotmat(Hitachi)

5. For HDG-2222T - HDG-4460T

HITACHI-1 Default

(Note) The T-series large tablet will be set as;

0.001", RUN mode, Lower-left origin,

9600bps, 7-data, Even parity, 1-stop,

ASCII format(Hitachi)

6. For HDG-1212E

HITACHI Default

(Note) The HDG-1212E will be set as;

0.025mm, REMOTE mode, Lower-left origin,

9600bps, 7-data, Even parity, 1-stop

ASCII format(Hitachi)

You can type the file name followed by command line arguments that tell the

driver the tablet type and configuration, or modify AUTOEXEC.BAT file so the

driver is loaded whenever you start your PC.:

ADIHDG [/n] [/M#####] [/C#] [/P#] [/U]

[/n]	is a one-digit number that specifies which serial port is being used

to communicate with the tablet.

If no port argument is given, the ADIHDG driver will be installed

on COM1.

/1  designates COM1    (Default)

/2  designates COM2

[/M#####]   The second argument selects the tablet model being used:

/M1111C selects the HDG-1111C

/M1212E selects the HDG-1212E   (Default)

/M1212D selects the HDG-1212D

/M1515D selects the HDG-1515D

/M1717D selects the HDG-1717D

/M1217D selects the HDG-1217D

/M2222S selects the HDG-2222/2222S

/M2436S selects the HDG-2436/2436S

/M3648S selects the HDG-3648/3648S

/M4460S selects the HDG-4460/4460S

/M2222T selects the HDG-2222T

/M2436T selects the HDG-2436T

/M3648T selects the HDG-3648T

/M4460T selects the HDG-4460T

If no /M##### is given, the driver assumes a default HDG-1212E.

[/C#]	is a designator for the type of cursor or stylus being used

by the driver:

The default cursor type is the /C4 (4-button cursor).

/CP	selects a PEN or stylus

/C4	selects a 4-button cursor    (Default)

/C12	selects a 12-button cursor

/C16	selects a 16-button cursor for HDG-1212E and Large

tablet only.

[Pointing Device Button Definitions]

The following lists the digitizer button functions that are executed when

using the ADIHDG driver with Standard configuration(i.e., unmodified acad.mnu

file) of AutoCAD.

This button mapping may be modified by editing the acad.mnu file.

For instructions to modify the acad.mnu file, refer to the AutoCAD "Reference

Manual" supplied with your AutoCAD.

Button              AutoCAD Function

Stylus Pen          Tip switch          Pick

Barrel Switch       Carriage Return

4-button cursor     Yellow              Pick

White               Carriage Return

Blue                Displays Pull-down Menu #1(Tools)

Green               Cancel

12-button cursor    1                   Pick

2                   Carriage Return

3                   Displays Pull-down Menu #1(Tools)

4                   Cancel

5                   Snap On/Off

6                   Ortho On/Off

7                   Grid On/Off

8                   Coordinate display On/Off

9                   Isoplane Top/Right/Left

*                   Tablet On/Off

0                   Not defined

#                   Not defined

16-button cursor    1                   Pick

for HDG-1212E       2                   Carriage Return

3                   Displays Pull-down Menu #1(Tools)

4                   Cancel

5                   Snap On/Off

6                   Ortho On/Off

7                   Grid On/Off

8                   Coordinate display On/Off

9                   Isoplane Top/Right/Left

A                   Tablet On/Off

0                   Not defined

B,C,D,E and F       Not defined

16-button cursor    1                   Pick

for Large tablet    2                   Carriage Return

3                   Displays Pull-down Menu #1(Tools)

4                   Cancel

5                   Snap On/Off

6                   Ortho On/Off

7                   Grid On/Off

8                   Coordinate display On/Off

9                   Isoplane Top/Right/Left

*                   Tablet On/Off

0                   Not defined

#,A,B,C and D       Not defined

[P#]		This argument allows you to assign the same function button

as HICOMSCAN driver under AutoCAD if [P0] is used.(The pick

button will be "0" button of 12-B cursor.)

This argument is valid only when 12-B cursor[/C12] or 16-B

cursor[/C16] is used.

If no [P#] is given, the pick button will be "1".

/P1  -- The pick button will be the "1" button. (Default)

(See the above function table)

/P0  -- The pick button will be the "0" button.

(Same function as HICOMSCAN driver)

(12-button cursor)

1                   Carriage Return

2                   Displays Pull-down Menu #1(Tools)

3                   Cancel

4                   Snap On/Off

5                   Ortho On/Off

6                   Grid On/Off

7                   Coordinate display On/Off

8                   Isoplane Top/Right/Left

9                   Tablet Off

*                   Not defined

0                   Pick

#                   Not defined

[/U]	This argument allows you to remove the driver from memory.

For example, if you want to install ADIHDG.COM with the HDG-1217D on COM1 and

12-button cursor, follows the step listed below.

- Insert "Hitachi drivers" diskette into A:.

- Type A: and press Enter key.

- Type as follows at DOS prompt(A:>).

A:>ADIHDG /M1217D /C12

After you install the ADIHDG driver, you are ready to start AutoCAD or

other AutoDeask's software products that support Real mode ADI digitizer.

And you may need to run the CONFIGURE program within AutoDesk's software

to select "Real mode ADI digitizer" as input device. To configure the Real

mode ADI driver, select "AutoDesk Device Interface" in the digitizer

configuration menu.

(Trademark)

ADI, AutoCAD and AutoShade are trademarks of Autodesk,Inc.##