ADI Driver for HP ThinkJet Printers

ADITJET.COM  is a TSR (Terminate and Stay Resident) program  that

uses the AutoDesk Device Interface (ADI) specification to drive a

Hewlett  Packard ThinkJet printer.  Switch 5 on the back  of  the

printer should be down (HP mode).  To use this program, you  must

load  it  into memory before you use AutoCAD or  AutoSketch.   To

load  it into memory, make the drive and/or directory  containing

ADITJET.COM the default drive/directory, and enter

ADITJET

at the system prompt, and press return.  This should be done once

each time you boot your computer.  If you want to unload  ADITJET

from memory later, enter

ADITJET U

and press return.

Before  you can use ADITJET.COM for the first time, you must  con-

figure  AutoCAD  or AutoSketch.  If you use AutoCAD,  select  the

"Configure  printer plotter" option from the configuration  menu,

and  select  "ADI  printer plotter" from the  list  of  available

devices.   (You  will need your disk of  printer  plotter  device

drivers  unless you have copied them to your hard disk.)  If  you

use AutoSketch, start it by entering SKETCH /R, and when you  get

to  the  Select plotter menu, select item 3.  Then,  with  either

program, answer the questions asked as shown below.  (Note:  With

AutoCAD, The first two questions are asked later, after the "dots

per inch" questions.)

Select output format: 2

Interrupt number in hex <7B>: (press Return)

Specify plot size in millimeters <N> (press Return)

Maximum horizontal (X) plot size in inches? <11.0> 6.66

Printer dots per inch in the horizontal (X) direction <100.0> 192

Maximum vertical (Y) plot size in inches? <8.5>

Answer  10.5  to the above question if you want  to  confine

your drawing to a single sheet of paper, or any larger  num-

ber  if  you  want to allow it to "spill  over"  onto  other

sheets.

Printer dots per inch in the vertical (Y) direction <100.0> 96

Does the printer/plotter support color? <N> (press Return)

Note: Some versions of AutoCAD will not ask the above  ques-

tion.

The  rest  of the configuration for AutoCAD is the  same  as  for

supported printers, and you can consult your AutoCAD installation

manual for more information.  When you are asked

Rotate plot 90 degrees? <N>

type  Y if you have a drawing that is wider than it is  high  and

you  want it to fill the page as much as possible, especially  if

1

you want a very wide drawing to spill over more than one sheet of

paper.  When you plot a drawing, you will have the opportunity to

change this and some of the other settings.

When  you  want to plot with AutoSketch, set up a plot  box,  and

fill  in the X and Y sizes yourself.  Be sure that they  are  not

greater  than the X and Y values you entered when you  configured

the  program.   If you have a drawing that is wider  than  it  is

high,  you will have to turn on the "Rotate plot 90 degrees"  op-

tion if the plot is to fill the paper.

2

########################################