General Instructions for

Patrick Swayne's ADI Printer Drivers

Introduction

This  is  a collection of drivers that  allow  several  different

printers to be used as printer plotters with AutoCAD (version 2.5

or  higher) or AutoSketch (version 2 or higher).  Most  of  these

printers  are  not directly supported by AutoCAD  or  AutoSketch.

Epson FX series printers are supported, but the FX driver in this

collection  provides much higher resolution.  These  drivers  use

the Autodesk Device Interface specification, and are accessed  by

AutoCAD and AutoSketch as ADI printer plotters.

Here  is  a list of the printer driver files supplied,  with  the

printers they support.

Driver              Printers supported

ADIEPFX.COM         Epson FX series and comp. (incl. Alps P2000)

ADIEPLQ.COM         Epson LQ series and compatible 24-pin prtrs.

ADIEPLQL.COM        "

ADIEPLQW.COM        "

ADIEPMX.COM         Epson MX series printers with Gaftrax Plus

ADIIDS.COM          IDS Prism P80 and P132 with Dot Plot

ADIMPI.COM          MPI 150, 99G, and Sprinter

ADITJET.COM         Hewlett Packard Thinkjet

SWEETP.BAS          A  BASIC  program that allows you  to  use  a

Sweet  P model 100 plotter or  Heath  IR-5208

plotter with AutoSketch.

There  is also a program called ADITEST.COM that can be  used  to

test the driver you choose with your printer.

This  collection also includes a driver that lets you use a  Sum-

maSketch graphics tablet with AutoSketch.  The documentation  for

this  driver,  which also details the files included  in  it,  is

called ADISUM.DOC.

The  specific instructions for using each printer driver  are  in

a .DOC file accompanying each driver.  For example, the  instruc-

tions  for ADIEPFX.COM are in ADIEPFX.DOC.  The file  ADIEPLQ.DOC

contains   instructions   for  ADIEPLQ.COM,   ADIEPLQL.COM,   and

ADIEPLQW.COM

Preparations Before Printing

Before you print a drawing using one of these drivers, you should

prepare your DOS printer driver.  These drives are all set up  to

use printer port LPT1.  (You can change the PORT number  (0=LPT1,

1=LPT2, etc.) in the assembly source code for a driver and  reas-

semble  it  if you need to use another port.)  If you  are  using

Zenith MS-DOS, and you have a parallel printer, run Configur  and

select  "Configure  an LPT device".  Select  "Configure  parallel

device" from the next menu, and LPT1 from the next menu.   Answer

1

N  to  "Strip parity on output" and "Map lower case to  upper  on

output".)   Enter  0 for the number of pad  characters,  and  any

number  you  desire for the timeout value (20 is  normal).   Then

write  the  new configuration to memory and disk.   Note:   These

drivers  set  the  timeout value to the maximum  while  they  are

running, and restore it when they are done with a plot.

If you are using a serial printer with Zenith MS-DOS, you can map

the COM port you are using to LPT1 with Configur.  If you use the

User Defined option when you configure your COM port, be sure  to

answer N to the Strip and Map questions, and set the timeout to a

high value (100 or more).

If you are using either of my programs FASTPRN.COM or  MAPLPT.COM

to drive a serial printer, type

FASTPRN ,,,,,,200

or

MAPLPT ,,,,,,200

and  press Enter.  You only need to enter the FASTPRN  or  MAPLPT

command  once  each time you boot up your computer, and  you  can

place the command in your AUTOEXEC.BAT file, if you wish.

Testing a Driver

You  can  test one of these drivers to see if it will  work  with

your  printer without using AutoCAD or AutoSketch.  Just run  the

driver program (which loads the driver into memory), and then run

ADITEST.COM.  For example, if you wanted to test ADIEPFX.COM, you

would  first make the drive/directory containing ADIEPFX.COM  and

ADITEST.COM  the  default, and enter these commands  at  the  DOS

prompt, pressing Enter or Return after each one.

ADIEPFX

ADITEST

If  the driver and your printer are working correctly, two  wedge

shapes will be printed on your paper, followed by a tiny box that

has the word TEST in it.  The size of the wedges and the box will

depend  on the resolution of your selected driver, and  the  word

TEST may be so small that you will need a magnifying glass to see

it.

If  you  see  the message "Printer reported a  problem"  on  your

computer screen, it could mean that your printer is off line, not

connected, etc.

Distribution

These copyrighted programs are provided free of charge.  They may

be  distributed freely as long as no charge is made for the  pro-

grams  themselves,  or any software package with which  they  are

2

included.  The programs and all related documentation files  must

be  distributed  together, and may not be  modified.   A  nominal

charge may be made for the disk containing these programs.

Warranty

These programs are provided AS IS without any warranty, expressed

or implied, including but not limited to fitness for a particular

purpose.

Disclaimer

In no event will Patrick Swayne or the Heath/Zenith Users'  Group

be  liable  to you for any damages, including any  lost  profits,

lost savings, or other incidental or consequential damages  aris-

ing out of the use of or inability to use these programs, even if

Patrick Swayne or the Heath/Zenith Users' Group has been  advised

of the possibility of such damages, or for any claim by any other

party.

3

########################################