This program produces an automatic via at (0,0) then makes it into

a block to be used later.  The following file setups need to be adhered to

1)	Make a layer called PAD, FAB.

2)	"LOAD" the PCB_PADS shape file.

3)	Have the "autovia.lsp" program in the current directory.

4)	type "(load "autovia")" this will run the autovia program.

At the "Enter name of shape to use for pad:" enter the name of one

of the shapes (ie.. S070 (square pad .070 inch square), R080 (round pad

.080 in round), or O100 (oval pad .100 long)).  The shape file has these

scaled so when they are placed they do not need to have the scale changed.

At the "Enter the hole diameter:" prompt type in the diameter of the

hole you would like to.  This part of the program will create an attribute

called "hole" it will be place at (0,0).

The last prompt, "Enter name of via file:" type in the name you would

like to save this file as, (ie.. via070r (via, .070 diameter, round)).  This

part of the program creates a "block".

If you would like to save the "block" file to disk type wblock and

follow the prompts.  The ACAD manual covers the subject if you have any

difficulties.

I hope this will be of use to someone.  If you have an interest in

using AutoCAD to produce printed circuits boards or have a different way to do

this please contact me,  my mail name is RWWALLACE.##################