Autodesk Product Support                              February 28, 1991

Erratum 17 Page Locking ADS Program for AutoCAD(R) 386/Release 11

Systems running AutoCAD 386 / Release 11 may hang if they contain

an Intel B1-step 80386 processor manufactured in 1985 and 1986.

Erratum 17 Problem Statement

AutoCAD 386 / Release 11 will experience system hangs on "B" step

386 DX CPU systems because the hardware/software configuration does

not contain the workaround for Erratum 17 of the "B" step 386 DX CPU.

If the first byte of a coprocessor (ESC) instruction is located on the

last byte of a page or segment, and the second byte is located on a page

or segment which would create a fault, then the processor will hang when

it tries to signal the fault.  The processor remains stopped until an

interrupt, NMI, or RESET occurs.  This erratum applies only to

coprocessor instructions in systems which use virtual memory.

How To Identify Whether a System Uses a "B" step 386 DX CPU

AutoCAD 386 / Release 11 will have problems with Erratum 17 only

with systems that use early, "B" step 386 DX CPU's, which should be

found only in certain 16 and 20MHz systems.  However, we have seen a

few clone machines which have used 20MHz chips in 25MHz implementa-

tions.  AutoCAD 386 / Release 11 will not have Erratum 17 problems with

newer systems that use "D" step 386 DX CPU's since the erratum was

removed from that stepping.

Autodesk is providing its Dealers and customers with a simple

program to determine if a system uses a "B" step 386 DX CPU.  At the

DOS prompt, execute the program CHKBSTEP.COM.  If your system contains

a "B" step 386 DX CPU, the following message will be printed to the

screen:

"This system is using a 386 DX B1-step part".

If your system does not contain a "B" step 386 DX CPU, the

following message will be printed to the screen:

"This system is not using a 386 DX B1-step part".

Only if your system contains a "B" step 386 DX CPU will you have

problems with Erratum 17.

Fixing the Problem

Autodesk is providing its Dealers and customers using AutoCAD 386

/ Release 11 on "B" step 386 DX CPU systems with a software program to

work around Erratum 17 problems.  This program, ERRATA17.EXP, is an

ADS (C Binding) application which scans the ACAD.EXE file and locks

down any code pages which may cause the processor to exhibit Erratum

17.  This fix only works for ACAD.EXE itself.  It will not help if the

Erratum 17 condition is caused by an ADI (Autodesk Device Interface) or

ADS application.

ERRATA17.EXP will lock into memory those pages which potentially

hang AutoCAD 386 on older 80386 CPU's that exhibit this problem.  There

is no advantage to using this program if you are not experiencing this

problem.

The first time ERRATA17 is run it will scan your ACAD.EXE and

build a table of memory locations that cause the problem, print them out,

and lock them into memory. This takes about 15 seconds.  Future

executions scan the table (ERRATA17.CFG) and lock the pages.  This

takes about 2 seconds.  If you ever change AutoCAD executables, the

program will recognize this and rescan the executable.

The default location for the configuration file, ERRATA17.CFG, is

the directory in which the ADS program, ERRATA17.EXP, was found.

Note:  ERRATA17.EXP has no commands.  Simply (xload)ing it causes

it to run.

It is only necessary to run this program once per AutoCAD session,

not to leave it loaded.  You may run it and remove it via your ACAD.LSP

file with the following lines:

(xload "ERRATA17")

(xunload "ERRATA17")

Some multi-tasking DOS configurations (with SHARE.EXE and 3COM

loaded, for example,) may cause problems when ERRATA17.EXP attempts

to open for reading an already executing ACAD.EXE file.  If you see the

message "Unable to open ACAD.EXE.", remove the offending program and

rerun ERRATA17.  After the first run, you should be able to return to

your standard configuration, since ERRATA17 will use the ERRATA17.CFG

file instead of opening ACAD.EXE.  Repeat this process if you ever

change AutoCAD executables.

Autodesk provides this information as a service to our Dealers and

customers.  We do not endorse or guarantee the suitability of the fixes

described above.  We advise you to check with your computer

manufacturer or supplier to determine the appropriate solution.

Erratum 21 Problem Statement

AutoCAD 386 / Release 11 will experience system hangs on certain

"B" step 386 DX CPU systems because the hardware configuration does

not contain the workaround for Erratum 21 of the "B" step 386 DX CPU.

This erratum erroneously forces a high state on Pin A31 of its address

line, when all of the following are true:

o  the data bus (dma channel) is put on hold

o  a 32-bit operation or instruction is being performed

o  virtual memory / paging is being used.

When the state on Pin A31 is forced from low to high in

conjunction with a floating-point operation, the system will usually hang.

Basically, each chip is waiting for the other to complete an operation,

and the system locks.

Note: Both 16MHz and 20MHz B-step chips are affected.

How To Identify Systems without the Erratum 21 Workaround

Unlike the Erratum 17 problem, only certain "B" step 386 DX CPU

systems will cause Erratum 21 problems with AutoCAD 386 / Release 11.

These systems do not contain the hardware workaround for Erratum 21;

there is no software workaround for this erratum.  Unfortunately, there

is no easy way to tell if your "B" step 386 DX CPU system does not have

the hardware workaround for Erratum 21.  There is no software program

or visual means to check for the workaround.

The only way to determine if your "B" step 386 DX CPU system

does not have the Erratum 21 workaround is to contact your system

manufacturer who will know whether it does or not, and should provide

you with a hardware upgrade if it doesn't.

However, if you suspect that your "B" step 386 DX CPU system does

not contain the Erratum 21 workaround, several third parties sell

hardware upgrades for the problem. For more information, please consult

the chapter on System Requirements in the AutoCAD 386 Installation and

Performance Guide, which explains a typical system manufacturer's

upgrade program (Compaq), and one from a third-party upgrade supplier

(Ironwood).