Orchid Technology Inc. / Panacea Inc.                         DLD-VGA(tm) v1.0

������������������������������������������������������������������������������

Copyright (C)1990  Panacea Inc.  All Rights Reserved.

This file covers the following topics:

*  Memory Allocation

*  Quick Installation

*  Environment variables and files used

*  Resolution & Boards supported

*  Environment Variables

*  Problems loading slides

MEMORY ALLOCATION

�����������������

For best performance when using the DLD with AutoCAD, you should balance your

system memory between EMS and Extended.  For example, with a total of 6MB of

RAM, optimum performance is achieved with 640KB conventional, 2MB EMS and

3MB Extended.

QUICK INSTALLATION

������������������

1. Run the batch file "DLDINST drive" from drive A: to copy the necessary

files to the subdirectory \DLD of the specified drive.  After the batch

file finished copying the files, change directory to subdirectory \DLD on

that specified drive.

2. Run DLDSETUP to configure the driver. Help is available at every

step by pressing the (?) key. This produces two files, DLDSETUP.DAT

which contains your setup information and FASTACAD.BAT.  For best

performance, answer YES when prompted for "Pre allocate EMS/Extended

Memory?"

3. Run either COLOR16.EXE or COLOR256.EXE, depending on whether you

are running either the 16 or 256 color version of the driver. This

utility allows you to customize your screen colors. It creates a file

called DLDCOLOR.DAT which contains your customized palette.

4. Run FASTACAD to load DLD-VGA.

5. Configure AutoCAD to use an ADI 4.0 Display Driver. This is done

by selecting #3, "Configure Acad", from the AutoCAD main menu, and

then selecting #5, "Display Device".

6. You are now ready to go.

Environment Variables (DLDCFG) and DLD-VGA Files

�����������������������������������������������

Running DLDSETUP will create two files: DLDSETUP.DAT - the

DLD-VGA configuration file; and a file called FASTACAD.BAT - a

batch file containing all the commands needed to run DLD-VGA.

Among the commands in the batch file is a line that sets an

environment variable called DLDCFG. DLDCFG is set to point to

the directory containing the DLD-VGA software, and is used by

DLDSETUP, COLOR16, COLOR256, DLDVGA, and DLDVG256, to locate

the files DLDSETUP.HLP, DLDCOLOR.HLP, DLDSETUP.DAT, and

DLDCOLOR.DAT. If DLDCFG is not set, those files are assumed to

be in the current directory. The FASTACAD.BAT is always

assumed to be in the current directory, regardless of whether

or not DLDCFG is set.

Resolutions

�����������

This version of DLD-VGA supports the Orchid ProDesigner Plus, the

ProDesigner II, and the ProDesigner/e from 320x200 to 1024x768 in

either 16 or 256 colors under AutoCAD and AutoSketch, and in all

256 color modes for AutoShade.

Environment Variables

���������������������

When the installation program creates the FASTACAD.BAT files, it

also includes settings for the AutoCAD Release 10 memory

variables. If you happen to be running AutoCAD 386, these

variables can be ignored, as AutoCAD 386 does not use them and

DLD-VGA should be pre-allocating the memory it requires. The

variables are, however, required in order to run DLD-VGA with

Release 10.

Slide Loading Bug

�����������������

One problem you might encounter if using older versions of

AutoCAD Release 10 (R10c2, for example), is that slides

containing solids are not loaded properly. Instead, you get a

small blob of color in the lower left corner of the display. This

is due to a problem in the older AutoCAD versions, not our

driver. Upgrade to c7 to fix the problem.