ScanPlot - Plot Range Finder

John Intorcio

This little program is meant as a tool for finding the maximum and

minimum coordintes used in an HPGL or DM/PL plot file created by

AutoCAD.

The synatx is:

SCANPLOT <Filename>

Extensions are not allowed, .PLT is assumed.

Upon invocation, ScanPlot reports the assumed langauge of the plot file

and begins scanning the commands contained therein.  It gives a vector

count to the screen so you can see that it's working.

On completion, ScanPlot returns the maximum and minimum points that are

used in the plot file.

ScanPlot can be used with PlotManager to determine the high and low

plotting ranges for a given plotter.  To use ScanPlot with PlotManager,

begin by creating two AutoCAD drawings.  Each of these drawings should

contain a single rectangle.  The first rectangle should be much wider

than high and the second, much higher than wide.  The idea is to make

the width the constraint on establishing extents on one and the height

the constraint on the other.

Next configure AutoCAD for the plotter model and sheet size in question

and plot each drawing to a file plotting extents and choosing the "Fit"

scaling option.  You might want to call these plots H and V for

Horizontal and Vertical rectangles.

Now run ScanPlot on each of the plot files.  The plot file created from

the Vertical rectangle will yield the maximum and minimum Y values and

the plot file created from the Horizontal rectangle will yield the

maximum and minimum X values.

You get the numbers like this:

Vertical rectangle plot:

Low Range:      ?,Minimum Y Value

High Range:     ?,Maximum Y Value

Horizontal rectangle plot:

Low Range:      Minimum X Value,?

High Range:     Maximum X Value,?

These values may then be used in defineing new plotters either by

editing PlotManager's emulation file or through the interactive

dialogue.