# Admont Software Inc.

# PLVCGL-W TM

# XeroxÓ/VersatecÓ AutoCADÓ Drivers

# AutoCadÓ Release 12 & 13 DOS

# AutoCadÓ Release 12 for Windows

# AutoCadÓ Release 13 for WindowsÓ, Windows for WorkGroups 3.11Ó and NTÓ.

# Operator and Installation Guide

# Publication 106-1.1

# May 1995

Admont Software Inc.

P.O. Box 76

Jericho, VT 05465-0076

802-899-2492 voice

802-899-2428 fax

Copyright Ó 1995 Admont Software Inc.

All Rights Reserved

No part of this manual may be copied or reproduced without prior written permission of Admont Software Inc.

Publication Number 106-1.1

Printed in the United States of America

AutoCADÓ is a registered trademark of Autodesk, Inc.

XeroxÓ is a registered trademark of Xerox Corp.

PLVCGL-W is a trademark of Admont Software Inc.

VersatecÓ is a registered trademark of Xerox Corp.

Admont Software Inc. reserves the right to make changes without notice to information contained in this document.  Admont Software Inc. assumes no responsibility for any errors or consequential damages that may result from use or misinterpretation of any information contained herein.

### Table of Contents

µTable of Contents	3

License Information & Activation Numbers	4

REGISTRATION & ACTIVATION ORDER FORM	6

To Our Readers	7

Plotters Supported & Features	8

Installing the PLVCGLW.EXP Driver on a DOS Version of AutoCAD	9

Installing the PLVCGL13.DLL Driver on a Windows or NT Version of AutoCAD Release 13	10

Installing the PLVCGL12.DLL Driver on a Windows  Version of AutoCAD Release 12	11

Appendix A Xerox/Versatec Plotters Supported	16

Appendix B Metric to Inch Conversion for Pen Width	21

AutoCAD PEN COLOR Assignments	22

### License Information & Activation Numbers

XE "License Information & Activation Numbers"§

The PLVCGL-W driver floppy contains three AutoCAD drivers:

PLVCGLW.EXP	- For Release 12 & 13 DOS AutoCAD Versions

PLVCGL12.DLL	- For Release 12 Windows AutoCAD Versions

PLVCGL13.DLL	- For Release 13 Windows AutoCAD Versions

on Windows, Windows for Workgroups, & NT.

Your site may have purchased a single workstation license or a site license.  During configuration you must provide an activation number that is tied to the AutoCAD serial number on that workstation.  The activation numbers are provided upon receipt of payment.  Until that time, the driver functions as a demo version with a large X across the plot.

Admont Software,  Inc.  -- Software License Agreement

This is a legal agreement between you (either an individual or an entity) and Admont Software Inc.  By opening the sealed software packet you are agreeing to be bound by the terms of this agreement.  If you do not agree to the terms of this agreement, promptly return the unopened software packet, manual, and other enclosed information to the place you obtained them for a full refund.

ADMONT SOFTWARE, INC. LICENSE

1.  GRANT OF LICENSE.  Admont Software, Inc. grants to you the right to use one copy of the enclosed software program (the SOFTWARE) on a single computer.  The SOFTWARE is in use on a computer when it is loaded into temporary memory (i.e. RAM) or installed into permanent memory (e.g. hard disk, CD-ROM, or other storage device) of that computer.   You may use the PLVCGLW.EXP, PLVCGL12.DLL and PLVCGL13.DLL AutoCAD drivers on 1 (one)  AutoCAD workstation on your network.  Use of PLVCGLW drivers on workstations outside of your network or in excess of 1 (one) require a site use license.

2.  COPYRIGHT.  The SOFTWARE is owned by Admont Software, Inc. and is protected by United States copyright laws and international treaty provisions.  Therefore, you must treat the SOFTWARE like any other copyrighted material (e.g. a book or musical recording) except that you may use the installation program to transfer the SOFTWARE to a single hard disk provided you keep the original solely for backup or archival purposes.  You may not copy the written materials accompanying the SOFTWARE.

3.  OTHER RESTRICTIONS.  You may not rent or lease the SOFTWARE.  This license is non-exclusive and nontransferable.  You may not reverse engineer, decompile, or disassemble the SOFTWARE.   You may not disclose any internal part of this software or permit it to be used by any person or entity other than your authorized officers, employees, and agents.

4.  DUAL MEDIA.  If the SOFTWARE package contains both 3.5" and 5.25" disks, then you may use only the disks appropriate for your single-user computer.  You may not use the other disks on another computer or loan, rent, lease, or transfer them to another user.

5.  REGISTRATION OF SOFTWARE.  You must complete and return to Admont Software, Inc.  the registration card included with the SOFTWARE.

LIMITED WARRANTY

LIMITED WARRANTY.   Admont Software, Inc. warrants that (a) the SOFTWARE will perform substantially in accordance with the accompanying written materials for a period of ninety (90) days from the date of receipt.  Any implied warranties on the SOFTWARE are limited to ninety (90) days.  Some states/countries do not allow limitations on duration of an implied warranty, so the above limitation may not apply to you.

CUSTOMER REMEDIES.  Admont Software Inc. and its suppliers' entire liability and your exclusive remedy shall be, at Admont Software, Inc.'s option, either (a) return of the price paid, or (b) repair or replacement of the SOFTWARE that does not meet Admont Software's Limited Warranty and which is returned to your supplier with a copy of your receipt.  This Limited Warranty is void if failure of the SOFTWARE has resulted from accident, abuse, or misapplication.  Any replacement SOFTWARE will be warranted for the remainder of the original warranty period or thirty (30) days, whichever is longer.  Outside the United States, these remedies are not available without proof of purchase from an authorized non-US. source.

NO OTHER WARRANTIES.  Admont Software, Inc. and its suppliers disclaim all other warranties, either express or implied, including, but not limited to implied warranties of merchantability and fitness for a particular purpose, with regard to the SOFTWARE and the accompanying written materials,.  This limited warranty gives you specific legal rights.  You may have others which vary from state/country to state/country.

NO LIABILITY FOR CONSEQUENTIAL DAMAGES.  In no event shall Admont Software, Inc. or its suppliers be liable for any damages whatsoever (including, without limitation, damages for loss of business profits, business interruption, loss of business information, or any other pecuniary loss) arising out of the use or inability to use this Admont Software Inc. product, even if Admont Software has been advised of the possibility of such damages.  Because some states/countries do not allow the exclusion or limitation of liability for consequential or incidental damages, the above limitation may not apply to you.

US. GOVERNMENT RESTRICTED RIGHTS

The SOFTWARE and documentation are provided with RESTRICTED RIGHTS.  Use, duplication, or disclosure by the Government is subject to restrictions set forth in subparagraph (c)(1)(ii) of The Rights in Technical Data and Computer Software clause at DFARS 252.227-7013 or subparagraphs (c)(1) and (2) of the Commercial Computer Software -- Restricted Rights at 48 CFR 52.227-19, as applicable.  Manufacturer is Admont Software, Inc. , P.O. Box 76, Jericho, Vermont 05465-0076.

If you acquired this product in the United States, this Agreement is governed by the laws of the State of Vermont.

Should you have any questions concerning this Agreement contact Admont Software, Inc. at P.O. Box 76, Jericho, Vermont 05465-0076.

### REGISTRATION & ACTIVATION

### ORDER FORM

XE "Registration & Activation Order Form"§

Instructions

Make a photocopy of this page, complete, and FAX to:

ADMONT SOFTWARE INC.,  FAX: 802-899-2428

NAME:		_______________________________________________

COMPANY:	 ______________________________________________

ADDRESS:	 ______________________________________________

ADDRESS: 	_______________________________________________

CITY, STATE, ZIP:  _____________________________________________

Voice Phone # : 	 ______________________________________________

FAX Phone #:     	_______________________________________________

To find out the AutoCAD serial number start AutoCAD and type CONFIG.  The AutoCAD serial number is displayed at the top of the screen.  The Activation number below is an Admont provided number, NOT the AutoCad Activation Key.

AutoCAD Serial Number (including all dashes)			Admont  Activation Number

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

________________________________			___________________

### To Our Readers

XE "To Our Readers"§

This guide contains step-by-step instructions to install and the PLVCGL-W AutoCAD drivers on your AutoCAD workstations.

This guide is written for a reader with an understanding of plotters and assumes experience working with computers and their peripherals as well as basic concepts used to operate your plotter.

As you use this guide, you may think of ways it can be improved.  We welcome your comments and suggestions.  You may send them to:

Admont Software Inc.

P.O. Box 76

Jericho, VT 05465-0076

Attn:  Technical Publications

### Plotters Supported & Features

XE "Plotters Supported & Features"§

Plotters Supported

PLVCGL-W drivers are designed to work with Xerox/Versatec equipment with VDS/VCGL support.  VCGL (Versatec Color Graphics Language) is supported on ServeWare 2.x, plotters with embedded controllers i.e. 8936-E, and external 800 series RPM controllers.

Call Admont Software Inc. for a current list of supported plotters.

Features

The PLVCGL-W drivers allow you to plot to your Xerox/Versatec plotting equipment using the standard AutoCAD "PLOT" command.  All 255 AutoCAD pens are supported.  You may edit the PLVCGL.CLR file to modify the pen colors as RGB values.  Since VCGL is the native high level language of Xerox/Versatec plotting equipment, plotting is fast and efficient.  Colors are locked into the VCGL file at PLOT generation time.

### Installing the PLVCGLW.EXP Driver on

### a DOS Version of AutoCAD

PLVCGLW.EXP is a protected mode ADI 4.2 AutoCAD driver.   This driver should be copied from the floppy diskette to the ACADDRV directory that contains your other AutoCAD drivers.   To determine the ACADDRV directory:

1:	Start AutoCAD.

2:	At the AutoCAD command prompt type SHELL and hit return twice.

3:	Type:  SET

4:	The string ACADDRV=c:\acad12\drv would indicate that the ACADDRV directory is c:\acad12\drv.  Copy the PLVCGLW.EXP file to this directory:

5:	>>COPY A:PLVCGLW.EXP C:\acad12\drv  (substitute your ACADDRV directory as the destination.

6:	Type: EXIT    to return to AutoCAD.

7:	Type CONFIG

8:	Select:  Add Plotter

9:	Select VCGL (1-Color) Driver if your plotter is a monochrome plotter

10:	Select VCGL (4-Color) Driver if your plotter is a color plotter.

11:	Select VCGL (3-Color) Driver if your plotter has no black ink.

12.	Select your plotter from the list.  If your exact plotter is not found, select one with the same resolution (dpi) and of equal or greater width.  For example, selecting a 8936-4 would allow plots of E-size through A-size and long plots to be selected from the AutoCAD PLOT menu.

13:	Enter the Activation Code if you have paid for and received an Activation Code, Enter 0 otherwise.

14:	Answer Y (yes) to the Simulated RGB question.  If you have  ServeWare 2.2 you may wish to experiment with Y and N answers.

15:	You may repeat steps 8-13 to configure multiple plotters.

16:	Continue with configuration and save configuration.

17:	Open a drawing and type PLOT.

### Installing the PLVCGL13.DLL Driver on

### a Windows or NT Version of AutoCAD Release 13

PLVCGL13.DLL is a 32-bit DLL mode ADI 4.2 AutoCAD driver.   This driver should be copied from the floppy diskette to the ACADDRV directory that contains your other AutoCAD drivers.   To determine the ACADDRV directory:

1:	Use the Windows File/Properties to see the drive:directory for AutoCAD Release 13 for windows.  For Example: C:\ACADR13\WIN

2.	The Driver directory is located in the DRV subdirectory.  In our example above this would be:  C:\ACADR13\WIN\DRV

3.	Copy PLVCGL13.DLL from the floppy to this DRV directory.

Example:  COPY A:PLVCGL13.DLL C:\acad13\win\drv

4.	Start AutoCAD Release 13.

5:	Type CONFIG

6:	Select:  Add Plotter

7:	Select VCGL (1-Color) Driver if your plotter is a monochrome plotter

8:	Select VCGL (4-Color) Driver if your plotter is a color plotter.

9:	Select VCGL (3-Color) Driver if your plotter has no black ink.

10.	Select your plotter from the list.  If your exact plotter is not found, select one with the same resolution (dpi) and of equal or greater width.  For example, selecting a 8936-4 would allow plots of E-size through A-size and long plots to be selected from the AutoCAD PLOT menu.

11:	Enter the Activation Code if you have paid for and received an Activation Code, Enter 0 otherwise.

12:	Answer Y (yes) to the Simulated RGB question.  If you have  ServeWare 2.2 you may wish to experiment with Y and N answers.

13:	You may repeat steps 8-13 to configure multiple plotters.

14:	Continue with configuration and save configuration.

15:	Open a drawing and type PLOT.

### Installing the PLVCGL12.DLL Driver on

### a Windows  Version of AutoCAD Release 12

PLVCGL12.DLL is a 16-bit DLL mode ADI 4.2 AutoCAD driver.   This driver should be copied from the floppy diskette to the ACADDRV directory that contains your other AutoCAD drivers.   To determine the ACADDRV directory:

1:	Use the Windows File/Properties to see the drive:directory for AutoCAD Release 13 for windows.  For Example: C:\ACAD12W

2.	The Driver directory is located in the main AutoCAD directory.  In our example above this would be:  C:\ACAD12W  You can verify this by looking for other files of the form PL*.DLL (for example PLPHPGL2.DLL).  If these files are not found in the main AutoCAD directory look for a DRV subdirectory or other subdirectories with the PL*.DLL files.  It is into this directory or subdirectory that you would copy the driver.

3.	Copy PLVCGL12.DLL from the floppy to this DRV directory.

Example:  COPY A:PLVCGL12.DLL C:\acad12W

4.	Start AutoCAD Release 12.

5:	Type CONFIG

6:	Select:  Add Plotter

7:	Select VCGL  Driver - By Admont Software Inc.

8:	Select the correct plotter model.  If your exact plotter model cannot be found select a model of the same size.  For example, if your plotter is 400 dpi and 36" wide, select 8936-4.  If it is 24" wide and 200 dpi, select 8924-2.

9:	Enter the Activation Code if you have paid for and received an Activation Code, Enter 0 otherwise.

10:	Answer Y (yes) to the Simulated RGB question.  If you have  ServeWare 2.2 you may wish to experiment with Y and N answers.

11:	You may repeat steps 8-13 to configure multiple plotters.

12:	Continue with configuration and save configuration.

13:	Open a drawing and type PLOT.

# First Time Plotting

# After the driver has been configured, it can be selected from the AutoCAD "PLOT" out device menu.  To test your plotter driver:

1.  OPEN a drawing file such as:

\ACAD\SAMPLE\CHROMA.DWG    or CLRWHEEL.DWG

2.  PLOT - Enter the AutoCAD PLOT Command

3.  Use your pointing device such as a mouse to select the button labeled:

DEVICE and DEFAULT SELECTION

4:	Use your pointing device to select the name of the VCGL Driver just configured.

# From this point on until you select another output plotting device, your plotter driver will be used whenever the AutoCAD PLOT command is used.  Continue with the following to complete the initial plotting test.

# 5.  Use your pointing device to select the "PEN ASSIGNMENTS" button.

# Pen Assignments

Color			Pen No.		Linetype		Pen Width

1			1			0			0.01

2			2			0			0.01

3			3			0			0.01

...

255			255			0			0.01

This is the standard AutoCAD Pen Assignment menu.  It is fully documented in the AutoCAD Release 12/13 documentation.  If you have difficulties using this menu, refer to the AutoCAD documentation.

This driver supports all 255 AutoCAD pens.  Initially the colors are mapped to the pens of the same number.  Colors for monochrome plotters are simulated by halftones.

PEN 7 is SOLID BLACK

Both the sample plots "CHROMA.DWG" and "CLRWHEEL.DWG" plot all 255 colors.  These plots are good choices to show the capability of the plotting system.

If you desire a solid black pen on all levels, assign pen #7 to all color numbers.

See Appendix to this manual for more information.

To change a color, use the pointing device to select (highlight) the desired color/pen line(s) (multiple lines can be selected).  Then use the pointing device to display a cursor in the PEN field and type a new pen number.

You can also use this menu to select a new pen width.  The pen units of width are the same as the plot units.  The units of width will be in inches unless you specified <M> in the config question:

"Size units (Inches or Millimeters)"

In the configure plotter section after the:

"Do you want to change anything?"

question in the Configure Plotter section of CONFIG.

A metric/inche conversion table can be found in the appendix.

Accept the PEN menu as modified by selecting the OK button at the bottom of the PEN ASSIGNMENTS menu.

PLOT SIZE SELECTION

Selecting "SIZE" from the AutoCAD "PLOT" menu gives you a selection of possible output drawing sizes.  Use your pointing device to select the desired size.

This is a standard AutoCAD Release 12 & 13 menu.  Refer to your AutoCAD manuals for more information.

PLOT ROTATION SELECTION

AutoCAD gives you the ability to rotate the information in your plot prior to output to the driver.  You can use this to save paper when drawing a smaller output size on a wider plotter.  To do this, the paper orientation must be changed from portrait to landscape and the plot drawing information must be rotated 90 degrees.

To change the paper orientation from the AutoCAD PLOT menu, select the "SIZE" button.  Define a USER size of width = 21, height = 33 (Note: these are the standard D-sizes with height and width reversed).  Select OK to return to the PLOT menu.

The plot drawing information must also be rotated to fit this new paper orientation.  Select the ROTATION and ORIGIN button and select 90 degree rotation.  Select OK to return to the PLOT menu, and OK to begin plotting.

For example, rotating a D size plot 90 degrees before output to an E-size electrostatic plotter will save 12"x36 (or 3 square feet) of paper.

This

Page

Intentionally

Blank

### Appendix A

### Xerox/Versatec Plotters Supported

Monochrome Table

8510    Cadmate (monochrome)

8910m   Color Cadmate (mono)

8924-2m 200 dpi (monochrome)

8925-2m 200 dpi (monochrome)

8936-2m 200 dpi (monochrome)

8944-2m 200 dpi (monochrome)

8954-2m 200 dpi (monochrome)

8924-4m 400 dpi (monochrome)

8925-4m 400 dpi (monochrome)

8936-4m 400 dpi (monochrome)

8944-4m 400 dpi (monochrome)

8954-4m 400 dpi (monochrome)

8810     Blaser (monochrome)

8810_la  Blaser A-landscape

8810_pa  Blaser A-portrait

8810_pb  Blaser B-portrait

8810_la5 Blaser A5-landscape

8810_pa5 Blaser A5-portrait

8810_lb5 Blaser B5-landscape

8810_pb5 Blaser B5-portrait

8810_la4 Blaser A4-landscape

8810_pa4 Blaser A4-portrait

8810_pb4 Blaser B4-portrait

8810_pa3 Blaser A3-portrait

8810_ex  Blaser executive

8810_fl  Blaser folio size

8810_lg  Blaser legal size

8810_en  Blaser envelope size

8624    200 dpi (monochrome)

8624hr  400 dpi (monochrome)

8625    200 dpi (monochrome)

8625hr  400 dpi (monochrome)

8636    200 dpi (monochrome)

8636hr  400 dpi (monochrome)

8770    400 dpi (monochrome)

c2736m  A-size Thermal  (mono)

c2756m  B-size Thermal  (mono)

c2766m  A3-size Thermal (mono)

c2726m  A4-size Thermal (mono)

c2776m  Spec. B Thermal (mono)

c2786m  Spec A3 Thermal (mono)

v80  200 dpi 10.56 in   (mono)

v81  100 dpi 10.24 in   (mono)

v82  100 dpi 10.56 in   (mono)

ce3224m 200 dpi (monochrome)

ce3225m 200 dpi (monochrome)

ce3236m 200 dpi (monochrome)

ce3244m 200 dpi (monochrome)

ce3424m 400 dpi (monochrome)

ce3425m 400 dpi (monochrome)

ce3436m 400 dpi (monochrome)

ce3444m 400 dpi (monochrome)

ecp42m  200 dpi (monochrome)

8836-2  200 dpi Laser (mono)

8836-4  400 dpi Laser (mono)

7222    200 dpi (monochrome)

7224    200 dpi (monochrome)

7225    200 dpi (monochrome)

7236    200 dpi (monochrome)

7244    200 dpi (monochrome)

7422    400 dpi (monochrome)

7424    400 dpi (monochrome)

7425    400 dpi (monochrome)

7436    400 dpi (monochrome)

7444    400 dpi (monochrome)

8222    200 dpi (monochrome)

8224    200 dpi (monochrome)

8236    200 dpi (monochrome)

8242    200 dpi (monochrome)

8272    200 dpi (monochrome)

c2552m 200 dpi Spectrum mono

c2562m 200 dpi Spectrum mono

c2556m 300 dpi Spectrum mono

c2566m 300 dpi Spectrum mono

c2558m 400 dpi Spectrum mono

c2568m 400 dpi Spectrum mono

Four Color Table

8910    Color Cadmate (4 color)

8924-2  200 dpi (4 color)

8925-2  200 dpi (4 color)

8936-2  200 dpi (4 color)

8944-2  200 dpi (4 color)

8954-2  200 dpi (4 color)

8924-4  400 dpi (4 color)

8925-4  400 dpi (4 color)

8936-4  400 dpi (4 color)

8944-4  400 dpi (4 color)

8954-4  400 dpi (4 color)

c2736a A-size 300 dpi Thermal  (4 color)

c2756a B-size 300 dpi Thermal  (4 color)

c2766a A3-size 300 dpi Thermal (4 color)

c2726a A4-size 300 dpi Thermal (4 color)

c2776a Spec. B 300 dpi Thermal (4 color)

c2786a Spec A3 300 dpi Thermal (4 color)

ce3224  200 dpi (4 color)

ce3225  200 dpi (4 color)

ce3236  200 dpi (4 color)

ce3244  200 dpi (4 color)

ce3424  400 dpi (4 color)

ce3425  400 dpi (4 color)

ce3436  400 dpi (4 color)

ce3444  400 dpi (4 color)

ecp42   200 dpi (4 color)

c2552  200 dpi Spectrum 4 color

c2562  200 dpi Spectrum 4 color

c2556  300 dpi Spectrum 4 color

c2566  300 dpi Spectrum 4 color

c2558  400 dpi Spectrum 4 color

c2568  400 dpi Spectrum 4 color

Three Color Table

8910s   Color Cadmate (3 color)

8924-2s 200 dpi (3 color)

8925-2s 200 dpi (3 color)

8936-2s 200 dpi (3 color)

8944-2s 200 dpi (3 color)

8954-2s 200 dpi (3 color)

8924-4s 400 dpi (3 color)

8925-4s 400 dpi (3 color)

8936-4s 400 dpi (3 color)

8944-4s 400 dpi (3 color)

8954-4s 400 dpi (3 color)

c2736   A-size 300 dpi Thermal (3 color)

c2756   B-size 300 dpi Thermal (3 color)

c2766   A3-size 300 dpi Thermal(3 color)

c2726   A4-size 300 dpi Thermal(3 color)

c2776   Spec. B 300 dpi Thermal(3 color)

c2786   Spec A3 300 dpi Thermal(3 color)

Windows AutoCAD Release 12 Table

8924-4  400 dpi (4 color)

8925-4  400 dpi (4 color)

8936-4  400 dpi (4 color)

8944-4  400 dpi (4 color)

8954-4  400 dpi (4 color)

8924-2  200 dpi (4 color)

8925-2  200 dpi (4 color)

8936-2  200 dpi (4 color)

8944-2  200 dpi (4 color)

8954-2  200 dpi (4 color)

c2736a A-size 300 dpi Thermal  (4 color)

c2756a B-size 300 dpi Thermal  (4 color)

c2766a A3-size 300 dpi Thermal (4 color)

c2726a A4-size 300 dpi Thermal (4 color)

8510    Cadmate 300 dpi (monochrome)

### Appendix B

### Metric to Inch Conversion for Pen Width

Millimeters				Inches

1					.03937

2					.07874

3					.11811

4					.15748

5					.19685

6					.23622

7					.27559

8					.31496

9					.35433

10					.3937

11					.43307

12					.47244

13					.51181

14					.55118

15					.59055

16					.62992

17					.66929

18					.70866

19					.74803

20					.7874

21					.82677

22					.86614

23					.90551

24					.94488

25					.98425

26					1.02362

27					1.06299

28					1.10236

29					1.14173

30					1.1811

### AutoCAD PEN COLOR Assignments

To see the default pen color/pen pattern assignments, open the AutoCAD sample file:  CHROMA.DWG and plot it on your plotter.

To Change the PLVCGLW default pen color assignments you will need to place a PLVCGL.CLR file in the same directory as the PLVCGL driver (PLVCGLW.EXP, PLVCGL12.DLL, or PLVCGL13.DLL).  The PLVCGL.CLR file can be found on the floppy and contains:

Pen #    Red_Value    Green_Value   Blue_Value   Opaque_flag

for all 255 AutoCAD Pens.  You may modify the Values within the range of 0-255 for the desired colors.

You will find the PLVCGL.CLR file on the distribution floppy:

PLVCGL.CLR	-	The default Pen Color Set

You can use a text editor to edit PLVCGL.CLR for your own pen mapping.  The format for each line is:

Pen #	Red	Green	Blue   Opaque

The value of pen # 7 is:		7	0	   0	  0	  0     for Black Transparent

or pen 202			202	255	   0	  0	  1     for Red Opaque

Note: The Opaque flag may not be supported on your plotter.  Call Admont Software Inc for details.

If the PLVCGL.CLR file exists in the ACADDRV directory, PLVCGLW reads the pen assignments from the file and overrides the default settings.

You may wish to begin by using the default pen settings in the distributed PLVCGL.CLR file with the AutoCAD sample file CHROMA.DWG:

Copy the PLVCGL.CLR file from the floppy to your ACADDRV directory.

Start AutoCAD.

Open the sample file CHROMA.DWG

PLOT

Exit AutoCAD

Select the visible effects (colors, monochrome tones) that you like and edit the

PLVCGL.CLR file to match your desired pen color/halftone settings.

Start AutoCAD, open CHROMA.DWG & PLOT to output the new color/halftone settings.