What's New in the AutoCAD Release 11 Update

Contents

2  Your AutoCAD Release 11 Update...DXFIX Utility

Phar Lap DOS Extender 2.6 update

4  New Device Drivers

New ADI Drivers

5  Solutions to Hardware Problems

Workarounds to B1-Step 80386 Chip Errata 17 and 21

6  Changes to AutoCAD Features

New Features

Corrections to AutoCAD Features

Corrections to DXF(tm)

IGES Changes and Corrections

Corrections to AutoLISP(r) and ADS(tm)

"Your AutoCAD Release 11 Update . . ."

Autodesk is committed to providing quality software to its customers. In

keeping with our commitment to quality, we are making an update of Release

11 available to AutoCAD 386 Release 11 customers so they can get the

maximum efficiency from the product.

This document outlines the AutoCAD Release 11 update and explains the

AutoCAD features affected by the update. The update is provided in 1.2 MB

and 720 KB disk formats for domestic and international versions. The

AutoCAD Release 11 update includes:

o  an updated set of AutoCAD 386 Release 11 disks,

o  "DXFIX," a utility that converts Release 11 DXF files for use with

AutoCAD Release 10,

o  an optional update of the Phar Lap DOS Extender 2.2d which is an

integral part of AutoCAD 386 Release 11,

o  New ADI(r) drivers including a super VGA driver,

o  a partial workaround for system hangs which may be encountered in

computers using Intel B1-step 80386 processor (due to errata 17

and 21) and a B1-step chip detection program.

"DXFIX Translation Utility"

DXFIX is a utility program that permits translation and filtering of ASCII

or binary DXF files, and is driven by an external specification file. It is

supplied with a specification file that reads AutoCAD Release 11 DXF files

and emits a DXF file compatible with AutoCAD Release 10. This utility can

benefit customers who must exchange drawings with colleagues, clients, or

vendors who are using an earlier release of AutoCAD. However, it should be

understood that, as in any translation effort, some information is

inevitably lost. In most cases, this loss is caused by the new features in

Release 11 which are supported by DXF groups and constructs not present in

Release 10. More information about DXFIX is available in the "DXFIX.doc"

file located on the Release 11 update's DXFIX/NEWDX/B1step Disk.

"Phar Lap DOS Extender 2.6 update"

The Phar Lap DOS Extender 2.6 update is optional and offers several

benefits to customers. Among them are:

o  A smaller kernel. The DOS Extender 2.6 provides a memory savings of

100 KB compared to the previous version of  the DOS Extender 2.2d.

The additional memory allows more space for shelled programs,

utilities, TSR programs, and other software. In addition, you can use

the shell room utility, "shroom," originally supplied with Release 11,

to expand shell space to within 8 KB of the memory available when

AutoCAD 386 is started. "Shroom.com" and "Shroom.doc" are located in

the sample directory of AutoCAD 386 Release 11.

o  Demand Paged Loading. The previous DOS Extender 2.2d loads the

entire program into memory and frees up pieces of the program when

memory is scarce. A new feature of the DOS Extender 2.6 reduces the

load time of AutoCAD 386 by 30 to 40 percent by loading the AutoCAD

code as it is needed. The DOS extender switch for Demand Paged Loading

is called DEMANDLOAD.

"DOS Extender Update"

o  XMS Memory Support. XMS (extended memory support) allows applications

to share extended memory more efficiently. XMS is necessary to access

Super Extended Memory, memory above 16 MB, on EISA machines.

o  Multiple Memory Support. DOS Extender 2.2d only takes extended memory

from one source. If a VCPI Expanded Memory Simulator (EMS) is

installed, then AutoCAD 386 only takes memory from that manager, even

if there is unused extended memory. The DOS Extender 2.6 can utilize

memory from VCPI, Extended, and XMS memory pools. The order of memory

allocation is:

Without XMS driver installed:

1) Direct Extended Memory

2) VCPI Memory Manager

3) Compaq Built-in Memory (Compaq machines only)

With an XMS driver installed: When an XMS driver is installed, all

memory is allocated from that driver.

o  Limited Windows 3.0 support. DOS Extender 2.6 will run in the DOS box

of Windows 3.0 in both real and standard modes. Since AutoCAD is a

graphics program, it must be run in the full screen mode of the DOS

box. Under standard mode, the DOS Extender gets its memory from XMS. In

order to make sure that AutoCAD gets XMS memory when it runs in the DOS

Box, you must ensure that a PIF file is set up which makes XMS memory

available to the application. You can do this by using the PIF editor.

DOS Extender version 2.6 will not support Windows 3.0 386 enhanced

mode. Autodesk does not warrant the suitability of Windows 3.0 as a

operating environment for AutoCAD 386.

Consult the "newdx.doc" file, located on the Release 11 update's

DXFIX/NEWDX/B1step Disk, for the procedure to update your copy of AutoCAD

386 Release 11 to DOS Extender Version 2.6. The Release 11 update also

contains documentation on the new DOS Extender switches that have been

added to version 2.6.

We believe that the benefits of version 2.6 will help our customers. If you

encounter any problems with version 2.6, revert back to using version 2.2d

and let us know about problems by submitting an "Autodesk Bug Report" form.

"New Device Drivers"

New ADI Drivers

Super VGA Driver

The update also includes a Super VGA ADI 4.0 display driver for AutoCAD,

AutoSketch, and AutoShade. The Super VGA driver (SVADI) is used in place

of the built-in display driver for the IBM Video Graphics Array (VGA). Its

advantages over the built-in VGA driver include:

o  Super VGA resolutions. The built-in driver operates at 640 x 480 dots

in 16 colors. SVADI supports 640 x 480, 800 x 600, and 1024 x 768

resolutions, in both 16 and 256 colors. Consult the documentation on

the SVADI disk for a list of supported VGA graphics boards.

o  More flexible configuration. SVADI allows you to change the size and

color of text areas, the text font size, flipscreen behavior, the

ed-green-blue shades of display colors, and more.

o  Dual screen operation. If you have both VGA and monochrome adapters in

your system, SVADI can display text on the monochrome and graphics on

the VGA.

o  Performance. SVADI is approximately 15 percent faster than the built-in

VGA driver on redraw operations.

Additional ADI Drivers

The Release 11 update also provides ADI drivers for these devices:

o  HP-GL/2 plotters

o  HP Long Plot

o  Epson LQ 24 pin printers

"Solutions to Hardware Problems"

Workarounds for B1-Step 80386 Chip Errata 17 and 21

Computers using Intel's B1-step 80386 processor chips manufactured in 1985

and 1986 may experience system hangs when using protected mode software such

as AutoCAD 386, due to the so-called Errata 17 and 21. For the convenience

of our dealers and customers, we have created a software solution for

Erratum 17 that applies to AutoCAD 386 Release 11. We are also providing a

program to determine if a system has a B1-step chip.

To determine if a system uses a B1-step 386 DX CPU, at the DOS prompt,

execute the program CHKBSTEP.COM,  located on the Release 11 update's

DXFIX/NEWDX/B1step Disk. If your system contains a B1-step 386 DX CPU, the

following message will be printed on the screen:

"This system is using a 386 DX B1-step part."

If the system does not contain a B1-step 386 DX CPU, the following message

will be printed on the screen:

"This system is not using a 386 DX B1-step part."

If a system does have the erratum 17 problem, you may use ERRATA17.EXP, a

program that provides a partial workaround for Erratum 17. It is an AutoCAD

Development System(tm) application that scans the ACAD.EXE file and locks

down any code pages that may cause the processor to exhibit Erratum 17.

This fix only works for ACAD.EXE itself. It will not help if the Erratum 17

condition is caused by an ADI (Autodesk Device Interface(tm) ) or ADS

application. ERRATA17.EXP will lock into memory those pages which

potentially hang AutoCAD 386 on B1-step 80386 CPUs that exhibit this

problem. There is no advantage to using this program if you are not

experiencing this problem.

There is no software workaround for the Erratum 21 hardware problem. The

solution is to replace the B1-step 80386 with a D-step 80386 or higher

chip. Several third parties sell hardware upgrades for the Erratum 21

problem.

"Changes to AutoCAD Features"

In the Release 11 update, we have provided improvements to many features

and added new features.

New Features

Saving Drawings

AutoCAD goes to much greater efforts to save a drawing under disk full

conditions. The key to a virtual memory system like AutoCAD 386 is

sufficient disk space. Even so, it is inevitable that disk full conditions

will be encountered. AutoCAD now retains a disk space buffer for times when

this occurs, allowing for successful saving of a drawing under these

conditions.

Support of Watcom C

ADS support library extensions are included for Watcom C 386 Version 8.0

with this release to enable use of Watcom C for ADS applications.

Corrections to AutoCAD Features

Polygons

Polygons created after drawing a spline fit polyline would contain a group

75 code value of 6 when the polygon was not spline fit. This error was

evident in the DXF output of a drawing exhibiting this case. This has been

corrected so that the group 75 value for polylines created with the polygon

command is independently set by direct operations on the entity only.

File Type Specification

When using the file dialogues, entering the paths of file names by relative

means (e.g., .\test) would fail saying "Wrong file type" unless the file's

type extension was supplied. This now works correctly whether or not the

file type is specified.

Line Entity

Drawing a line entity by entering a deferred tangent or perpendicular

specification (to an arc or circle) as the first point and entering a

relative point offset as the second point exhibited different behavior than

in Release 10. This case has been corrected to match Release 10's behavior,

which is to interpret the relative point specification as the direction and

length for the line, and calculate the tangent or perpendicular point of

the circle or arc for the line direction.

OBJECT SNAP (OSNAP)

OSNAP INT would fail to find an intersection point where one line had been

drawn to the NEArest, MIDpoint, or ENDpoint (using object snap) on another

line that is almost perpendicular to the X-Y plane of the current UCS.

AutoCAD now identifies the intersection requested and properly completes

this construction.

OSNAP NEArest to a line that is drawn perpendicular to the X-Y plane of the

current UCS would fail by not returning the nearest point on the line from

the current view direction. This has been corrected.

If a block containing a circle was inserted with a rotation angle, the

QUAdrant object snap points of the circle did not also rotate as

documented. This behavior was corrected.

Filmroll Files

The mesh output in a filmroll file applied the last value of SURFTYPE to

all meshes output, regardless of the surftype used in creating the original

mesh. This has been corrected so that meshes output to filmroll files

contain the proper surftype with which they were created.

AUDIT

AUDIT behaved abnormally for model space viewports with width and/or height

values of 0.0. AUDIT would report incorrect numbers of errors and/or errors

corrected during the operation. Auditing a drawing with these cases now

functions appropriately with accurate messages.

HIDE

Hide would incorrectly hide 3Dface entities with invisible edges if

SPLFRAME were set to 0. This was corrected.

TEXT

A third TEXT command attempt would result in a fatal error if the first

text execution had a null string entered for the text and the second one

was undone. This has been fixed.

HATCH

Hatching around text entities of styles other than STANDARD would fail to

avoid the text properly by misjudging the location of the text and hatching

in the wrong place. This is corrected to hatch around text properly.

ZOOM

ZOOM to extents in model space with TILEMODE set to 0 would produce  an

abnormal display if there was only a very tiny entity, e.g., a point

entity. Attempting to execute DVIEW following this zoom operation would

hang the system. This has been corrected.

In some cases, ZOOM XP would not update the extended data of paper space

viewports. This was corrected.

UNDO

UNDO after a PAN operation in a model space viewport would regenerate

additional viewports. This was corrected to operate without causing

unneeded regenerations as the reference manual indicates it should.

Entity Layers

In some cases, changing the layer of an entity to a layer that was frozen

would not remove it completely from the display, even after a redraw. This

has been corrected.

Entities in a block insertion that are on separate active layers will be

displayed even though the layer of the block insertion is turned off. In

some cases, these displayed entities would not plot. This was corrected.

EXTERNAL REFERENCE (Xref)

Binding an unresolved external reference caused an internal error when

handles were turned on. This was corrected.

When an external reference A which referenced Xref B which referenced Xref

C was attached to a drawing (DWG->A->B->C), and external reference B was

also independently referenced to the same drawing (DWG->B->C), detaching

external reference A would detach the independent reference of B when it

should not have. This has been corrected.

Several other internal errors for external reference operations were fixed.

In some cases, Audit or Recovery on a drawing where an external reference

has been attached and then detached would report "Incorrect layer index xx"

errors. This has been corrected.

A memory fault error would occur if the user attempted to Xref Attach a

bad, very small, or non-drawing file. AutoCAD now handles this input error

without a memory fault.

Server Authorization Configuration

Answering no to the question "Do you wish to change it?" during server

authorization configuration would sometimes corrupt the ACAD.PWD file. This

has been corrected.

Server Authorization file (ACAD.PWD) crash recovery has been substantially

improved, and should be handled automatically more often.

Paper Space

In some cases, when the paper space limits were the same as the viewport

size, plotting the limits or extents would result in an effective plotting

area having zero width. This was corrected.

Dimensioning

Leader and Center dimension commands would draw lines, arrows and text in

color BYBLOCK rather than BYLAYER when DIMCLR was set to BYBLOCK. These

dimension commands now handle color the same as other dimension commands.

Diameter dimensions would draw the center lines too long by a factor of the

circle or arc radius if DIMCEN was set to a negative  value. This was

corrected.

STRETCH would not move linear dimension text if positioned at a user

specified location. This was corrected.

Using the suffix option "<>" with dimensions would create an empty text

string resulting in entities which were difficult to select and erase. In

contrast, prefix "<>" option would not. The suffix option was corrected to

operate like the prefix option.

Corrections to DXF

Executing DXFIN followed by UNDO then REDO resulted in an internal error.

This has been fixed.

A partial entities-only DXFIN failed with the message:  "Undefined block

xxx|yyy Invalid" if the target drawing contained an Xref dependent block.

This was fixed.

DXFOUT failed to output the DIMGAP field of the DIMSTYLE table. AutoCAD now

uses group code 147 for DIMGAP. Previous versions of Release 11 were broken

in this regard and DXF applications dependent on the previous group code

for DIMGAP will need to be updated.

Entities-only DXFIN of a drawing that contained one external reference and

had tilemode set to 0 into a target drawing with tilemode set to 0 with at

least one entity would result in an internal error. This has been

corrected.

DXFIN of a file that contains dimstyles did not set the DIMSCALE header

variable properly. The value would be set to the value of DIMSCALE of the

last dimstyle read from the DXF file. This has been corrected so that

DIMSCALE itself is not dependent on the last dimstyle read from the file.

IGES Changes and Corrections

IGESOUT of an extruded line would sometimes add the elevation to the

thickness of an extruded line by mistranslating the coordinates of the

tabulated cylinder (entity type 122). This has been corrected.

IGESIN sometimes overextended oblique dimension lines when the minimum

resolution in the IGES file was set too high. AutoCAD now tries to handle

this case for IGES files from other sources.

IGESOUT of a multifaceted polyface mesh into Paper Space which had been

INSERTed resulted in the loss of all but one of the faces of the PFACE

mesh. AutoCAD now groups all the individual faces that were created into a

single IGES 402 form 7 (group without back pointers) to address this

problem.

Corrections to AutoLISP and ADS

(nentsel) and ads_nentsel( ) were inconsistent in treating dimension

entities. When selecting a dimension entity, nentsel would not descend into

the dimension's anonymous block. If the dimension entity was contained in

another block definition, nentsel descended into the dimension's anonymous

block. AutoCAD now descends consistently returning the anonymous block

information in all cases.

(entmake) and ads_entmake( ) failed by displaying the message "Invalid

COMPLEX entity" when attempting to make a block and that block contained

insert entities. This has been corrected to create the block as expected.

When defining a block, (entmake) and ads_entmake( ) reported an incorrect

name for the block when the block definition contained an insert entity.

The correct name of the current block definition is now returned.

When using (entmake) and ads_entmake( ) to redefine an anonymous block, the

message returned would be incomplete. The message now returned includes the

block's index properly identifying the block created.

(tblsearch), (tblnext), ads_tblsearch( ) and ads_tblnext( ) failed to

return the DIMGAP field of DIMSTYLE tables. This was corrected.

(entmake) failed to take an (entget) or ads_entget( ) result as an argument

unless you saved it in a symbol and passed the symbol to (entmake). Passing

a list returned by (entget) is now accepted by (entmake).

Attempting to activate viewport entities contained in a block insertion by

passing (ssget "x") filtered input is now trapped and  disallowed.

Turning off the Paper Space viewport by passing (ssget "x") filtered input

is now trapped and disallowed.

Using (entmake) and ads_entmake( ) with the list returned by (entget)

failed to use the Z coordinate returned with the 2D polyline vertices, and

instead used the current elevation. This  has been corrected.

Nesting (entget) functions would cause an internal error. This has been

corrected.

Creating the maximum number of selection sets a second time after freeing them

the first time could result in an fatal error. This has been corrected.

(entget) using the wildcard "*" didn't always return all the extended entity

data attached. This was corrected.

Corrections to Support Files

Several corrections and clarifications to the HELP file and INSTALL program

dialogue are included in this update version of AutoCAD.

CFIGPHAR.EXE is now installed in the correct directory, \ACAD instead of

\ACAD\SAMPLES.

Canceling some commands and programs executed from the screen and pull-down

menus would cause scrolling messages at the Command: prompt. This has been

corrected.

The paragraph text editing support program PTEXT could crash the  system if a

backspace key were pressed when the cursor was on the first character of the

line. This problem no longer occurs.

Drawing arcs at certain angles would appear to draw straight lines as well as

arcs when using the double line draw routine DLINE. This was corrected.

ACAD.MNU has been enhanced to allow DTEXT more flexibility in setting text

styles and other options like those that are available when entering the

command at the command prompt.

ACAD.MNU has been enhanced to allow DVIEW more flexibility by offering more

options like those that are available when entering the command at the command

prompt.

In the support routine XPLODE, the specification of BYBLOCK and BYLAYER under

the color option were reversed. This was corrected.

In the sample routine FPLOT, asymmetrical meshes would sometimes be produced,

andthe start point would be incorrect. This was  corrected.

ADI, ATC, AutoCAD, Autodesk, Autodesk Logo, AutoLISP, AutoShade, and

AutoSketch are registered in the U.S. Patent and Trademark Office by Autodesk,

Inc.

ACAD, Autodesk Training Center, Advanced User Interface, AUI, Autodesk Device

Interface, Autodesk Animator, AutoFlix, AutoShade, CA Lab, and DXF are

trademarks of Autodesk, Inc.

RenderMan is a registered trademark of Pixar and used under license by

Autodesk.

(c)  Copyright 1991 Autodesk, Inc.