This package contains the following items:

1. TEK4662.DOC, this text

2. TEK4662.COM is an Autodesk Device Interface driver for the

Tektronix 4662 plotter for use with Autocad or Autosketch.

3. TEK4662.ASM is the assembler source for the above.

4. TEK4662.BAS is a GWBASIC program that may be customized for

plotting arbitrary plot files on the 4662.

5. PLOTBOX.SKD (for Autosketch) and PLOTBOX.DXF (for Autocad)

are simple patterns to aid in plotter usage, as described below.

Setup features are as follows:

1. The TERMINAL connector, J104, on the plotter must be

connected to COM1 through a standard RS232 cable.  (COM2 may be

used instead by changing one equate in the assembler source.)  The

LOCAL button on the plotter must be locked down.

2. The driver is programmed for a plotter device address of

C (from the choices A, B, C or D) as set up on the rear panel

switches.  This, too, can be changed by modifying one equate.  The

driver is programmed for 1200 baud. Switch settings of A = 8, B =

0, C = 3 and D = 3 will work.

3. Maximum plot size is 15 inches for X and 10 inches for Y.

4. The 4662 plotter does 4096 steps in 15 inches, so steps per

inch is 4096/15 = 273.066667 for both X and Y.

5. The interrupt number is the default 78h.

6. To use the driver, execute TEK4662.COM before entering

Autosketch or Autocad.

If you will be plotting on paper smaller than the standard

B-size, you can make paper placement easier through the following

steps: Import the file PLOTBOX.SKD (for Autosketch) or PLOTBOX.DXF

(for Autocad) into your drafting program.  Prepare the plotter for

plotting, with a pen in it, but with no paper.  Plot the file.

This will draw a box right on the plotter bed that will indicate

the limits of the plotting area.

Limitations:

1.  This driver has been extensively used with Autosketch, but

has not been tested on Autocad.

2.  This driver doesn't support the multiple-pen attachment.

Since it was written for single-color plots, the driver treats the

change-pen and new-pen commands as no-ops.

3.  The present version of this driver doesn't test whether

the plotter is on, connected, and ready to plot.  Furthermore, it

doesn't provide any way to escape from the block-transmit wait

loop.  Therefore, be sure the plotter is ready to go (LOAD button

up, etc.) before you start the plot.  If you have just made changes

in your drawing, be sure to SAVE it before you plot.  If the system

hangs up, you will have to reboot.

The file TEK4662.BAS is a GWbasic program for use with non-

Autodesk CAD packages that don't use the ADI interface.  It is

usable with any package that can write plotter commands to a disk

file.  Customize it for your program by following the instructions

in the listing.

If you find these programs useful, please register by sending

$10 to:

Charles E. Cohn

6311 Mark Trail

Austell, GA 30001-5126

Comments and questions may also be sent to the above address,

or you may call (404)944-7510.

#############################################################################################################################################################################################################################################################################################################################################################################################################################