XPLODE.LSP operates like "EXPLODE" but it will explode 2D blocks

with unequal scales.  It does this using the principle that a

circle, when tilted into the z axis, looks like an ellipse.  Thus

if an equally scaled block is tilted the proper amount into the Z

axis it can be made to look unequally scaled.  This program

rescales the block equally and then uses trigonometry to determine

how much to tilt the block so that it looks like it did before and

explodes it.  PROJECT.LSP, as supplied with Autocad is then used

to project the entities back onto their original plane.

Blocks are not projected onto the WCS unless that is where they

started out.  Nested blocks don't get exploded, but they do appear

properly scaled.  They are on the tilted plane though and if

XPLODEd they will only look right from that tilted plane.  To get

them back on the same plane with the other entities use the

regular EXPLODE and PROJECT.  It may not be too hard to

incorporate this into the program, but I haven't tried.

To get a better idea of what is happening, issue the command

VPOINT 1,-1,1 before running XPLODE.LSP

To load and execute the program do the following:

(load "xplode")

XPLODE

Walter Warren, Compuserve 73750,3535

I know this works on my system with R12c2 and it was originally written

for R10.