Contents:       CMOSGET.COM     save settings

CMOSPUT.COM     restore settings

CMOS.DOC        this file

FATBAK11.ZIP    backup FAT and root directory

BOOTSECT.ZIP    backup hard disk boot sector

Purpose:        Saves the CMOS configuration settings on AT and

later systems so you can reload them instantly

after changing batteries, avoiding having to

reconfigure your system from scratch.

Format:         [d:] [path] CMOSGET

[d:] [path] CMOSPUT

Remarks:        IBM keeps its configuration for the AT and later

machines in a CMOS chip that's maintained by a

battery when the machine is off. A failing

battery can cause errors at boot, triggering a

bad configuration message, or your clock may

begin to behave erratically. Batteries have a

life of about 2 - 3 years. When you remove the

battery, you will lose your configuration record

and have to run SETUP all over again. Many of you

don't even have the SETUP program (your dealer

probably set your machine up for you, and didn't

bother to give you the diagnostics disk.) Bet you

don't know what numerical hard disk type you

have. You need to know this to access the hard

disk.

CMOSGET and CMOSPUT make this easy. CMOSGET reads

the contents of the CMOS chip and writes it to

standard output, redirecting this to output to a

file saves 64 bytes of information.

CMOSPUT does the complement: it reads the

standard input and redirects the saved data to

the CMOS chip.

You can't simply restore the old data, however,

since you would end up with the time and date

that were active when you saved the information,

and would have to use SETUP  to update them.

CMOSPUT does this for you by getting the current

time and date from dos, using the BIOS to update

the real-time clock.

Example:        The complete procedure is as follows: First,

create a bootable diskette by using the FORMAT a:

command with the /s option. Onto this diskette,

copy the SETUP program used by your machine and

the CMOSPUT.COM program here. (Later when the

BIOS detects that CMOS has bad data, it will

refuse to boot from the hard disk, and you will

need to boot from this floppy.)

Run CMOSGET to save the CMOS memory data in a

file on the diskette with the command:

CMOSGET > A:CMOS.DAT

Change the battery. Boot the machine from the

floppy you prepared and ignore the error

messages. Set the date and time with the dos TIME

and DATE commands. Run CMOSPUT with the command:

CMOSPUT < CMOS.DAT

Now remove the diskette and reboot the computer.

If everything works, you should get no errors and

the correct time and date. If there is a problem,

you can always reboot with the same diskette and

run the SETUP program.

Comments:       Do this today. You will need it.

Programs by Robert L. Hummel